3GPP TSG-RAN WG2 Meeting #111	R2-20XXXXX

- Electronic, 17 – 28 August 2020

Agenda item:	8.10.3.1

Source:	ZTE Corporation, Sanechips

Title:	Offline-106: [NTN] Idle mode issues

Document for:	Discussion & Decision

# Introduction

This is a summary of the following offline discussion on idle mode issues for NTN:

- [AT111][106][NTN] Idle mode issues (ZTE)

Scope: Discuss the proposals in R2-2006872, R2-2006973, R2-2007171 and proposals 1 and 2 in R2-2007574. The intention is to identify design alternatives, collect company views and, whenever possible, also narrow down the proposals.

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Thursday 2020-08-20 16:00 UTC

Initial deadline (for rapporteur's summary in R2-2008187): Thursday 2020-08-20 18:00 UTC

This document covers the following contributions submitted to RAN2#111-e meeting:

R2-2006872	Consideration on system information and cell (re)selection in NTN	ZTE corporation, Sanechips	discussion	Rel-17	NR_NTN_solutions-Core

R2-2006973	IDLE mode procedure	Qualcomm Inc	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007171	Discussion on RRC_IDLE mode issues in NTN	Huawei, HiSilicon	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007574	Considerations on satellite ephemeris	THALES	discussion	Rel-17 NR_NTN_solutions-Core

Companies are invited to provide their views for each issue.

# Discussion:

## #Issue 1: General principle for cell (re)selection in NTN

In NR, for cell selection, UE follow the S-criterion to select a cell with qualified RX level (RSRP) and RX quality (RSRQ). For cell reselection, UE follow the reselection priority configured via system information or RRCRelease message for each frequency and reselect to the best cell (highest ranked cell based on R-criterion) if the cell is not barred or reserved.

In NTN, the same principle should be kept in general to ensure that UE selects a cell with good RX level and quality while network can control the distribution of users via reselection priority for load balancing and radio resource management [1].

Q1) Do companies agree that the cell selection following S-criterion and cell reselection following reselection priority and R-criterion in NR should be taken as a baseline in NTN?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Samsung | Yes | While R16-based cell selection and reselection can be used as the baseline, NTN-specific enhancements are needed to improve reliability and processing load on the UE. Furthermore, both Earth-moving beams and quasi-Earth-fixed beams (i.e., beams that cover a Fixed-Earth area for a limited time) need to be supported. |

On this basis, some assistance information or further enhancement can be considered to assist or fasten the cell (re)selection procedure so that UE find a suitable NTN cell as soon as possible.

## #Issue 2: Ephemeris and location assisted cell reselection

In terrestrial systems, a UE can determine it is near a cell edge due to a clear difference in RSRP as compared to cell centre. Such an effect may not be as pronounced in non-terrestrial deployments, resulting in a small difference in signal strength between two beams in a region of overlap.

Figure 1. A sketch of near-far effect in different scenarios: (a) Terrestrial Network; (b) NTN

Ephemeris information and UE location information can be used to help UEs perform measurement, identify cell edge and perform cell selection/reselection accordingly, in addition to PCI and frequency information included in the broadcast system information.

Thus, it has been proposed to introduce satellite ephemeris and UE location assisted cell selection and reselection [3].

Q2) Do companies agree that satellite ephemeris and UE location assisted cell selection and reselection should be introduced for NTN?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Samsung | Yes | Various pieces of information, in addition to satellite ephemeris and UE location, should be considered. For example, the movement of the satellite can be exploited to encourage/discourage cell reselection to a suitable neighbor cell. A gNB can create a compact neighbor list based on satellite ephemeris and provide such list to the UE to guide the cell search (i.e., support for “guided” cell search in addition to autonomous cell search). An indicator in System Information can inform the UE about the availability of such guided cell search so that the UE can make use of this feature. This can reduce power consumption at the UE and help the UE find more reliable neighbors. |

## #Issue 3: Network type and scenario indication

UE can be divided into the following categories:

- UE supporting TN only

- UE supporting NTN only

- UE supporting both TN and NTN

TN only UE shall select a TN cell while a NTN only UE shall select a NTN cell. UE supporting both TN and NTN shall be able to select from the two different types of cells as needed.

Based on the about understanding, it is obvious that awareness of the network type is helpful for UE supporting or with preference for a specific network type to find a suitable cell during cell selection and reselection procedure.

With regards to how to identify the network types, the following solutions can be considered:

- Implicit network type indication: Allocate specific PCI, PLMN ID, or frequency band for NTN cell.

- Explicit network type indication: Per cell or per frequency network type indication in system information.

In NR, there are 1008 unique physical-layer cell identities (PCI) and each PCI is linked to a specific synchronization signal sequence. Reserving part of the 1008 PCIs for NTN will take up the resources in TN while defining new PCIs requires considerate investment in RAN1 to define new synchronization signal sequences, which is not preferable at least in this release.

As mentioned in TR38.821, using common PLMN ID for NTN cells and TN cells is not precluded. Thus, UE cannot rely on the PLMN ID to differentiate the network type.

“Deployment of PLMNs with specific PLMN IDs for NTN cells and TN cells, or between different type of NTN platforms (GEO or LEO), is considered as a preferred option, however the configuration of common PLMN identities is not precluded.”

Whether to allocate specific frequency band for NTN/LEO/Non-GEO/LEO network should be discussed and decided in RAN4, which is totally out of RAN2 scope.

With the above consideration, proving a network type indication per cell or frequency via system information is preferable as it is a pure RAN2 solution without impact in other working groups.

Thus, it has been proposed to broadcast network type indication (e.g. connectToNTN) in system information to assist cell selection and reselection [1] [3].

Q3.1) Do companies agree that network type indication (e.g. connectToNTN) should be broadcast for NTN cell?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Samsung | Yes | We can define a generic term “NTN Type” and define suitable NTN types (TN vs. NTN, GEO vs. LEO, Quasi-Earth-fixed vs. Earth-moving beams, and so on) to facilitate various operations including cell selection/reselection, time/timer based parameters at PHY, MAC, RLC, and PDCP layers. RAN2 can send LS to SA2 about NTN Type so that even the core network can make use of NTN Type. For example, a paging timer can be properly determined by an AMF and charging can be carried out per network type. |

Furthermore, due to the different characteristics of the NTN scenario (e.g. GEO cell with wide coverage and LEO cell with low latency), UE may have preference for one specific scenario during cell selection and reselection.

Thus, it has also been proposed to broadcast the NTN scenario (e.g. GEO or LEO) to assist cell selection and reselection [1] [3].

Q3.2) Do companies agree that NTN scenario (e.g. GEO or LEO) should be broadcast for NTN cell?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Samsung | Yes | NTN Type can be defined in a suitable manner to cover various scenarios. |

## #Issue 4: NTN specific reselection priority

The cell reselection priority can be provided to UE via system information or RRCRelease message so that UE will be guided to frequency with higher priorities. From UE’s perspective, it may rank the cells in frequencies with highest priority based on R-criterion and read the system information of the highest cell to see if it is suitable to camp on.

For the common reselection priority in system information, network broadcast it without knowing the supported network type of UE. For a TN cell, it may configure a TN only frequency with highest priority while UE supporting both NTN and TN will be guided to that frequency and get tracked in TN only frequencies while it could have reselect to a NTN cell at its own will.

Thus, it has been proposed to provide NTN specific cell reselection priorities [1]. TN only UE will follow the legacy reselection priority for TN cells while NTN only UE will follow the NTN specific reselection priorities in which frequencies with NTN cells are prioritized so that both types of UE can find a suitable cell quickly. For UE supporting both TN and NTN, a default reselection priority set (e.g. the legacy reselection priority for TN or the reselection priority for NTN) can be defined or configured for UE to apply.

Q4) Do companies agree that NTN specific cell reselection priority should be introduced?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Samsung | Yes | Additionally, a finer granularity than (TN, NTN) distinction can be helpful. For example, there could be a preference for LEO vs. GEO (or vice versa). RAN2 can discuss which NTN Types can be useful for cell selection and reselection. |

## #Issue 5: Provision of satellite ephemeris

The satellite ephemeris data contains the information about the orbital trajectories of artificial satellites. UE with capability on timing and frequency pre-compensation can derive the satellite location based on the ephemeris data and perform time and frequency pre-compensation accordingly. Thus, it has been proposed to provide the satellite ephemeris to UE [1] [3] [4].

Q5.1) Do companies agree that the satellite ephemeris should be provided to UE?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Samsung | Yes | RAN2 can discuss what type of ephemeris info is most relevant/useful. |

Before discussing the signaling details of satellite ephemeris provision, we should make it clear what kind of information will be provided to UE, i.e. the format of the satellite ephemeris.

As captured in TR38.821, there are different possible formats of ephemeris data:

- Option 1: Orbital parameters (including orbital plane parameters and satellite level parameters). A description table for the orbital parameters and the corresponding illustrations are as below.

Table 1: Essential Elements of Ephemeris

| Orbital plane parameters |  | Square root of semi major axis（semi-major axis） |
| --- | --- | --- |
|  |  | Eccentricity（eccentricity） |
|  |  | Inclination angle at reference time（inclination） |
|  |  | Longitude of ascending node of orbit plane（right ascension of the ascending node） |
|  |  | Argument of perigee（argument of periapsis） |
| Satellite level parameters |  | Mean anomaly at reference time（true anomaly and a reference point in time） |
|  |  | Ephemeris reference time（the epoch） |

Figure 2: Satellite Orbit and Keplerian Elements

- Option 2: The satellite location in coordinates (x, y, z), e.g. ECEF coordinates. For anything else than GEO, additionally a velocity vector (vx, vy, vz) and again a reference point in time are needed.

Q5.2) For the satellite ephemeris format, which option do companies prefer?

- Option 1: Orbital parameters (including orbital plane parameters and satellite level parameters).

- Option 2: The satellite in coordinates (x, y, z), e.g. ECEF coordinates.

| Company | Option 1 or 2 | Comments (if any) |
| --- | --- | --- |
| Samsung | Option 2 pending RAN1 verification | In general, we prefer minimum # of bits and hence Option 2. However, let’s make sure from RAN1 that Position, Velocity, and Time are adequate for the UE to carry out suitable time and frequency pre-compensation, because such compensation is likely within RAN1 scope. |

## #Issue 6: Introduction of NTN specific SIB

As described in WID [5], additional assistance information specifically for NTN, for example, satellite ephemeris, pre-compensation offset, initial BWP configurations for multiple beams is needed to be broadcast in the system information to help UE in satellite measurements and cell reselection. The information can be specific to satellite or cell.

Some satellite specific information, e.g., RTD variation and ephemeris, may be decoupled with the cell specific system information and scheduled periodically, though it is possible to carry it in existing SIB, e.g., SIB9 which contains information related to GPS time and Coordinated Universal Time (UTC). This is because the continuous change in satellite specific information does not need to trigger UE to acquire the existing SIBs, i.e., SIB1/SIB9. In this case, it is more efficient if a new SIB is introduced for satellite specific information.

Thus, it has been proposed to introduce a new SIB to carry the satellite specific information [2] [4].

Q6) Do companies agree that a new SIB should be introduced to carry the satellite specific information?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Samsung | Yes | The content of SIB and its structure (e.g., periodicity) can be discussed to ensure reliability and signaling efficiency. |

Additional Issues

Samsung requests RAN2 to consider following additional topics that affect System Information being sent in an enhanced version of an existing SIB or a new NTN SIB.

## #Issue 7: Broadcast Signaling and Groupcast/Multicast Signaling

Certain type of information may be applicable to one group of UEs and not others. Hence, groupcast/multicast signaling (different from the traditional SI signaling) can also be considered to increase the signaling efficiency. Such signaling can especially be useful for handover. Please see Proposal 4 in [6].

## #Issue 8: Size of RNTI for the NTN

Additionally, RAN2 can discuss the need for an increased RNTI size to create “High Capacity- RNTIs” (HC-RNTIs) to support smartphones and a massive number of delay-tolerant IoT devices with intermittent data traffic (especially for GEOs). The gNB, in an implementation-specific manner, can chose a suitable size for HC-RNTI. An indication of a suitable “HC-RNTI” size would likely be needed in System Information. Please see Proposal 4 in [7].

## #Issue 9: Tracking Area Management for the NTN

Tracking area management needs to be discussed as part of Idle mode issues. We suggest a comparison of various approaches (e.g., multi-TAI broadcast per cell, time-based TAU, and virtual tracking areas (VTAs)). Please see Proposal 1 in [8].

# Conclusion:

## 3.1 List of agreeable proposals

To be added

## 3.2 List of proposals to be discussed online

To be added

# Reference

[1] R2-2006872	Consideration on system information and cell (re)selection in NTN ZTE corporation, Sanechips

[2] R2-2006973	IDLE mode procedure	Qualcomm Inc

[3] R2-2007171	Discussion on RRC_IDLE mode issues in NTN	Huawei, HiSilicon

[4] R2-2007574	Considerations on satellite ephemeris	THALES

[5] RP-201256 Solutions for NR to support non-terrestrial networks (NTN) THALES

[6] R2-2006953, “Control Plane Enhancements for the Connected Mode in an NTN- Overall Observations and Proposals,” Samsung.

[7] R2-2006941, “NTN WI- Overall Observations and Proposals,” Samsung.

[8] R2-2006945, “Control Plane Enhancements for Idle and Inactive Modes in an NTN- Overall Observations and Proposals,” Samsung.