3GPP TSG-RAN WG2 Meeting #111 electronic	                                                                                R2-200xxxx

17st August – 28th August 2020

Agenda Item:	4.1

Source: 	ZTE (email discussion rapporteur)

Title:  	                  Report of [AT111-e][301][NBIOT R15] NPRACH carrier selection (ZTE)

Document for:	Discussion and Decision

# Scope of the offline email discussion

This document contains the summary of the offline email discussion “[AT111-e][301][NBIOT R15] NPRACH carrier selection”, as indicated below:

- [AT111-e][301][NBIOT R15] NPRACH carrier selection (ZTE)

Status: Started

Scope: Discuss which changes, if any, are agreeable.

Intended outcome: Report in R2-2008301 and Agreeable CRs (if needed)

Deadline: Tuesday 25 1100 UTC.

# Offline email discussion

- R2-2006838	36331_R15_Clarification for NPRACH carrier selection	ZTE Corporation, Sanechips, MediaTek Inc	CR	Rel-15	36.331	15.10.0	4354	-	F	NB_IOTenh2-Core

In the above CR, potential ambiguities have been discussed. There were some initial discussion on the CR in the Monday online session:

- R2-2006838	36331_R15_Clarification for NPRACH carrier selection	ZTE Corporation, Sanechips, MediaTek Inc	CR	Rel-15	36.331	15.10.0	4354	-	F	NB_IOTenh2-Core

- QC wonder what happens when NW does not want mixed mode UE to use legacy list carriers, and thinks the UE would end up maintaining 2 lists for use with CFRA and CBRA. HW have the same understanding as QC on issue2, and also NW configuration can avoid most of the problems raised.

- Ericsson thinks MAC may also be impacted if we change this.

In the following sections, companies are invited to provide their further comments on the separated issues.

Issue 1: carrier for CFRA or dedicated SR without HARQ-ACK

Q1: Companies are invited to provide their understanding on whether the anchor carrier and the carriers in ul-ConfigList can be configured for CFRA or dedicated SR without HARQ-ACK even in the case that ul-ConfigListMixed is present but nprach-Distribution is absent.

| Company | Yes or No? | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | Per our understanding, nprach-Distribution is mainly for UE distribution among legacy carrier list and new carrier list in the scenario that UE selects random access carrier. Since the resources for CFRA or physical layer SR is explicitly indicated by eNB and that can be based on eNB implementation, it’s no need to apply such nprach-Distribution for CFRA or physical layer SR cases. |
| Qualcomm | No | We don’t quite see the reason why NW would want UE supporting mixed mode to not use legacy nprach carrier for random access but ok to configure UE to use leacy nprach carrier via dedicated signalling (i.e PDCCH order or nprach for SR configuration). We think the same rules apply for nprach-Distribution for both CBRA and CFRA; that is if nprach-Distribution is not present then only the NPRACH carriers in mixed mode list can be used for PDCCH order. If distribution is configured, then legacy NPRACH list and mixed mode NPRACH list are considered as one list. |
| MediaTek | Yes | We don’t see any reason to limit that the NW has to use the mixed carrier list only for CFRA and physical Layer SR when nprach-Distribution is not present. For those UEs spport mixed operation mode, there is no trouble of using the non-mixed carrier in UL, and using a mixed carrier in DL. Allowing to use non-mixed list in this case can improve the flexibility of NW resource allocation, help balancing work loads of NPRACH carriers. |
| Huawei, HSilicon | No but | Based on the existing specifications, we think that RRC uses nprach-Distribution to build the list of NPRACH resources when reading SIB2/SIB22/SIB23. This is list is then provided to MAC and is used for all, i.e. initial access, RACH in connected mode, PDCCH order in connected mode and dedicated SR. We have sympathy for the proposal that the list of carriers used for CFRA could be different from the list of carriers used for access, this would allow to use a larger set of carriers for CFRA for the case the UE is supporting mixed mode operation. We think this would require some changes to : - RRC: to specify that the UE in connected mode used a combined list for CFRA, irrespective of nprach-Distribution. -MAC: to specify that the UE uses two separate lists for UE initiated RACH and PDCCH order. |
| Ericsson | Not decided | As mentioned there will be changes needed also in MAC; we would like to first check all the relevant updates and decide. Was there any mismatch seen in the field? Was NW configuring without mixed list and UE expecting to receive using mixed list only? |

If there are both Yes and No in the answers for Q1, possible different understanding may exists between UE and eNB. We think anyway a clarification would be needed.

Q2: If answer for Q1 is Yes, whether the following clarification can be agreed? If no, please provide your suggestion.

Ul-ConfigList, ul-ConfigListMixed For FDD: List of UL non-anchor carriers and associated configuration that can be used for random access. E-UTRAN configures UL non-anchor carriers operating in mixed operation mode only in ul-ConfigListMixed and only a UE that supports mixed operation mode uses the carriers in ul-ConfigListMixed. A given carrier is either signalled in the ul-ConfigList or in ul-ConfigListMixed. If ul-ConfigListMixed is present and at least one of the carriers in ul-ConfigListMixed is configured for random access: - If nprach-Distribution is present, the UE supporting mixed operation mode creates a combined list of UL carriers for contention based random access by appending ul-ConfigListMixed to the ul-ConfigList while maintaining the order among both ul-ConfigList and ul-ConfigListMixed; the total number of signalled UL non-anchor carriers cannot be more than maxNonAnchorCarriers-NB-r14. - If nprach-Distribution is absent, the UE supporting mixed operation mode uses the list of UL carriers for contention based random access provided in ul-ConfigListMixed and considers nprach-ProbabiliyAnchor being set to zero for each NPRACH resource, i.e. the anchor carrier is not used for contention based random access. Otherwise, the nprach-Distribution field is not applicable and the UE shall ignore the value. For TDD: E-UTRAN configures ul-ConfigList-r15 and includes the same number of entries as in dl-ConfigList. The UL carrier frequency of the non-anchor carrier is same as the DL carrier frequency.

| Company | Yes or No? | Detailed comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Qualcomm | No | As per answer to Q1, this change is not right. |
| MediaTek | Yes |  |
| Huawei, HiSilicon | No | See out answer to Q1. If we were to agree on the proposal, then this would not be sufficient, we would need to specify which list is used by the UE for contention free RACH in both RRC and MAC specifications |
| Ericsson | No | Agree with Huawei |

Q3: If answer for Q1 is No, whether clarification is needed?

| Company | Yes or No? | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | We suggest to add a clarification, e.g., at least in Chairman Note as following: RAN2 has common understanding that anchor carrier and carriers in dl-ConfigList also cannot be used for CFRA and dedicated SR without HARQ-ACK in the case that ul-ConfigListMixed is present but nprach-Distribution is absent. |
| Qualcomm | May be | Not sure such statement is necessary as eNB should provide correct UE specific configuration taking into account UE capability and cell configuration i.e. if UE supports mixed mode then network should not configure NPRACH carrier from legacy list if nprach-Distribution is absent in the cell. |
| MediaTek | Yes | UE vendors and NW vendors may have different understanding on this question, plus, a clarification has no harm. |
| Huawei, HiSilicon | No | As explained in Q1, we think that the specification is clear, RRC builds the list when reading system information and then this list is used for all. |

Issue 2: Index of the carriers in ul-ConfigListMixed

Q4: If ul-ConfigListMixed is present and nprach-Distribution is absent, which option is the assumption on the identity/index of the carriers in the ul-ConfigListMixed? (Here we assume the number of carriers in ul-ConfigList is m and the number of carriers in ul-ConfigListMixed is n)

- Option 1: identity/index of the carriers in the ul-ConfigListMixed is in the range [1...n]

- Option 2: identity/index of the carriers in the ul-ConfigListMixed is in the range [m+1, m+n]

| Company | Preferred option? | Detailed comments |
| --- | --- | --- |
| ZTE | Slightly prefer Option 2 | Generally, both of the options are acceptable to us. The main point is to keep consistent understanding between UE and eNB. There may be the case that two UEs (support or not support mixed operation mode) both use the first carrier in ul-ConfigList and ul-ConfigListMixed respectively. With option 1, even they are using the different carriers, they have same assumption on the carrier index (e.g., carrier_id is 1), and then they may monitor the RAR with same RA-RNTI that will cause unnecessary collision. Company mention eNB can configure different PDCCH carrier for these two different UL carriers to avoid collision on RAR reception. We agree the feasibility but think it is undesired restriction. So we slightly prefer option 2. |
| Qualcomm | Option 1 | The broadcast information determines the list of nprach carriers (i.e. it is either the mixed list only or the legacy+mixed list). Therefore, index value will be same as the number of entries in the constructed list (i.e. if there were five carriers then index range would be 1 – 5 regardles of the list has only mixed mode carriers or both legacy and mixed mode carriers). |
| MediaTek | Option2 | During the IODT test, we found that the NW vendors have different understanding on this question. A clarification is necessary. Both options are acceptable. We agree with ZTE’s comments, so option2 is preferred. |
| Huawei, HiSilicon | Option 1 | We assume the issue here is independent of issue 1 and only addresses the initial access case. With this assumption, we think there is no indication anywhere that the UE should handle two lists, one combined list and one separate list, simultaneously. Also it would be very strange to use two different numbering for the same carrier in the same procedure. |
| Ericsson | Option 2 | With option 2, DL and UL are aligned, as DL carrier index is a combination of dl-ConfigList and dl-ConfigListMixed, irrespective of nprach-Distribution |

Q5: If answer for Q4 is Option 2, whether the following clarification can be agreed?

ul-ConfigList, ul-ConfigListMixed For FDD: List of UL non-anchor carriers and associated configuration that can be used for random access. E-UTRAN configures UL non-anchor carriers operating in mixed operation mode only in ul-ConfigListMixed and only a UE that supports mixed operation mode uses the carriers in ul-ConfigListMixed. A given carrier is either signalled in the ul-ConfigList or in ul-ConfigListMixed, and the total number of signalled UL non-anchor carriers cannot be more than maxNonAnchorCarriers-NB-r14. If ul-ConfigListMixed is present and at least one of the carriers in ul-ConfigListMixed is configured for random access: - If nprach-Distribution is present, the UE supporting mixed operation mode creates a combined list of UL carriers for random access by appending ul-ConfigListMixed to the ul-ConfigList while maintaining the order among both ul-ConfigList and ul-ConfigListMixed. - If nprach-Distribution is absent, the UE supporting mixed operation mode uses the list of UL carriers for random access provided in ul-ConfigListMixed and considers nprach-ProbabiliyAnchor being set to zero for each NPRACH resource, i.e. the anchor carrier is not used for random access. - Regardless of the nprach-Distribution is present or not, the UE supporting mixed operation mode always assume the identity/index of the carriers in the list of UL carriers would be the index in the combined list by appending ul-ConfigListMixed to the ul-ConfigList. Otherwise, the nprach-Distribution field is not applicable and the UE shall ignore the value. For TDD: E-UTRAN configures ul-ConfigList-r15 and includes the same number of entries as in dl-ConfigList. The UL carrier frequency of the non-anchor carrier is same as the DL carrier frequency.

| Company | Yes or No? | Detailed comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Qualcomm | No | See answer to question 4. |
| MediaTek | No | An elaboration on the “the identity/index of carriers in the list of UL carriers” would be helpful, like where it would be used. Because this is not a well-defined definition, reader may wonder what this is refer to. |
| Huawei, HiSilicon | No | See our answer to Q4. The proposed change would not be a clarification but a change of behavior. |
| Ericsson | Yes |  |

Q6: if answer for Q4 is Option 1, whether clarification is needed?

| Company | Yes or No? | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | In order to avoid any inconsistent understanding between UE and eNB, we think the following clarification may still be needed. With the clarification, UE and eNB can have consistent understanding on carrier index. The eNB could further guarantee the suitable configuration to try to avoid collision by implementation. Ul-ConfigList, ul-ConfigListMixed For FDD: List of UL non-anchor carriers and associated configuration that can be used for random access. E-UTRAN configures UL non-anchor carriers operating in mixed operation mode only in ul-ConfigListMixed and only a UE that supports mixed operation mode uses the carriers in ul-ConfigListMixed. A given carrier is either signalled in the ul-ConfigList or in ul-ConfigListMixed. If ul-ConfigListMixed is present and at least one of the carriers in ul-ConfigListMixed is configured for random access: - If nprach-Distribution is present, the UE supporting mixed operation mode creates a combined list of UL carriers for random access by appending ul-ConfigListMixed to the ul-ConfigList while maintaining the order among both ul-ConfigList and ul-ConfigListMixed; the total number of signalled UL non-anchor carriers cannot be more than maxNonAnchorCarriers-NB-r14. - If nprach-Distribution is absent, the UE supporting mixed operation mode uses the list of UL carriers for random access provided in ul-ConfigListMixed and considers nprach-ProbabiliyAnchor being set to zero for each NPRACH resource, i.e. the anchor carrier is not used for random access. The UE would assume the index of UL carriers for random access starts from 1. Otherwise, the nprach-Distribution field is not applicable and the UE shall ignore the value. For TDD: E-UTRAN configures ul-ConfigList-r15 and includes the same number of entries as in dl-ConfigList. The UL carrier frequency of the non-anchor carrier is same as the DL carrier frequency. |
| Qualcomm | May be | Don’t think such clarification is needed. Our understanding is all lists have index 1 for first entry. If such clarification is needed then it needs to be stated also for ‘ul-ConfigList’ for Ues not support mixed mode. |
| MediaTek | Yes | As per answer to Q4, a calrification is necessary. |
| Huawei, HiSilicon | No | We think it is true for all cases that the first carrier in ul-Config starts at 1, this is clear from the list definition 1.. maxNonAnchorCarriers-NB-R14. Note that in MAC the NPRACH resources for the anchor carrier are provided separately, see section 5.1.1: - if the UE is an NB-IoT UE: - the available set of PRACH resources supported in the Serving Cell on the anchor carrier, nprach-ParametersList, and on the non-anchor carriers, in ul-ConfigList. |

# Summary

Conclusion:

Updated CRs in the draft folder:

# References

[1] R2-2006838	 36331_R15_Clarification for NPRACH carrier selection 	ZTE Corporation, Sanechips, MediaTek Inc	CR	Rel-15	36.331	15.10.0	4354	-	F	NB_IOTenh2-Core

[2] R2-2006840	36331_R16_Clarification for NPRACH carrier selection	 ZTE Corporation, Sanechips, MediaTek Inc	CR	Rel-16	36.331	16.1.1	4356	-	A	NB_IOTenh2-Core

# Participants

| Qualcomm | Mungal | mdhanda@qti.qualcomm.com |
| --- | --- | --- |
| Huawei, HiSilicon | Odile | odile.rollinger@huawei.com |
| Ericsson | Ritesh | Ritesh.shreevastav@ericsson.com |