3GPP TSG-RAN WG2 Meeting #111 electronic	                                                                                R2-200xxxx

17st August – 28th August 2020

Agenda Item:	7.2.4

Source: 	ZTE (email discussion rapporteur)

Title:  	Report of [AT111-e][403][eMTC R16] Clarification on subframe level resource reservation (ZTE)

Document for:	Discussion and Decision

# Scope of the offline email discussion

This document contains the summary of the offline email discussion “[AT111-e][403][eMTC R16] Clarification on subframe level resource reservation (ZTE)”, as indicated below:

- [AT111-e][403][eMTC R16] Clarification on subframe level resource reservation (ZTE)

Scope: Check with RAN1 whether the intention was to use the definition of subframe bitmap reservation and consider introducing a new parameter.

Intended outcome: Report from the discussion in R2-2008233

Deadline: Tuesday 25 1100 UTC.

# Offline email discussion

In RAN2#111 e-meeting, there are some initial discussion for the following CR:

R2-2006858	Clarification on subframe level resource reservation for eMTC	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.1.1	4358	-	F	LTE_eMTC5-Core

- Huawei thinks subframe reservation bitmap is already available in SIB1 and wonders why it is not possible to use that. It was RAN1’s intention not to introduce subframe bitmap.

- QC thinks it is possible for the eNB to know the UE capabilities, but this is not the case for the common signalling.

The brief difference on resource reservation between NB-IoT and eMTC can be found in the following table (taking DL as example):

|  | NB-IoT | eMTC |
| --- | --- | --- |
| R13 SIB | downlinkBitmap-r13-> SystemInformationBlockType1-NB | fdd-DownlinkOrTddSubframeBitmapBR-r13-> SystemInformationBlockType1-BR-r13 |
| R13 dedicated signalling | downlinkBitmapNonAnchor-r13-> DL-CarrierConfigDedicated-NB-r13-> CarrierConfigDedicated-NB-> | N/A |
| R16 SIB | N/A | slotBitmap-r16-> resourceReservationConfigCommonDL-r16-> SystemInformationBlockType29-r16 |
| R16 dedicated signalling | subframeBitmap-r16-> resourceReservationConfigDL-r16-> PhysicalConfigDedicated-NB slotConfig-r16-> resourceReservationConfigDL-r16-> PhysicalConfigDedicated-NB | slotBitmap-r16-> resourceReservationDedicatedDL-r16-> PhysicalConfigDedicated |

In SIB1-BR, the R13 bitmap-level resource reservation definition is as following:

SystemInformationBlockType1-v1310-IEs ::= SEQUENCE { hyperSFN-r13 BIT STRING (SIZE (10)) OPTIONAL, -- Need OR eDRX-Allowed-r13 ENUMERATED {true} OPTIONAL, -- Need OR cellSelectionInfoCE-r13 CellSelectionInfoCE-r13 OPTIONAL, -- Need OP bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { si-WindowLength-BR-r13 ENUMERATED { ms20, ms40, ms60, ms80, ms120, ms160, ms200, spare}, si-RepetitionPattern-r13 ENUMERATED {everyRF, every2ndRF, every4thRF, every8thRF}, schedulingInfoList-BR-r13 SchedulingInfoList-BR-r13 OPTIONAL, -- Cond SI-BR fdd-DownlinkOrTddSubframeBitmapBR-r13 CHOICE { subframePattern10-r13 BIT STRING (SIZE (10)), subframePattern40-r13 BIT STRING (SIZE (40)) } OPTIONAL, -- Need OP fdd-UplinkSubframeBitmapBR-r13 BIT STRING (SIZE (10)) OPTIONAL, -- Need OP startSymbolBR-r13 INTEGER (1..4), si-HoppingConfigCommon-r13 ENUMERATED {on,off}, si-ValidityTime-r13 ENUMERATED {true} OPTIONAL, -- Need OP systemInfoValueTagList-r13 SystemInfoValueTagList-r13 OPTIONAL -- Need OR } OPTIONAL, -- Cond BW-reduced nonCriticalExtension SystemInformationBlockType1-v1320-IEs OPTIONAL }

In R16, resource reservation configuration for eMTC UE can be provided via both SIB and dedicated signaling. Moreover, same IE structure ResourceReservationConfigDL-r16/ResourceReservationConfigUL-r16 are used for both SIB and dedicated signaling.

ResourceReservationConfig information element -- ASN1START ResourceReservationConfigDL-r16 ::= SEQUENCE { periodicityStartPos-r16 PeriodicityStartPos-r16, resourceReservationFreq-r16 CHOICE { rbg-Bitmap1dot4 BIT STRING (SIZE (6)), rbg-Bitmap3 BIT STRING (SIZE (8)), rbg-Bitmap5 BIT STRING (SIZE (13)), rbg-Bitmap10 BIT STRING (SIZE (17)), rbg-Bitmap15 BIT STRING (SIZE (19)), rbg-Bitmap20 BIT STRING (SIZE (25)) } OPTIONAL, -- Need OP slotBitmap-r16 CHOICE { slotPattern10ms BIT STRING (SIZE (20)), slotPattern40ms BIT STRING (SIZE (80)) }, symbolBitmap1-r16 BIT STRING (SIZE (7)) OPTIONAL, -- Cond Bitmap1 symbolBitmap2-r16 BIT STRING (SIZE (7)) OPTIONAL, -- Cond Bitmap2 ... } ResourceReservationConfigUL-r16 ::= SEQUENCE { periodicityStartPos-r16 PeriodicityStartPos-r16, slotBitmap-r16 CHOICE { slotPattern10ms BIT STRING (SIZE (20)), slotPattern40ms BIT STRING (SIZE (80)) } OPTIONAL, -- Cond FDDandTDDnoDL symbolBitmap1-r16 BIT STRING (SIZE (7)) OPTIONAL, -- Cond Bitmap1 symbolBitmap2-r16 BIT STRING (SIZE (7)) OPTIONAL, -- Cond Bitmap2 ... } PeriodicityStartPos-r16 ::= CHOICE { periodicity10ms NULL, periodicity20ms INTEGER(0..1), periodicity40ms INTEGER(0..3), periodicity80ms INTEGER(0..7), periodicity160ms INTEGER(0..15), spare3 NULL, spare2 NULL, spare1 NULL } -- ASN1STOP

Furthermore, according to the UE capability definition, there exist the R16 eMTC UE that support DL resource reservation with subframe-level but not support DL resource reservation with slot-level.

Therefore, companies are invited to provide their understanding on behavior for such UE in the following cases in order to see whether issue exists.

Q1: Companies are invited to provide their understanding on UE process in the following cases (taking DL as example):

- Case1:

  - fdd-DownlinkOrTddSubframeBitmapBR-r13 is present in SIB1-BR

  - resourceReservationConfigCommonDL is present in SystemInformationBlockType29

  - resourceReservationConfigDedicatedDL is set to setup and resourceReservationDedicatedDL is included

- Case 2:

  - fdd-DownlinkOrTddSubframeBitmapBR-r13 is present in SIB1-BR

  - resourceReservationConfigCommonDL is present in SystemInformationBlockType29

  - resourceReservationConfigDedicatedDL is set to setup but resourceReservationDedicatedDL is not included

- Case 3:

  - fdd-DownlinkOrTddSubframeBitmapBR-r13 is present in SIB1-BR

  - resourceReservationConfigCommonDL is present in SystemInformationBlockType29

  - resourceReservationConfigDedicatedDL is absent or set to release.

- Case 4:

  - resourceReservationConfigCommonDL is not present

  - resourceReservationConfigDedicatedDL is set to setup and resourceReservationDedicatedDL is included

| Company | Comments |
| --- | --- |
| ZTE | Based on the following description in RAN2 spec: resourceReservationConfigDedicatedDL Indicates whether the DL resource reservation is enabled for the UE, e.g. for NR coexistence. If the field is set to setup and resourceReservationDedicatedDL is not included, then resourceReservationConfigCommonDL in SystemInformationBlockType29 applies.. And also the following description in RAN1 spec (TS 36.213): For BL/CE UEs, the set of BL/CE DL subframes is indicated as follows - If higher layer parameter resourceReservationDedicatedDL is configured, - for PDSCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space including PDSCH transmission without a corresponding MPDCCH, - if the Resource reservation field in the DCI is set to 0, then the set of BL/CE DL subframes corresponds to all downlink subframes and special subframes during the PDSCH transmission; - if the Resource reservation field in the DCI is set to 1, then the set of BL/CE DL subframes corresponds to all downlink subframes and special subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all OFDM symbols of all PRBs of the PDSCH transmission are reserved in the subframe); - for MPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space, - the set of BL/CE DL subframes corresponds to all downlink subframes and available special subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all OFDM symbols of all PRBs of the MPDCCH transmission are reserved in the subframe). - In all other cases, the set of BL/CE DL subframes is indicated by the higher layers according to fdd-DownlinkOrTddSubframeBitmapBR [11].. We understand setup of dedicated resource reservation configuration can be used to enable R16 configuration. In case1, the content of dedicated resource reservation configuration is also provided, dedicated resource reservation configuration would be used instead of the configuration in SIB29. In case 2, the configuration in SIB29 would be used. In case 3, the R13 configuration would be used. In case 4, dedicated resource reservation configuration would be used. |

Q2: Companies are invited to provide their comments on whether there is any issue for such UE that support DL subframe-level resource reservation but not support DL slot-level resource reservation with in above cases?

| Company | Yes or No? | Comments |
| --- | --- | --- |
| ZTE | Yes | In case 1 and case 4, dedicated resource reservation configuration is provided and slotBitmap-r16 would be mandatorily provided. UE would use this dedicated configuration. If the two bits for each subframe in slotBitmap-r16 are coded to “01” or “10”, such special UE may think such dedicated configuration is invalid. In case 2, the UE would use configuration in SIB29. If two bits for each subframe in slotBitmap-r16 in resourceReservationConfigCommonDL are coded to “01” or “10”, such special UE may also think the configuration is invalid. Same issue exists for UL. Moreover, in RAN1 spec, the wording “- If higher layer parameter resourceReservationDedicatedDL is configured,” intend to cover all the cases that R16 configuration is provided and enabled. But it looks inconsistent with RAN2 description on the conditions. Further checking may be needed. |
| Huawei | Yes. | The issue is that RAN1 had intended to re-use the Rel-13 subframe-level reservation and therefore did not include it in the needed parameters for eMTC (but did include for NB-IoT). However, this existing bitmap is in SIB and not dedicated signaling. In addition, the RAN1 parameters list indicates the R16 reservation should be independent from the R13 bitmap. In practice this is not a real issue for dedicated signaling – in case the UE indicates support of subframe-level reservation but not slot level, then eNB should not configure “01” or “10” – it is normal the eNB configures only what the UE supports, and since we have separate capability bits to indicate slot level and subframe level then NW can configure based on this. In SIB it is an issue - because a UE not supporting slot level reservation will just ignore the slot level configuration and use the R13 subframe bitmap. However, there is no way to provide a R16 subframe level reservation independently from the R13 configuration without using the slotlevel bitmap. This makes it unclear what a UE supporting subframe level but not slot level should apply when the eNB configures per slot. The introduction of CHOICE slot/subframe as NB-IoT supports is simply a signaling optimization. |

Q3: If the answer to Q2 is Yes, companies are invited to provide their preference on the above change options or indicate other option in the table below.

Option 1:

slotBitmap Slot-level resource reservation configuration. Value slotPattern10ms corresponds to 10ms slot pattern and slotPattern40ms corresponds to 40ms slot pattern, see TS 36.213 [23] for DL and TS 36.211 [21] for UL. The first/leftmost 2-bits corresponds to the subframe #0 of the radio frame satisfying SFN mod periodicity = start position, as indicated by periopdicityStartPos. Two bits for each subframe coded as: 00: both slots are not reserved 01: the first slot is not reserved, the second slot is reserved 10: the first slot is reserved, the second slot is not reserved 11: both slots are reserved. If the field is not included in UL configuration, the value of the field from DL configuration applies. For the UE that supports subframe-level resource reservation but does not support slot-level resource reservation, if the resource reservation is enabled for it, the two bits for each subframe should be set to “00” or “11”.

Option 2:

resourceReservationConfigDedicatedDL Indicates whether the DL resource reservation is enabled for the UE, e.g. for NR coexistence. If the field is set to setup and resourceReservationDedicatedDL is not included, then resourceReservationConfigCommonDL in SystemInformationBlockType29 applies. If resourceReservationConfigCommonDL is not provided or the two bits for each subframe in slotBitmap in resourceReservationConfigCommonDL are set to “01” or “10”, and for the UE that supports subframe-level resource reservation but does not support slot-level resource reservation, if this field is set to setup, resourceReservationDedicatedDL should also be included with two bits for each subframe in slotBitmap set to “00” or “11”.

resourceReservationConfigDedicatedUL Indicates whether the UL resource reservation is enabled for the UE, e.g. for NR coexistence. If the field is set to setup and resourceReservationDedicatedUL is not included, then resourceReservationConfigCommonUL in SystemInformationBlockType29 applies. If resourceReservationConfigCommonUL is not provided or the two bits for each subframe in slotBitmap in resourceReservationConfigCommonUL are set to “01” or “10”, and for the UE that supports subframe-level resource reservation but does not support slot-level resource reservation, if this field is set to setup, resourceReservationDedicatedUL should also be included with two bits for each subframe in slotBitmap set to “00” or “11”.

Option 3:

To introduce a new subframe-level configuration for R16 eMTC, e.g.,

ResourceReservationConfig information element

-- ASN1START

ResourceReservationConfigDL-r16 ::= SEQUENCE {

periodicityStartPos-r16		PeriodicityStartPos-r16,

resourceReservationFreq-r16	CHOICE {

rbg-Bitmap1dot4				BIT STRING (SIZE (6)),

rbg-Bitmap3					BIT STRING (SIZE (8)),

rbg-Bitmap5					BIT STRING (SIZE (13)),

rbg-Bitmap10				BIT STRING (SIZE (17)),

rbg-Bitmap15				BIT STRING (SIZE (19)),

rbg-Bitmap20				BIT STRING (SIZE (25))

}	OPTIONAL, -- Need OP

subframeBitmap-r16			CHOICE {

subframePattern10ms			BIT STRING (SIZE (10)),

subframePattern40ms			BIT STRING (SIZE (40))

},

slotBitmap-r16				CHOICE {

slotPattern10ms				BIT STRING (SIZE (20)),

slotPattern40ms				BIT STRING (SIZE (80))

},

symbolBitmap1-r16			BIT STRING (SIZE (7))	OPTIONAL,	-- Cond Bitmap1

symbolBitmap2-r16			BIT STRING (SIZE (7))	OPTIONAL,	-- Cond Bitmap2

...

}

ResourceReservationConfigUL-r16 ::= SEQUENCE {

periodicityStartPos-r16		PeriodicityStartPos-r16,

subframeBitmap-r16			CHOICE {

subframePattern10ms			BIT STRING (SIZE (10)),

subframePattern40ms			BIT STRING (SIZE (40))

},

slotBitmap-r16				CHOICE {

slotPattern10ms				BIT STRING (SIZE (20)),

slotPattern40ms				BIT STRING (SIZE (80))

} OPTIONAL,	-- Cond FDDandTDDnoDL

symbolBitmap1-r16			BIT STRING (SIZE (7))	OPTIONAL,	-- Cond Bitmap1

symbolBitmap2-r16			BIT STRING (SIZE (7))	OPTIONAL,	-- Cond Bitmap2

...

}

PeriodicityStartPos-r16 ::=		CHOICE {

periodicity10ms					NULL,

periodicity20ms					INTEGER(0..1),

periodicity40ms					INTEGER(0..3),

periodicity80ms					INTEGER(0..7),

periodicity160ms				INTEGER(0..15),

spare3 NULL, spare2 NULL, spare1 NULL

}

-- ASN1STOP

Option 4:

slotBitmap Slot-level resource reservation configuration. Value slotPattern10ms corresponds to 10ms slot pattern and slotPattern40ms corresponds to 40ms slot pattern, see TS 36.213 [23] for DL and TS 36.211 [21] for UL. The first/leftmost 2-bits corresponds to the subframe #0 of the radio frame satisfying SFN mod periodicity = start position, as indicated by periopdicityStartPos. Two bits for each subframe coded as: 00: both slots are not reserved 01: the first slot is not reserved, the second slot is reserved 10: the first slot is reserved, the second slot is not reserved 11: both slots are reserved. If the field is not included in UL configuration, the value of the field from DL configuration applies. For a UE that supports subframe-level resource reservation but does not support slot-level resource reservation, the 2 bits are interpreted as: 00: subframe is not reserved 01: subframe is reserved 10: subframe is reserved 11: subframe is reserved.

| Company | Preferred option | Comments |
| --- | --- | --- |
| ZTE | Option 2 | Option 1 is initially proposed but there had concern that such restriction may be also applicable to SIB case as this is the common IE structure. So a sentence “if the resource reservation is enabled for it” is added. Moreover, Option 2 can be considered and also feasible to solve the issue. Option 2 only have impact on dedicated signaling. The only concern may be that, it may be not so good to mention an inner IE in field description for this IE ResourceReservationConfigDL/ResourceReservationConfigUL. We think option 3 may be not needed if option 1 or option 2 can solve the issue. |
| Huawei | Option 4 | We should have made the signaling the same as NB-IoT – i.e. to have a CHOICE between subframe or slot level, so this should be option 3, not as it has been presented above. As mentioned above this is just a signaling optimization, and the real problem is with common signaling, not dedicated signaling, therefore we should clarify what the UE applies in case the SIB indicates slot level reservation but the UE does not support it. Note that the bit interpretation is such that if the eNB indicates any slot in the subframe as reserved then UE reserves the whole subframe if slot level is not supported. |

# Summary

Conclusion:

Updated CRs in the draft folder:

# Participants

| Company | Name | e-mail address |
| --- | --- | --- |
| ZTE | Ting | lu.ting@zte.com.cn |
| Huawei | Brian | Brian DOT alexander DOT martin AT huawei DOT com |