3GPP TSG-RAN WG2 Meeting #111 electronic				R2-20xxxx

Online, August 17th – 28th, 2020

Agenda Item:		6.10.3	RRC corrections

Source:			Huawei, Ericsson

Title:				Summary on [AT111-e][888] RRC corrections

WI code(s):			NR_SON_MDT-Core

Document for:		Discussion and Decision

## 1	Introduction

This paper is to provide a summary on email 888.

- [AT111-e][888] RRC  corrections (Huawei, Ericsson)

Scope: all the open issues and not treated issues in 6.10.3

Step one is to achieve agreements to address the issues. Step 2 is to provide agreeable CRs and reviewing the CRs.

Intended outcome: Agreeable CR packages (3-4). All the NBC CRs should be explicitly marked. 38.331 CR for editorial changes is R2-2008284 provided by Huawei. 38.331 CR for MDT is R2-2008285 provided by Huawei. 38.331 CR for SON is R2-2008286 provided by Ericsson. If needed 36.331 CR is R2-2008287 provided by Ericsson.

Deadline of Step 1: Tuesday 25 03:00 UTC

Deadline of Step 2: Thursday 27 04:00 UTC

Status: will start as the emeeting starts

Basically this paper is based on the R2-2008281, and section 2 is updated based on the latest RAN2 agreements. So companies are welcome to put your comments in relevant tables. It is noted that some companies already provided comments.

All related Tdocs are listed in section 4. In previous RAN2 meeting, some categories were allocated for proposals in order for efficient discussions. So this paper is to use the same method,

- (a) a potential easy agreement, e.g. Proposals where consensus exists, that seem straightforward to agree

- (b) need further discussion. These should be tagged with e.g. [FFS] so they are clearly visible, and should indicate what the primary controversy is

- (c) Proposals with big impacts, so suggested to not be considered in R16.

As further analysis, the following categories are considered for proposals:

[NBC]: non-backward compatible changes

[New func]: changes for new functionality

[Critical change]: critical or necessary changes, e.g. correct the issues in the current spec, or make the SON and MDT better

[Others]: proposals other than [NBC], [New func] and [Critical change]

## 2	Issue summary

### 2.1	Guideline

Section 2.1 to 2.4 are discussions based on the contributions under the AI 6.10.3.

From the summary rapporteur point of view, it is suggested to put the following categories for the contributions:

- ASN1 related

  - MDT

  - SON

  - Others

- Non-ASN1 related

  - MDT

  - SON

  - Others

- Editorial changes

### 2.2	ASN1 related

#### 2.2.1	MDT

[6], CATT

- Rename the MeasResult2NR-r16, MeasResultList2NR-r16, MeasResult2EUTRA-r16 and MeasResultList2EUTRA-r16 included in UEInformationResponse message to MeasResult3NR-r16, MeasResultList3NR-r16, MeasResult3EUTRA-r16 and MeasResultList3EUTRA-r16.

- Rename the MeasResult2EUTRA-r16 used in MCGFailureInformation message to MeasResult3EUTRA-r16.

3) Correct InterFreqTargetInfo in AreaConfiguration field descriptions to interFreqTargetList.

// (b)

[NBC] for 1) and 2), as the field names are changed.

[RAN2 conclusion]: Not pursued. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm |  | No strong opinion. |
| ZTE |  | Seems no necessary, but we don’t have strong opinion here. |
| Ericsson |  | First two changes are not required and there is no confusion in our opinion. Third change is editorial and can be merged with Editorial CR. |
| CATT |  | MeasResult2NR is used both in R15 and R16, but have different meaning, so we prefer to avoid any confusion. |

[11], Samsung

Proposal	Change interFreqTargetList to a list of target frequencies that may include serving frequency and specify that UE creates a log entry only when having results for any of the indicated target frequencies (as in LTE)This version of the CR includes the following changes

// (b). [10] is the CR related to the discussion paper [11], so should be discussed first.

[NBC][New func]. It changes the structure of the field InterFreqTargetList.

[RAN2 conclusion]: Noted. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | No | Prefer to keep the current desgin. |
| Qualcomm | No | Prefer to keep the current design. |
| ZTE | No | We think current design is sufficient. |
| Ericsson | No | Current specification based method is clear and it works fine for the purpose. |
| CATT | No | Not essential |

[10], Samsung

This version of the CR includes the following changes

- Clarification is added that UE performs periodic logging if camped cell is part of areaConfig and the UE has results available for the frequencies and cells indicated by InterFreqTargetInfo

- Clarify that network only includes frequencies listed in SIB4 in InterFreqTargetInfo and that logging is not supported for frequencies measured only for the purpose of early measurement provision

  - Procedure is updated accordingly to cover the case InterFreqTargetInfo is not configured

- Change interFreqTargetList to targetFreqList and clarify that it may include serving frequency

  - Specify that UE does not perform logging if no results are available concerning any of the target frequencies

- Clarify that network does not configure areaConfiguration when configuring event triggered reports of type outOfCoverage

- Cover the UE behavior within the procedural specification (i.e. remove some aspects from the field descriptions)

// (b)

[NBC][New func] for the change “interFreqTargetList to targetFreqList”.

[RAN2 conclusion]: Not pursued. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | No | No for the change “interFreqTargetList to targetFreqList”. No strong opinion for other changes. |
| Qualcomm |  | No strong opinion. |
| ZTE | No | We think interFreqTargetList is only applicable for neighboring cell measurement, and won’t be used for area scope checking. |
| Ericsson | No | Two parts to this CR. - Regarding the interFreqTargetList and areaConfig, we believe this is not required and the current specification is clear on this. - Regarding making areaConfig optional for OOC event triggered logged MDT, we believe this is also not required as the UE logs the measurements even if the areaConfig is configured as per the procedural text. 2> else if the reportType is set to eventTriggered, and eventType is set to outOfCoverage: 3> perform the logging at regular time intervals as defined by the loggingInterval in VarLogMeasConfig only when the UE is in any cell selection state; 3> perform the logging immediately upon transitioning from the any cell selection state to the camped normally state; |
| CATT |  | Apart from changing the name of interFreqTargetList, we can discuss further for the wording of the rest changes, we tend to agree the intention as the current text is not clear enough. |

[15], Samsung

Proposal: IE InterFreqTargetList moves out of IE AreaConfiguration.

// (b)

[NBC]. It moves the IE interFreqTargetList out of the IE AreaConfiguration.

[RAN2 conclusion]: Not pursued. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | No | See no strong reason of this change. |
| Qualcomm | No | The current structure allows UE to reduce the number of inter-frequency targets within an areaconfiguration. Thus, do not agree with the change. |
| ZTE | No | It has been discussed in previous meeting and we agree not to move the IE outside the AreaConfiguration. |
| Ericsson | No | As noted by ZTE, this was discussed and not agreed in the past. |
| CATT | No | Not essential |

[32], ZTE

In 5.5a.3.2, the procedure part on how to include neighboring cell measurement when UE is configured is included;

The separated available bits in LogMeasReport is replaced by the common available IE UEMeasurementsAvailable.

// (b)

[NBC]. It replaces some availability indicators with a common one.

[RAN2 conclusion]: Not pursued. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm |  | Regarding UEMeasurementsAvailable, I think this is based on agreement in the RAN2#110-e, however, in the CR the structure of UEMeasurementsAvailable seems incomplete. UEMeasurementsAvailable should be defined. Regarding 5.5a.3.2 changes, no strong opinion but should be properly organized. |
| ZTE | Yes | Otherwise it is not specified how to set neighboring cell measurement when interFreqTargetList is configured; the next change is to align the available bit setting behavior in MDT. |
| Ericsson | No | The procedural text proposed in the contribution is already captured in the field description and we should avoid moving field description related text to the procedural text and vice-versa. If configured, it indicates the frequency for which UE is requested to perform measurement logging for neighbour cells. UE should perform measurement logging for the frequency in SIB4 of the current serving cell whose DL-carrierfrequency is included in the InterFreqTargetList. If not configured, the UE should perform measurement logging for all the neighbour cells. The ASN.1 change is not an essential change and do not change any functionality and we should avoid optimizations at this stage and only fix open issues that are ‘wrong’ in the spec. Therefore reject |
| CATT | No | Not essential |

[37], Samsung

Change the MeasResultListLoggingNR-r16 to the MeasResultLogging2NR-r16 in MeasResultListLogging2NR-r16.

// (b)

[NBC]. The name and meaning of the IE MeasResultListLoggingNR-r16 are changed.

[RAN2 conclusion]: The changes are agreed and will be merged into one big MDT CR. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm | Yes |  |
| ZTE | Yes | The intention is to link the measurement results to the frequency band the measurements are taken, which is helpful. |
| Ericsson | Yes |  |
| CATT | Yes |  |

[42], Samsung

1/ Update the term 'global cell id' to global cell identity' to align with procedure text.

2/ Remove to store the loggingInterval in VarLogMeasConfig explicitly in the procedural text.

3/ Update the procedural text such that the reportType is set to eventTriggered with typo correction.

4/ Change the field name or update field descriptions in ASN.1 i.e.

- measResultServingCell  measResultServCell in LogMeasInfo

- Update "as indicated by NR within absoluteTimeInfo" in the field description of absoluteTimeStamp

- Remove the field description of MeasResultServCell in RLF-Report

- Typo is fixed i.e. 'indicated  indicate' in the field description of rlf-Report

5/ Change the absoluteTimeStamp to absoluteTimeInfo in VarLogMeasReport to align with procedure text

// (b). Some changes are impacting ASN1, while others are not

[NBC] for 4/. Some IE names are changed (to be aligned with the names used in the procedural text).

[RAN2 conclusion]:

=>	For 4/, keep the field names as they are, and update the names in the procedural text

=>	Update the names in the procedural text  and all the other changes are agreed and merged into the big editorial CR.

So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | For 4/, we suggest to keep the field names as they are, and we can update the names in the procedural text. |
| Qualcomm |  | Seems reasonable. |
| ZTE |  | All seems editorial, can be merged into Rapporteur CR. |
| Ericsson |  | Agree with ZTE that this CR is editorial and can be merged with rapporteur CR. |
| CATT | Yes | Agree with ZTE that this CR is editorial and can be merged with rapporteur CR |

#### 2.2.2	SON

##### 2.2.2.1	SCG failure report

[16], Samsung

Proposal: Avoid that UE duplicates the location info provided upon SCG failure merely to facilitate use by both MN and SN.

// (b)

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | In our paper R2-2007753 presented in this meeting, we have a different view, and we think the current design is basically ok but may need some enhancements for user consent purposes. |
| Qualcomm | Yes | We should remove the duplication of location information. |
| ZTE | No | We don’t think this is duplicated information since SN can configure differently location configurations in otherConfig from MN. Therefore, the location info to be reported for MN and SN can be different. |
| Ericsson | No | This was discussed in the past and not agreed for the following reasons. - The locationInfo included directly in SCGFailureInformation is based on the MN configured WLAN / BT measurements whereas the lo0cationInfo included in measResultSCG-Failure of SCGFailureInformation message is based on SN configured WLAN / BT measurements. - The locationInfo included directly in SCGFailureInformation is encoded in MN RRC format whereas the locationInfo included in measResultSCG-Failure of SCGFailureInformation message is encoded in SN RRC format. |
| CATT | No | We discuss this issue before, so we stick to the current text. |

##### 2.2.2.2	RA report

[20], Ericsson

Proposal 1	Modify the existing cellID, in order to allow the UE to include either the SpCell or the SCell in which RA occurred when the raPurpose is set to ulUnSynchronized.

Proposal 2	Clarify the use case of ulUnSynchronized in the raPurpose field description according to the attached TP.

Proposal 3	In the raPurpose field description, change noSRPUCCHResourceAvailable into noPUCCHResourceAvailable, since the flag noSRPUCCHResourceAvailable is not in the ASN.1.

// (b)

[Critical change].

[RAN2 conclusion]: Changes are agreed and merged into big SON CR. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm | Yes |  |
| ZTE | Yes | Since we agree to support logging of RA event triggered by TA addition in SCell, in such case SCell info shall be stored. |
| Ericsson | Yes |  |
| CATT | Yes |  |

##### 2.2.2.3	RLF report

[21], Ericsson

Proposal 1	RAN2 is requested to agree on one of the following options to resolve the ambiguity in the current specification for the scenarios when the UE includes both reconnectCellId and reestablishmentCellId in the RLF report.

a.	The UE shall include a flag to indicate whether the re-establishment was successful or not in the RLF report.

b.	The UE includes the reconnectCellId only when the re-establishment was unsuccessful

// (b)

[New func].

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm | No | In the scenario “UE declares RLF -> UE performs re-establishment successfully -> re-establishment cell does not fetch the RLF report -> UE gets released to RRC IDLE -> UE comes back to connected in another cell and this cell fetches the RLF report.” I think, UE should not add the reconnectcellID as UE performed the reestablishment successfully. Thus, flag is not needed to differentiate III and IV in Ericsson proposal. |
| ZTE | No without confirmed by RAN3 | The reconnectedCellID info is added upon request from RAN3 and in their LS (R2-2004334), it is stated “6)Time interval between HOF/RLF and successful RRC re-connection: This information helps the network to understand whether the re-connection cell could be used to detect the root cause of failure event.” it seems RAN3 think the current info included in RLF report is sufficient. Considering this info has been used by RAN3 for MRO usage, it is preferred to consult RAN3 before we make any changes. |
| Ericsson | Yes | This is a required change for MRO purposes and what Qualcomm mentions above is not correct. Even when the UE performs reestablishment successfully, there is no guarantee that the reestablishment cell fetches the RLF report immediately. If the RLF report is not fetched, then the reconnectionCellID will also be stored in the RLF report. This causes confusion for the MRO algorithm as it is not clear whether the HO parameter needs to be tuned towards reestablishment cell or reconnection cell. Even when the reestablishment is not successful, the UE includes the reestablishment cell ID as long as it selects that cell for reestablishment. This is as per the current procedural text and thus the cause of confusion for MRO. |
| CATT | No | We think the reconnectionCellID is only record if reestablishmentCellId is not set, the network does not need to get both. |

[38], Samsung

Change the upper limit from 12 to 16 in plmn-IdentityList within VarRLF-Report

// (b)

[Critical change].

[RAN2 conclusion]: Changes are agreed and merged into big SON CR. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| Ericsson | Yes |  |
| CATT | Yes |  |

##### 2.2.2.4	CEF report

[34], ZTE

- RPLMN has been replaced by selected PLMN;

- On IE to indicate UL/NUL type used in RA procedure has been introduced in CEF report and the corresponding procedure part has been added in subclause 5.3.3.7 and 5.3.13.5.

// (b)

[NBC]. The new IE ulCarrierIndication-r16 is added before "..." and it is NBC.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | For the introduction of new IE ulCarrierIndication-r16, it should be backward compatible. |
| Qualcomm |  | No strong opinion. |
| ZTE |  | The intention is to align the CEF report content specified between 37.320 and 38.331. If the inclusion of such indicator is agreeable, then we can update the CR to a backward compatible one. |
| Ericsson | NO | SUL/NUL indication is implicit and it was agreed that based on the used frequency resources network can derive this information. Therefore reject this. |
| CATT |  | Show some sympathy to this proposal |

[12], Samsung

- Add the corresponding procedural texts in 5.3.3.7 and 5.3.13.5. [Samsung] The resume failure have been missed

- Add the field, maxTxPowerReached into the IE, ConnEstFailReport.

- Remove the field description on numberOfPreamblesSent in the IE ConnEstFailReport

// (b). In LTE TS 36.331, for the field maxTxPowerReached, it is mentioned as below. It refers to TS 36.321 and thus we need to firstly check whether NR TS 38.321 has supported the functionality or not.

[Samsung] In LTE TS36.331, we can see the following procedural texts:

- 5.3.3.6 T300 expiry (skipped) 3> set maxTxPowerReached to indicate whether or not the maximum power level was used for the last transmitted preamble, see TS 36.321 [6];

Here, the intention referring TS36.321 is just to introduce the power ramping operation during normal random access procedure, but not to provide any specific UE behaviour related to the field maxTxPowerReached. So, we cannot see the field maxTxPowerReached in TS36.321.

Accordingly, we need to only update TS38.331 for the field maxTxPowerReached, based on the RAN2 agreement.

[Critical change].

[RAN2 conclusion]: Not pursued. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | For maxTxPowerReached, we would like to check whether NR TS 38.321 has supported the functionality or not. |
| Qualcomm |  | maxTxPowerReached would be needed to figure out the cause of RACH failure. However, agree with Huawei that we should check if it is supported in TS 38.321. |
| ZTE |  | We think based on current RA report, NW can know the power ramped during RA procedure, and derive whether the maximum transmitted power has reached or not. |
| Ericsson |  | We believe the maxTxPowerReached alone does not suffice as the network does not get to know in which RA attempt did the UE reach the max TX power. Therefore, if we are including the maxTxPowerReached, then there is also a need to include the RA attempt in which this stage was reached. As this requires multiple changes, we propose to look into this in rel-17 and not include anything further in rel-16 which does not convey the correct information to the network. |
| CATT | No | Agree with Ericsson |

#### 2.2.3	Others

[31], ZTE

- “if available” is deleted from the procedure part, UE always includes locationCoordinate if detailed information is available for reporting;

- locationCoordinate and locationTimestamp in commonLocationInfo IE is change from optional to mandatory;

// (b)

[NBC]. It removes some "OPTIONAL" for some Ies.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | We wonder whether it is essential issue for “OPTIONAL”. |
| Qualcomm |  | Do not agree. These are the optional fields. If UE have these fields available, then UE will add them, however, not in favour of making then mandatory. For example, if certain reason, UE does not have location coordinate available, UE cannot report them. |
| ZTE | Yes | When UE includes detailed location information, the coordinates info shall also be included, otherwise the included other parameters in detailed location info is useless, which is also the same behavior as specified in LTE. |
| Ericsson |  | We wonder whether it is possible that the UE has WLAN information available but could not compute the location coordinates based on this. In such a case, it would still be good to include only WLAN measurements along with time stamps, thus not making locationCoordinates mandatory. Therefore, keep the OPTIONAL tag for locationCoordinate. In principle, we are okay with removing OPTIONAL for locationTimeStamp but as this is not a critical change and thus would like to hear other companies’ views as this is a NBC change in principle |
| CATT |  | Not essential, prefer to change stage 2 spec |

### 2.3	Non-ASN1 related

#### 2.3.1	MDT

[1], CATT

Add “if reestablishmentCellId in VarRLF-Report is not set” as a condition together with the current condition to decide whether to include re-connection cell id in RLF report.

// (b)

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSicon |  | No strong opinion. |
| Qualcomm | Yes |  |
| ZTE | No | Please refer to our previous comment in 2.2.2.3 RLF report. |
| Ericsson | Partly yes | This covers half of the problem as we have listed in 2.2.2.3. We propose to combine this solution with the solution proposed in our contribution 2007668 |
| CATT | Yes | We think the reconnectionCellID is only record if reestablishmentCellId is not set, the network does not need to get both. |

[9], Samsung

This version of the CR includes the following changes

- Clarification is added that for the entries included in the UEInformationResponse message the UE includes all optional fields stored in VarLogMeasReport. I.e. UE does not omit optional fields like detailed location info in order to report a larger number of entries

- Statements regarding the setting of the BT and WLAN information are clarified to reflect that this concerns information within logged measurement entries (rather than separate entries, as the current text erroneously suggests)

- Cover the UE behavior within the procedural specification (i.e. remove some aspects from the field descriptions)

- Change also clarifies that these fields can only be set if there are remaining entries

// (b)

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSicon |  | No strong opinion. |
| Qualcomm |  | No strong opinion. However, like the current way of reporting, where WLAN/BT measurement can be reported separately. |
| ZTE |  | Maybe no. We think the original intention is to leave it to UE implementation. |
| Ericsson |  | The current version of the spec already ensures that the UE always includes the entire set of the logMeasInfo for a given sample. When the first segment of logged MDT results are sent, then the UE needs to include a flag that it still has logged MDT samples available and also include flags that says whether the remaining measurements have WLAN and BT measurements or not. So what is proposed is not required. 3> if the VarLogMeasReport includes one or more additional logged measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message: 4> include the logMeasAvailable; 3> if the VarLogMeasReport includes one or more additional logged Bluetooth measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message: 4> include the logMeasAvailableBT; 3> if the VarLogMeasReport includes one or more additional logged WLAN measurement entries that are not included in the logMeasInfoList within the UEInformationResponse message: 4> include the logMeasAvailableWLAN; |
| CATT |  | No strong opinion. |

[17], Samsung

Proposal: RAN2 clarifies how to determine whether a cell is part of the area Indicated by AreaConfiguration.

// (b). It seems that there is no ASN1 impact, otherwise this paper should be moved to section 2.2.1 MDT.

[Critical change].

[RAN2 conclusion]:

=>	Not pursued

=>	Can be discussed in the email discussion on stage2 correction.

So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSicon |  | We do not have strong opinion. For the field CGI-Info-Logging, it is clear on the NCGI setting, e.g. for cellIdentity, plmn-Identity, trackingAreaCode, so we think it may be clear for the UE to also apply this rule for the areaConfiguration check. |
| Qualcomm |  | No strong opinion. |
| ZTE |  | Share the same view as Huawei, the same principle when setting the global cell identity can be reused. |
| Ericsson |  | We think that checking based on only the first PLMN in the SIB1 would reduce the usecase associated to sending a PLMNISList as part of the logged MDT configuration. Therefore, we prefer that the UE checks that atleast one PLMN ID + (TAC ID / Cell ID) combination in SIB1 matches with the configured PLMNIDlist and the Area config provided in the MDT configuration. |
| CATT |  | No strong opinion |

[18], Nokia

Changes contain:

- Correction of typos in 5.5a.3

- Replacing “8” by reference to maxRAReport

- Adding “else” branch to the procedures on addign RA-report entries

- Clarification on VarRA-report release that clarifies the UE starts counting the 48h timer only when the RA Report is filled

- Correction of typos in 5.7.10.4

// (a). Changes seem reasonable

[QC] Do not agree regrading clarification on VarRA-report release. In the 109-bis meeting, we agreed that UE should relaxed with condition of maximum RA-report and if no new RA entries is obtained within last 48 hours since the last RA-report entry then UE can clear var-RA report.  For example, if eight entries are not met, UE is not going to keep the RA-report forever. Furthermore, significance of the RA-report reduces significantly with time and the network does not extract the RA-report within last 48 hours since last entry then there is no point of UE storing it. [see 109-bis meeting agreements]

[Huawei] ok to change it from (a) to (b).

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm | No | See our comment regrading the varRA-report release. |
| ZTE |  | No strong view, but we do share some sympathy as Qualcomm. |
| Ericsson |  | ‘else’ condition is not required as it is not adding any behavioural change. Following change is not required. (the VarRA-report is filled with a number of random access procedures equal to maxRAReport). Rest of the changes are editorial and hence merge with rapporteur CR. |
| CATT |  | Only agree the editorial change. |

[24], Huawei

For SON and MDT reporting of availability indicators, the condition is clarified that:

if the RRCReconfiguration message was received via SRB1, but not within mrdc-SecondaryCellGroup or E-UTRA RRCConnectionReconfiguration:

// (b)

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes |  |
| ZTE |  | Seems unnecessary, we think it is no harm to set the availability bit, SN just doesn’t request UE to report. There is no critical consequence. |
| Ericsson |  | In principle, this is correct, but the network can handle this and there is no impact if this CR is not agreed. We should keep the bar high for Rel-16 CRs and therefore reject. |
| CATT |  | No strong view |

[25], Huawei

In section 5.5a.3.2, the following changes are made:

- For UE storing logged measurement results, the following behaviour is used:

when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following

- When the UE performs logged measurements, for WLAN, Bluetooth and Sensor measurements, separate descriptions are added.

- The following text is removed from the Note:

or as used for evaluation of reporting criteria or for measurement reporting according to 5.5.3 in RRC_CONNECTED

// (b)

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes |  |
| Ericsson | No | First change is not required. Second change is also not required as the UE would not have WLAN or BT measurements based on the procedural text. 4> if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons; 4> if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs; Third change is editorial. |
| CATT |  | No strong view |

[26] Huawei

Similar as LTE definition, add UE behaviours of releasing the field obtainCommonLocation for the following cases:

- 5.3.7.2    RRC re-establishment

- 5.3.7.3    cell reselection during RRC re-establishment

- 5.3.13.2   RRC resume

// (b)

[Critical change].

[RAN2 conclusion]: The changes are agreed in principle and should be checked though email discussion[888]. So suggest to continue collecting comments.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes | Similar as LTE spec. |
| ZTE | Yes |  |
| Ericsson | May be not | In principle we agree with the change but there is no impact even if this CR is not agreed. So, do we need to have this CR as ‘correction’ to rel-16? |
| CATT | Yes |  |

[36] Samsung

Update the field description of CGI-InfoEUTRALogging such that

- The cellIdentity belongs to the first PLMN entry of plmn-IdentityList (when connected to EPC) or of plmn-IdentityList-r15 (when connected to 5GC) in SystemInformationBlockType1

- The plmn-Identity corresponds to the first PLMN entry of plmn-Identity (when connected to EPC) or of plmn-IdentityList-r15 (when connected to 5GC) that contained the reported cellIdentity.

// (b)

[Critical change].

[RAN2 conclusion]: changes are agreed and will be merged into big MDT CR. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes |  |
| ZTE |  | No strong view |
| Ericson | Yes |  |
| CATT |  | No strong view |

#### 2.3.2	SON

##### 2.3.2.1	Inter-RAT SON for TS 36.331

[1], CATT

Add “if reestablishmentCellId in VarRLF-Report is not set” as a condition together with the current condition to decide whether to include re-connection cell id in RLF report.

// (b)

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm | Yes |  |
| ZTE | No | As commented in previous issue, we don’t think RAN2 can decide without confirmed by RAN3. |
| Ericsson | Partly yes | This covers half of the problem as we have listed in 2.2.2.3. We propose to combine this solution with the solution proposed in our contribution 2007668 |
| CATT | Yes |  |

[5], CATT

1) One condition of including NR related cell ID in LTE VarRLF-Report is changed from “UE supports Radio Link Failure Report for inter-RAT MRO” to “UE supports Radio Link Failure Report for inter-RAT MRO NR”.

2) One condition of including E-UTRA re-connection cell ID in NR VarRLF-Report is changed from “UE supports RLF report for inter-RAT MRO” to “UE supports RLF report for inter-RAT MRO EUTRA”.

3) Add “failure” follow “last radio link”. It is an editorial change.

// (b)

[CATT] We think proposal 3) is a CAT a issue, it’s quite simple and obvious.

[Huawei] Suggest to discuss all changes together.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | Ok for 3), and no strong opinion for other changes. |
| Qualcomm |  | Agree for 3). I understand the intention of CATT, but I believe adding NR or EUTRA at end of inter-RAT MRO makes it more confusing. However, do not have strong preference. |
| ZTE |  | P3 is editorial, can be merged into Rapporteur CR. No strong view for the rest. |
| Ericsson |  | P3 to be merged with rapporteur CR. P1 and P2 is also not essential and the network knows whether to use it for EUTRA MRO or NR MRO and as there are no UE impacts, we prefer not to change anything. |
| CATT |  | We should align the wording between 38.306 and 38.331, so prefer to have it. |

[28], Huawei

The following changes are made:

- Clarify the HOF case occured between E-UTRA/EPC and E-UTRA/EPC, between E-UTRA/5GC and E-UTRA/5GC and from E-UTRA/5GC to NR.

- Clarify the RLF case occured from E-UTRA/EPC to E-UTRA, between E-UTRA/5GC and E-UTRA/5GC and from NR to E-UTRA/5GC.

- Clarify that the RRCConnectionReconfiguration message including the mobilityControlInfo is embedded in NR RRC message upon handover to E-UTRA from NR.

// (b)

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Basically our CR has two parts: one is to align the procedural text with stage-2 definition for inter-RAT SON, and the other is the improvements on wordings. |
| Qualcomm |  | No strong opinion. Some part of the CR seems reasonable. |
| ZTE |  | Fine for the last one, no strong view for the rest. |
| CATT |  | No strong opinion |

##### 2.3.2.2	Inter-RAT SON for TS 38.331

[2], CATT

- Add “if reestablishmentCellId in VarRLF-Report is not set” as a condition together with the current condition to decide whether to include re-connection cell id in RLF report.

- Add “if the UE supports RLF report for inter-RAT MRO NR” as a condition for including NR re-connection cell id in EUTRA RLF report.

// (b)

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm | Yes |  |
| ZTE | No | Please refer to our previous comment in 2.2.2.3 RLF report. |
| CATT | Yes |  |

[8], Google

Only the UE with cross-RAT RLF reporting capability is required to transmit the cross-RAT RLF report to the network.

// (b). This CR seems to be covered by the above CR ([2], CATT).

[Google] We do not think CATT’s CR covers Google’s CR. In CATT’s CR, the change is in section 5.3.3.4. It specifies the UE behaviours for “store” the cell id in VarRLF-Report. In Google CR, the change is in section 5.7.10.3. It specifies the UE behaviours for “transmit” the EUTRA RLF report.

[Huawei] Ok. The two CRs ([2][8]) are for separate changes.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm | Yes. |  |
| ZTE |  | Seems to be a rare issue, no strong view. |
| CATT |  | No strong opinion |

[27], Huawei

The following changes are made:

- Clarify the HOF case occured between NR and E-UTRA connected to 5GC

- Clarify that the RRCReconfiguration message including the reconfigurationWithSync is embedded in E-UTRA RRC message upon handover to NR from E-UTRA

- Add the text decription to set the failedPCellId-EUTRA for cross-RAT RLF report according to the connectionFailureType.

// (b)

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes. |  |
| ZTE |  | Fine with the second one, no strong view for the rest. |
| CATT |  | No strong opinion |

##### 2.3.2.3	Inter-RAT SON for TS 38.306 (to be hanlded in main session)

[3], CATT

- Add “nrReconnectCellId” in “Radio Link Failure Report for inter-RAT MRO NR”.

- Change field name “previousNR-CellId” to “previousNR-PCellId”. It is a editorial change.

- Change field name “failedNR-CellId” to “failedNR-PCellId”. It is a editorial change.

// (a). Seems editorial changes

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes |  |
| ZTE | Yes | But this seems to be handled in main session. |

[4], CATT

1) Merged the “Reconnection Report for inter-RAT MRO EUTRA” into the “Radio Link Failure Report for inter-RAT MRO EUTRA” capability.

2) Add the supporting of “previousPCellId” report as a sub-capability of “Radio Link Failure Report for inter-RAT MRO EUTRA”.

3) Add the “frequency+PCI” as one of the cell identification branch of HOF case to align with the description and ASN in TS38.331.

// (b)

[New func].

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. Sightly prefer the current text. |
| Qualcomm |  | Merging is fine as agreed. But prefer current text style. |
| ZTE | Yes | This was agreed in email discussion [AT110-e][802][SONMDT] UE capabilities (CMCC, vivo). Again, seems to be handled in main session. |

##### 2.3.2.4	CEF report

[7], CATT

Following changes are introduced:

- Add TA information in CEF report when T319 expire.

- Correct some typo.

// (a). Seems reasonable

[Critical change].

[RAN2 conclusion] changes are agreed and will be merged into big MDT CR. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| CATT | Yes |  |

[14], Samsung

Proposal: Upon setup failure or resume failure, UE sets the plmn-Identity to RPLMN if available.

// (b)

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm |  | No strong opinion. |
| ZTE |  | Maybe no. It seems we already have the same discussion in previous meeting, and majority thinks current procedure is fine. |
| CATT |  | Agree with ZTE |

[41], Samsung

1/ Change the 'cell identity' to the 'global cell identity' when the UE decides whether to reset the numberOfConnFail to 0 in 5.3.3.7 and 5.3.13.5

2/ Clarity that the UE includes the cell level and SS/PBCH block level RSRQ and SS/PBCH block indexes of the failed cell in measResultFailedCell based on the available SSB measurements.

3/ Update the reference section for setting perRAInfoList correctly

// (b)

[Critical change].

[RAN2 conclusion]:

=>	The third change is agreed and merged into big editorial CR.

=>	The second change should be discussed in the email discussion [888].

=>	The first change is not agreed.

So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm |  | Seems reasonable |
| ZTE |  | (1)(3) seems editorial change, which is fine. As for (2), we think current description is sufficient, not need to over-specify. |
| CATT |  | No strong opinion |

##### 2.3.2.5	RLF report

[19], Nokia

Changes contain:

- Adding determination of RLF cause for the BH RLF

// (b)

[Critical change].

[RAN2 conclusion] Should be handled in other session. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm | No | IAB is not the part of Rel-16 SON/MDT WI. |
| ZTE |  | Seems reasonable, since the cause has been included in ASN.1, it is fine to add in the procedure part. |
| CATT |  | IAB is not the part of Rel-16 SON/MDT WI |

[29], Huawei

Capture “clear the information included in VarRLF-Report, if any” in all the fast MCG recovery messages received by the UE, including:

- Reception of an RRCConnectionReconfiguration including the mobilityControlInfo;

- Reception of an RRCConnectionRelease;

- Reception of the MobilityFromEUTRACommand by the UE

// (b)

Note: this CR is indeed for TS 36.331, but the Tdoc info shows TS 38.331 (it is a mistake).

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Qualcomm | Yes |  |
| ZTE | Yes | Seems reasonable. |
| CATT | Yes |  |

[39], Samsung

Clarify that the UE set the measResultLastServCell in VarRLF-Report to include both cell and sorted beam measurements in case of RLF or HO failure.

// (b)

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm |  | No strong opinion. |
| ZTE |  | No strong view. |
| CATT | Yes |  |

##### 2.3.2.6	RA report

[33], ZTE

- One more condition to include RA entry has been added in 5.7.10.4;

- The note indicate UE not to store RA entry in RA report has been moved from 5.7.10.5 to 5.7.10.4;

- The inclusion of RA resource in RA report has been updated to include the RA resource information if it is used.

- The field description of  dlRSRPAboveThreshold has been updated.

// (b)

[Critical change].

[RAN2 conclusion]

=>	The first change will be further discussed in email [888]

=>	The last change is not agreed

=>	The second and third changes are agreed and will merged into big SON CR.

So please companies provide your comments on the first change if any.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm |  | Seems okay. |
| ZTE | Yes | For 1, if not agreed, UE might not set the RA report when the EPLMN List is not full and there are still places for adding new RA entry. For 2, if not agreed, UE might include RA information triggered due to NR-U, in this case UE cannot set the RApurpose which is mandatory in the procedure part, and this will provide misleading information to NW. For 3, if not agreed, UE will include RA resource that is not used in the RA procedure, which is misleading. 4 is just editorial change. |
| CATT |  | No strong view |

##### 2.3.2.7	User consent for RLF report, CEF report and SCG failure report

[22], Huawei

Proposal 1: For LTE RLF report and CEF report, it is proposed to apply location info configuration, i.e. obtainLocation, bt-nameListConfig, wlan-NameListConfig.

Proposal 2: In LTE 36.331, apply location info configuration (i.e. location, WLAN measurement, Bluetooth measurement) to measResultSCG-FailureMRDC.

Proposal 3: In LTE 38.331, apply location info configuration (i.e. locationInfo, WLAN measurement, Bluetooth measurement, Sensor measurement) to SCGFailureInformationEUTRA.

Proposal 4: In LTE 36.331, apply location info configuration (i.e. location, WLAN measurement, Bluetooth measurement) to SCGFailureInformationNR.

Proposal 5: In LTE 38.331, apply location info configuration (i.e. locationInfo, WLAN measurement, Bluetooth measurement, Sensor measurement) to MeasResultSCG-Failure.

Proposal 6: The RLF/CEF/SCG failure reporting and MDT use the same user consent.

// (b)

[New func][Critical change]. It is related to user consent clarifications (as RAN2 sent a LS to SA5 on user consent for RLF/CEF/SCG failure at RAN2-110-e meeting).

[RAN2 conclusion] Postponed. So no need to discuss it here.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Even if there are lots of proposals in the discussion paper, the basic idea is not complex, i.e. be sure that the following RAN2 agreements have been correly implemented in the LTE and NR specs. From RAN2 understanding network will not configure UE to report location information for SON/MDT purpose if network doesn’t get the user consent from this UE. |
| Qualcomm |  | Our preference is to remove the unnecessary duplication of location information and agree with the Samsung proposal. Also, I am not sure if separate user consent is required form NR and LTE, in such case the same user consent cannot be used for RLF/CEF and SCGFailure. |
| ZTE |  | Seem reasonable. |
| CATT |  | I’m wondering whether we can agree this without any feedback from SA3 though we agree the intention. |

#### 2.3.3	Others

[40], Samsung

- Update 'upon entering NR or E-UTRA' to 'upon entering camped normally state in NR or E-UTRA' in the procedural text.

- Change the term 'outside NR' to 'in any cell selection state and/or camped on any cell state in NR or LTE'

// (b)

Note: this CR is related to mobility history information.

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon |  | No strong opinion. |
| Qualcomm |  | Seems okay |
| ZTE |  | No strong view. |
| CATT | Yes |  |

### 2.4	Editorial changes

[23], Huawei

// (a). Suggest to have one CR (Category D CR) for agreeable editorial changes.

[13], Samsung

// (a)

[35], Quectel

Delete the RLF content setting in VarRLF-Report from section 5.3.10.3.

// (a)

For all above editorial changes:

| Company | Agree (Yes/No) | Comment |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Ok with the above 3 CRs. We also note that there may be other editorial changes in other CRs (mentioned in section 2.2 and 2.3). We think that either we could include every editorial change in a big CAT D CR, or we just focus on the above 3 CRs (i.e. other editorial changes can be discussed and agreed in spearate CRs). |
| Qualcomm | Yes |  |
| ZTE | Yes | Can have all agreeable editorial change merged in Rapporteur CR. |
| CATT | Yes |  |

### 2.5	Summary on the agreeable changes

Changes on MDT for TS 38.331:

| Tdoc | RAN2 conclusion | NBC? (Yes/No) | Note |
| --- | --- | --- | --- |
| [37] | The changes are agreed and will be merged into one big MDT CR. | Yes |  |
| [26] | The changes are agreed in principle and should be checked though email discussion[888]. | No |  |
| [36] | changes are agreed and will be merged into big MDT CR. | No |  |
| [7] | changes are agreed and will be merged into big MDT CR. | No |  |

Changes on SON for TS 38.331:

| Tdoc | RAN2 conclusion | NBC? (Yes/No) | Note |
| --- | --- | --- | --- |
| [20] | Changes are agreed and merged into big SON CR. | No |  |
| [38] | Changes are agreed and merged into big SON CR. | No |  |
| [33] | => The first change will be further discussed in email [888] => The last change is not agreed => The second and third changes are agreed and will merged into big SON CR. | No |  |

Editorial changes on MDT and SON for TS 38.331:

| Tdoc | RAN2 conclusion | Note |
| --- | --- | --- |
| [42] | => For 4/, keep the field names as they are, and update the names in the procedural text => Update the names in the procedural text and all the other changes are agreed and merged into the big editorial CR. |  |
| [41] | => The third change is agreed and merged into big editorial CR. => The second change should be discussed in the email discussion [888]. => The first change is not agreed. |  |

Changes on MDT and SON for TS 36.331: (if needed)

| Tdoc | RAN2 conclusion | NBC? (Yes/No) | Note |
| --- | --- | --- | --- |

## 3	Conclusion

[To be updated]

## 4	Tdocs under AI 6.10.3	RRC corrections

- R2-2006644	Correction about Including Re-connection Cell ID in RLF Report	CATT	CR	Rel-16	36.331	16.1.1	4351	-	F	NR_SON_MDT-Core

- R2-2006645	Correction about Including Re-connection Cell ID in RLF Report	CATT	CR	Rel-16	38.331	16.1.0	1723	-	F	NR_SON_MDT-Core

- R2-2006646	Correction on RLF Report for Inter-RAT MRO NR	CATT	CR	Rel-16	36.306	16.1.0	1778	-	F	NR_SON_MDT-Core

- R2-2006647	Correction on RLF Report for Inter-RAT MRO EUTRA	CATT	CR	Rel-16	38.306	16.1.0	0365	-	F	NR_SON_MDT-Core

- R2-2006648	Clarification on RLF Report for Inter-RAT MRO	CATT	CR	Rel-16	36.331	16.1.1	4352	-	F	NR_SON_MDT-Core

- R2-2006649	Clarifications for MeasResult2NR and MeasResult2EUTRA  Relevant IEs	CATT	CR	Rel-16	38.331	16.1.0	1724	-	F	NR_SON_MDT-Core

- R2-2006650	Add TA Information in CEF Report when T319 Expire	CATT	CR	Rel-16	38.331	16.1.0	1725	-	F	NR_SON_MDT-Core

- R2-2007214	Correction on cross-RAT RLF report	Google Inc.	CR	Rel-16	38.331	16.1.0	1801	-	F	NR_SON_MDT-Core

- R2-2007224	Clarification on MDT regarding reporting of WLAN and BT information	Samsung Telecommunications	CR	Rel-16	38.331	16.1.0	1804	-	F	NR_SON_MDT-Core

- R2-2007225	Clarification on MDT regarding area configuration and target frequencies	Samsung Telecommunications	CR	Rel-16	38.331	16.1.0	1805	-	F	NR_SON_MDT-Core

- R2-2007226	Specification of target frequencies for measurement logging	Samsung Telecommunications	discussion	Rel-16	NR_SON_MDT-Core

- R2-2007372	Corrections on ConnEstFailReport	Samsung	CR	Rel-16	38.331	16.1.0	1838	-	F	NR_SON_MDT-Core

- R2-2007373	Correction on RA-Report	Samsung	CR	Rel-16	38.331	16.1.0	1839	-	F	NR_SON_MDT-Core

- R2-2007382	Logging PLMN Info in VarConnEstFailReport	Samsung 	discussion	NR_SON_MDT-Core

- R2-2007384	Moving IE InterFreqTargetList out of IE AreaConfiguration 	Samsung	discussion	NR_SON_MDT-Core

- R2-2007385	Avoiding Duplication of Location Info upon SCG Failure	Samsung	discussion	NR_SON_MDT-Core

- R2-2007386	How to Determine Whether a Cell is Part of the Area Indicated by AreaConfiguration	Samsung	discussion	NR_SON_MDT-Core

- R2-2007510	Correction on RA-Report release	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	1853	-	F	NR_SON_MDT-Core

- R2-2007511	Correction to RLF cause determination for backhaul failure	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	1854	-	F	NR_SON_MDT-Core

- R2-2007657	On the need of SCell indication in the RA-report	Ericsson	discussion

- R2-2007668	On the inclusion of reconnectCellId	Ericsson	discussion

- R2-2007753	Discussion on user consent	Huawei, HiSilicon	discussion	Rel-16	NR_SON_MDT-Core

- R2-2007754	Editorial corrections on MDT and SON in NR	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1892	-	D	NR_SON_MDT-Core

- R2-2007755	Correction on MDT and SON configurations for MR-DC	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1893	-	F	NR_SON_MDT-Core

- R2-2007756	Correction on logged MDT	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1894	-	F	NR_SON_MDT-Core

- R2-2007757	Correction on the release of obtainCommonLocation	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1895	-	F	NR_SON_MDT-Core

- R2-2007758	Correction on Inter-RAT SON for 38.331	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1896	-	F	NR_SON_MDT-Core

- R2-2007759	Correction on Inter-RAT SON for 36.331	Huawei, HiSilicon	CR	Rel-16	36.331	16.1.1	4408	-	F	NR_SON_MDT-Core

- R2-2007760	Correction on clearing VarRLF-Report regarding T316	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1897	-	F	NR_SON_MDT-Core

- R2-2007761	Summary on 6.10.3 RRC Corrections	Huawei	discussion	Rel-16	NR_SON_MDT-Core	Late

- R2-2007776	Correction on GNSS location information reporting in MDT	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.1.0	1900	-	F	NR_SON_MDT-Core

- R2-2007777	Correction to 38331 on logged MDT	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.1.0	1901	-	F	NR_SON_MDT-Core

- R2-2007778	Correction to 38331 on RA report	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.1.0	1902	-	F	NR_SON_MDT-Core

- R2-2007779	Inclusion of UL carrier indication in CEF report	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.1.0	1903	-	B	NR_SON_MDT-Core

- R2-2007954	 Correction to RLF content setting in VarRLF-Report	Quectel	draftCR	Rel-16	38.331	16.1.0	D	NR_SON_MDT-Core

- R2-2008000	Correction on the field description of CGI-InfoEUTRALogging	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.1.0	1962	-	F	NR_SON_MDT-Core

- R2-2008001	Correction to measResultNeighCells for logged measurements	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.1.0	1963	-	F	NR_SON_MDT-Core

- R2-2008002	Correction to the upper limit of PLMN identities in VarRLF-Report	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.1.0	1964	-	F	NR_SON_MDT-Core

- R2-2008003	Corrections to measResultLastServCell for RLF and HO failure	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.1.0	1965	-	F	NR_SON_MDT-Core

- R2-2008004	Corrections to mobility history information	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.1.0	1966	-	F	NR_SON_MDT-Core

- R2-2008005	Miscellaneous corrections related to connection establishment/resume failure information	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.1.0	1967	-	F	NR_SON_MDT-Core

- R2-2008006	Miscellaneous corrections related to logged measurements	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.1.0	1968	-	F	NR_SON_MDT-Core