3GPP TSG RAN WG2 #112-e	draft R2-20xxxxx

Online, 02 – 13 Nov 2020

Source: 	Huawei, HiSilicon

Title: 	[draft] Summary for Offline [008][NR15] inter-node RRC (Huawei)

Agenda Item:	5.4.1.4

Document for:	Discussion and decision

# Introduction

This document contains a list of TDocs covered in the following offline discussion:

- [AT112-e][008][NR15] inter-node RRC (Huawei)

Treat R2-2008727, R2-2010542, R2-2009242, R2-2009243, R2-2010357, R2-2010976, R2-2009159, R2-2009160, R2-2009161, R2-2010359, R2-2010360, R2-2009257, R2-2009258.

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Discussion stop at Wed Nov 11, 1200 UTC

Companies are invited to share their views on each TDoc submitted.

# Discussion

## Band Selection (R2-2008727, R2-2010542, R2-2009242, R2-2009243, R2-2010357)

Contents of the RAN3 LS (R2-2008727):

1. Overall Description: RAN3 has discussed the band selection scenario as described below: There is a cell with multiple overlapping bands, e.g., two, n2 and n25. Also, let’s assume there are UEs that can access both bands. A UE selects the first band in the frequencyBandList (broadcast in SIB1) that it supports, according to supportedBandListNR reported in UE-NR-Capability. It may be needed to select for this UE another band in the current serving cell during RRC connection setup phase or after RRC connection is established. If this happens, in a split gNB the gNB-DU needs to inform the gNB-CU of this change (note that the gNB-CU is not assumed to decode the CellGroupConfig IE), which may be used to update the measurement configuration for the UE. RAN3 has discussed whether the new band selected for the UE can be indicated over F1 using the Selected BandCombinationIndex as defined in TS 38.331. Regardless of the split gNB case, RAN3 kindly requests RAN2 to confirm that it is possible for the network to change the band on which the UE is served in the scenario above. RAN3 would also like to ask whether the Selected BandCombinationIndex, as defined in TS38.331, can be used to indicate the new single band selected in the scenario above. 2. Actions: To RAN2 group: ACTION: RAN3 kindly asks RAN2 to address the questions in this LS.

RAN2 needs to confirm whether it is possible for the network to change the band selected by the UE during RRC connection setup phase or after RRC connection is established, and whether the Selected BandCombinationIndex can be used to indicate the new single band selected.

Q1.1: Do you think it is possible for the network to change the band selected by the UE a) during RRC connection setup phase, and b) after RRC connection is established?

| Company | During RRC connection setup (YES/NO) | After RRC connection is established (YES/NO) | Comments |
| --- | --- | --- | --- |
| Huawei | No | Yes | During setup procedure, the UE capability has not been transferred to the network, therefore the network cannot deduce which band is selected by the UE and unable to change it. After RRC connection is established, the network is able to fetch UE capability, deduce the band selected by the UE, and change the band on which the UE is served through: RRCReconfiguration → CellGroupConfig → SpCellConfig → ReconfigurationWithSync → ServingCellConfigCommon → DownlinkConfigCommon → FrequencyInfoDL → MultiFrequencyBandListNR. |
| Qualcomm Incorporated | No | Yes | This requires Reconfiguration with Sync, which requires AS security to have been activated. |

With regard to CU-DU interaction, RAN3 discussed whether the Selected BandCombinationIndex in DU to CU RRC Information can be reused to indicate the new band. Since the band is determined by DU, DU needs to notify CU so that CU can generate the RRC signalling and send it to the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| […] |  |  |  |  |  |  |
| Selected BandCombinationIndex | O |  | OCTET STRING | BandCombinationIndex, as defined in TS 38.331 [8]. For (NG)EN-DC and NR DC operation, this IE should be included so that gNB-CU is informed of the selected Band Combination. | YES | ignore |
| […] |  |  |  |  |  |  |

Q1.2: Do you think RAN3 can reuse the Selected BandCombinationIndex to indicate the new single band selected by DU?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | No | The band indication for the new band selected by the network should be based on the frequency band list in SIB1, not based on the band combinations in UE capability. Since they are different lists, it is preferred not to reuse Selected BandCombinationIndex. Moreover, since RAN2 supports fallback band combinations, not all band combinations will be explicitly included in the UE capability. We noticed that there is CellGroupConfig in DU to CU RRC Information, which can include the Uu signalling of indicating the new band. Therefore this IE can be reused. |
| Qualcomm Incorporated | No | The UE may not explicitly signal a single-band band combination in the band combination UE capability. So Selected BandCombinationIndex cannot unambiguously indicate a specific band. |

Q1.3: Do you agree to send an LS to RAN3?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | Yes | The details can be discussed after consensus is reached for the above two questions. |
| Qualcomm Incorporated | Yes |  |

## R2-2010976	Intra-band EN-DC deployment issue

Proposal 1 : RAN2 to clarify in the description of the scellFrequenciesSN-EUTRA and scellFrequenciesSN-NR what the "frequency" means (i.e. carrier center frequency or the SSB frequency).

Q2.1: Companies are invited to provide views on the above proposal.

| Company | Carrier centre frequency/ SSB frequency | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | SSB frequency for NR / Center frequency for EUTRA | At least the combination of a serving frequency and measuredFrequency should be used by MN to identify the type of measurements configured by SN, i.e. either intra-frequency or inter-frequency. This is mentioned in section 7.2 of 37.340. There could be other intended purposes. |

Proposal 2: RAN2 to discuss how to exchange PSCell/Scell(s) carrier center frequency and channel bandwidth to ensure UE capability is respected in intra-band EN-DC deployments.

Q2.2: Companies are invited to provide views on the above proposal.

| Company | Comments |
| --- | --- |
| Qualcomm Incorporated | The identified problem looks real. NR uses somewhat complicated way of expressing the exact location of channel bandwidth, like point A, SCS specifics and so on. Something similar to FrequencyInfoDL/UL may need to be added. |

## R2-2009159, R2-2009160, R2-2009161	Clarification to usage of MN and SN configuration restrictions

Proposal 1: Capture in Rel-15 RRC that ConfigRestrictInfoSCG may be used in both MN- and SN-initiated procedures, but ConfigRestrictModReqSCG can only be used in SN-initiated procedures.

Q3.1: Do you agree with the above proposal in R2-2009159?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | No | If ConfigRestrictInfoSCG is used in SN-initiated procedures, the consequences could be: if SN cannot accept the configuration from MN, it cannot inform MN through the existing procedure, and adding new coordination mechanism increases the complexity. |
| Qualcomm Incorporated | Yes |  |

Q3.2: Do you agree with the changes in R2-2009160 and R2-2009161?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | No | Same comment as Q3.1. |
| Qualcomm Incorporated | Yes |  |

## R2-2010359, R2-2010360	Clarification on scg-CellGroupConfigEUTRA

Q4: Do you agree with the changes?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | Yes |  |
| Qualcomm Incorporated | Yes |  |

## R2-2009257, R2-2009258	Correction to RRC resume and re-establishment

Q5: Do you agree with the changes?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | Yes | Reasonable changes. |
| Qualcomm Incorporated | Yes |  |

# Conclusion

To be added.