3GPP TSG-RAN WG2 Meeting #112 electronic							R2-2009900

Online, November 2 – 13, 2020

Agenda Item:	8.5.3

Source: 	SONY

Title:	Considerations in unlicensed URLLC

Document for:	Discussion and Decision

# Introduction

In the last meeting, RAN2 had initial discussion for the aspects related to URLLC in unlicensed controlled environments, but no agreements were reached. However, some progress had been made in RAN1 and some of the agreements are copied here:

Agreements:

For semi-static channel access mode,

o    Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT.

o    FFS: FFP Periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT.

Agreements:

- For semi-static channel access mode,

- FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling.

  - FFS on to be provided by SIB-1

- FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

Agreements:

- At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

In this contribution, we discuss some aspects relating to semi-static channel access and CG-PUSCH harmonization.

# Semi-Static Channel Access

RAN1 agreed that the start of the Fixed Frame Period (FFP) in semi-static channel access (SCA), i.e. the FFP offsets for the UE and the gNB, can be different and therefore are independently configured. It is FFS whether the period, i.e. the duration of the FFP for the UE can also be different.  If the FFP duration between the UE and the gNB cannot be different, then long FFP duration may lead to high latency for UE UL transmissions. For example in Figure 1, the FFP duration of the gNB and UE are the same but with different starting FFP offset. If UL data arrives at time t2, after the start of the UE FFP, then the UE cannot transmit it straight away even if there are CG-PUSCH resources, since the UE needs to perform an LBT and the next CCA phase is at time t3.  Hence there is a delay between time t4 and t2 which may not be acceptable for URLLC transmissions.

Figure 1: Delay in UL transmissions due to UE’s long FFP duration

Observation 1: If the FFP duration between the gNB and UE cannot be different, the UE may face delay for its UL transmission.

If the FFP durations for gNB and UE can be different, then UE with URLLC traffic can be configured with shorter FFP duration, thereby providing more opportunities for the UE to initiate a COT. For example in Figure 2, the UE has a shorter FFP duration than that for the gNB and once again, UL data arrives in the middle of the UE’s FFP at time t2.  Comparing to the example in Figure 1, since the UE has a shorter FFP duration, it is able to access the channel earlier, thereby reducing the delay in transmitting its UL data. Hence, it is beneficial that the FFP duration (period) for the gNB and UE can also be different.

Figure 2: UE's FFP duration smaller than gNB's FFP duration

Proposal 1: The FFP duration (period) for the gNB and UE can be configured to be different.

Since different UEs may have different traffic, e.g. eMBB vs URLLC, it is beneficial that they can have different FFP configurations for the starting offset and duration, so that the gNB can configure a shorter FFP duration for a UE with URLLC traffic to provide more transmission opportunities and longer FFP duration for a UE with eMBB traffic.  Similarly, a UE can support multiple traffic types, e.g. eMBB and URLLC, and so it is also beneficial that a UE can be configured with multiple FFP configurations.

Proposal 2: The starting FFP offset and FFP duration are independently configured for each UE.

In RAN1, it is FFS whether to support UE initiated COT for semi-static channel access in Idle Mode. If a UE cannot initiate a COT in Idle Mode, then the UE can only perform a PRACH as a responding device, that is, the gNB is required to transmit in an FFP, e.g. a SIB, in order for the UE to perform a PRACH.  Such transmissions may introduce interference and may also deprive a UE in Connected Mode from initiating a COT, e.g. if the UE is performing LBT whilst the gNB is transmitting the SIB. Hence, it is beneficial that UE initiated COT for semi-static channel access is also supported in Idle Mode.

Proposal 3: UE initiated COT for semi-static channel access is supported in Idle Mode.

# CG-PUSCH Harmonisation

For CG harmonisation, RAN1 concluded that Table 1 in R1-2005376 [2] can be a starting point for discussion. However, we further studied and expanded the table here to include our views for Rel-17 [3].

Table 1: Summary for CG-PUSCH Harmonisation

| CG features | Rel-16 URLLC | Rel-16 NR-U | Rel-17 unlicensed URLLC |
| --- | --- | --- | --- |
| Multiple CG configurations | Supported | Supported | Already supported |
| HARQ process number/ ID determination | Associated with the configured/indicated first TO, calculated based on the equation defined in TS 38.321 | Decide and reported by the UE in CG-UCI | Use NR-U’s scheme |
| Management of HARQ process number/ ID among multiple CG configurations | Not shared between different CG configurations in the same BWP | Can be shared between different CG configurations in the same BWP | Use NR-U's scheme |
| RV determination | One of the three RV sequence can be configured and associated with TO {0,0,0,0}; {0,3,0,3}; {0,2,3,1} | Decide and reported by the UE in CG-UCI | Use NR-U’s scheme |
| Flexible initial transmission occasion (TO) | If the CG is configured with Configuredgrantconfig-StartingfromRV0 set to 'off', the initial transmission only starts at the first TO of the K repetitions; otherwise, the initial transmission TO depends on the configured RV sequence and K repetitions. | Multiple consecutive potential TOs are configured by cg-nrofPUSCH-InSlot-r16 and cg-nrofSlots-r16, can start initial transmission at any TOs depending on the LBT results. | Multiple consecutive potential TOs should be configured by cg-nrofPUSCH-InSlot-r16 and cg-nrofSlots-r16 Support of cross slot TO configuration should be further considered. |
| Repetition scheme(s) | PUSCH repetition Type A and PUSCH repetition Type B | Similar as PUSCH repetition Type B without supporting segmentation. (no support of cross-slot resource allocation, and if collide with invalid symbol(s), drop the repetition) | Support PUSCH segmentation |
| CG-Downlink feedback information (DFI) | No support. If Re-scheduling UL grant is not received, UE assumes ACK. | Support, If CG-DFI is not received, UE assumes NACK. | Should be supported |
| CG Re-transmission timer | No support | Support and always configured | Should support, but configurable |
| CG Re-transmission scheme | Only support Re-transmission scheduled by UL grant | Support automatic re-Transmission on the same or different CG configuration decided by UE, and support re-Transmission scheduled by UL grant | Use NR-U’s scheme. |
| UE behaviour when SFI is not detected | CG-PUSCH cannot be transmitted on semi-static ‘F’ symbols if SFI is not detected, otherwise it follows the SFI indication. | If enableConfiguredUL is enabled by RRC then CG-PUSCH can be transmitted on semi-static ‘F’ symbols if SFI is detected and ‘F’ is indicated as UL, and transmitted on semi-static ‘F’ symbols if SFI is not detected If enableConfiguredUL is disabled by RRC then CG-PUSCH cannot be transmitted on semi-static ‘F’ symbols regardless whether SFI is detected | Support enableConfiguredUL. enableConfiguredUL should be enabled together with some other URLLC parameters (e.g. Type B repetition) |
| L1 priority | Supported | Not supported | Should be supported |

Proposal 4: for CG harmonisation, we propose the right column of Table 1 for CG harmonisations of unlicensed URLLC in Rel-17.

# Conclusion

In this contribution, we have discussed some aspects of unlicensed URLLC, and we have the following observation and proposals.

Observation 1: If the FFP duration between the gNB and UE cannot be different, the UE may face delay for its UL transmission.

Proposal 1: The FFP duration (period) for the gNB and UE can be configured to be different.

Proposal 2: The starting FFP offset and FFP duration are independently configured for each UE.

Proposal 3: UE initiated COT for semi-static channel access is supported in Idle Mode.

Proposal 4: for CG harmonisation, we propose the right column of Table 1 for CG harmonisations of unlicensed URLLC in Rel-17.

# References

[1] R1-2005571, “Enhancements for unlicensed band URLLC/IIoT,” Sony, RAN1#102e

[2] R1-2005376, “Enhancements for unlicensed band URLLC/IIoT,” Vivo, RAN1#102e

[3] R1-2008357, “Considerations in unlicensed URLLC,” Sony, RAN1#103e