3GPP TSG-RAN2 Meeting #112-e	R2-2009908

# Online, 17th Aug 2020 - 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | 0057 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on 38.323 for the PDCP duplication with more than two RLC entities for SRB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT-Core |  |  |  |  |  | Date: |  |  | 2020-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In RAN2#109 meeting and RAN2#109 bis meeting, we have achieved the following agreements: - => Rel-16 PDCP duplication is applied to SRBs. - => For SRBs, all secondary RLC entities are activated when configured. - => Confirm that duplication is always activated for all RLC entities for SRB (meaning e.g. that duplicationState has no meaning for SRB). - In both stage 2 and stage 3 specification, it is only defined that the PDCP duplication shall be activated for an SRB if more than two RLC entities are associated, but it is still not clear how many RLC entities shall be activated for PDCP duplication in the current spec. In our understanding ,it needs be clarified in spec in order to make UE behavior on PDCP duplication of SRB clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1: Add a clarification that if more than 2 RLC entities are configured for one SRB, the PDCP duplication for all associated RLC entities other than the primary RLC entties shall be activated. Impact analysis Impacted 5G architecture options: NR SA, NR-DC, NE-DC Impacted functionality: Multiple configured grant configurations Inter-operability: 1: If UE implement by following CR while NW does not, no inter-operability issue. 2: If NW implement by following CR while UE does not, UE may not send the enough copies to NW if the SRB is configured with more than 2 RLC enities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE maynot activate the PDCP duplication for all RLC entities for an SRB assocaited with more than two RLC entities. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------------------------  The change start -------------------

5.11.1	Activation/Deactivation of PDCP duplication

For the PDCP entity configured with pdcp-Duplication , the transmitting PDCP entity shall:

-	for SRBs:

-	activate the PDCP duplication for all associated RLC entities other than the primary RLC entity;

-	for DRBs:

-	if the activation of PDCP duplication is indicated for the DRB:

-	activate the PDCP duplication for the DRB;

-	if the activation of PDCP duplication is indicated for at least one associated RLC entities:

-	activate the PDCP duplication for the indicated associated RLC entities;

-	activate the PDCP duplication for the DRB;

-	if the deactivation of PDCP duplication is indicated for the DRB:

-	deactivate the PDCP duplication for the DRB;

-	if the deactivation of PDCP duplication is indicated for at least one associated RLC entities:

-	deactivate the PDCP duplication for the indicated associated RLC entities;

-	if all associated RLC entities other than the primary RLC entity are deactivated for PDCP duplication:

-	deactivate the PDCP duplication for the DRB.

---------------------------------------  The change end --------------------