3GPP TSG-RAN WG2 Meeting #113 electronic                                                                 R2-2100244

Online, Jan 25 – Feb 5, 2021

Source:	OPPO

Title:	Paging Collision Avoidance

Agenda Item:	8.3.2

Document for:	Discussion and Decision

- Introduction

In RAN#90e meeting, a revised WID scope of Rel-17 Multi-SIM WI was agreed [1]:

- Specify, if necessary, enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs [RAN2]

  - RAT Concurrency: Network A can be NR or LTE. Network B can either be LTE or NR.

  - Applicable UE architecture: Single-Rx/Single-Tx.

- Specify mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose) [RAN2]:

  - RAT Concurrency: Network A is NR. Network B can either be LTE or NR.

  - Applicable UE architecture: Single-Rx/Single-Tx, Dual-Rx/Single-Tx

- Unless SA2 find an alternative solution or decides otherwise, specify mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR[ RAN2].

  - RAT Concurrency: Network A is either LTE or NR. Network B is either LTE or NR.

  - Applicable UE architecture: Single-Rx/Dual-Rx/Single-Tx

UE SIMs may belong to same or different operators.

USIM can be a physical SIM or eSIM.

Coordination with relevant WGs, such as SA2, should be considered where relevant.

For objective 1, specification change should focus on NR side and the change on LTE side is only for IDLE mode (i.e. related to EPC enhancement in SA2)

And the following agreements were made in RAN2#112e meeting for objective 1 [2]:

From RAN2 point of view, Option 1, 2a, 2b, and 3 are feasible to solve the paging collision issue in 5GS. Each have different effectiveness (as per analysis during the email discussion). When indicating reply to SA2, indicate both feasibility as well as effectiveness.

Indicate to SA2 that RAN2 continues to further evaluate the pros and cons of options 1, 2a, 2b, 3.

Option 4 is still allowed (but RAN2 will not specify UE implementation).

Clarifying "No E-UTRA impact" can be done in RANP.

Option 2c can be evaluated later as it doesn't work alone.

Enhancement for 5GS should be prioritized since it can handle paging collision issue in both NR+NR and NR+LTE scenarios.

In this contribution, we will focus on objective 1, i.e. paging collision avoidance.

- Discussion

Based on RAN2 agreements above, the following alternatives can be further discussed in RAN2:

Option 1: UE -requested 5G-GUTI reassignment for one USIM using the Mobility Registration Update. However, it should be noted the 5G-GUTI is systematically reassigned by the network during the Mobility Registration Update procedure (as of Rel-15) requires. Proposed for 5GS only.

-    Option 2: Changes related to the UE_ID (UE Identity Index) that is used for calculation of PF/PO only:

-    Option 2a    Calculation of PF/PO by using an Alternative UE_ID I. The UE ID sent in the paging message is not impacted by this Alternative ID that is only used for PO/PF calculations Proposed for both EPS and 5GS.

-    Option 2b   Calculation of PF/PO by using a UE_ID which is derived from IMSI+offset value. The offset value is negotiated between UE and MME. Proposed for EPS only.

-    Option 3 Repeating paging in the RAN on consecutive POs. for MUSIM devices.

To make the choice, RAN2 should evaluate the pros and cons for the above options, please see our analysis in the table below:

Table 1 Pros and Cons for solutions on paging collision avoidance

|  | NAS Impact | AS Impact | Effective |
| --- | --- | --- | --- |
| Option1 | Yes | No | This issue can be solved usually if an assistant info is added into 5G-GUTI reassignment request |
| Option2a | Yes | Yes | This issue can be solved usually |
| Option2b | Yes | Yes | This issue can be solved usually |
| Option3 | Maybe | Yes | This issue can be solved usually |

According to the analysis in table 1, Option1 is simple and effective if an assistant info is added into 5G-GUTI reassignment request. Option2a/2b is too complex as both NAS and AS impact are involved, so we think it’s not worthy to adopt this option to solve this low possibility issue. For Option3, this solution is friendly for UE but wastes the recourses in Uu interface. In our view, Option1 and Option3 can be the way forward in RAN2.

Proposal 1: For paging collision avoidance, RAN2 only consider the following solutions:

- Option1

- Option3

- UE implementation

- Conclusion

In conclusion, we propose the followings:

Proposal 1: For paging collision avoidance, RAN2 only consider the following solutions:

- Option1

- Option3

- UE implementation

- References

[1] 3GPP RP-202895 Revised WID of Rel-17 Multi-SIM WI

[2] 3GPP RAN2#112e Chairman Notes