3GPP TSG-RAN WG2 Meeting #112-e                                                    R2-2100336

Electronic, 25 January – 5 February 2021‎‎

Source:	CATT

Title:	Consideration on measurement for NTN system

Agenda Item:	8.10.3.2

Document for:	Discussion and Decision

Introduction

Measurement is one of the key topics that should be addressed in NTN WI. If the SMTC measurement gap configuration doesn’t consider the propagation delay difference, UE will miss the SSB in measurement gap. The initial discussion for SMTC and measurement gap configuration was triggered in email discussion [Offline-106][NTN][1]. Threshold based measurement initiation triggering condition in TN system is not workable for NTN system. RAN2 should further discuss the new measurement initiation triggering condition.

In this contribution, we will further discuss about the new measurement initiation triggering condition and SMTC measurement gap configuration.

Discussion

SMTC and gap configuration

In TN system, the typical cell radius is several hundred of meters, the propagation delay difference between serving and neighbor cell(less than 1 us) is usually much less than length of SMTC (at least 1ms), the impact caused by propagation delay difference between serving and neighbor cell is quite small, the risk for UE to miss the SSB of neighbor cell is quite small.

But for NTN system, the situation is totally different. As mentioned before, the propagation delay difference between serving and neighbor satellites may be quite large, e.g. about 0~serval hundreds of kilometers for LEO and 0~serval thousands of kilometers for GEO, which equates to 0~serval ms similar with the length of SMTC window, sometimes even larger. Due to suffer from different delay between SMTC window configured via serving satellite and SSB burst of neighbor satellite, a big timing gap will be perceived from UE side.

- Due to suffer from different delay between SMTC window configured via serving satellite and SSB burst of neighbor satellite, a big timing gap will be perceived from UE side.

The situation on propagation delay difference in NTN system is quite different than that in TN system. To simplify the analysis, we illustrate the issue in Figure 1:

Figure 1 Two different links for transparent payloads scenario (C2) in NTN system

In Figure 1, G represents the location of NTN gateway and the location of satellites is marked by S1 and S2, while U represents the location of UE. Assume UE is in the coverage overlapping area between satellite S1 and satellite S2, the current serving satellite is S1. Due to the moving of satellite, UE should do measurement of the neighbour cell to control its mobility, e.g. cell reselection or HO. LUS1G represents the length of service link 1 which consists of user link LUS1 and feeder link LS1G, while LUS2G represents the length of service link 2 which consists of user link LUS2 and feeder link LS2G.

The length difference between LUS1G and LUS2G may be quite huge, e.g. about 0~serval hundreds of kilometres for LEO and 0~serval thousands of kilometres for GEO.

More addition, the propagation delay difference between different path is changing along with the moving of satellites. As time goes by, the configured measurement window may miss the actual SSB window of gNB2 as the actual SSB window has moved away. But the maximal propagation delay difference can be calculated by the gateway based on satellite ephemeris and the minimal elevation angle of service link.

Figure 3 Timing from gNB1 side and UE side

The network should compensate UE measurement window itself considering propagation delay difference between satellites, e.g. the impact caused by propagation delay difference between satellites is invisible to UE.

Extend the length of the measurement gap based on the max propagation delay difference between serving and neighbour satellite to avoid UE missing the SSB burst of neighbour satellites.

- Extend the length of the measurement gap based on the max propagation delay difference between serving and neighbor satellite to avoid UE missing the SSB burst of neighbour satellites.

Measurement report triggering condition

In TN system, the measurement events are based on RSRP or RSRQ. UE is configured threshold parameters by the network in system information triggering the measurement report.

But in NTN system, the situation is not same. The near-far effect is quite different between NTN and TN system. To simplify the analysis, the issue is illustrated in Figure 1:

Figure 3 A sketch of near-far effect in different scenarios: (a) Terrestrial Network; (b) NTN

In TN system, UEs can determine the edge and center of a cell by the RSRP or RSRQ due to the near- far effect.  However the near-far effect is not pronounced in the NTN system as Figure 3 shows. The RSRP/RSRQ value difference between satellite cell center and satellite cell edge is typically less than 3dB, while the RSRP/RSRQ value difference between TN cell center and TN cell edge is typically more than 15dB. Thus, existing measurement mechanism is not suitable for NTN system.

- Existing measurement mechanism is not suitable for NTN system.

The handover or cell re-selection is usually due to the satellite moving. RAN2 should discuss the Location-based measurement report triggering condition. UE location info and satellite Ephemeris info can be used to trigger intra-frequency or inter-frequency measurements in NTN system.

- UE location info and satellite Ephemeris info can be used to trigger intra-frequency or inter-frequency measurements in NTN system.

Conclusion

In section 2, we made the following observations:

- Due to suffer from different delay between SMTC window configured via serving satellite and SSB burst of neighbor satellite, a big timing gap will be perceived from UE side.

- Existing measurement mechanism is not suitable for NTN system.

Based on the discussion in the section 2, we propose the following:

- Extend the length of the measurement gap based on the max propagation delay difference between serving and neighbor satellite to avoid UE missing the SSB burst of neighbour satellites.

- UE location info and satellite Ephemeris info can be used to trigger intra-frequency or inter-frequency measurements in NTN system.

Reference

- R2-2010768  [Post112-e] [106] [NTN] SMTC and Gaps (CATT)

- R2-2008834	 Open Issues for Measurements in NTN  CATT  discussion Rel-17 NR_NTN_solutions-Core