3GPP TSG RAN WG2 Meeting #113e                                                        	   R2-2100364

E-Meeting, 25th January – 5th February, 2021

Agenda Item:	8.16.4

Source: 	Intel Corporation

Title:	RAN2 impact on support of IMS emergency call for SNPN

Document for: 	Discussion/Decision

# Introduction

The WID for eNPN [1] was agreed with the following objective:

- Support SNPN along with subscription / credentials owned by an entity separate from the SNPN including:

  - The broadcasting of information to enable SNPN selection for UEs with subscription/credentials owned by an entity separate from the SNPN [RAN2]

  - The associated cell selection/reselection and connected mode mobility support [RAN2/RAN3]

  - The necessary modifications over network interfaces (e.g. NG, Xn, F1, E1 etc) [RAN3]

- Support UE onboarding and provisioning for NPN including:

  - The UE onboarding relevant parameter broadcast from SI B[RAN2]

  - The associated cell selection/reselection, cell access control and the connected mode mobility support [RAN2/RAN3]

  - The necessary modifications over network interfaces (e.g. NG, Xn, F1, E1 etc) [RAN3]

- Support of IMS voice and emergency services for SNPN [RAN2]

  - Broadcasting of relevant parameters [RAN2]

This objective was set according to the SA2 TR [2], where the impact is provided with more details.

In this contribution, we analysed the RAN2 impact for the emergency call support for SNPN and proposed the changes needed from the RAN2 side and areas of study/discussion.

# Discussion

The main impact to RAN2 is the support of the emergency services for UEs in limited service state (i.e. for unauthenticated UE where UE has no USIM).  For this, the Solution#23 in the TR [2] includes this impact:

NG-RAN of SNPN

-	Include related broadcast indicator that the cell supports Emergency Services over NG-RAN for UEs in limited service state, and if the NG-RAN is shared by more than one network, and the networks do not have the same support for Emergency Services, the broadcast indicator is related to those networks that supports Emergency Services.

In the TS38.331, the existing ims-EmergencySupport for PLMN UEs in limited service state is signaled over per cell basis:

SIB1 ::=        SEQUENCE {

cellSelectionInfo                   SEQUENCE {

q-RxLevMin                          Q-RxLevMin,

q-RxLevMinOffset                    INTEGER (1..8)                      OPTIONAL,   -- Need S

q-RxLevMinSUL                       Q-RxLevMin                          OPTIONAL,   -- Need R

q-QualMin                           Q-QualMin                           OPTIONAL,   -- Need S

q-QualMinOffset                     INTEGER (1..8)                      OPTIONAL    -- Need S

}                                                                 OPTIONAL,   -- Cond Standalone

cellAccessRelatedInfo               CellAccessRelatedInfo,

connEstFailureControl               ConnEstFailureControl                    OPTIONAL,   -- Need R

si-SchedulingInfo                   SI-SchedulingInfo                        OPTIONAL,   -- Need R

servingCellConfigCommon             ServingCellConfigCommonSIB               OPTIONAL,   -- Need R

ims-EmergencySupport                ENUMERATED {true}                        OPTIONAL,   -- Need R

eCallOverIMS-Support                ENUMERATED {true}                        OPTIONAL,   -- Need R

ue-TimersAndConstants               UE-TimersAndConstants                    OPTIONAL,   -- Need R

uac-BarringInfo                     SEQUENCE {

uac-BarringForCommon                UAC-BarringPerCatList                OPTIONAL,   -- Need S

uac-BarringPerPLMN-List             UAC-BarringPerPLMN-List              OPTIONAL,   -- Need S

uac-BarringInfoSetList              UAC-BarringInfoSetList,

uac-AccessCategory1-SelectionAssistanceInfo CHOICE {

plmnCommon                           UAC-AccessCategory1-SelectionAssistanceInfo,

individualPLMNList                   SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AccessCategory1-SelectionAssistanceInfo

}                                                                        OPTIONAL    -- Need S

}                                                                            OPTIONAL,   -- Need R

useFullResumeID                     ENUMERATED {true}                        OPTIONAL,   -- Need R

lateNonCriticalExtension            OCTET STRING                             OPTIONAL,

nonCriticalExtension                SIB1-v1610-IEs                           OPTIONAL

}

For UE initiating IMS emergency call in limited service state in the PLMN case, the UE just needs to initiate the connection if the network broadcast that it supports emergency call for limited service state and the RAN will select the AMF that support IMS emergency call.

Observation#0: Current specifications support a single bit in cell to indicate IMS emergency call support as the PLMN supporting emergency call is not relevant to a limited service UE.

Based on the SA2 TR, it seems to imply the emergency call support is per SNPN as it is possible different SNPNs sharing the same gNB may not have the same support for Emergency Services. However, similar to the current IMS emergency call support indication for limited service state, , there is no need for separate emergency support per SNPN broadcast over the SIB.

Observation#1: Even though the SA2 TR indicate that the emergency call support is per SNPN, such indication is only needed per cell since gNB will select the AMF that support emergency support.

Another question is whether a new per cell indication is needed for SNPN or the existing ims-EmergencySupport for PLMN UEs can be extended for SNPN. For this, a new per cell indication is needed for indicating emergency call support for SNPN to ensure backward compatibility since Rel-15 gNB that supports sharing between SNPN and PLMN networks may support emergency support for PLMN network but not for SNPN.  A Rel-17 UE supporting only SNPN in limited service state will only initiate IMS emergency call in a cell indicating SNPN IMS emergency call.

Observation#2: New per cell indication is needed for indicating emergency support for SNPN in limited service state to ensure backward compatibility.

It is natural that such indication should be in SIB1

Proposal#1: New per cell RRC signaling is needed in SIB1 for indicating support of IMS emergency support for SNPN.  A Rel-17 UE supporting only SNPN in limited service state will only initiate IMS emergency call in a cell indicating SNPN IMS emergency call

If Proposal#1 is agreed, there may be a need to identify that this is an SNPN only UE that is making the IMS emergency call and an indication may be needed in the RRC Setup Complete message.

# Conclusion

It is requested that RAN2 discussed the following observations and proposals:

Observation#0: Current specifications support a single bit in cell to indicate IMS emergency call support as the PLMN supporting emergency call is not relevant to a limited service UE.

Observation#1: Even though the SA2 TR indicate that the emergency call support is per SNPN, such indication is only needed per cell since gNB will select the AMF that support emergency support

Observation#2: New per cell indication is needed for indicating emergency support for SNPN to ensure backward compatibility.

Proposal#1: New per cell RRC signaling is needed in SIB1 for indicating support of IMS emergency support for SNPN. A Rel-17 UE supporting only SNPN in limited service state will only initiate IMS emergency call in a cell indicating SNPN IMS emergency call

# References

[1] RP-202363 Revised WID: Enhancement of Private Network Support for NG-RAN

[2] TR23.700-07 v1.2.0 Study on enhanced support of non-public networks