3GPP TSG-RAN WG2 Meeting #113                                                     	 R2- 2100379

E-Meeting, USA, Jan 25th – Feb 5th, 2021

Agenda item: 	8.10.2.1

Source:	Intel Corporation

Title: 	Pre-compensation for NTN

Document for:	Discussion and Decision

# Introduction

During RAN2 112 e-meeting, it has been agreed the following:

Agreements:

- RAN2 working assumption (for RRC idle. FFS for Inactive/Connected): Rel-17 UE with pre-compensation capability obtains UE specific UE-gNB RTT based on its GNSS in LEO/GEO. FFS how this is calculated and what/if anything needs to be broadcasted for the different pre-compensation methods (e.g. common TA) to help the UE to obtain the full UE-gNB RTT.

- If the UE-gNB RTT is pre-compensated, preamble ambiguity is not an issue in Rel-17 NTN (i.e. no enhancements are necessary). FFS how and by whom the possibly multiple components of UE-gNB RTT are pre-compensated

- From RAN2 perspective, for UE with UE-specific pre-compensation as a baseline it is up to gNB implementation to ensure sufficient time on UE side for the Msg3 transmission.

This contribution focuses discussion on the detail of UE pre-compensation in NTN.

# Discussion

One of the primary differences in NTNs compared to terrestrial cellular networks is the significantly longer propagation delay between the UE (on the ground) and the satellite. Propagation delays can result in the uplink signal from different UEs being received at the gNB at very different times. In order to be able to receive multiple uplink signals, the gNB assigns timing advances to UEs ensure that the receive times of signals from UEs are the same. The timing advance value is communicated to the UE in the random access procedure, in the random access response in legacy.

Due to long propagation delay in NTN, it has been discussed extensively and agreed during last meeting that UE with pre-compensation capability obtains UE specific RTT (between UE and gNB) based on GNSS information. And it is FFS on how the pre-compensation is calculated.

In general, the total delay is consisting of two parts:

- Part 1: Between UE and satellite (service link delay)

- Part 2: Between satellite and gNB (feeder link delay)

The service link delay should be obtained by the UE based on GNSS information. RAN1 also has related discussion and the potential solutions are as follow:

- Option 1: The User specific TA is estimated by the UE based on its GNSS acquired position together with the serving satellite ephemeris indicated by the network:

  - FFS: Details on serving satellite ephemeris indication

- Option 2: The User specific TA is estimated by the UE based on the GNSS acquired reference time at UE together with reference time as indicated by the network

Although both options will work to compensate the service link delay, we think that option 1 may be a simpler option considering mobility. Both satellite and UE may be in mobility. In option 1, the network will broadcast the satellite ephemeris information and UE will calculate the distance between the satellite using the GNSS information. The distance can then map to the RTT for the service link delay compensation. If UE or the satellite are in mobility, the information can be calculated based on updated UE location information or future information satellite broadcast. In option 2, UE will need to acquire the reference time when updating the pre-compensation calculation. Therefore, option 1 is preferred.

Proposal 1: UE pre-compensation for service link are calculated as follow:

- Network broadcast satellite ephemeris information (at least current information)

- UE uses the satellite ephemeris information and GNSS information to calculate the distance between UE and satellite

- UE uses the distance to calculate RTT of the service link delay

As for part 2, the feeder link delay, it can simply broadcast by the network since the value will be common for all UEs.

Proposal 2: Network broadcast the feeder link delay.

When considering mobility, it is possible for the network to broadcast multiple delay based on its movement. For instant, it can broadcast a current feeder link delay and the next N feeder link delay based on its mobility. That way, the UE only needs to acquire the broadcast after N times.

Proposal 3: RAN2 to discuss the network broadcast multiple feeder link delay based on its mobility.

# Conclusion

Proposal 1: UE pre-compensation for service link are calculated as follow:

- Network broadcast satellite ephemeris information (at least current information)

- UE uses the satellite ephemeris information and GNSS information to calculate the distance between UE and satellite

- UE uses the distance to calculate RTT of the service link delay

Proposal 2: Network broadcast the feeder link delay.

Proposal 3: RAN2 to discuss the network broadcast multiple feeder link delay based on its mobility.

# Reference

- “Solutions for NR to support non-terrestrial networks”, Thales, RP-201256

- “Solutions for NR to support non-terrestrial networks (NTN)”, TS38.821