3GPP TSG-RAN WG2 Meeting #113-e	R2-2100387

# Electronic meeting, January 25 – February 5, 2021

Agenda item: 	6.15

Source:		Intel Corporation

Title:	 DC location information reporting

Document for:	Discussion and Decision

# Introduction

In RAN #90e, the following proposal is endorsed regarding DC location reporting from RP-202893.

RAN to task RAN2 to complete the Rel-16 RRC based DC location reporting signalling for 2 UL CCs in RAN#91e, i.e. RAN2 should provide either agreed or, if agreement is not possible, technically endorsed CRs to RAN#91e, addressing the case of 2 UL CCs. It would be better if solutions also consider the signalling overhead and future proofness aspects.

In this document, we would like to discuss signaling details to report DC location.

# Detailed signalling

There could be two approaches depending on how future proofness requirement is interpreted and considered in the signalling design. One way is to define signalling only for 2CCs and the case more than 2CCs is introduced in non-critical extension in the later release. Although Rel-16 signaling doesn’t support the cases more than 2 CCs but there is no issue in future proofness. The other way is to define signalling structure to support all possible CA and BWP combinations. This approach is more future proof. However, even in this approach, it may not support all CA cases due to the practical limitation of the maximum number of pairs. As shown in the below table, it would be hard to support up to 32 CCs considering the required # of DC information entries is exponentially increase. The number of DC location information is roughly calculated as (M+1)^N where M is the number of BWPs (=8) and N is the number of carriers.

| # of CCs | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # of DC information | 81 | 729 | 6561 | 59049 | 531441 | 4782969 | 43046721 |

Observation 1: the future proofness can be supported by introducing the signalling more than 2 CCs in non-critical extension in the later release.

Observation 2: even if generic signalling were introduced to support more than 2CCs, it is not practical to define the maximum number of DC location information to support up 32 CCs.

We provide more detailed signalling structure as follows.

Approach 1: DC location information only for 2 CCs

The example ASN.1 structure is shown below. The size of IE is 64 to accommodate up to 8 BWPs for normal UL and SUL in both PCell and SCell respectively. We assume that no BWP in one serving cell case is covered by the existing signalling.

UplinkTxDirectCurrentCell-v16xy ::=           SEQUENCE {

uplinkDirectCurrentpercellBWP-pair-r16     SEQUENCE (SIZE (1..81)) OF UplinkTxDirectCurrentBWP-pair-v16xy

}

UplinkTxDirectCurrentBWP-pair-v16xy ::=            SEQUENCE {

bwp-Id1									 BWP-Id,	--BWP id of PCell

bwp-Id2									 BWP-Id,	--BWP id of SCell

sul-indication-PCell					 ENUMERATED {true}		OPTIONAL,

sul-indication-SCell					 ENUMERATED {true}		OPTIONAL,

shift7dot5kHz                           BOOLEAN,

txDirectCurrentLocation                 INTEGER (0..3301)

}

Approach 2: DC location information more than 2 CCs

In this approach 2, in each DC location information, the list of serving cell information and the BWP is included. The signalling structure itself is generic to support any number of serving cells.

UplinkTxDirectCurrentCell-v16xy ::=           SEQUENCE {

uplinkDirectCurrentperCA-BWP-combination-r16     SEQUENCE (SIZE (1..maxDirectCurrentperCA-BWP-combination)) OF UplinkTxDirectCurrentperCA-BWP-combination-v16xy

}

UplinkTxDirectCurrentperCA-BWP-combination -v16xy ::=            SEQUENCE {

ca-BWP-combination SEQUENCE (SIZE (1.. maxNrofServingCells)) OF CA-BWP,

shift7dot5kHz                           BOOLEAN,

txDirectCurrentLocation                 INTEGER (0..3301)

}

CA-BWP ::= SEQUENCE {

servingCellId            		                ServCellIndex,

bwp-Id                           		    	 BWP-Id,

sul-indication             		 ENUMERATED {true} OPTIONAL

}

There is no significant difference between approach 1 and approach 2. As discussed above, approach 2 is more general to support the large number of CCs. But, the max # of DC information entries (i.e. maxDirectCurrentperCA-BWP-combination) should limited to the practical number e.g. 81 or 729 considering the maximum number of CCs that Rel-16 really needs to support.

Proposal 1: Either Approach 1 or approach 2 can be introduced for DC information reporting.

- Approach 1: DC location information only for 2 CCs

- Approach 2: DC location information more than 2 CCs

Proposal 2: if approach 2 is agreed, the max # of DC information entries (i.e. maxDirectCurrentperCA-BWP-combination) should limited to the practical number e.g. 81 or 729.

# Relationship with existing DC reporting

There could be several ways to define the relationship with the existing DC location reporting.

- Approach 1: the UE reports Rel-16 DC location information only if gNB indicates to report Rel-16 DC location information. Otherwise, the UE reports only Rel-15 DC location information.

- Approach 2: the UE reports Rel-15 and Rel-16 DC location information when gNB requests to report DC location information. The legacy gNB will ignore Rel-16 DC location information although the UE reports it.

Both approaches are feasible. Approach 1 requires additional specification efforts for gNB request and UE reporting. However, considering the signalling overhead, we should avoid sending Rel-16 DC location information if it is not actually used.

Proposal 3: gNB request for Rel-16 DC location information is introduced.

# UE capability

According to the latest RAN4 feature list, DC location reporting is defined as optional capability with FRx differentiation.

| 7-5 | DC location for intra-band CA | indicate whether UE support Additional DC location reporting for intra-band UL CA |  | Yes | N/A | The gNB cannot correctly calculate the DC location of intra-band CA | 3 | No need | FR1 and FR2 |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: UE capability for DC location information should be introduced.

# Conclusion

In this document, we discussed signaling details to report DC location. Based on the discussion, we propose following proposals.

Observation 1: the future proofness can be supported by introducing the signalling more than 2 CCs in non-critical extension in the later release.

Observation 2: even if generic signalling were introduced to support more than 2CCs, it is not practical to define the maximum number of DC location information to support up 32 CCs.

Proposal 1: Either Approach 1 or approach 2 can be introduced for DC information reporting.

- Approach 1: DC location information only for 2 CCs

- Approach 2: DC location information more than 2 CCs

Proposal 2: if approach 2 is agreed, the max # of DC information entries (i.e. maxDirectCurrentperCA-BWP-combination) should limited to the practical number e.g. 81 or 729.

Proposal 3: gNB request for Rel-16 DC location information is introduced.

Proposal 4: UE capability for DC location information should be introduced.

# References

[1] R4-2011906	LS on additional DC location reporting for intra-band UL CA