3GPP TSG-RAN WG2 #113e	Tdoc R2-2100484

Electronic meeting, 2021-01-25 - 2021-02-05

Agenda Item:	6.16

Source:	Ericsson

Title:	Clarify the usage of voiceFallbackIndication for emergency service

Document for:	Discussion, Decision

# 1	Introduction

In the last meeting there was a discussion on the use of the voiceFallbackIndication during fallback from 5GS to EPS for emergency services.  However, the discussion did not fully conclude, and there was one issue that was postponed for next meeting.

R2-2009240	Clarify the usage of voiceFallbackIndication for emergency service	ZTE Corporation, Sanechips	discussion	Rel-16	TEI16

-	[029] Rapporteur: Continue to discuss P2 in R2-2009240 (whether to extend voiceFallbackIndication-r16 to "emergency service fallback" scenario).

[029] RAN2 confirms network is allowed to include voiceFallbackIndication-r16 in RRCRelease when triggers "EPS fallback for IMS voice" and QoS Flow establishment request for Emergency Services (No spec change is needed).

[029] Regarding how to support “first attempt E-UTRAN cell upon HO failure” in case of emergency service fallback, postpone the discussion to next meeting. Following options can be considered:

- Opt 1: leave it to UE implemetation;

- Opt 2: reuse voiceFallbackIndication-r16 sent by network (FFS on new capability).

[029] this topic is postponed (expected next meeting)

In this contribution we address the open issue above.

# 2	Discussion

### 2.1 Emergency call using EPS fallback for IMS voice

EPS fallback for IMS voice is used when IMS voice is not supported in 5GS and is achieved through either release with redirect or inter-system handover. The redirect or handover is triggered by the NG-RAN node when 5GC requests the establishment of a QoS flow for IMS voice.

As part of TEI-16, EPS fallback for IMS voice was enhanced and a voiceFallbackIndication was introduced in the release and handover message in NR to make the UE aware when EPS fallback occurs (see [3], [4]).

- For release with redirect the voiceFallbackIndication allows the UE to set the correct establishment cause in the subsequent RRC connection setup in EPS

- For handover the voiceFallbackIndication allows the UE to select an E-UTRA cell instead of an NR cell if the handover fails.

Since emergency calls are IMS voice calls, the EPS fallback may also be triggered when the UE initiates an emergency call. From TS 23.502:

### 4.13.4	Emergency Services

#### 4.13.4.1	General

If the 5GS supports Emergency Services, the support is indicated to UE via the Registration Accept message on per-TA-list and per-RAT basis, as described in TS 23.501 [2].

If the 5GS supports Emergency Services Fallback, the support is indicated to UE via the Registration Accept message on per-TA-list and per-RAT basis, as described in TS 23.501 [2].

The UE shall follow the domain selection rules for emergency session attempts as described in TS 23.167 [28].

If the 5GC has indicated Emergency Services Fallback support for the TA and RAT where the UE is currently camping, and if the UE supports emergency services fallback, the UE shall initiate the Emergency Services Fallback procedure described in clause 4.13.4.2.

At QoS Flow establishment request for Emergency Services, the procedure described in clause 4.13.6.2 Inter RAT Fallback in 5GC for IMS voice or the procedure described in clause 4.13.6.1 EPS fallback for IMS voice may be triggered by the network, when configured.

For redirect based EPS fallback triggered by an emergency call there is no need for the voiceFallbackIndication as the UE is aware that an emergency call is ongoing and therefore knows that it should set the establishment cause to “emergency”. For handover based EPS fallback the indication still serves a purpose though since it ensures that the UE selects an E-UTRA cell in case the handover fails. For this reason R2-2009240 [1] proposed to only include the voiceFallbackIndication in the handover based EPS fallback but not in the redirect based EPS fallback when the fallback is triggered by an emergency call. However, in the offline discussion [2] a majority of the companies considered this to be an unnecessary network restriction as the UE will set the establishment cause to “emergency” even if the voiceFallbackIndication is present in the redirect message. Hence it was agreed that:

[029] RAN2 confirms network is allowed to include voiceFallbackIndication-r16 in RRCRelease when triggers "EPS fallback for IMS voice" and QoS Flow establishment request for Emergency Services (No spec change is needed).

### 2.2 Emergency calls using emergency services fallback

Emergency services fallback is another way of handling emergency calls when IMS voice is not supported in 5GS. When emergency services fallback is used the 5GC indicates to the UE already during the attach procedure that emergency services is not supported in 5GS. When the UE wants to initiate an emergency call it sends an emergency services fallback request to 5GC which in turn notifies NG-RAN which will redirect or handover the UE to EPS. The main difference compared to EPS fallback for IMS voice is therefore that the UE is aware already from the beginning that emergency services are not supported in 5GS and that it will need move to EPS for emergency calls. From TS 23.502:

### 4.13.4	Emergency Services

#### 4.13.4.1	General

If the 5GS supports Emergency Services, the support is indicated to UE via the Registration Accept message on per-TA-list and per-RAT basis, as described in TS 23.501 [2].

If the 5GS supports Emergency Services Fallback, the support is indicated to UE via the Registration Accept message on per-TA-list and per-RAT basis, as described in TS 23.501 [2].

The UE shall follow the domain selection rules for emergency session attempts as described in TS 23.167 [28].

If the 5GC has indicated Emergency Services Fallback support for the TA and RAT where the UE is currently camping, and if the UE supports emergency services fallback, the UE shall initiate the Emergency Services Fallback procedure described in clause 4.13.4.2.

At QoS Flow establishment request for Emergency Services, the procedure described in clause 4.13.6.2 Inter RAT Fallback in 5GC for IMS voice or the procedure described in clause 4.13.6.1 EPS fallback for IMS voice may be triggered by the network, when configured.

In the offline discussion for R2-2009240 it was also discussed whether the voiceFallbackIndication should be included in the redirect or handover message also for emergency services fallback. Several companies noted that for the redirect case the indication is not needed since the UE is aware of the ongoing emergency call and therefore knows that it should set the establishment cause to “emergency”. For the handover case there were different views whether the indication is needed:

[029] Regarding how to support “first attempt E-UTRAN cell upon HO failure” in case of emergency service fallback, postpone the discussion to next meeting. Following options can be considered:

- Opt 1: leave it to UE implemetation;

- Opt 2: reuse voiceFallbackIndication-r16 sent by network (FFS on new capability).

As pointed out earlier, one difference in emergency services fallback compared to EPS fallback for IMS voice is that the UE is aware that emergency calls are not supported in 5GS. If the UE wants to place an emergency call and is handed over from 5GS to EPS but the handover fails, the UE knows that it should select an E-UTRA cell rather than an NR cell to speed to up the call establishment. Therefore, the voiceFallbackIndication does not seem needed in the handover case either.

The voiceFallbackIndication is not included in a release with redirect message triggered by Emergency Services fallback.

The voiceFallbackIndication is not included in a handover message triggered by Emergency Services fallback.

If we want to ensure the UE prioritizes E-UTRA cells after a failed handover triggered by emergency services fallback we could add a requirement or informative note in the handover failure section in the RRC specification.

Consider adding a requirement or informative note in RRC to ensure that the UE first attempts to select E-UTRA cells after a failed handover triggered by emergency services fallback.

# Conclusion

In the previous sections we made the following proposals:

Proposal 1	The voiceFallbackIndication is not included in a release with redirect message triggered by Emergency Services fallback.

Proposal 2	The voiceFallbackIndication is not included in a handover message triggered by Emergency Services fallback.

Proposal 3	Consider adding a requirement or informative note in RRC to ensure that the UE first attempts to select E-UTRA cells after a failed handover triggered by emergency services fallback.

# References

R2-2009240, Clarify the usage of voiceFallbackIndication for emergency service, ZTE

[AT112-e][029][NR TEI16] Misc Corrections II (ZTE)

R2-2002326, Introduction of voice fallback indication (CR 38.331), Qualcomm, T-Mobile USA, Verizon, China Telecom, Softbank, Ericsson, Sharp

R2-2000581, Introduction of voice fallback indication (CR 36.331), Qualcomm, T-Mobile USA, Verizon, China Telecom, Softbank, Ericsson, Sharp

R2-2006308, Correction on establishment cause value upon enhanced EPS voice fallback (CR 36.331), Qualcomm, Ericsson, Softbank