3GPP TSG-RAN WG2 Meeting #113 electronic	R2-2100571

Online, Jan 25 – Feb 5, 2021

Source: 			ZTE Corporation, Sanechips

Title: 	Define and Constrain Reduced Capability for RedCap

Agenda item:		8.12.2.1

Document for: 	Discussion and Decision

- Introduction

Some consideration on UE type definition and signaling design principles are provided in this contribution.

- Discussion

Till now, it is till FFS the details and number of device types for RedCap UE. In previous RAN2 meetings, it has been agreed and captured in TR that the number of different UE types should be minimized to reduce market fragmentation. And UE types should be introduced only when where essential to control UE accesses and differentiate them from other non-RedCap UEs[3]:

The network should be able to control whether RedCap UEs can access the cell and differentiate them from other, non-RedCap UEs. The number of different UE types should be minimised to reduce market fragmentation, and UE types should be introduced only where essential to control UE accesses and differentiate them from other non-RedCap UEs.

During initial access procedure, the NW need to distinguish RedCap UE to know the minimal capabilities, so it can schedule the RedCap UE properly. Because NW cannot obtain the full capabilities before UE enter connected state, RedCap UE type acts as a capability signaling. Thus, the UE type definition should be able to indicate at least the reduced capabilities which has impact on initial access processing and needs to be identified during initial access.

Proposal 1: The definition of UE type should be associated with minimum reduced capabilities which has impact on initial access and need to be identified during initial access procedure.

One principle is that UE type definition should not be linked directly to use cases. From RAN2 point of view, the UE type definition should be based on the common radio capabilities of these use cases and other potential use cases. In other words, the access control mechanism should be ignorant of specific use case. Access control for a specific use case can be done in NAS layer and based on subscription data.

Proposal 2: The definition of UE type should be use case ignorant i.e. access control is not designed for specific use cases.

For the number of UE types, company showed different views,because of the various requirements of target use cases. These various requirements may lead to different level of capability reduction and different impact on spectrum efficiency. It seems to define multiple UE type is justify for differentiation in access control.

However, from NW point of view, only one UE type is beneficial in reducing scheduling complexity, resource partition and complexity in access control mechanism. If more than one UE type is defined per FR, further resource partition may be needed for UE type identification which will reduce the resource efficiency. And complexity in cell barring and UAC will also be increased. Thus when taking the cost in complexity and resource efficiency into consideration, it is questionable to define multiple device types for more granular access control.

Proposal 3: To only define one UE type per FR.

In NR, some advanced capability is defined to support bit peak rate use case, such as carrier aggregation/dual connectivity. Some capabilities are defined to support high reliable and low delay use case, such as URLLC, NR-IIoT. It may be meaningful to determine whether these advanced capabilities can be supported by redcap UE.

From prevent market fragmentation point of view, to limit the scope of capabilities will limit number of different UE implementation. And because redcap UE is targeted to design low cost, low complexity and compact device, it seems not all advanced capabilities need to be supported by redcap UE. If the RedCap UE is allowed to support the advanced capability, the scenarios of RedCap UE will be extended if it is allowed by the subscription information (e.g. low-end mobile phone).

Therefore, it is reasonable to have some limitations in capability signaling to prevent redcap UE from indicating some advanced capabilities. As captured in TR, with a separate capability container, it is easy and more clear to limit the scope which can be signaled by RedCap UE.

Proposal 4: To limit the scope of capabilities which RedCap UE can signal. At least capabilities related to CA and DC should not be reported. FFS on other capabilities.

- Conclusion

Proposal 1: The definition of UE type should be associated with minimum reduced capabilities which has impact on initial access and need to be identified during initial access procedure.

Proposal 2: The definition of UE type should be use case ignorant i.e. access control is not designed for specific use cases.

Proposal 3: To define only one UE type per FR.

Proposal 4: To limit the scope of capabilities which RedCap UE can signal. At least capabilities related to CA and DC should not be reported. FFS on other capabilities.

- Reference

- RAN2#111e Chairman notes

- RAN2#112-2 chairman notes

- 3GPP, TR 38.875, v1.0.