3GPP TSG-RAN WG2#113-e Meeting	Tdoc R2-2100587

# Online, January 25th- Feburary 5th 2021											(Revision on R2-2010085)

Agenda Item:		8.13.3.2

Souce:	Samsung

Title:	Immediate MDT with MR-DC and Intermediate MDT for early measurements

Document for:		Discussion and decision

# Introduction

RAN2 agreed, during RAN2#111-e that studies regarding the support of logged and Immediate MDT in MR-DC should be prioritized. In this contribution we cover Immediate MDT while continueing from the final status in R16. We furthermore discuss MDT for measurements provided upon transition to connected (although maybe somewhat in-between Immediate and Logged MDT).

In brief, the paper includes following proposals:

- Intermediate MDT

  - Support inclusion of detailed location info for early measurement reporting upon transition to connected

- Immediate MDT with MRDC

  - Clarify whether whether for MDT measurements on particular frequency (M1, M8) network can only re-use measurements already configured for RRM purposes

  - If network should be able to configure measurements purely for Immediate MDT reasons, discuss and conclude if and how the inter-node coordination introduced as part of the RRM framework is affected by IMM MDT

  - If so, consider to introducing re-negotiation for the configuration restrictions regarding measurements (i.e. allowing SN to request additional measurements for regular RRM purposes)

# Discussion

## Related agreeents from R16 status

During the RAN2#106 meeting RAN2 reached several agreements regarding MDT with MRDC, see below.

Agreements

1	Immediate MDT configurations are supported for DC scenario.

2	Logged MDT configurations can come from SN node in DC scenario.

3	The existing MDT framework is the baseline for the SCG cells related MDT configuration.

4	The triggers for MDT measurements associated to MCG and SCG are separate.

5	MN-SN coordination is required for MDT measurements’ configuration and reporting in DC framework.

6	If SRB3 is not configured, SN related measurements are transmitted to MN via SRB1/2 and then forwarded to SN.

7	If SRB3 is configured, MN related measurements are transmitted to MN via SRB1/2, SN related measurements are transmitted to SN via SRB3

At a later meeting RAN2 however reduced the scope of MDT operations with MRDC due to lack of time to complete the work, see below.

Agreements:

1	Only immediate MDT is supported for EN-DC scenario in R16 MDT

2 	In signaling based immediate MDT, MME provides MDT configuration for both MN and SN towards MN including multi RAT SN configuration, specifically E-UTRA and NR MDT configuration. MN then forwards the NR MDT configuration towards SN (EN-DC scenario, SN is always NR).

3 	In management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently. Inform other working group that Management based MDT should not overwrite signaling based MDT.

4	For immediate MDT configuration, MN and SN can independently configure and receive measurement from the UE.

## Intermediate MDT for early measurements

Immediate MDT so far merely involved the introduction of a field by which network and request UE to include available detailed location information in a MeasurementReport message. We think it makes sense to introduce the same option to request the UE to include location information for early measurement reporting and hence we propose:

Proposal 1	Introduce support for including detailed location information when reporting early measurements

## Immediate MDT in MRDC

### Inter-node handling in MRDC in R16 and R17, general

It seems that for R16 RAN3 introduced a Trace Start procedure was in TS 38.423 by which MN can request SN to start a trace session. The Trace Start message includes the field MDT Activation which values can be set to (Immediate MDT only, Logged MDT only, Immediate MDT and Trace,…). Furthermore, as no clear restrictions are specified, we wonder if RAN3 progressed beyond previous RAN2 agreements and hence what we should assume R16 status to be regarding:

- Support of MRDC cases other than EN-DC

- Use of the procedure to request SN to activate a logged MDT session

- Use of the procedure to request a management based Immediate MDT session

We assume TS 37.320 is still leading and correctly reflecting R16 status (but does not seem to clarify that only MN can configure logged MDT).

#### 5.4.1.3            Immediate MDT for MR-DC

Immediate MDT is supported for EN-DC scenario.

In signalling based immediate MDT, MME provides MDT configuration for both MN and SN towards MN including multi RAT SN configuration, specifically E-UTRA and NR MDT configuration. MN then forwards the NR MDT configuration towards SN (EN-DC scenario, SN is always NR).

In management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently. For both MN and SN, Management based MDT should not overwrite signalling based MDT.

For immediate MDT configuration, MN and SN can independently configure and receive measurement from the UE.

Proposal 2	RAN2 is requested to confirm R16 status as reflected in 37.320 is accurate and to confirm merely following changes are to be done for R17:

- Support of MRDC cases other than EN-DC

- Support of MN forwarding MDT configuration to SN for management based Immediate MDT session

### Inter-node coordination for IMM MDT involving measurements of frequencies

General

Immediate MDT is re-using the RRM measurement reporting framework. This framework includes coordination between MN and SN regarding the measurements each node performs. We a.o. discussed which node is responsible for which type of measurements i.e. that in general SN controls SCG mobility while MN may initiate SN change involving change of primary frequency for load balancing purposes. Furthermore, the framework includes coordination to ensure UE capability limitations concerning measurement performance are respected (covering separate limitations regarding frequencies to measure, number of measurement per serving frequencies and across all non-serving frequencies and total number of measurements). We would like to understand whether or not these inter-node aspects are affected by the introduction of immediate MDT in R16. I.e this concerns the MDT measurements concerning a particular frequency e.g. M1, M8).

We understand that the MDT configuration that MN signals to SN does not include any information regarding which frequency to measure. We wonder if this means that each node will decide by itself for which frequencies to configure MDT? Would it meaning that both MN and SN may configure IMM MDT for same frequency?

As this has so far not been raised, perhaps the common understanding is that for IMM MDT, network will never configure additional RRM measurements merely for MDT purposes. I.e. is it so that for M1/ M8 MDT measurements, network can only re-use measurements that it already needs to configure for RRM purposes (i.e. merely adds includeLocationInfo to such existing measurements)? Anyhow, as we think this has so far not been discussed/ concluded we propose:

Proposal 3	RAN2 is requested to discuss and conclude whether for MDT measurements on particular frequency (M1, M8) network can only re-use measurements already configured for RRM purposes

- I.e. that network cannot configure measurements purely for Immediate MDT reasons

Impact IMM MDT on RRM framework

In case RAN2 conclusion that network may configure measurements purely for Immediate MDT (M1, M8) reasons, there is a need to discuss if and how the inter-node coordination introduced as part of the RRM framework is affected by IMM MDT. Some further remarks regarding this:

- Both for signalling and management based MDT, there are several aspects to be decide by RAN (i.e. nothing included within the received MDT request) e.g.

  - Which frequencies to measure for MDT purposes

  - For a particular frequency, whether to configure multiple measurements for MDT purposes

  - Certain measurement configuration details/ parameters

- For R17, there seem to be two main options

  - MN decides which frequencies to measure, and possibly some further detailed aspects

  - MN merely forwards the MDT requests, and leaves it up to SN to configure MDT measurements for frequencies it controls

  - A combination i.e. MN forwards the received MDT request, but also provides a list of frequencies for which MN configures MDT measurements

- The question is now how to use existing fields defined for coordinating the RRM measurements that SN may configure for RRM purposes. I.e. so far MN decide the number of measurements SN is allowed to configure observing the following limits:

  - Serving frequencies (maxIntraFreqMeasId) i.e. at most 9 per serving freq, split beteen MN and SN

  - Non-serving freqencies (maxInterFreqMeasId) i.e. at most 10 across all neighbouring freqs, split between MN and SN

- As discussed in the previous, we think we should avoid/ limit specification changes and think the simplest option would be to introduce re-negotiation for the configuration restrictions regarding measurements. We could also allow SN to use this option only to request additional measurements for regular RRM purposes

Proposal 4	RAN2 is requested to discuss and conclude if and how the inter-node coordination introduced as part of the RRM framework is affected by IMM MDT:

- If some impact is foreseen, RAN2 is requested to consider an option involving minimal specification changes i.e. to introduce re-negotiation for the configuration restrictions regarding measurements (i.e. allowing SN to request additional measurements for regular RRM purposes)

# Conclusion & recommendation

The paper discusses R17 enhancements concerning logged and immediate MDT with MRDC. It furthermore covers some related early measurement aspects. The document includes the following proposals that RAN2 is requested to discuss and conclude:

Proposal 1	Introduce support for including detailed location information when reporting early measurements

Proposal 2	RAN2 is requested to confirm R16 status as reflected in 37.320 is accurate and to confirm merely following changes are to be done for R17:

- Support of MRDC cases other than EN-DC

- Support of MN forwarding MDT configuration to SN for management based Immediate MDT session

Proposal 3	RAN2 is requested to discuss and conclude whether for MDT measurements on particular frequency (M1, M8) network can only re-use measurements already configured for RRM purposes

- I.e. that network cannot configure measurements purely for Immediate MDT reasons

Proposal 4	RAN2 is requested to discuss and conclude if and how the inter-node coordination introduced as part of the RRM framework is affected by IMM MDT.:

- If some impact is foreseen, RAN2 is requested to consider an option involving minimal specification changes i.e. to introduce re-negotiation for the configuration restrictions regarding measurements (i.e. allowing SN to request additional measurements for regular RRM purposes)

# References

[1] TS 38.331 Radio Resource Control