3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-2100607

Elbonia, 25 January – 05 February 2021

Agenda item:	6.10.3

Source:	Nokia, Nokia Shanghai Bell, Ericsson

Title:	Logged MDT Information extension

WID/SID:	NR_SON_MDT-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

It was agreed during RAN2#112 that IDC indication should be introduced in Logged MDT. However, the current ASN.1 in Rel-16 of MDT does not allow the addition of IE in the report as it is in LTE. This causes a problem for the inclusion in Rel-17 of IDC information in the logged MDT report.

In this contribution we discuss how to approach the issue as Rel-16 correction.

# 2	Discussion

## 2.1	Extension in Logged MDT

In LTE, the Information Element defined for logged measurement performed by the UE are signaling back to the network is as follows:

LogMeasInfo-r10 ::=		SEQUENCE {

locationInfo-r10					LocationInfo-r10				OPTIONAL,

relativeTimeStamp-r10				INTEGER (0..7200),

servCellIdentity-r10				CellGlobalIdEUTRA,

measResultServCell-r10				SEQUENCE {

rsrpResult-r10						RSRP-Range,

rsrqResult-r10						RSRQ-Range

},

measResultNeighCells-r10			SEQUENCE {

measResultListEUTRA-r10				MeasResultList2EUTRA-r9		OPTIONAL,

measResultListUTRA-r10				MeasResultList2UTRA-r9		OPTIONAL,

measResultListGERAN-r10				MeasResultList2GERAN-r10	OPTIONAL,

measResultListCDMA2000-r10			MeasResultList2CDMA2000-r9	OPTIONAL

}	OPTIONAL,

...,

[[	measResultListEUTRA-v1090			MeasResultList2EUTRA-v9e0	OPTIONAL

]],

[[	measResultListMBSFN-r12				MeasResultListMBSFN-r12		OPTIONAL,

measResultServCell-v1250			RSRQ-Range-v1250			OPTIONAL,

servCellRSRQ-Type-r12				RSRQ-Type-r12				OPTIONAL,

measResultListEUTRA-v1250			MeasResultList2EUTRA-v1250	OPTIONAL

]],

[[	inDeviceCoexDetected-r13			ENUMERATED {true}			OPTIONAL

]],

[[	measResultServCell-v1360			RSRP-Range-v1360			OPTIONAL

]],

[[	logMeasResultListBT-r15				LogMeasResultListBT-r15		OPTIONAL,

logMeasResultListWLAN-r15			LogMeasResultListWLAN-r15	OPTIONAL

]],

[[	anyCellSelectionDetected-r15		ENUMERATED {true}			OPTIONAL

]],

[[	measResultListNR-r16				MeasResultCellListNR-r15	OPTIONAL

]]

}

Observation 1: In LTE the extension mechanism “…” is used for adding new measurements or logged information.

However, in 38.331 the same IE contains no extension mechanism.

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16        OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL

}

This create a problem if new elements need to be added to the IE LogMeadInfo-r16.

Observation 2: The timestamped entry of NR Rel-16 Logged Measurement Information (LogMeasInfo-r16) is non-extendible.

This brings unnecessary limitation. We believe this was not intentional design, as further extensions to MDT reports are foreseen. For Rel-17 WID, it was already agreed to include IDC indication in Logged MDT, as it is in LTE.

The inclusion of IDC for logged MDT has been agreed in previous RAN2 meeting. By the analysis on how LTE baseline should be adopted for IDC inclusion in NR MDT reports (see Annex), we noticed the problem. Namely, the ASN.1 signaling extension for inclusion of IDC indicator should be part of extended LogMeasInfo-r16, however, the IE (as it is currently) can’t be directly extended since it lacks the extension marker (i.e. ellipsis).

Observation 3: LogMeasInfo-r16 needs a correction to be extendible in Rel-16, otherwise extension in Rel-17 version of the IE will not follow generic ASN.1 extension principles but will require special handling.

## 2.2	Potential solutions

We identify there are 4 solutions to tackle issue with non-extendible LogMeasInfor-r16:

- Addition of “…” in LogMeasInfo-r16 in Rel-16 in NBC way

- Extend the message using existing tools in Rel 17 (with a duplicated list or a new parallel list as in solution 3).

- Parallel list with extendibility is introduced

- Dummify the current list and create a new version

All solutions are provided in Annex A of this document for reference. Generally, the following can be noted on the solutions:

- Solution 1 creates non backward compatible ASN.1 (with isolated impact to UEs implementing logged MDT), as to minimize the number of UE / Nodes impacted by non-backward compatible change Rel-16 CR would be preferred.

- Solution 2 require special handling of the logged data and reporting, will increase signaling size if the entire structure is duplicated (since if we create new IE, the UE will have to fill in both Rel-16 and Rel-17 versions of the IE)

- Solution 3 retains backward compatibility and can be done either in Rel-16 or Rel-17, however this introduces (as in Solution 2) the need to handle both types of logged IEs

- Solution 4 could require a CR in Rel-16 to minimize the number of UE / Nodes impacted by non-backward compatible changes, avoids handling duplicated content and is relatively clean solution (it is a common way to invalidate IEs also for other features).

## 2.3	Comparison of the solutions

Here we list the pro and cons of the two possible solutions.

| Solution | Pros | Cons |
| --- | --- | --- |
| - ASN.1 change with addition of “…” in LogMeasInfo-r16 | Cleanest solution Enables to achieve the same design of Logged MDT in NR as in LTE Simple addition of new measurements / features NBC is isolated only to UEs implementing Rel-16 NR Logged MDT | Late ASN.1 NBC change in Rel-16 (Requires Rel-16 CR) |
| - Message extension only with existing Rel-17 tools (non-critical extension to define either duplicated or a new parallel list) | Avoids Rel-16 NBC change Can be done in Rel-17 | Specification of the logged MDT report handling gets more difficult Increase signaling size if the entire structure is duplicated. The new Rel-17 IE is created and the UE will have to fill in both Rel-16 and Rel-17 versions of the IE and signal both To understand both IEs presence, there may be a separate availability indicator or update of the UE capability needed -> impacts to ASN.1 for Rel-17 report handling get overcomplex (see description under the Table) |
| - Parallel list with extendibility | Retains backward compatibility Can be done in Rel-16 or in Rel-17 | Specification of the logged MDT report handling get more difficult More difficult coding of the message in UE (encode 2 list in parallel) The UE will have to fill in both Rel-16 and Rel-17 versions of the IE and signal both |
| - The current list is dummified and new version of the LogMeasInfo is created | Relatively clean solution Simple addition of new measurements / features | Late NBC change in Rel-16 (Requires Rel-16 CR) The first list IE need to be sent |

Table 1: Comparison of solution for non-extendible LogMeasInfo-r16.

For some more detailed observations we list also particular observations for:

Solution 2: Non-critical extension

Pros:

-	Does not introduce Non Backward compatible change in Release 16

-	Message extension is smaller using “noncritical extension” structure

Cons

-	Specification of the logged MDT report handling gets more difficult

-	In case of the list is duplicated, if the logged MDT result is received by a Rel-16 gNB but the UE includes Rel-17 extension, the additional change needed may be a separate rel-17 specific logged MDT available indicator in the UE-MeasurementAvailable. So, a UE that has rel-17 based MDT report contents would indicate the availability using logMeasAvailable-r17 whereas the legacy UEs would use logMeasAvailable-r16.

#### –	UE-MeasurementsAvailable-r16

The IE UE-MeasurementsAvailable is used to indicate all relevant available indicators for UE mesurements.

UE-MeasurementsAvailable information element

-- ASN1START

-- TAG-UE-MeasurementsAvailable-START

UE-MeasurementsAvailable-r16 ::=              SEQUENCE {

logMeasAvailable-r16                         ENUMERATED {true}               OPTIONAL,

logMeasAvailableBT-r16                       ENUMERATED {true}               OPTIONAL,

logMeasAvailableWLAN-r16                     ENUMERATED {true}               OPTIONAL,

connEstFailInfoAvailable-r16                 ENUMERATED {true}               OPTIONAL,

rlf-InfoAvailable-r16                        ENUMERATED {true}               OPTIONAL,

...,

logMeasAvailable-r17                         ENUMERATED {true}               OPTIONAL

}

-- TAG-UE-MeasurementsAvailable-STOP

-- ASN1STOP

RAN2 should discuss which is the preferred solution

Proposal: RAN2 should discuss the preferred solution for Logged MDT information extension in Rel-16, considering there are further extensions expected in Rel-17.

# 3	Conclusion

This document has made the following observations and proposals:

Observation 1: In LTE the extension mechanism “…” is used for adding new measurements or logged information.

Observation 2: The timestamped entry of NR Rel-16 Logged Measurement Information (LogMeasInfo-r16) is non-extendible.

Observation 3: LogMeasInfo-r16 needs a correction to be extendible in Rel-16, otherwise extension in Rel-17 version of the IE will not follow generic ASN.1 extension principles but will require special handling. Proposal: RAN2 to discuss the preferred solution for Logged MDT information extension in Rel-16, considering there are further extensions expected in Rel-17.

# A	Annex

## A.1	Rel-16 proposal for solution 1: Addition of “…”

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16        OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL,

...

}

## A.2	Rel-16 proposal for solution 2: Non Critical extension (with duplication of LogMeasInfo-r16)

UEInformationResponse-r16-IEs ::=    SEQUENCE {

measResultIdleEUTRA-r16              MeasResultIdleEUTRA-r16             OPTIONAL,

measResultIdleNR-r16                 MeasResultIdleNR-r16                OPTIONAL,

logMeasReport-r16                    LogMeasReport-r16                   OPTIONAL,

connEstFailReport-r16                ConnEstFailReport-r16               OPTIONAL,

ra-ReportList-r16                    RA-ReportList-r16                   OPTIONAL,

rlf-Report-r16                       RLF-Report-r16                      OPTIONAL,

mobilityHistoryReport-r16            MobilityHistoryReport-r16           OPTIONAL,

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-vxyz-IE17-OPTIONAL

}

UEInformationResponse-vxyz-IE17 ::=     SEQUENCE {

ueInformationResponse-r17           UEInformationResponse-r17           OPTIONAL,

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

LogMeasReport-r17 ::=                SEQUENCE {

logMeasInfoList-r17                  LogMeasInfoList-r17,

...

}

LogMeasInfoList-r17 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r17

LogMeasInfo-r16 ::=                  SEQUENCE {

new-Rel-17-Elements                  NEW Rel-17-Elements	OPTIONAL,

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16        OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL

}

## A.3	Rel-17 proposal for solution 2: Non Critical extension (with a parallel list)

UEInformationResponse-r16-IEs ::=    SEQUENCE {

measResultIdleEUTRA-r16              MeasResultIdleEUTRA-r16             OPTIONAL,

measResultIdleNR-r16                 MeasResultIdleNR-r16                OPTIONAL,

logMeasReport-r16                    LogMeasReport-r16                   OPTIONAL,

connEstFailReport-r16                ConnEstFailReport-r16               OPTIONAL,

ra-ReportList-r16                    RA-ReportList-r16                   OPTIONAL,

rlf-Report-r16                       RLF-Report-r16                      OPTIONAL,

mobilityHistoryReport-r16            MobilityHistoryReport-r16           OPTIONAL,

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-vxyz-IE17-OPTIONAL

}

UEInformationResponse-vxyz-IE17 ::=     SEQUENCE {

ueInformationResponse-r17           UEInformationResponse-r17           OPTIONAL,

nonCriticalExtension                SEQUENCE {}                         OPTIONAL

}

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

LogMeasReport-r17 ::=                SEQUENCE {

logMeasInfoList-r17                  LogMeasInfoList-r17,

...

}

LogMeasInfoList-r17 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r17

LogMeasInfo-r17 ::=                  SEQUENCE {

new-Rel-17-Elements                  NEW Rel-17-Elements OPTIONAL,

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16        OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL

}

## A.4	Rel-16 proposal for solution 3: Parallel list

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

logMeasInfoList-r16_ext              LogMeasInfoList-r16_ext

}

LogMeasInfoList-r16_ext ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16_ext

LogMeasInfo-r16_ext::=                  SEQUENCE {

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16        OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL

}

## A.5	Rel-16 proposal for solution 4: Dummify the first list

UEInformationResponse-r16-IEs ::=    SEQUENCE {

measResultIdleEUTRA-r16              MeasResultIdleEUTRA-r16             OPTIONAL,

measResultIdleNR-r16                 MeasResultIdleNR-r16                OPTIONAL,

logMeasReport-r16                    LogMeasReport-r16                   OPTIONAL,

connEstFailReport-r16                ConnEstFailReport-r16               OPTIONAL,

ra-ReportList-r16                    RA-ReportList-r16                   OPTIONAL,

rlf-Report-r16                       RLF-Report-r16                      OPTIONAL,

mobilityHistoryReport-r16            MobilityHistoryReport-r16           OPTIONAL,

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

nonCriticalExtension                 SEQUENCE {}                         OPTIONAL

}

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

dummy                                LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...,

[[

logMeasInfoList-r16-vxyz             LogMeasInfoList-r16-vxyz,

]]

}

LogMeasInfoList-r16-vxyz                 SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo- r16-vxyz

LogMeasInfo-r16-vxyz ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16        OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL,

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16        OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16            OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL

}