3GPP TSG-RAN WG2 Meeting #113e	R2-2100727

Online, Jan 25 – Feb 05, 2021

Agenda Item		:	8.2.3

Source		:	LG Electronics Inc.

Title		:	Support for CHO and CPAC coexistence

Document for		:	Discussion and Decision

1. Introduction

In this contribution, we discuss to support configuration for CHO and CPAC coexistence.

2. Discussion

In the RAN2#112e meeting, RAN2 made following agreements for Rel-17 CPAC [1]:

For CPA and MN initiated Inter-SN CPC, the MN generates and transmits the conditional configuration message (i.e. RRCReconfiguration/RRCConnectionReconfiguration message) to the UE.  The RRCReconfiguration provided by the candidate PSCell(s) is encapsulated in the final conditional reconfiguration message to the UE. The MN is not allowed to alter the RRCReconfiguration provided by the candidate PSCell(s).

Proposal 1: Option 1 should be used for the generation of conditional reconfiguration for SN initiated inter-SN conditional PSCell change.

Option 1:	The MN generates CPC. The source SN sets the execution condition and communicates it to the MN. The MN generates the conditional reconfiguration message including the execution condition(s) provided by the source SN and RRCReconfiguration provided by the candidate PSCell(s).

FFS how many candidate cells (UE and network impacts should be clarified).

FFS whether the number of candidate cells for CPAC different from that of CHO.

Before we discuss FFS, RAN2 may need to confirm that the UE can be configured with both CHO and CPAC simultaneously.  In our view, the simultaneous configuration of CHO and CPAC may already happen in Rel-16 because SN initiated intra-SN CPC is configured without MN involvement. We do not see any serious problem or specification impacts in supporting the coexistence of CHO and CPAC configurations. Thus, RAN2 should be encouraged to support the coexistence of CHO and CPAC configurations.

Proposal 1. Support the coexistence of CHO and CPAC configurations in UE.

One issue to support the coexistence of CHO and CPAC configurations is to address the maximum number of candidate cells which the UE should support and to ensure that network configuration on CPAC should not exceed the UE capability.

In our view, in all CPAC cases in Rel-17, network can ensure that the CPAC configuration does not exceed the UE capabilities. This is because in those cases, MN can know the potential capability issue of a certain CPAC configuration and resolve it before sending the problematic configuration to the UE. This means, in turn, MN is required to generate the conditional reconfiguration for the UE. For example, while MN is preparing CPA after configuring CHO, if the MN finds out that there is a problem with the number of candidate cells, the MN can resolve the problem by CPA modification before sending it to the UE.

However, in case of the SN initiated intra SN CPC scenario w/o MN involvement, the UE may be configured with CHO and CPC that exceeds the R16 UE’s capabilities. Currently the R16 UE can only support up to 8 candidate cells for conditional mobility. Hence, if the total maximum number of candidate cells of the CHO and the CPC exceeds 8, unexpected error or failure could occur.

To solve the problem of exceeding the capability, the  simplest way is to extend UE capabilities, i.e. increasing the supported number of candidate cells from 8 to 16. We think this capability extension does not really burden UE, in that R16 UE is already capable of supporting much larger number of configuring measurement objects, measurement report configurations, and measurement IDs,  as shown in the Annex below, which are somehow relevant to the capabilities of evaluating conditional mobility execution conditions. The capability extension only requires doubled memory for storing the candidate cell configurations.

Proposal 2. UE supports up to 16 candidate cells for conditional mobility

3. Conclusion

In this contribution, we have the following proposals:

Proposal 1. Support the coexistence of CHO and CPAC configurations in UE.

Proposal 2. UE supports up to 16 candidate cells for conditional mobility

4. References

[1] R2-2010701	Report from session on LTE legacy, LTE TEI16 and NR/LTE Rel-16 Mobility	Nokia

5. Annex – TS 38.331-g20

6.3.2	Radio resource control information elements

MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers intra-frequency, inter-frequency and inter-RAT mobility as well as configuration of measurement gaps.

MeasConfig information element

-- ASN1START

-- TAG-MEASCONFIG-START

MeasConfig ::=                      SEQUENCE {

measObjectToRemoveList              MeasObjectToRemoveList                                              OPTIONAL,   -- Need N

measObjectToAddModList              MeasObjectToAddModList                                              OPTIONAL,   -- Need N

reportConfigToRemoveList            ReportConfigToRemoveList                                            OPTIONAL,   -- Need N

reportConfigToAddModList            ReportConfigToAddModList                                            OPTIONAL,   -- Need N

measIdToRemoveList                  MeasIdToRemoveList                                                  OPTIONAL,   -- Need N

measIdToAddModList                  MeasIdToAddModList                                                  OPTIONAL,   -- Need N

s-MeasureConfig                     CHOICE {

ssb-RSRP                            RSRP-Range,

csi-RSRP                            RSRP-Range

}                                                                                                       OPTIONAL,   -- Need M

quantityConfig                      QuantityConfig                                                      OPTIONAL,   -- Need M

measGapConfig                       MeasGapConfig                                                       OPTIONAL,   -- Need M

measGapSharingConfig                MeasGapSharingConfig                                                OPTIONAL,   -- Need M

...,

[[

interFrequencyConfig-NoGap-r16      ENUMERATED {true}                                                   OPTIONAL    -- Need R

]]

}

MeasObjectToRemoveList ::=              SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectId

MeasIdToRemoveList ::=                  SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasId

ReportConfigToRemoveList ::=            SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigId

-- TAG-MEASCONFIG-STOP

-- ASN1STOP

MeasIdToAddModList

The IE MeasIdToAddModList concerns a list of measurement identities to add or modify, with for each entry the measId, the associated measObjectId and the associated reportConfigId.

MeasIdToAddModList information element

-- ASN1START

-- TAG-MEASIDTOADDMODLIST-START

MeasIdToAddModList ::=              SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod

MeasIdToAddMod ::=                  SEQUENCE {

measId                              MeasId,

measObjectId                        MeasObjectId,

reportConfigId                      ReportConfigId

}

-- TAG-MEASIDTOADDMODLIST-STOP

-- ASN1STOP

6.4	RRC multiplicity and type constraint values

–	Multiplicity and type constraint definitions

maxCellInter                            INTEGER ::= 16      -- Maximum number of inter-Freq cells listed in SIB4

maxCellIntra                            INTEGER ::= 16      -- Maximum number of intra-Freq cells listed in SIB3

maxCellMeasEUTRA                        INTEGER ::= 32      -- Maximum number of cells in E-UTRAN

maxCellMeasIdle-r16                     INTEGER ::= 8       -- Maximum number of cells per carrier for idle/inactive measurements

maxCellMeasUTRA-FDD-r16                 INTEGER ::= 32      -- Maximum number of cells in FDD UTRAN

maxNrofObjectId                         INTEGER ::= 64      -- Maximum number of measurement objects

maxNrofMeasId                           INTEGER ::= 64      -- Maximum number of configured measurements

maxReportConfigId                       INTEGER ::= 64

maxCellReport                           INTEGER ::= 8