3GPP TSG-RAN WG2 #113 electronic	Tdoc R2-2101435

Electronic Meeting, Jan 25 – Feb 05, 2021

Agenda Item:	5.4.3

Source:	Ericsson

Title:	On the use of UE simultaneous Rx/Tx capability

Document for:	Discussion, Decision

# 1	Introduction

In RAN#112-e, an LS was received from RAN4 on simultaneous Rx/Tx capability (R4-2016988). This contribution further discusses the LS and a possible reply to RAN4.

# 2	Discussion

In the LS, RAN4 firstly expresses their understanding on the UE capability fallback behaviour as follows:

Simultaneous Rx/Tx capability for TDD-TDD and TDD-FDD inter-band CA, SUL and EN-DC band combinations has been discussed in RAN4. It is identified that there are some ambiguity on the applicability of simultaneous Rx/Tx condition for an inter-band combination, especially for the combination having more than two bands. As an example, for CA_n39-n41-n79, if UE supports simultaneous Rx/Tx capability for CA_n41-n79 but not for CA_n39-n41, the simultaneous Rx/Tx capability shall not be reported for CA_n39-n41-n79. However, since the capability of the fallback mode is different from the higher order band combination, simultaneous Rx/Tx capability for CA_n41-n79 shall be reported additionally.

The above is mostly in line with TS 38.331, since the UE may advertise fallback band combinations where it includes additional features compared to a corresponding superset band combination. In other words, the UE is only required to exclude from its report band combinations regarded as fallback band combinations with no additional features compared to its corresponding superset band combination (as clarified in 38.331, also copied below).

1>	for each band combination included in the list of "candidate band combinations":

2>	if the network (E-UTRA) included the eutra-nr-only field, or

2>	if the requested rat-Type is eutra:

3>	remove the NR-only band combination from the list of "candidate band combinations";

NOTE 4:	The (E-UTRA) network may request capabilities for nr but indicate with the eutra-nr-only flag that the UE shall not include any NR band combinations in the UE-NR-Capability. In this case the procedural text above removes all NR-only band combinations from the candidate list and thereby also avoids inclusion of corresponding feature set combinations and feature sets below.

2>	if it is regarded as a fallback band combination with the same capabilities of another band combination included in the list of "candidate band combinations", and

2>	if this fallback band combination is generated by releasing at least one SCell or uplink configuration of SCell according to TS 38.306 [26]:

3>	remove the band combination from the list of "candidate band combinations";

RAN2 confirms RAN4 understanding that the UE may report a fallback band combination for which it supports additional functionality compared to its corresponding superset band combination.

For a band combination with different simultaneous Rx/Tx capability for the fallback mode, RAN4’s understanding is that the network shall also consider the fallback mode capability to decide the UL/DL scheduling among all bands for this band combination. It’s not clear whether the current RAN2 specification supports this kind of understanding. If not or if it can only be derived implicitly, RAN4 would like to see some explicit clarification in the RAN2 specification.

Secondly, RAN4 asks whether RAN2 specifications already support the following behaviour:

Such aspect is not currently present in RAN2, since the UE capability design follows a hierarchical structure where each feature is defined in its applicable level. For instance, the parameters applicable for a certain band combination are defined in a corresponding per band combination structure (as depicted from 38.331 below).

#### –	CA-ParametersNR

The IE CA-ParametersNR contains carrier aggregation and inter-frequency DAPS handover related capabilities that are defined per band combination.

That implies that if other features should be applicable for a certain band combination (e.g. as in RAN4 example where they may want simultaneous Rx/Tx capability to be applicable to the “n39-n41” portion of the CA_n39-n41-n79 band combination), such feature should be defined for this given band combination.

Moreover, such hierarchical structure also allows the independent report of band combinations, i.e. since each band combination contains the features applicable to it, the inclusion/omission of band combinations in the UE report should not affect other band combinations reported by the UE. For instance, the UE may not be able to report all the band combinations it needs due to size limitations (as detailed in 38.331, also copied below), thus it is not possible to rely on the report of e.g. band combination A to derive what the UE supports for band combination B.

1>	if the requested rat-Type is nr:

2>	include into supportedBandCombinationList as many NR-only band combinations as possible from the list of "candidate band combinations", starting from the first entry;

Therefore, the RAN4 understanding that the support of simultaneous Rx/Tx capability needs to be derived from two different band combinations is not in line with the current signaling structure and is not currently present in the RAN2 specifications.

The understanding that the support of simultaneous Rx/Tx capability needs to be derived from two different band combinations is not in line with the current signaling structure and is not currently present in the RAN2 specifications.

While new signaling can be added to account for this, it is advised to first clarify to RAN4 that such behaviour is not in according to RAN2 signaling design – if RAN4 would really see a need to account for such behaviour, it would be up to RAN2 how to design this behaviour in according to the UE capability signaling structure.

Inform RAN4 that the UE capability signaling does not account for the indication of support of a feature that needs to be derived from multiple band combinations. If the current design of simultaneous Rx/Tx capability is not enough, RAN4 can clarify which further cases need to be covered so that RAN2 can design the according signaling for such cases.

For the question raised by RAN2 on whether the simultaneous RxTx UE capability is needed for inter-band NR-DC (for TDD-TDD and TDD-FDD band combinations), RAN4 thinks that the capability is needed, and same principles used for simultaneousRxTxInterBandCA as well as clarification consideration above shall also be applied for NR-DC

Lastly, RAN4 replies on a previous RAN2 question regarding applicability of simultaneous Rx/Tx capability for NR-DC:

For NR-DC, since the signaling already allows this capability to be reported for both CA-ParametersNR and CA-ParametersNRDC, the behaviour expressed by RAN4 is already possible in RAN2 specifications.

Inform RAN4 that the UE capability signaling already allows the simultaneous Rx/Tx capability to be reported differently for NR CA and NR-DC.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The understanding that the support of simultaneous Rx/Tx capability needs to be derived from two different band combinations violates the current signaling structure and is not currently present in the RAN2 specifications.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 confirms RAN4 understanding that the UE may report a fallback band combination for which it supports additional functionality compared to its corresponding superset band combination.

Proposal 2	Inform RAN4 that the UE capability signaling does not account for the indication of support of a feature that needs to be derived from multiple band combinations. If the current design of simultaneous Rx/Tx capability is not enough, RAN4 can clarify which further cases need to be covered so that RAN2 can design the according signaling for such cases.

Proposal 3	Inform RAN4 that the UE capability signaling already allows the simultaneous Rx/Tx capability to be reported differently for NR CA and NR-DC.

# 4	References

R4-2016988, Draft LS on simultaneous Rx/Tx capability, RAN4#97-e, November 2-13