3GPP TSG RAN WG2 Meeting #113-e		R2-21xxxxx

Online 25 Jan – 5 Feb 2021

Title:	[Draft] LS on neighbour cell measurement in NB-IoT RRC_CONNECTED state

Response to:

Release:	Release 16

Work Item:	NB_IOTenh4_LTE_eMTC6-Core

Source:	??? [to be RAN2]

To:	RAN4

Cc:	RAN1?

Contact person:	??

??

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

RAN2 is currently working on below objective where RAN4 is also marked as one of the responsible group.

- Specify signaling for neighbor cell measurements and corresponding measurement triggering before RLF, to reduce the time taken to RRC reestablishment to another cell, without defining specific gaps. [NB-IoT] [RAN2, RAN4].

Based on the below Figure, the intention of this objective can be expressed as to reduce the time between reference points C and D through specifying signalling for neighbor cell measurements and corresponding measurement triggering between reference points A and C, i.e., aim to identify suitable cell before RLF is declared.

RAN2 has also identified the scenarios shown in the table below that needs to be considered for NB-IoT connected mode neighbour cell measurements.

| Scenario | Serving & Neighbour cell anchor carrier frequency | UE connected mode operating Frequency |
| --- | --- | --- |
| A | Intra-frequency | Anchor carrier |
| B | Intra-frequency | Non-Anchor carrier |
| C | Inter-frequency | Non-Anchor carrier but same as Neighbour cell Anchor carrier |
| D | Inter-frequency | Anchor carrier |
| E | Inter-frequency | Non-Anchor carrier but different from Neighbour cell Anchor carrier |

For each of the above scenario, RAN2 has the following questions:

- For how long the neighbour cell can be considered as known after it has been detected/re-confirmed?

- How long does it take to perform cell detection both in normal and in extended coverage?

- For how long the NRSRP/NRSRQ measurements can be consider as valid?

- How long does it take to perform NRSRP/NRSRQ measurements?

# 2	Actions

To 3GPP RAN4

ACTION: RAN2 respectfully asks RAN4 take into account the above information and provide feedback to the above questions.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #113-bis-e	12 – 20 April 2021

TSG RAN WG2 Meeting #114-e	19 – 27 May 2021