3GPP TSG-RAN WG2 Meeting #113e                                                 draft-R2-2102064

E-Meeting: January 25- February 05, 2021

Agenda item:	7.2.3

Source:	Qualcomm Incorporated

Title:	[AT113-e][404][eMTC R16] PDCCH-based HARQ-ACK for multi-TB scheduling

Document for:	Discussion and Decision

# Introduction

RAN1 has agreed a change in PDCCH-based HARQ ACK when using multiple TBs transmission. The corresponding RAN2 CR is provided in [1]. Based on [1], the following email discussion is started.

- [AT113-e][404][eMTC R16] PDCCH-based HARQ-ACK for multi-TB scheduling (Qualcomm)

Status: Started

Scope:

Week 1: Check whether the intention is agreeable and there is sufficient support in principle; collect initial comments.

Week 2: Agreeable CR, if there is sufficient support

Intended outcome:

Week 1: Report in R2-2102064

Week 2: Agreeable 36.321 CR in R2-210XXXX (to be announced later)

Deadline: Wednesday 2021-01-27 4:45 UTC

This document is to provide summary on this email discussion.

# Discussion

RAN1 has agreed following text in TS 36.212 (R1-2009613, CR#0353) such that HARQ specific PDCCH-based HARQ ACK is used for transmissions with scheduling of multiple TBs. The new change is shown in color font.

If ce-PUSCH-MultiTB-Config is not enabled and the Resource block assignment in format 6-0A is set to all ones, or ce-PUSCH-MultiTB-Config is enabled and mpdcch-UL-HARQ-ACK-FeedbackConfig is configured and the 6 MSB bits of the Scheduling TBs for Unicast Field are set to '110111', format 6-0A is used for the indication of ACK feedback. 8 bits including the 6 LSB bits of the Scheduling TBs for Unicast Field and 2 MSB bits of Repetition number are used to indicate HARQ-ACK by bitmap, where the order of the bitmap to HARQ process index mapping is such that HARQ process indices are mapped in ascending order from MSB to LSB of the bitmap. For each bit of the bitmap, value 1 indicates ACK, and value 0 is reserved. And all the remaining bits except Flag format 6-0A/format 6-1A differentiation and DCI subframe repetition number are set to zero.

However, in TS 36.321, the PDCCH-based HARQ-ACK is assumed to be the acknowledgement for all HARQ processes for which PUSCH transmission is completed regardless of multiple TBs transmission as shown below.

- if the PDCCH indicates an UL HARQ-ACK feedback for an asynchronous UL HARQ process for a UE configured with mpdcch-UL-HARQ-ACK-FeedbackConfig; and - if the PUSCH transmission is completed: - stop drx-ULRetransmissionTimer for all UL HARQ processes.

To correct this, a draft CR for TS 36.321 is provided in [1].

Question 1: Do you agree with the intent of the change in [1]? Please elaborate if further change/clarification is needed in the comments.

| Company | Yes/No | Comments |
| --- | --- | --- |

Question 2: Is the any change needed in the specification other than TS 36.321? if yes, please elaborate.

| Company | Yes/No | Comments |
| --- | --- | --- |

# Conclusion

TBD..

# References

[1] R2-2100735,	PDCCH-based HARQ-ACK for a specific HARQ process with multi-TB scheduling, Qualcomm Incorporated, RAN2#113e.