3GPP TSG-RAN WG2 Meeting #113 electronic				R2-21xxxxx

Online, 25th Jan – 5th Feb, 2021

Agenda Item:		8.13.3	MDT

Source:			Huawei (Summary rapporteur)

Title:				Report of [AT113-e][844][NR/R17 SON/MDT]  Logged MDT part I

WI code(s):			NR_ENDC_SON_MDT_enh-Core

Document for:		Discussion and Decision

## 1	Introduction

In Rel-17 SON and MDT session, it was agreed to have the following email discussion:

- [AT113-e][844][NR/R17 SON/MDT]  Logged MDT part I (Huawei)

-	Discussion on 2.3.1 of R2-2102250

Intended outcome: Agreeable WF

Deadline: Thursday 04/02/2021

This report is to collect companies’ opinion and provide some summary proposals to move the discussion forward.

## 2	Logging early measurements

### 2.1	List of relevant proposals from contribution [1] to [8]

[1], China Telecom

Proposal 2: early measurement results can be used for logged MDT purpose under the logged MDT framework.

[2], OPPO

Proposal 1: RAN2 to agree that down selection from following three options should be made to let the network clearly understand the reason of expiration of the T331 or the mismatch between the camped cell and the validity area list why early measurement results are not included in the logged measurement report anymore:

- Option A: the configuration of the validityAreaList and/or the validityCellList and measIdleDuration-r16 IE is included in the logged measurement report.

- Option B: include an indication in a new LogMeasInfo-r16 entry in the logMeasReport-r16 IE indicating that early measurement results are not included in the log measurement report from now on (corresponding to the time stamp included in the entry) due to a reason other than cell-level signal not hearable.

- Option C: include indications in all related LogMeasInfo-r16 entries in the logMeasReport-r16 IE indicating that early measurement results are not included in the log measurement report due to a reason other than cell-level signal not hearable.

Proposal 2: RAN2 to agree that down selection from following two options should be made to let the network clearly understand the reason why some previously recorded early measurement results are missed in the following logged measurement entries is the updating of the early measurement configuration:

- Option A: include the updated carrier frequency information (measIdleCarrierListEUTRA and/or measIdleCarrierListNR IE) in the logged measurement report and add a new indication to mark the timestamp from which the updated configuration becomes valid.

- Option B: include an indication for the carrier frequencies for which the early measurements are no longer performed due to updating of the configuration of the early measurement.

[3], Qualcomm

Proposal 1: Introduce the UE capability to indicate whether UE supports logging of non-SIB4 frequencies, i.e., UE capability to indicate UE can perform logging of early measurements.

Proposal 2: Allow the logging and reporting of early measurements for logged MDT purposes. RAN 2 is requested to discuss the following choices:

- The network configures the location information in the early measurements. UE reports the early measurements together with location information. Note that, in the early measurement/idle mode measurements, UE reports a single latest measurement instance.

- The network can use a flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes. Upon such an indication, UE can log measurements on non-cellReselection (carrier frequencies not part of SIB4).  AreaConfig and/or InterFreqTargetInfo can be used for filtering of SIB4 and non-SIB4 frequencies.

- The network may configure a separate report for logging early measurements. This report logs the measurements on carrier frequencies configured in early/idle mode measurements. This can also have different logging periodicity as compared to loggedMeasurementConfiguration.

[4], Ericsson

Proposal 3	Measurements related to the carrier frequencies configured in the early measurement configuration are logged under logged MDT reports.

Proposal 4	OAM should be able to control the inclusion or exclusion of measurements based on early measurement configuration in the logged MDT report.

Proposal 5	OAM needs to be able to distinguish between measurements included in the logged MDT report based on the early measurement configuration and the measurements that are included based on cell reselection SIBs.

Proposal 6	Measurement values related to Early measurement carriers are incorporated into logged MDT report irrespective of the qualityThreshold criterion configured in the early measurement configuration.

[5], ZTE

Proposal 4: RAN2 support logging measurement on frequencies/cells as configured for Early measurements in logged MDT.

Proposal 5: AreaConfiguration is enhanced to include the frequencies/cells as configured for Early measurements (i.e.  without checking  the frequencies broadcast in system information) to support logging measurement on the corresponding frequencies/cells configured.

[6], Huawei

Proposal 4: UE includes the early measurement results to the logged MDTs.

[7], Samsung

Proposal 1	Introduce support for including detailed location information when reporting early measurements

[8], Samsung

Proposal 1	Support logged MDT for non-SIB4 frequencies in R17

### 2.2	Discussion

Among all contributions, [2] and [3] have some candidate solutions so that both are suggested for discussions.

[2], OPPO

Proposal 1: RAN2 to agree that down selection from following three options should be made to let the network clearly understand the reason of expiration of the T331 or the mismatch between the camped cell and the validity area list why early measurement results are not included in the logged measurement report anymore:

- Option A: the configuration of the validityAreaList and/or the validityCellList and measIdleDuration-r16 IE is included in the logged measurement report.

- Option B: include an indication in a new LogMeasInfo-r16 entry in the logMeasReport-r16 IE indicating that early measurement results are not included in the log measurement report from now on (corresponding to the time stamp included in the entry) due to a reason other than cell-level signal not hearable.

- Option C: include indications in all related LogMeasInfo-r16 entries in the logMeasReport-r16 IE indicating that early measurement results are not included in the log measurement report due to a reason other than cell-level signal not hearable.

For above proposal, please companies provide your comments in the following table.

| Company | Preferred option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | We think that QC paper [3] provides some basic solutions, so it can be discussed first. And we have provided our preferences and analysis for [3]. The above propsoal 1 seems to be about more details on top of [3] (or a further step). |
| vivo | See comments | We agree that early measurements could be re-used for logged MDT purpose, when available. If we want to indicate something to NW for MDT purpose, then the indication must be relevant to issues that need to be fixed. Since this type of measurements are not mandatorily available, we wonder why should NW be aware of the reason that the early measurements are absent (or partly absent) in the delivered report. |
| oppo | Either | If not specific reason to be known by the network, we believe the network needs to at least know the reason of previously logged early measurement results being absent in the following logged measurement entries is due to weak signal level or not. Correspondingly, the MNO could choose to optimize the deployment of SN or not, which is vital. |
| Qualcomm |  | Our understanding is same as Huawei, once we agree on general framework, we can discuss the details. In our proposal, we outlined all of the possible choices and based on the option we select the discussion can be totally be in different direction. |
| Nokia, Nokia Shanghai Bell | None | In our understanding Logged MDT measurement results have different purpose than logging early measurement and they are used differently. Logging early measurements enable the network to configure CA/DC as soon as possible – thus they are taken by the network for decision, while logged MDT are for offline postprocessing. |
| Ericsson |  | We see some benefits of option-B. However, we believe it is too early to discuss the details related to the signalling (i.e., the proponent discusses about the new LogMeasInfo-r16 and we believe that one can discuss the exact fields to be used at a later stage). We also agree with Qualcomm and Huawei that these proposals are too detailed for the first set of agreements on this topic! |
| Samsung |  | Same view as expressed by Huawei. More specifically, we think following proposals are best discussed first (supported by more than one company): - Support logging of results available based on early measurement reporting (EMR) i.e. for freqs not in SIB4/ SIB5 <QC, E///, Hua, ZTE, Sam> - Configuration of logging of EMR can be done based on OAM i.e. need not require SN configuration <E///, Sam> - Support reporting of detailed location for EMR (as reported in UEInformationResponse or RRCResumeComplete when network requests EMR by idleModeMeasurementReq) <QC, Sam> |
| ZTE | None | We share the same concerns from other companies that the proposals now on the table might be too detailed to agree at this stage. In our understanding, the intention of logging early measurement is to allow UE to perform measurements on frequency configured for early measurements based on logged MDT configuration, and it is possible for logged MDT to configure separately the validtyArea that is different from that is for early measurement based on NW’s requirement. |
| CATT |  | We have the same view as Huawei, the details can be discussed after the basic solutions. |
| Apple |  | We share Huawei’s understanding that we’d better discuss the framework first. |
| Sharp | None | We share the same view with Huawei and Qualcomm, we should discuss general proposals first, all this options are details. |

Proposal 2: RAN2 to agree that down selection from following two options should be made to let the network clearly understand the reason why some previously recorded early measurement results are missed in the following logged measurement entries is the updating of the early measurement configuration:

- Option A: include the updated carrier frequency information (measIdleCarrierListEUTRA and/or measIdleCarrierListNR IE) in the logged measurement report and add a new indication to mark the timestamp from which the updated configuration becomes valid.

- Option B: include an indication for the carrier frequencies for which the early measurements are no longer performed due to updating of the configuration of the early measurement.

For above proposal, please companies provide your comments in the following table.

| Company | Preferred option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | We think that QC paper [3] provides some basic solutions, so it can be discussed first. And we have provided our preferences and analysis for [3]. The above propsoal 1 seems to be about more details on top of [3] (or a further step). |
| vivo |  | Please see comments for P1. |
| oppo | either | See comments for P1. |
| Qualcomm |  | Our understanding is same as Huawei, once we agree on general framework, we can discuss the details. In our proposal, we outlined all of the possible choices and based on the option we select the discussion can be totally be in different direction. |
| Nokia, Nokia Shanghai Bell | No | We think Nw can use Early measurements logging if it has available together with Logged MDT results but see no urgent need to change the two frameworks |
| Ericsson |  | Here also we see some benefits of option-B but we can discuss this proposal after agreeing on [3] |
| Samsung |  | Similar view as expressed by Huawei and Qualcomm |
| ZTE | None | Those proposals can be postponed until we figure out the general framework. |
| CATT |  | We have the same view as Huawei, the details can be discussed after the basic solutions. |
| Apple |  | Same as above. |
| Sharp |  | Same comment to P1. |

Summary:

Most of companies think that P1&P2 of [2] are details so that [3] can be discussed first. So it is suggested to not make any conclusions regarding [2] for this meeting.

[3], Qualcomm

Proposal 1: Introduce the UE capability to indicate whether UE supports logging of non-SIB4 frequencies, i.e., UE capability to indicate UE can perform logging of early measurements.

For above proposal, please companies provide your comments in the following table.

| Company | Agree or not? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Generally ok and it is related to solutions (i.e. P2 as below). |
| vivo | See comments | Before discussing whether we should introduce the UE capability, we need to first discuss the benefits of supporting logging of non-SIB4 frequencies, if the benefits are acknowledged by the majority, we can come back to this proposal. |
| oppo | Yes |  |
| Qualcomm | Yes | It is needed if we want UE to collect non-SIB4 frequencies in the logged MDT report. |
| Nokia, Nokia Shanghai Bell | Agree with vivo |  |
| Ericsson | UE capabilities should be discussed after discussing the newly added report contents (See comments) | We think both non-SIB4 and non-SIB5 should be considered because the early measurement configuration included both intra-RAT (for CA and DC) and inter-RAT (for DC) related configuration. MeasIdleConfigSIB-r16 ::= SEQUENCE { measIdleCarrierListNR-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16 OPTIONAL, -- Need S measIdleCarrierListEUTRA-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16 OPTIONAL, -- Need S ... } MeasIdleConfigDedicated-r16 ::= SEQUENCE { measIdleCarrierListNR-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierNR-r16 OPTIONAL, -- Need N measIdleCarrierListEUTRA-r16 SEQUENCE (SIZE (1..maxFreqIdle-r16)) OF MeasIdleCarrierEUTRA-r16 OPTIONAL, -- Need N measIdleDuration-r16 ENUMERATED{sec10, sec30, sec60, sec120, sec180, sec240, sec300, spare}, validityAreaList-r16 ValidityAreaList-r16 OPTIONAL, -- Need N ... } We are fine with introducing new capability but we believe first we need to discuss the feature as such and then discuss the need for one or more capability bits. So, we are supportive of UE logging non-SIB4 and non-SIB5 frequencies in logged MDT but we can discuss the new UE capabilities later (we don’t want to introduce different capability for non-SIB4 and non-SIB5 etc. Therefore, lets discuss the report contents first). |
| Samsung | Yes | We think logging of the frequencies measured for EMR purposes is equally relevant than for other/ camping frequencies, both for intra-freq (outside SIB4) and inter-RAT (outside SIB5) We agree with Ericsson that discussion of UE capability is secondary |
| ZTE | Can be postponed | Capability issue is normally discussed after detailed procedure is defined. |
| CATT | Yes |  |
| Apple | Yes |  |
| Sharp | Yes, but | Agree with other companies that the capability can be discussed later when the contents/procedure of early measurement report for logged MDT is clear enough. |

Summary:

Capability issue can be discussed after detailed solution is defined.

Proposal 2: Allow the logging and reporting of early measurements for logged MDT purposes. RAN 2 is requested to discuss the following choices:

- The network configures the location information in the early measurements. UE reports the early measurements together with location information. Note that, in the early measurement/idle mode measurements, UE reports a single latest measurement instance.

- The network can use a flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes. Upon such an indication, UE can log measurements on non-cellReselection (carrier frequencies not part of SIB4).  AreaConfig and/or InterFreqTargetInfo can be used for filtering of SIB4 and non-SIB4 frequencies.

- The network may configure a separate report for logging early measurements. This report logs the measurements on carrier frequencies configured in early/idle mode measurements. This can also have different logging periodicity as compared to loggedMeasurementConfiguration.

For above proposal, please companies provide your comments in the following table.

| Company | Preferred option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | ii and iii | For ii (or option 2), generally we think it is simple and we have the following comments. Firstly, the option 2 does not have much impacts on both logged measurements and early measurements. Secondly, we understand that there should be no impacts on measurements behaviours, i.e. the UE only needs to put the early measurements in logged measurements. In general, the potential impacts due to the option 2 are: - Stage-2 description - The flag from network side and UE capability definition For iii (or option 3), we see some benefits (e.g. flexibility) compared with option 2, and it also has some more complexities. We are open for it. [Huawei2] As we know that early measurements is a feature between UE and RAN, and it is mainly used for CA/DC setup purpose. In other words, the existing early measurements has nothing to do with CN/OAM. For i, regarding how to do it, there are two understandings in our understanding: - Keep the current framework for the early measurements, and just add location information into it. Then we wonder the benefits of doing it - Extend the early measurements to be more like logged MDT. If so, we think there may be lots of work to do |
| vivo | Option 1 | Option 1 is a straight-forward choice, NW could configure location information in the early measurement configuration, and upon reception of the early measurement with location tagged, gNB should save the received early measurement to Trace Record. |
| oppo | ii and iii | Regarding option 1, the reason why location information should be used as tag is not obvious. Further clarification is needed. |
| Qualcomm | Option 1 is simplest with narrow scope. While option 2 and option 3 can provide some wider scope with some added complexity. | Here are the trade-offs of different choices: - Option 1 is the simplest choice, - Pro: there is limited UE and BS impact. UE report location information in the early measurements and based on early measurements, the network can establish the relationship between logged MDT and early measurements. - Cons: In early measurements a single measurement instance is reported, not very useful considering idle/inactive mode optimizations - Option 2 has limited UE and network impact - Pro: UE can log both non-SIB4 and SIB4 frequencies in the logged MDT report. Network can exploit this to have a better coverage optimization. - Cons: Does not provide the time/space multiplexing for common frequencies for common frequencies of SIB4 and non-SIB4 - Option 3 has high UE and network impact - Pro: Can provide the framework for exploiting space and time multiplexing for common frequencies of SIB4 and non-SIB4. - Cons: Extra complexity. Extra memory. |
| Nokia, Nokia Shanghai Bell | See comments | The two frameworks have different purposes. Relationship between Early measurements and Logged MDT measurement results would imply the possibility to analyse radio performance for mobility vs. CA/DC setup. We understand this use case is not urgent in the WI. Both frameworks can work and can be used independently. |
| Ericsson | ii with changes. iii can be postponed | For ii, we see this as an option for what to log and when to log. We would like to clarify two things. - add non-SIB4 and non-SIB5 frequencies for the same reason as explained in previous question. - We believe the following flag introduced (‘The network can use a flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes’) is in logged MDT configuration. If this is the correct understanding, then we would like rephrase ii as follows. The network can use a flag in logged MDT configuration to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes. Upon such an indication, UE can log measurements on non-cellReselection (carrier frequencies not part of SIB4 or SIB5). AreaConfig and/or InterFreqTargetInfo can be used for filtering of SIB4 and non-SIB4 frequencies. For iii, we see this as an option for how to log. We believe there could be benefits of introducing explicit logs within the logged MDT report to capture the early measurement frequency related measurements. However, we believe it is too early to discuss the details. We think that it is good to first discuss what measurements (as discussed in option-ii) to be logged based on early measurement configuration in the logged MDT and then we could discuss how to log it. |
| Samsung | Option i and ii | We think i and ii are independent so we think it is no need to chose - Option i: We think this is useful and straightforward (merely doing for measurements reported upon transition to connected same as done for regular RRM) - Option ii: We think this is useful, straightforward and supported by several companies - Option iii: We think this is an extension to ii but we see no real need to transfer the results of these frequencies separately (i.e. it will introduce additional changes/ complexities for which we see no real need). Anyhow, can be concluded later A regarding some further details of ii: In case InterFreqTargetInfo is used, network can explicitly indicate the non-SIB4 frequencies it wants results for. For other cases, Otherwise, a flag in logged MDT configuration is needed to indicate interest to have such results |
| ZTE | Option ii with clarification | First we’d like not to impact early measurement structure because it might need cross-WI coordination since this function is used by DCCA mainly, and we shall go with available options that can be fully decided within our WI. As we explained in previous comments we think NW shall be allowed to configure differently validArea for logging of early measurements in logged MDT from that for early measurement. For logging for logged MDT purpose, UE shall follow the configuration specified via loggedMeasurementConfiguration not that for early measurement. Also we are wondering if the indication is really needed, since NW shall be able to configure the frequencies directly in e.g., AreaConfiguration, presence of such frequencies/cells can be an implicit indication that UE is supported to log measurement other than that indicated in SIB4 or SIB5. |
| CATT | ii | For option i, we think it is inappropriate to change the original early measurement configuration for MDT purposes. For option ii, we can agree to include the early measurement results in logged measurement for non-SIB4 frequencies to have a better coverage optimization. For option iii, extra complexity and memory are introduced. |
| Apple | FFS | Though Option 1 is the simplest, it requires a mandatory location reporting from UE side and user privacy is a major concern to us. |
| Sharp | ii | Agree with Ericsson’s understanding of ii and iii. |

Summary:

11 companies provided comments.

The following table summarizes companies’ opinions. It is noted that some companies did not provide explicit preferences and they may need more time to double check them. It is noted that some discussions during emails are also captured here.

|  | Option 1 | Option 2 | Option 3 |
| --- | --- | --- | --- |
| Supports | Vivo, Samsung (2/11) | Huawei, OPPO, Ericsson, Samsung, ZTE, CATT, Sharp (7/11) | Huawei, OPPO, Ericsson (3/11) |
| Clarifications | [Vivo] upon reception of the early measurement with location tagged, gNB should save the received early measurement to Trace Record. [CATT] we think it is inappropriate to change the original early measurement configuration for MDT purposes. [Apple] Though Option 1 is the simplest, it requires a mandatory location reporting from UE side and user privacy is a major concern to us. [Ericsson] Enhancing the early measurement report with location information itself is not part of the ‘logged MDT enhancements’ work. | [Ericsson] What to log and when to log. add non-SIB4 and non-SIB5 frequencies; the flag is introduced in logged MDT configuration. [Samsung] In case InterFreqTargetInfo is used, network can explicitly indicate the non-SIB4 frequencies it wants results for. For other cases, Otherwise, a flag in logged MDT configuration is needed to indicate interest to have such results [ZTE] we’d like not to impact early measurement structure, and NW shall be allowed to configure different validArea for EM in MDT and legacy EM. [Ericsson] the OAM can include a flag in the logged MDT configuration that OAM is interested in EMR related measurements in the logged MDT report | [Ericsson] how to log. See some benefits but too early to discuss the details. |
| Pros | there is limited UE and BS impact. UE report location information in the early measurements and based on early measurements, the network can establish the relationship between logged MDT and early measurements. | UE can log both non-SIB4 and SIB4 frequencies in the logged MDT report. Network can exploit this to have a better coverage optimization. | Can provide the framework for exploiting space and time multiplexing for common frequencies of SIB4 and non-SIB4. |
| Cons | In early measurements a single measurement instance is reported, not very useful considering idle/inactive mode optimizations | Does not provide the time/space multiplexing for common frequencies for common frequencies of SIB4 and non-SIB4 | Extra complexity. Extra memory. |

In addition to the above summary table, there are also some opinions for solutions:

- Option 1 and option 2&3 are independent. (Nokia, Samsung)

- Option 2 and 3 are for different purposes. (Ericsson)

From email rapporteur’s point of view, option 2 has lots of supports (7/11) so it is suggested to progress on it (with some clarifications). Option 3 has some benefits as well as complexities and only , so it is suggested to not conlcude on it for now. For option 1, it may need some discussions in order to clarify the meaning.

So it is proposed:

Proposal 1: Agree on option 2:

- The network can use a flag in logged MDT configuration to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes. Upon such an indication, UE can log measurements on non-cellReselection (carrier frequencies not part of SIB4 or SIB5).  AreaConfig and/or InterFreqTargetInfo can be used for filtering of SIB4 and non-SIB4 frequencies.

Proposal 2: Option 1 is enhancing the early measurement report with location information. It is proposed to clarify the usage before the details of solutions:

- Alternative 1: RAN is to use the EMR and it follow legacy framework of EMR

- Alternative 1a: TCE/OAM is to use the EMR and it is left to network implementation how RAN and TCE/OAM exchange information

- Alternative 1b: TCE/OAM is to use the EMR and it needs further discussions and it may be related to TRACE functionality

As mentioned in the above summary table, for option 1, some companies think it should not couple with logged MDT enhancements. So it is also proposed to decide on it.

Proposal 3: Option 1 is not part of the ‘logged MDT enhancements’ work.

If you have other comments, e.g. other proposals (not covered by above proposals) also need discussions, please add the comments in the following table.

| Company | Comments |
| --- | --- |
| Samsung | As indicated in response to first question, we hope its possible to make one more agreement - Configuration of logging of EMR can be done based on OAM i.e. need not require SN configuration <E///, Sam> |

Summary:

Only one company provided comments, and it is suggested to firstly progress on [3].

## 3	Conclusion

Based on discussions in section 2, it is proposed:

Proposal 1: Agree on option 2:

- The network can use a flag in logged MDT configuration to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes. Upon such an indication, UE can log measurements on non-cellReselection (carrier frequencies not part of SIB4 or SIB5).  AreaConfig and/or InterFreqTargetInfo can be used for filtering of SIB4 and non-SIB4 frequencies.

Proposal 2: Option 1 is enhancing the early measurement report with location information. It is proposed to clarify the usage before the details of solutions:

- Alternative 1: RAN is to use the EMR and it follow legacy framework of EMR

- Alternative 1a: TCE/OAM is to use the EMR and it is left to network implementation how RAN and TCE/OAM exchange information

- Alternative 1b: TCE/OAM is to use the EMR and it needs further discussions and it may be related to TRACE functionality

Proposal 3: Option 1 is not part of the ‘logged MDT enhancements’ work.

## 4	Reference

- R2-2100287	Discussion on logged MDT in MR-DC	China Telecommunication	discussion	Rel-17

- R2-2101960	Consideration of logged MDT enhancements	OPPO	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

- R2-2101341	Logged measurement Enhancements	QUALCOMM INCORPORATED	discussion	Rel-17

- R2-2101418	On logged MDT related enhancements	Ericsson	discussion

- R2-2101591	Consideration on Logged MDT enhancements and early measurements	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2101697	Discussion on logged MDT enhancements	Huawei, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

- R2-2100587	Immediate MDT with MR-DC and Intermediate MDT for early measurements	Samsung Telecommunications	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

- R2-2101945	Progressing Logged MDT for R17 concerning MR-DC, IRAT and IDC	Samsung Telecommunications	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core	R2-2100588