3GPP TSG-RAN WG2 Meeting #113 electronic				R2-21xxxxx

Online, 25th Jan – 5th Feb, 2021

Agenda Item:		8.13.3	MDT

Source:			Huawei (Summary rapporteur)

Title:				Report of [AT113-e][844][NR/R17 SON/MDT]  Logged MDT part I

WI code(s):			NR_ENDC_SON_MDT_enh-Core

Document for:		Discussion and Decision

## 1	Introduction

In Rel-17 SON and MDT session, it was agreed to have the following email discussion:

- [AT113-e][844][NR/R17 SON/MDT]  Logged MDT part I (Huawei)

-	Discussion on 2.3.1 of R2-2102250

Intended outcome: Agreeable WF

Deadline: Thursday 04/02/2021

This report is to collect companies’ opinion and provide some summary proposals to move the discussion forward.

## 2	Logging early measurements

### 2.1	List of relevant proposals from contribution [1] to [8]

[1], China Telecom

Proposal 2: early measurement results can be used for logged MDT purpose under the logged MDT framework.

[2], OPPO

Proposal 1: RAN2 to agree that down selection from following three options should be made to let the network clearly understand the reason of expiration of the T331 or the mismatch between the camped cell and the validity area list why early measurement results are not included in the logged measurement report anymore:

- Option A: the configuration of the validityAreaList and/or the validityCellList and measIdleDuration-r16 IE is included in the logged measurement report.

- Option B: include an indication in a new LogMeasInfo-r16 entry in the logMeasReport-r16 IE indicating that early measurement results are not included in the log measurement report from now on (corresponding to the time stamp included in the entry) due to a reason other than cell-level signal not hearable.

- Option C: include indications in all related LogMeasInfo-r16 entries in the logMeasReport-r16 IE indicating that early measurement results are not included in the log measurement report due to a reason other than cell-level signal not hearable.

Proposal 2: RAN2 to agree that down selection from following two options should be made to let the network clearly understand the reason why some previously recorded early measurement results are missed in the following logged measurement entries is the updating of the early measurement configuration:

- Option A: include the updated carrier frequency information (measIdleCarrierListEUTRA and/or measIdleCarrierListNR IE) in the logged measurement report and add a new indication to mark the timestamp from which the updated configuration becomes valid.

- Option B: include an indication for the carrier frequencies for which the early measurements are no longer performed due to updating of the configuration of the early measurement.

[3], Qualcomm

Proposal 1: Introduce the UE capability to indicate whether UE supports logging of non-SIB4 frequencies, i.e., UE capability to indicate UE can perform logging of early measurements.

Proposal 2: Allow the logging and reporting of early measurements for logged MDT purposes. RAN 2 is requested to discuss the following choices:

- The network configures the location information in the early measurements. UE reports the early measurements together with location information. Note that, in the early measurement/idle mode measurements, UE reports a single latest measurement instance.

- The network can use a flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes. Upon such an indication, UE can log measurements on non-cellReselection (carrier frequencies not part of SIB4).  AreaConfig and/or InterFreqTargetInfo can be used for filtering of SIB4 and non-SIB4 frequencies.

- The network may configure a separate report for logging early measurements. This report logs the measurements on carrier frequencies configured in early/idle mode measurements. This can also have different logging periodicity as compared to loggedMeasurementConfiguration.

[4], Ericsson

Proposal 3	Measurements related to the carrier frequencies configured in the early measurement configuration are logged under logged MDT reports.

Proposal 4	OAM should be able to control the inclusion or exclusion of measurements based on early measurement configuration in the logged MDT report.

Proposal 5	OAM needs to be able to distinguish between measurements included in the logged MDT report based on the early measurement configuration and the measurements that are included based on cell reselection SIBs.

Proposal 6	Measurement values related to Early measurement carriers are incorporated into logged MDT report irrespective of the qualityThreshold criterion configured in the early measurement configuration.

[5], ZTE

Proposal 4: RAN2 support logging measurement on frequencies/cells as configured for Early measurements in logged MDT.

Proposal 5: AreaConfiguration is enhanced to include the frequencies/cells as configured for Early measurements (i.e.  without checking  the frequencies broadcast in system information) to support logging measurement on the corresponding frequencies/cells configured.

[6], Huawei

Proposal 4: UE includes the early measurement results to the logged MDTs.

[7], Samsung

Proposal 1	Introduce support for including detailed location information when reporting early measurements

[8], Samsung

Proposal 1	Support logged MDT for non-SIB4 frequencies in R17

### 2.2	Discussion

Among all contributions, [2] and [3] have some candidate solutions so that both are suggested for discussions.

[2], OPPO

Proposal 1: RAN2 to agree that down selection from following three options should be made to let the network clearly understand the reason of expiration of the T331 or the mismatch between the camped cell and the validity area list why early measurement results are not included in the logged measurement report anymore:

- Option A: the configuration of the validityAreaList and/or the validityCellList and measIdleDuration-r16 IE is included in the logged measurement report.

- Option B: include an indication in a new LogMeasInfo-r16 entry in the logMeasReport-r16 IE indicating that early measurement results are not included in the log measurement report from now on (corresponding to the time stamp included in the entry) due to a reason other than cell-level signal not hearable.

- Option C: include indications in all related LogMeasInfo-r16 entries in the logMeasReport-r16 IE indicating that early measurement results are not included in the log measurement report due to a reason other than cell-level signal not hearable.

For above proposal, please companies provide your comments in the following table.

| Company | Preferred option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | We think that QC paper [3] provides some basic solutions, so it can be discussed first. And we have provided our preferences and analysis for [3]. The above propsoal 1 seems to be about more details on top of [3] (or a further step). |
| vivo | See comments | We agree that early measurements could be re-used for logged MDT purpose, when available. If we want to indicate something to NW for MDT purpose, then the indication must be relevant to issues that need to be fixed. Since this type of measurements are not mandatorily available, we wonder why should NW be aware of the reason that the early measurements are absent (or partly absent) in the delivered report. |

Proposal 2: RAN2 to agree that down selection from following two options should be made to let the network clearly understand the reason why some previously recorded early measurement results are missed in the following logged measurement entries is the updating of the early measurement configuration:

- Option A: include the updated carrier frequency information (measIdleCarrierListEUTRA and/or measIdleCarrierListNR IE) in the logged measurement report and add a new indication to mark the timestamp from which the updated configuration becomes valid.

- Option B: include an indication for the carrier frequencies for which the early measurements are no longer performed due to updating of the configuration of the early measurement.

For above proposal, please companies provide your comments in the following table.

| Company | Preferred option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | We think that QC paper [3] provides some basic solutions, so it can be discussed first. And we have provided our preferences and analysis for [3]. The above propsoal 1 seems to be about more details on top of [3] (or a further step). |
| vivo |  | Please see comments for P1. |

[3], Qualcomm

Proposal 1: Introduce the UE capability to indicate whether UE supports logging of non-SIB4 frequencies, i.e., UE capability to indicate UE can perform logging of early measurements.

For above proposal, please companies provide your comments in the following table.

| Company | Agree or not? | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Generally ok and it is related to solutions (i.e. P2 as below). |
| vivo | See comments | Before discussing whether we should introduce the UE capability, we need to first discuss the benefits of supporting logging of non-SIB4 frequencies, if the benefits are acknowledged by the majority, we can come back to this proposal. |

Proposal 2: Allow the logging and reporting of early measurements for logged MDT purposes. RAN 2 is requested to discuss the following choices:

- The network configures the location information in the early measurements. UE reports the early measurements together with location information. Note that, in the early measurement/idle mode measurements, UE reports a single latest measurement instance.

- The network can use a flag to indicate if an early measurement/idle mode configuration has relevance for logged measurement purposes. Upon such an indication, UE can log measurements on non-cellReselection (carrier frequencies not part of SIB4).  AreaConfig and/or InterFreqTargetInfo can be used for filtering of SIB4 and non-SIB4 frequencies.

- The network may configure a separate report for logging early measurements. This report logs the measurements on carrier frequencies configured in early/idle mode measurements. This can also have different logging periodicity as compared to loggedMeasurementConfiguration.

For above proposal, please companies provide your comments in the following table.

| Company | Preferred option | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | ii and iii | For ii (or option 2), generally we think it is simple and we have the following comments. Firstly, the option 2 does not have much impacts on both logged measurements and early measurements. Secondly, we understand that there should be no impacts on measurements behaviours, i.e. the UE only needs to put the early measurements in logged measurements. In general, the potential impacts due to the option 2 are: - Stage-2 description - The flag from network side and UE capability definition For iii (or option 3), we see some benefits (e.g. flexibility) compared with option 2, and it also has some more complexities. We are open for it. |
| vivo | Option 1 | Option 1 is a straight-forward choice, NW could configure location information in the early measurement configuration, and upon reception of the early measurement with location tagged, gNB should save the received early measurement to Trace Record. |

If you have other comments, e.g. other proposals (not covered by above proposals) also need discussions, please add the comments in the following table.

| Company | Comments |
| --- | --- |

## 3	Conclusion

[To be added]

## 4	Reference

- R2-2100287	Discussion on logged MDT in MR-DC	China Telecommunication	discussion	Rel-17

- R2-2101960	Consideration of logged MDT enhancements	OPPO	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

- R2-2101341	Logged measurement Enhancements	QUALCOMM INCORPORATED	discussion	Rel-17

- R2-2101418	On logged MDT related enhancements	Ericsson	discussion

- R2-2101591	Consideration on Logged MDT enhancements and early measurements	ZTE Corporation, Sanechips	discussion	Rel-17

- R2-2101697	Discussion on logged MDT enhancements	Huawei, HiSilicon	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

- R2-2100587	Immediate MDT with MR-DC and Intermediate MDT for early measurements	Samsung Telecommunications	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

- R2-2101945	Progressing Logged MDT for R17 concerning MR-DC, IRAT and IDC	Samsung Telecommunications	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core	R2-2100588