3GPP TSG RAN WG2#113-e	R2-2102206

e-Meeting, 25th January - 5th February, 2021

3GPP TSG-RAN WG4 Meeting #98-e	R4-2103728

Electronic Meeting, 25 January – 5 February, 2021

Title:	LS related to RSS based RSRQ for LTE-MTC

Response to:

Release:	Release 16

Work Item:	LTE_eMTC5-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

#### Name:	Li Zhang

E-mail Address:	zhangli164@huawei.com

Attachments: -

1. Overall Description:

RAN4 has defined RSS based RSRP measurement requirements in 36.133 in Rel-16 eMTC WI.

As RSS based RSRQ measurement is not defined in 36.214, RAN4 understands that the existing criterion for cell selection and cell reselection which is based on both RSRP and RSRQ, may not be applicable when a cell is measured based on RSS. There are two options from RAN4 perspective:

- Option 1: Remove RSRQ from the cell selection and cell re-selection criterion when a cell is measured using RSS.

- Option 2: Define RSRQ for RSS measurements.

RAN4 kindly asks RAN2 to decide which option should be pursued (other options not precluded if seen suitable by RAN2), and to update their specification accordingly. RAN4 will update the RSS based measurement(s) based on RAN1/RAN2 decision, if needed.

2. Actions:

To RAN2:

RAN4 kindly asks RAN2 to decide which option should be pursued to address the cell selection and cell reselection criterion based on RSS measurements, and to update the specification accordingly.

3. Date of Next TSG-RAN4 Meetings:

TSG-RAN4 Meeting #98bis-e		  	12 Apr – 20 Apr, 2021

TSG-RAN4 Meeting #99-e		19 Apr – 27 May, 2021