3GPP TSG-RAN WG2 Meeting #113 bis electronic						R2-21xxxxx
Online, April 12 – April 20, 2021

Agenda Item:	5.4.1.3

Source: 	OPPO

Title:  	Report of [AT113bis-e][008][NR15] Other & LTE (OPPO)

Document for:	Discussion and decision

# Introduction

This document is to kick off the following email discussion:

- [AT113bis-e][008][NR15] Other & LTE (OPPO)

Scope: Treat R2-2103877, R2-2103878, R2-2104279, R2-2102905, R2-2102906, R2-2102907, R2-2102908, R2-2102903, R2-2102904, R2-2103643, R2-2103644, R2-2104234, R2-2104238,

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed-in-principle CRs.

Deadline: Schedule A

- Phase 1: collect companies’ view, by 2021-04-15 12:00 UTC;

- Phase 2: rapporteur will share summary report based on input of phase 1 for review, by 2021-04-19 18:00 UTC.

Contact Information

| Company | Email |
| --- | --- |
| Huawei | davd.lecompte@huawei.com |
| Fujitsu | ohta.yoshiaki@fujitsu.com |
| MediaTek | Chun-fan.tsai@mediatek.com |
| ZTE | liu.jing30@zte.com.cn |
| Samsung | sy0123.jung@samsung.com |
| Nokia | amaanat.ali@nokia.com |
| OPPO | wangshukun@oppo.com |
| NTTDOCOMO | tianyang.min.ex@nttdocomo.com |
| Apple | yuqin_chen@apple.com |
| Qualcomm | mambriss@qti.qualcomm.com |
| Ericsson | pradeepa.ramachandra@ericsson.com |
| Intel | Sudeep.k.palat@intel.com |

# Discussion

Companies are requested to add their comments for each of the treated CRs of this email discussion in the boxes below.

## Cell ID

[1] R2-2103877	Clarification on CGI reporting	Apple	draftCR	Rel-15	38.331	15.13.0	F	NR_newRAT-Core

[2] R2-2103878	Clarification on CGI reporting	Apple	draftCR	Rel-16	38.331	16.4.1	A	NR_newRAT-Core

In [1][2], the company wants to clarify that the SN node cannot configure the CGI reporting for all MR-DC case.

Q1: Do companies agree the changes of the CR in [1][2]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | almost | 1st change: suggest "In (NG)EN-DC and NR-DC, network does not configure report of type ReportCGI-EUTRA for the SCG." 2nd change: "In MR-DC" is good but no need for the other change |
| MediaTek | Intention is okay | The wording from Huawei is better. |
| ZTE | Agree | We share the comments from HW. |
| Samsung | Agree | Same view as Huawei |
| OPPO | Yes | We share the comments from HW. |
| Apple (proponent) | Agree | We can accept with Huawei suggestion. |
| QCOM | Yes |  |
| Ericsson | Agree | Wording proposal from Huawei is better. |
| Intel | Yes | We are OK with the changes and also the alternative proposed by Huawei to use “network” instead of SN or S_gNB. |

[3] R2-2104279	Discussion on ambiguity of cell ID in RAN sharing	vivo	discussion	Rel-15	NR_newRAT-Core

In [3], the company thinks it doesn’t make clear which cellIdentity will be used for SI validity check in TS 38.331. When UE performs SI validity checking, it needs to compare the cellIdentity received from the serving cell with the cellIdentity of the stored version of the SIB in UE. However, there may be many cellIdentities for RAN sharing cells since there are more than one PLMNs in the cell and one PLMN may correspond to one cellIdentity.

1> delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid; 1> for each stored version of a SIB: 2> if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell: 3> if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell; 2> if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell: 3> if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell;

So the company proposes:

Proposal: The cellIdentity corresponding to the first PLMN in a cell’s PLMN list combination should be taken into account while judging the validity of stored cell-specific SIBs.

Q2: Do companies agree the above proposal copied from [3]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | yes |  |
| MediaTek | Yes |  |
| Samsung | No strong view | Agree with the intention, but as the PLMN ID cannot be included twice, it seems obvious to use the cell ID corresponding to the first PLMN ID in the PLMN-IdentityInfoList for SI validity checking. We believe that it's editorial issue so this clarification may be included in Rap CR if needed. |
| Lenovo | No | There is already the definition below in TS 38.331 that the cellIdentity refers to the first PLMN: Global cell identity: An identity to uniquely identifying an NR cell. It is consisted of cellIdentity and plmn-Identity of the first PLMN-Identity in plmn-IdentityList in SIB1. |
| Apple | No strong opinion | We generally agree with the intention. |
| Qcom | No | The proposed change seems to be contra-intuitive because the UE may prefer to use the selected PLMN. No need to specify the UE behaviour to address such corner case, it should be left to UE implementation. |
| Intel | Yes |  |
| Ericsson | Yes | Change is non-controversial/editorial, and can be made in Rapporteur CR. |

## SMTC

[4] R2-2102905	Clairifcation on field descritpion of SMTC in ReconfigurationWithSync for NR-DC	OPPO	CR	Rel-15	38.331	15.13.0	2484	-	F	NR_newRAT-Core

[5] R2-2102906	Clairifcation on field descritpion of SMTC in ReconfigurationWithSync for NR-DC	OPPO	CR	Rel-16	38.331	16.4.1	2485	-	A	NR_newRAT-Core

In [4][5], the company wants to clarify the field description of SMTC in ReconfigurationWithSync. In RAN2#110e meeting, SMTC issue for NR PSCell addation in NR-DC is discussed and SMTC is missing for this case. So CRs [R2-2006334/R2-2006245] are agreed. RAN2 agreed that SMTC in ReconfigurationWithSync is used for PSCell addition only when PSCell addition is for NR-DC case. However, the field descriptioin of SMTC in ReconfigurationWithSync is not clear.

Q3: Do companies agree the changes of the CR in [4][5]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Nokia | Not essential | Without the change the behavior is still correct, so we don’t see any need to change. |
| Huawei, HiSilicon | Partially | The possible misunderstanding in current specification is that this field could be used for PSCell addition for (NG)EN-DC. Adding "in MR-DC" as in the proposed CR is not useful and makes the sentence more difficult to read. Adding "in NR-DC" as proposed is ok but the coversheet needs improvement: - the CR affects (NG)EN-DC and not NR-DC - the reason for change and consequences if not approved should say that NR PSCell addition when the UE is in E-UTRA RRC_CONNECTED could fail if the UE expects the field to be set but this is not done by legacy networks. |
| MediaTek | Partially | Intention is fine. We think only adding “in NR-DC” may be needed as commented by Huawei. But indeed the change is not critical, and we are fine not to change it. |
| ZTE | See comments | It is ok to add the “for NR-DC”, but the addition of “for MR-DC” is incorrect. Because NE-DC is not applied. If companies think clarification is really needed. We suggest to reword it as below: “The SSB periodity/offset/duration configuration of target cell for NR PSCell change, and NR PCell change in case of (NG)EN-DC and NR-DC, and for NR PSCell addition in case of NR-DC.” |
| Samsung | No | Intention is correct but we think there is no room for misunderstanding. |
| OPPO | Yes | The CRs intends to make the field description clear. And we can agree with ZTE comments. |
| Apple | Partly | Agree with Huawei. We are fine with adding “for NR-DC”. |
| QCOM |  | It seems an unnecessary details as current wording are clear enough, but we will go with the majority |
| Ericsson | No | Non-functional clarification that the PSCell addition case covers only NR-DC, which is technically correct, but not really needed. At least the part with “for MR-DC” is not needed, as PSCell change is only for MR-DC. |
| Intel | Yes, but | Agree with Huawei that cover page should be updated – the summary of change, impact analysis and consequences if not approved should provide technical details. |

[6] R2-2102907	Clairifcation on usage of SMTC in the measObjectNR if not configured	OPPO	CR	Rel-15	38.331	15.13.0	2486	-	F	NR_newRAT-Core

[7] R2-2102908	Clairifcation on usage of SMTC in the measObjectNR if not configured	OPPO	CR	Rel-16	38.331	16.4.1	2487	-	A	NR_newRAT-Core

During handover, or NR PSCell addition, or NR PSCell change, the SMTC will be used to search the target cell. If the SMTC is not configured, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. However, there are two SMTC in measObjectNR, i.e. smtc1 and smtc2. It is not clear which SMTC will be used. In [6][7], the company wants to clarify how to choose SMTC to use in measObjectNR.

Q4: Do companies agree the changes of the CR in [6][7]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | This description is in MeasObjectNR, there is no need to duplicate it |
| MediaTek | No | This is over specified and it is too late to have additional UE requirement. |
| ZTE | No | We understand it is up to UE implementation whether to use smtc1 or smtc2. Is it necessary to capture such details of UE implementation in specification? In addition, this is for redirection that UE will search all candidate cells on the given frequency. Should it be more suitable and simpler for UE to only use smtc 1 (larger periodicity)? [OPPO] there is no wording to say only smtc1 is used. It is not clear, so we need the CR. |
| Samsung | No | We think no further clarification is needed. |
| OPPO | Agree | We think it is better to make it clear from UE point of view. |
| Apple | No | Not essential. |
| QCOM | No | The spec is clear enough, and since the spec didn’t specify which SMTC to be used (SMTC1 vs SMTC2), it should be left to the UE implementation on selecting a suitable SMTC. |
| Ericsson | No | This is not needed. |

[8] R2-2102903	Clairifcation on SCell without SSB	OPPO	CR	Rel-15	38.331	15.13.0	2482	-	F	LTE_NR_DC_CA_enh-Core

[9] R2-2102904	Clairifcation on SCell without SSB	OPPO	CR	Rel-16	38.331	16.4.1	2483	-	A	NR_newRAT-Core

In R15, the feature of SCell without SS/PBCH blcok is supported. So the absoluteFrequencySSB IE in FrequencyInfoDL IE can be absent for SCcell addition case. And all SSB related configuration should be absent, of course including SMTC in SCellConfig.

The SMTC maybe configured for NR SCell addition and it is obvious that the SMTC can be configured only for the the SCell with SSB case.

In [8][9], the company wants to clarify that SMTC for NR SCell addition is only configured when the target SCell is with SS/PBCH block case. And if absoluteFrequencySSB IE in FrequencyInfoDL IE is absent, SMTC in SCellConfig is also absent.

Q5: Do companies agree the changes of the CR in [8][9]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | This specification is clear and the proposed text is unclear |
| MediaTek | Not essential | Intention is fine but we don’t think change really needed as there should be no confusing in implementation. But we are fine to have this if majorities prefer. |
| ZTE | See comment | The changes are editorial, the second change is not needed. The first change can be revised as: “The SSB periodicity/offset/duration configuration of target cell for NR SCell addition if the SCell is configured with SSB”. And prefer to merge it into the rapporteur CR. |
| Samsung | No | No room for misunderstanding |
| OPPO | Yes | For SCell without SSB, the SMTC can not be configured. We can accept the change from ZTE. For the second change, it is for the SCell without SSB and SMTC will not be configured. So the change is correct. |
| Apple | No | Not essential. |
| Qcom |  | We support the intention, but not sure if needed … will go with majority |
| Ericsson | No | This is not needed. |
| Intel | No | We don’t think it is essential correction. |

## CSI measurement

[10] R2-2103643	Clarification of CSI measurement configuration	Ericsson	CR	Rel-15	38.331	15.13.0	2517	-	F	NR_newRAT-Core

[11] R2-2103644	Clarification of CSI measurement configuration	Ericsson	CR	Rel-16	38.331	16.4.1	2518	-	A	NR_newRAT-Core, TEI16

The field descriptiions for csi-IM-ResourceSetList and nzp-CSI-RS-ResourceSetList (IE CSI-ResourceConfig) indicates that they are used for “beam measurement and reporting in a CSI-RS resource set”. This is not complete since they are used also for other CSI measurements.

In [10][11], the company propose to clarify the wording.

Q6: Do companies agree the changes of the CR in [10][11]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Nokia | Yes | This is purely editorial and could be merged to rapporteur’s corrections. |
| Huawei, HiSilicon | Yes | Agree with Nokia |
| MediaTek | Yes | Agree to be put in rapporteur’s CR. |
| ZTE | Yes | Agree with Nokia |
| Samsung | Yes | The change is correct and it can be merged into Rap CR. |
| OPPO | Yes |  |
| Apple | Yes | Agree with Nokia. |
| Qcom | Yes | Change make sense |
| Ericsson | Yes (proponent) |  |

## RRC full config

[12] R2-2104234	Clarification on RRC full config for intra-SN PSCell change	NTT DOCOMO, INC., Ericsson, Fujitsu, Huawei, HiSilicon	CR	Rel-15	36.331	15.13.0	4638	-	F	NR_newRAT-Core

Moved from 5.4.1.1

[13] R2-2104238	Clarification on RRC full config for intra-SN PSCell change	NTT DOCOMO, INC., Ericsson, Fujitsu, Huawei, HiSilicon	CR	Rel-16	36.331	16.4.0	4639	-	A	NR_newRAT-Core

Moved from 5.4.1.1

In current TS36.331, for (NG)EN-DC, it is specified that if the target SgNB cannot comprehend the NR SCG configuration provided by the source SgNB, the MN may release and add the NR SCG part of the configuration and generates drb-ToRleaseList for the SN terminated RBs towards the UE. In [12][13], the companies think it is unclear whether the rule above applies only for inter-SN PSCell change or both inter-SN and intra-SN PSCell change and propose that it applies only to the inter-SN PSCell change case.

Q7: Do companies agree the changes of the CR in [12][13]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Nokia | No | As per discussion in R2-1800845 and the tdoc here it is clear that the fullConfig is indicated to MN even for the intra-SN case then why MN should not generate this to keep same behavior as the other case? Also, the interop statements are incorrect as there is no need to involve UE actions here. Nothing broken according to current spec. i.e. same behaviour for the intra-SN Pscell Change and inter-SN Pscell change. |
| Huawei, HiSilicon | Yes |  |
| Fujitsu | Yes | As per discussion in R2-1714228, the agreement is only about inter-SN PScell change. RAN2 didn’t agree on intra-SN PScell change. The specification should be corrected accordingly. 1: In case the target eNB understands the MCG part of the configuration but the target SgNB doesn’t understand the SCG part • SN indicates to the MN that it has applied full SCG configuration • Impacted bearers in indicated in the drb-toReleaseList • MN sets the en-DC-release flag to TRUE in the LTE RRCConnectionReconfiguration message sent to the UE. |
| ZTE | Partial Yes | We are fine with the added sentence. While for “inter-SN PSCell change” case, we are actually wondering about the necessity of always generating “drb-ToReleaseList” for SN terminated RBs in LTE. From SN point of view, the target SN can anyway decode the DRB information included in radioBearerConfig of source SCG context. So if full configuration is needed for SN-CU, the target SN can also generate drb-ToReleaseList included in radioBearerConfig, to inform UE to release old SN terminated DRBs. And no problem at UE side is foreseen. We are not challenging the existing text, but would like to say: there should be no problem if E-UTRAN does not generate drb-ToReleaseList even for inter-SN PSCell change case (e.g. in case of intra-vendor scenario). |
| Samsung | Yes |  |
| OPPO | Yes |  |
| NTTDOCOMO | Yes | As Proponent. RAN3 introduced an RRC full config indication in X2-AP for intra-SN PSCell change case without informing RAN2 (R3-183923, R3-183924). It seems RAN2 hasn’t discussed the intra-SN PSCell change case. There exists following two interpretations and it is very confusing that one indication can be interpreted with two meanings without any clarification in the spec. Thus, this issue shall be clarified. Interpretation1: RRC full config indication in X2-AP indicates full config of NR SCG configuration and radio bearer config for inter-SN PSCell change case while only indicates full config of NR SCG configuration for intra-SN PSCell change case. Interpretation2: RRC full config in X2-AP indicates full config of NR SCG configuration and radio bearer configuration for both inter-SN PSCell change and intra-SN PSCell change cases. As I discussed with most of the network vendors in offline, their understanding is for intra-PSCell change case, as the PDCP termination point is not changed, there is no need to release the drb i.e. interpretation1. @Nokia I understand your understanding is interpretation2 above which can be clarified as following. We are actually fine with either way, but this issue must be clarified in the spec. “In case of (NG)EN-DC, for both inter-SN and intra-SN PSCell change, the target SgNB may be unable to comprehend the NR SCG configuration provided by the source SgNB. In such a case, release and addition may be applied for the NR SCG part of the configuration. NOTE 1: When using release and addition for the NR SCG configuration, E-UTRAN includes drb-ToReleaseList for the SN terminated RBs. |
| Apple | Yes |  |
| Qcom |  | Coversheet needs update, in “Proposed change affects” to remove the cross from “ME” |
| Ericsson | Yes (proponent) |  |
| Intel | No | The scenario where the target SN cannot comprehend the source SN configuration is only relevant for inter-SN case. This a general descriptive text and it can be generic without differentiating between intra and inter SN. The correct text is not incorrect and we don’t think it is essential to add this clarification. |

# Conclusions

Based on the discussion above, we propose:

# Reference