3GPP TSG-RAN WG2 Meeting #113 bis electronic						R2-21xxxxx
Online, April 12 – April 20, 2021

Agenda Item:	5.4.1.3

Source: 	OPPO

Title:  	Report of [AT113bis-e][008][NR15] Other & LTE (OPPO)

Document for:	Discussion and decision

# Introduction

This document is to kick off the following email discussion:

- [AT113bis-e][008][NR15] Other & LTE (OPPO)

Scope: Treat R2-2103877, R2-2103878, R2-2104279, R2-2102905, R2-2102906, R2-2102907, R2-2102908, R2-2102903, R2-2102904, R2-2103643, R2-2103644, R2-2104234, R2-2104238,

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed-in-principle CRs.

Deadline: Schedule A

- Phase 1: collect companies’ view, by 2021-04-15 12:00 UTC;

- Phase 2: rapporteur will share summary report based on input of phase 1 for review, by 2021-04-19 18:00 UTC.

Contact Information

| Company | Email |
| --- | --- |

# Discussion

Companies are requested to add their comments for each of the treated CRs of this email discussion in the boxes below.

## Cell ID

[1] R2-2103877	Clarification on CGI reporting	Apple	draftCR	Rel-15	38.331	15.13.0	F	NR_newRAT-Core

[2] R2-2103878	Clarification on CGI reporting	Apple	draftCR	Rel-16	38.331	16.4.1	A	NR_newRAT-Core

In [1][2], the company wants to clarify that the SN node cannot configure the CGI reporting for all MR-DC case.

Q1: Do companies agree the changes of the CR in [1][2]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |

[3] R2-2104279	Discussion on ambiguity of cell ID in RAN sharing	vivo	discussion	Rel-15	NR_newRAT-Core

In [3], the company thinks it doesn’t make clear which cellIdentity will be used for SI validity check in TS 38.331. When UE performs SI validity checking, it needs to compare the cellIdentity received from the serving cell with the cellIdentity of the stored version of the SIB in UE. However, there may be many cellIdentities for RAN sharing cells since there are more than one PLMNs in the cell and one PLMN may correspond to one cellIdentity.

1> delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid; 1> for each stored version of a SIB: 2> if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell: 3> if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell; 2> if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell: 3> if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell;

So the company proposes:

Proposal: The cellIdentity corresponding to the first PLMN in a cell’s PLMN list combination should be taken into account while judging the validity of stored cell-specific SIBs.

Q2: Do companies agree the above proposal copied from [3]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |

## SMTC

[4] R2-2102905	Clairifcation on field descritpion of SMTC in ReconfigurationWithSync for NR-DC	OPPO	CR	Rel-15	38.331	15.13.0	2484	-	F	NR_newRAT-Core

[5] R2-2102906	Clairifcation on field descritpion of SMTC in ReconfigurationWithSync for NR-DC	OPPO	CR	Rel-16	38.331	16.4.1	2485	-	A	NR_newRAT-Core

In [4][5], the company wants to clarify the field description of SMTC in ReconfigurationWithSync. In RAN2#110e meeting, SMTC issue for NR PSCell addation in NR-DC is discussed and SMTC is missing for this case. So CRs [R2-2006334/R2-2006245] are agreed. RAN2 agreed that SMTC in ReconfigurationWithSync is used for PSCell addition only when PSCell addition is for NR-DC case. However, the field descriptioin of SMTC in ReconfigurationWithSync is not clear.

Q3: Do companies agree the changes of the CR in [4][5]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Nokia | Not essential | Without the change the behavior is still correct, so we don’t see any need to change. |

[6] R2-2102907	Clairifcation on usage of SMTC in the measObjectNR if not configured	OPPO	CR	Rel-15	38.331	15.13.0	2486	-	F	NR_newRAT-Core

[7] R2-2102908	Clairifcation on usage of SMTC in the measObjectNR if not configured	OPPO	CR	Rel-16	38.331	16.4.1	2487	-	A	NR_newRAT-Core

During handover, or NR PSCell addition, or NR PSCell change, the SMTC will be used to search the target cell. If the SMTC is not configured, the UE uses the SMTC in the measObjectNR having the same SSB frequency and subcarrier spacing, as configured before the reception of the RRC message. However, there are two SMTC in measObjectNR, i.e. smtc1 and smtc2. It is not clear which SMTC will be used. In [6][7], the company wants to clarify how to choose SMTC to use in measObjectNR.

Q4: Do companies agree the changes of the CR in [6][7]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |

[8] R2-2102903	Clairifcation on SCell without SSB	OPPO	CR	Rel-15	38.331	15.13.0	2482	-	F	LTE_NR_DC_CA_enh-Core

[9] R2-2102904	Clairifcation on SCell without SSB	OPPO	CR	Rel-16	38.331	16.4.1	2483	-	A	NR_newRAT-Core

In R15, the feature of SCell without SS/PBCH blcok is supported. So the absoluteFrequencySSB IE in FrequencyInfoDL IE can be absent for SCcell addition case. And all SSB related configuration should be absent, of course including SMTC in SCellConfig.

The SMTC maybe configured for NR SCell addition and it is obvious that the SMTC can be configured only for the the SCell with SSB case.

In [8][9], the company wants to clarify that SMTC for NR SCell addition is only configured when the target SCell is with SS/PBCH block case. And if absoluteFrequencySSB IE in FrequencyInfoDL IE is absent, SMTC in SCellConfig is also absent.

Q5: Do companies agree the changes of the CR in [8][9]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |

## CSI measurement

[10] R2-2103643	Clarification of CSI measurement configuration	Ericsson	CR	Rel-15	38.331	15.13.0	2517	-	F	NR_newRAT-Core

[11] R2-2103644	Clarification of CSI measurement configuration	Ericsson	CR	Rel-16	38.331	16.4.1	2518	-	A	NR_newRAT-Core, TEI16

The field descriptiions for csi-IM-ResourceSetList and nzp-CSI-RS-ResourceSetList (IE CSI-ResourceConfig) indicates that they are used for “beam measurement and reporting in a CSI-RS resource set”. This is not complete since they are used also for other CSI measurements.

In [10][11], the company propose to clarify the wording.

Q6: Do companies agree the changes of the CR in [10][11]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Nokia | Yes | This is purely editorial and could be merged to rapporteur’s corrections. |

## RRC full config

[12] R2-2104234	Clarification on RRC full config for intra-SN PSCell change	NTT DOCOMO, INC., Ericsson, Fujitsu, Huawei, HiSilicon	CR	Rel-15	36.331	15.13.0	4638	-	F	NR_newRAT-Core

Moved from 5.4.1.1

[13] R2-2104238	Clarification on RRC full config for intra-SN PSCell change	NTT DOCOMO, INC., Ericsson, Fujitsu, Huawei, HiSilicon	CR	Rel-16	36.331	16.4.0	4639	-	A	NR_newRAT-Core

Moved from 5.4.1.1

In current TS36.331, for (NG)EN-DC, it is specified that if the target SgNB cannot comprehend the NR SCG configuration provided by the source SgNB, the MN may release and add the NR SCG part of the configuration and generates drb-ToRleaseList for the SN terminated RBs towards the UE. In [12][13], the companies think it is unclear whether the rule above applies only for inter-SN PSCell change or both inter-SN and intra-SN PSCell change and propose that it applies only to the inter-SN PSCell change case.

Q7: Do companies agree the changes of the CR in [12][13]?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| Nokia | No | As per discussion in R2-1800845 and the tdoc here it is clear that the fullConfig is indicated to MN even for the intra-SN case then why MN should not generate this to keep same behavior as the other case? Also, the interop statements are incorrect as there is no need to involve UE actions here. Nothing broken according to current spec. i.e. same behaviour for the intra-SN Pscell Change and inter-SN Pscell change. |

# Conclusions

Based on the discussion above, we propose:

# Reference