3GPP TSG-RAN WG2 #113bis-e	R2-21xxxxx

E-meeting, 12 – 20 April 2021

Agenda item:	6.1.4.2

Source: 	Samsung

Title: 	Discussion summary (Phase 1) - [AT113bis-e][022][NR16] IAB LTE Changes (Samsung)

Document for:	Discussion and decision

# Introduction

This tdoc summarised changes proposed under AI 6.1.4.2, corresponding to the following at-meeting discussion:

- [AT113bis-e][022][NR16] IAB LTE Changes (Samsung)

Scope: Treat R2-2102800, R2-2103558, R2-2103598, R2-2103601, R2-2104166, R2-2104177, R2-2104178

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed-in-principle CRs, if any

Deadline: Schedule A

Proposed changes in each individual submission to this AI are first briefly summarised and numbered by the rapporteur. This is followed by a proposal from the rapporteur on a way forward for each individual proposed change (or set of related changes, grouped under the same issue). Occasionally this involves asking interested companies to choose between two or more proposed alternatives. Inputs are then collected from individual companies on rapporteur’s proposals, and a way forward from the rapporteur is given. A draft CR capturing the proposed changes (and capturing companies’ input below) is provided separately.

# Detailed discussion on proposals (Phase 1)

Table below captures individual issues and accompanying information and background, before making a proposal for a way forward for each of the issues raised:

| Issue no. | Issue description and relevant tdoc(s) | Rapporteur proposal | Companies’ views on Proposals |
| --- | --- | --- | --- |
| 1 | DLInformationTransfer and ULInformationTransfer message definitions do not contain F1-C related information R2-2102800 (CATT), R2-2104166 (Huawei, HiSilicon) | It is true that message definitions in 6.2.2 do not mention F1-C. However, the DL and UL information transfer procedures in 5.6.1 and 5.6.2 clearly mention F1-C handling. This being said, it does make sense to align the procedure description and message definition. Proposal 1: Mention F1-C in relevant message definitions in 6.2.2. Actual text to be agreed during CR drafting phase. | [CATT] Agree. The procedure description and message definition should be aligned. [Ericsson]: OK [LG]: We can go with majority, but the clarification should be simple. [Intel] Agree with proposal. Besides, we are wondering whether we also need to update SRB2 definition in TS36.331 4.2.2 as: “SRB2 is for RRC messages which include logged measurement information as well as for NAS messages, IAB-DU specific F1-C related information, all using DCCH logical channel” [QC] OK [vivo] Agree. [Lenovo] Agree [Nokia]: Agree [Samsung] Agree. |
| 2 | DLInformationTransfer and ULInformationTransfer message definitions do not capture fully the agreed restriction that, for the EN-DC case, the SRB2 on LTE leg should exclusively be used for F1-C transfer R2-2103598 (Samsung/Opt A), R2-2103601 (Samsung/Opt B) | It is true that based on message definitions in 6.2.2, SRB1 transfer of F1-C message is allowed in some special instances (when SRB2 is not established yet). It should be noted that the UL information transfer procedure (5.6.2) does mention that “When F1-C related information has to be transferred, the IAB-MT shall initiate the procedure only if SRB2 is established.” However, procedure description and message information should be aligned (see also Issue 1). OptA makes it explicit that SRB1 is never used for F1-C. OptB on the other hand allows F1-C transfer via SRB1 in all cases where SRB2 transfer is not functioning. Since OptA is better aligned with existing text in 5.6.2, the following is proposed: Proposal 2: Make it clear in relevant message definitions in 6.2.2 that SRB1 is never used for F1C (not even when SRB2 is not established yet). Actual text to be agreed during CR drafting phase. | [Huawei]: It seems 38.300 has already clarified this “The F1-C message is carried over LTE RRC using SRB2 between IAB-node and MeNB and via X2AP between MeNB and IAB-donor”. In addition, we may need to first clarify if we allow to use SRB1 if SRB2 is not established yet. [CATT] We think option A is the correct intention. In 36.331 & 5.6.2.2 Initiation, it is stated “When F1-C related information has to be transferred, the IAB-MT shall initiate the procedure only if SRB2 is established.” It seems clear for UL but not clear for DL. We are fine to make it clear. [Ericsson]: It is not clear why we should have different behaviour for F1 dedicated signalling compared with other information transferred in the same UL/DLInformationTransfer. For example in 5.6.2.2, the description of F1-C transferring is the same as the one for CDMA2000, but no exception is mentioned in ULInformationTransfer description for it. So we do not believe that this change is needed. [LG]: For the F1-C over LTE RRC, the original intention is option A and it is clearly specified in section 5.6.2, i.e., “When F1-C related information has to be transferred, the IAB-MT shall initiate the procedure only if SRB2 is established.” In our understanding, if there is no established SRB2, IAB-MT shall not initiate the procedure. This means that if FI-C over LTE RRC is initiated, there should be the SRB2. Thus, there is no ambiguity/contradiction in the spec and no change would be preferred. For the Huawei’s comment (“first clarify if we allow to use SRB1 if SRB2 is not established yet.”): the relevant discussion has already done for CP-UP separation in “[Post113-e][058][IAB17]”. The clear majority is SRB2 only. It would be good to avoid duplicated discussion. [Intel] As rapporteur captured, IAB-MT initiates UL information transfer only if SRB2 is established. Moreover, in TS37.340 subclause 7.11, it clearly indicate that “SRB2 is used for transporting the F1-AP message encapsulated in SCTP/IP or F1-C related SCTP/IP packet between IAB-MT and MN [10]”. We agree that only SRB2 on LTE leg is used for F1-C transfer, while thinking the changes may not be necessary. [QC] SRB1 may be used when SRB2 is not available. [vivo] We also believe Option A is the correct understanding, according to the agreement made at RAN2#108 meeting: RAN2#108 => SRB2 is used for transport of all F1AP messages in EN-DC. As for the changes made in the CR, we tend to agree P2 makes the text clearer. [Lenovo] Prefer option A and agree with the proposal, since it has been agreed that only SRB2 can be used for F1-c transport, and option A is compatible with the description of section 5.6.2.2 in 36.331. Unless strong motivation introduced, option B is excluded. [Nokia]: This case applies to EN-DC where the NR SN has to be added before the transfer of F1-C over LTE can happen. However, we were wondering if it is possible that SRB2 is not established at that point. [Samsung] Agree. In 6.2.2 (DLInformationTransfer and ULInformationTransfer), it is not made explicitly clear that only SRB2 can be used for transfer of F1-C related information. Like several companies have noted, it is true that this is made clear elsewhere in the spec (in 5.6.2.2) where it is stated that “When F1-C related information has to be transferred, the IAB-MT shall initiate the procedure only if SRB2 is established.” However, this is only for the UL - there is no similar clarification for DL. CATT have also observed this. |
| 3 | Initiation of UL information transfer procedure via LTE leg (for the case of F1-C) is implied to be mandatory R2-2104166 (Huawei, HiSilicon) | As correctly pointed out in the relevant contribution, F1-C transfer can be performed via LTE leg or NR backhaul. This being said, the assumption should be that the existing text in 5.6.2.2 is applicable taking into account any relevant configuration, and as such should be ok as-is. However, the rapporteur believes there is benefit in agreeing the first change to 5.6.2.2. The second change is unnecessary since it refers to use of SRB2 restriction and ‘shall’ is the correct term to use. Proposal 3: Agree the first change to Section 5.6.2.2: change ‘an IAB-MT in RRC_CONNECTED initiates the UL information transfer procedure whenever…’ to ‘may initiate’. Proposal 4: Do not agree the second change to Section 5.6.2.2. | [CATT] P3: Rewrite the sentence “In addition, an IAB-MT in RRC_CONNECTED initiates the UL information transfer procedure whenever there is a need to transfer F1-C related information.” We think the yellow highlighted part imply that F1-C related information is transmitted in LTE only when needed. P4: agree P4. [Ericsson]: P3: OK P4: No strong view [LG]: Ok with the rapporteur’s proposals. [Intel] Agree with P3 and P4. [QC] OK [vivo] Agree P3/P4. [Lenovo] Agree with P3, and no strong view with P4. [Nokia] Agree P3 and P4 [Samsung] Agree P3 and P4. |
| 4 | Incomplete/incorrect handling of failure to deliver UL information transfer message, for the case where message contains F1-C info R2-2104166 (Huawei, HiSilicon), R2-2103598 (Samsung/OptA), R2-2103601 (Samsung/OptB) | The key issue here is whether IAB-DU should be informed of message failure (as proposed in R2-2103598), or not (R2-2104166, R2-2103601). Proposal 5: Agree to make it clear that upper layers are not informed of failure to deliver UL information transfer message, for the case where message contains F1-C info. Actual text to be agreed during CR drafting phase. Proposal 6: Companies to express their preference between notifying the IAB-DU of message failure (R2-2103598 – let’s call it Option A), and not notifying the IAB-DU (R2-2104166/ R2-2103601 – let’s call it Option B). Actual text to be agreed during CR drafting phase. | [Huawei]: Fine with P5. i.e. not notify IAB-DU. [CATT] P5: We can ask RAN3 if this information is needed. However, even if upper layer can do nothing with the failure information, we think the revision is an optimization and too late in Rel-16. [Ericsson]: Agree with CATT. This is just an optimization that can be left to IAB node implementation, given that Rel.16 is closed. [LG]: OK with Proposal 5, i.e., option B which is not notifying the IAB-DU. If majority want to leave it up to IAB node implementation, it is also fine to us. [Intel] We are wondering what is the scenario of this failure delivery of ULInformationTransfer? Is it considered only when MCG link is failed? [QC] P5: OK, P6: should be left to implementation [vivo] P5, ok. P6, our preference is Option A, i.e., to notify the IAB-DU of the message failure, which aligns with DLInformationTransfer case where DL f1c information is forwarded to IAB DU not upper layer. But we are also fine to leave it to IAB-node implementation if this is the majority view. [Lenovo] Prefer for Option A. [Nokia] P5:OK [Samsung] Agree to P5. Wrt P6, we prefer Option A. |
| 5 | DedicatedInfoF1c message may not always contain SCTP wrapper, based on recent RAN3 agreement. R2-2103558 (Nokia, Nokia Shanghai Bell) | The existing wording in 6.3.6 indeed could imply that F1-C related packet is always encapsulated in SCTP and a change is needed. However, if a reference to 36.423 is added (as proposed in R2-2103558), this may be sufficient to fix the issue. The rapporteur is additionally not sure that the further change proposed in R2-2103558 – to change ‘F1-C related SCTP/IP packet’ to ‘F1-C (SCTP/)IP packet is the cleanest way of implementing this. To fully avoid ambiguity, perhaps we should say something like ‘F1-C IP packet with or without SCTP encapsulation’? Proposal 7: Add reference to 36.423. Proposal 8: Companies to express their preference between change as proposed in R2-2103558 (Option A), change as proposed above by the rapporteur (Option B), or not having any further changes (Option C) on top of adding the reference as in P7. | [Huawei]: Fine with either option A or B, since this is more like text improvement. [CATT] P7: support P8: Either option A or B is OK. [Ericsson]: P7: OK P8 (Option C): We agree with Rapporteur, the brackets just add confusion. Maybe better to just introduce the reference [LG]: It’s sort of clean-up of the specification. We are ok with P7 and can live with majority for P8. [Intel] The same issue is discussed in [014] email discussion. Option A or Option B is preferred. [QC] P7: OK, P8: Should be “F1-C related IP packet”. IKE HS does not have SCTP. [vivo] P7: support. P8: Option B is better, the added brackets together with the original slash seem to be more confusing. [Lenovo] P7: Agree to add the reference. P8: Both option A and B are workable. No strong view. [Nokia] P7 OK P8 – Option A or B, with the note that if Option B is adopted the wording should be “F1-C related IP packet with or without encapsulated SCTP packet” [Samsung] P7 OK. Wrt P8, we have no preference. |
| 6 | Typo – change ‘IAB donor-CU’ to ‘IAB-donor-CU’ R2-2104166 (Huawei, HiSilicon) | ‘IAB-donor-CU’ is the correct format (see e.g. 38.300). Proposal 9: change ‘IAB donor-CU’ to ‘IAB-donor-CU’. | [CATT] Support [Ericsson]: P9 OK [LG] Fine with P9. [Intel] Agree [QC] OK [vivo] OK, text improvement [Lenovo] Agree [Nokia] P9 OK [Samsung] OK. |
| 7 | The ‘need’ code of dedicatedInfoF1c-r16 IE and dedicatedInfoType-r16 IE in the ULInformationTransfer should be removed. R2-2104166 (Huawei, HiSilicon) | As the message in question is an UL message, this makes sense. Proposal 10: Remove the ‘need’ code of dedicatedInfoF1c-r16 IE and dedicatedInfoType-r16 IE in the ULInformationTransfer. | [CATT] support [Ericsson]: P10: OK [LG] Fine with P10. [Intel] Agree [vivo] Agree [Lenovo] Agree [Nokia] OK [Samsung] OK. |

# Summary and way forward

On Proposal 1:

Proposal 1: Mention F1-C in relevant message definitions in 6.2.2. Actual text to be agreed during CR drafting phase.

All 9 respondents agree with the proposal. There is also a separate proposal from one of the companies to also modify the SRB2 definition accordingly, and the rapporteur agrees with this.

Regarding Proposal 2:

Proposal 2: Make it clear in relevant message definitions in 6.2.2 that SRB1 is never used for F1C (not even when SRB2 is not established yet). Actual text to be agreed during CR drafting phase.

4 out of 10 respondents are ok with the Proposal as-is, while 6 companies agree with the intention but feel changes are not needed since the essence is captured elsewhere in this or other specs.  3 out of these 6 companies seem to have a preference for no changes, while tolerating changes. Given all this, the rapporteur proposes that the Proposal is still implemented, since the clarifications elsewhere in the spec are only made for the UL transfer message (as noted by several companies).

With regards to Proposals 3 & 4:

Proposal 3: Agree the first change to Section 5.6.2.2: change ‘an IAB-MT in RRC_CONNECTED initiates the UL information transfer procedure whenever…’ to ‘may initiate’.

Proposal 4: Do not agree the second change to Section 5.6.2.2.

All but one respondent are ok with P3, while everyone is either ok with P4 or without a strong view. The Rapporteur proposes P3 and P4 are implemented.

With regards to Proposals 5 & 6:

Proposal 5: Agree to make it clear that upper layers are not informed of failure to deliver UL information transfer message, for the case where message contains F1-C info. Actual text to be agreed during CR drafting phase.

Proposal 6: Companies to express their preference between notifying the IAB-DU of message failure (R2-2103598 – let’s call it Option A), and not notifying the IAB-DU (R2-2104166/ R2-2103601 – let’s call it Option B). Actual text to be agreed during CR drafting phase.

7 out of 10 companies are ok with P5 and the rapporteur proposes it is implemented. Regarding P6, there is stronger opposition to making any changes and the rapporteur proposes that the specs should not mandate notifying of the IAB-DU.

On Proposals 7 & 8:

Proposal 7: Add reference to 36.423.

Proposal 8: Companies to express their preference between change as proposed in R2-2103558 (Option A), change as proposed above by the rapporteur (Option B), or not having any further changes (Option C) on top of adding the reference as in P7.

All respondents are ok with Proposal 7, while for Proposal 8, Option B seems to be the majority view (many respondents have no preference).

Proposal 9 (correcting a typo) has support of all respondents:

Proposal 9: change ‘IAB donor-CU’ to ‘IAB-donor-CU’.

Proposal 10 also has support of all respondents:

Proposal 10: Remove the ‘need’ code of dedicatedInfoF1c-r16 IE and dedicatedInfoType-r16 IE in the ULInformationTransfer.

# References

- R2-2102800	Clarification on DLInformationTransfer and ULInformationTransfer	CATT

- R2-2103558	Clarification on IP packet type in DedicatedInfoF1c	Nokia, Nokia Shanghai Bell

- R2-2103598	Transfer of F1C traffic over LTE leg in IAB - Option A	Samsung Electronics GmbH

- R2-2103601	Transfer of F1C traffic over LTE leg in IAB - Option B	Samsung Electronics GmbH

- R2-2104166	Miscellaneous corrections for TS 36.331 on F1 over LTE for IAB	Huawei, HiSilicon

- R2-2104177	Correction on ULInformationTransfer failure handling for IAB in 36.331 [Opt A]	Samsung R&D Institute UK

- R2-2104178	Correction on ULInformationTransfer failure handling for IAB in 36.331 [Opt B]	Samsung R&D Institute UK