3GPP TSG RAN WG2 #113-bise	                                       	R2-21xxxxx

e-Meeting,  12th – 20th April, 2021

Title:	(draft) Reply LS on handover with PSCell

Response to:	R4-2103674

Release:	Release 17

Work Item:	NR_RRM_enh2-Core

Source:	ZTE Corporation (To be RAN2)

To:	RAN4

Cc:

Contact Person:

Name:	Jing Liu

E-mail Address:	liu.jing30@zte.com.cn

Attachments: -

1. Overall Description:

RAN2 thanks RAN4 for the LS on handover with PSCell.

Regarding Question 1:

- Question 1: what is the RRC processing delay for following cases of handover with PSCell?

RAN2’s answer is given in below table:

| Scenario | Source PCell | Target PCell | Target PSCell | RRC procedure delay for HO with PSCell |
| --- | --- | --- | --- | --- |
| NR SA to EN-DC | NR (incl. FR1 and FR2) | LTE | NR (incl. FR1 and FR2) | [50ms] |
| EN-DC to EN-DC | LTE | LTE | NR (incl. FR1 and FR2) | 20ms |
| NE-DC to NE-DC | NR FR1 | NR FR1 | LTE | 16ms |
| NR-DC to NR-DC | NR FR1 | NR FR1 | NR FR2 | 16ms |

Regarding the RRC processing delay for NR SA to EN-DC, RAN2 understands the RRC processing delay (i.e. 50ms) defined for inter-RAT handover from NR to E-UTRAN can be applied, but it is up to RAN4 to make a final decision. And RAN2 understands RAN4 will specify the RRC processing delay of “NR SA to EN-DC” in TS 36.133. In addition, RAN2 will update TS 38.331 and TS 36.331 to clearly capture the RRC processing delay for other cases listed in above table.

Regarding Question 2:

- Question 2: Regarding HO with PSCell triggered by single RRC HO command, which of following options is in line with RAN2 definition when UE fails to synchronize to the expected PSCell?

RAN2’s answer is Option 1. In case of MN handover with PSCell change, irrespective whether the UE fails to synchronize to the target PSCell, the UE can always  access to target PCell, and if UE fails to synchronize to the target PSCell it reports SCGFailureInformation to target PCell. In this case, the SCG configuration (including the configuration for PSCell) is kept and all SCG transmissions are suspended, it is up to target MN to decide whether to reconfigure/release SCG after receiving SCGFailureInformation message from the UE.

2. Actions:

To RAN4

ACTION: 	RAN2 respectfully asks RAN4 to take above response into account in their future work.

3. Date of Next TSG-RAN2 Meetings:

3GPP  RAN2#114-e 	                19 May – 27 May 2021			Electronic Meeting

3GPP  RAN2#115-e 	                16 August – 27 August 2021			Electronic Meeting