3GPP TSG-RAN2 Meeting #113bis-e	R2-210xxxx

Online, April 12 – April 20, 2021

Title:	[Draft] Reply LS to SA3 on FBS detection

Response to:	S3-210756

Release:	Rel-17

Work Item:	FS_5GFBS

Source:	RAN2

To:	SA3

Cc:	RAN3

Contact Person:

#### Name:	Rui Wang

E-mail Address:	wangrui46@huawei.com

Attachments:

1. Overall Description:

RAN2 would like to thank SA3 for their further clarifications and feedback in the reply LS on false base station detection. Regarding SA3’s question on the feasibility, from RAN2’s perspective it would be feasible to report a list of MIB/SIBs hashes.

Regarding hash value reporting

- Reporting hashes of system information is currently not supported. The logged MDT, mobility history information and CGI reporting procedures could be modified but that requires additional work.

- Regarding MIB/SIBs reading, the existing RAN2 procedures already require UEs to read MIB and SIBs:

  - UEs in RRC_IDLE/INACTIVE state UEs read MIB/SIB1/other interested SIBs of camped cell. Calculating the hash of these MIB/SIBs is not expected to have a negative impact on power consumption. However, calculating the hash of additional SIBs (i.e. SIBs currently not acquired by the UE) will cause extra power consumption and is not preferred from RAN2 viewpoint.

  - UE in RRC_CONNECTED state UEs, when configured with CGI reporting, read MIB and SIB1 of neighbour cells. The existing CGI reporting creates interruptions whenever the UE needs to tune in to another cell. Too frequent request of CGI reporting in CONNECTED will not only cause extra power consumption but will also create frequent interruptions . Those impacts become much worse if something else than MIB/SIB1 needs to be acquired. Thus, from RAN2 viewpoint, the scope of CGI reporting (i.e. acquired MIB/SIB1) is preferred not to be expanded and the procedure should not be triggered often.

Regarding reporting reject_info and signal_info mentioned by SA3

- The reporting RSRP/RSRQ/RSSI/beam level information of SSB or CSI-RS is supported since Rel-15. And reporting connection establishment failure and radio link failure is supported in Rel-16. If other information is required to be reported, additional RAN2 work will be required.

2. Actions to SA3:

To SA3: RAN2 kindly asks SA3 to take the above information into account.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2 Meeting #114										19 - 27 May 2021, online