3GPP TSG-RAN2 Meeting #113bis-e	R2-210xxxx

Online, April 12 – April 20 2021

Title:	[Draft] Reply LS to SA3 on FBS detection

Response to:	S3-210756

Release:	Rel-17

Work Item:	FS_5GFBS

Source:	Huawei [to be RAN2]

To:	SA3

Cc:	RAN3

Contact Person:

#### Name:	Rui Wang

E-mail Address:	wangrui46@huawei.com

Attachments:

1. Overall Description:

RAN2 would like to thank SA3 for their further clarifications and feedback in the reply LS on false base station detection. Regarding SA3’s question on the feasibility, from RAN2’s perspective, it would be feasible to report the hashes of MIB/SIB(s) with the following explanations:

Reporting of hash values

- Reporting hashes of system information is currently not supported. The logged MDT, mobility history information, and CGI reporting procedures could be enhanced to introduce calculating and reporting of the hashes but this will require additional RAN2 work.

- Regarding reading of MIB/SIBs,

existing RAN2 procedures that already require UEs to read MIB and SIB(s) should be re-used. Calculating and reporting the hashes of these MIB/SIBs is not expected to have a significant negative impact on power consumption when existing measurement and reporting is re-used. However, reading additional SIB(s) (i.e. SIBs currently not acquired by the UE) and calculation of the hashes will negatively impact power consumption.

- In current specifications, the following are supported:

  - UEs in RRC_IDLE/INACTIVE state read MIB/SIB1 and other interested SIB(s) of the camped cell.

  - UEs in RRC_CONNECTED state, when configured with CGI reporting, read MIB and SIB1 of the neighbour cells. The request of CGI reporting in CONNECTED creates interruptions to the serving cell communications when the UE needs to tune in to the other cell to read MIB and SIB1. Those impacts will become worse if something else than MIB/SIB1 needs to be acquired.

Reporting reject_info and signal_info

- The reporting RSRP/RSRQ/RSSI/beam level information of SSB or CSI-RS is supported since Rel-15. In addition, reporting of connection establishment failure and radio link failure is supported in Rel-16. If other information is required to be reported, additional RAN2 work will be required.

- RAN2 does not consider any new procedures or measurements for the SA3 request other than what is described above.

2. Actions to SA3:

To SA3: RAN2 kindly asks SA3 to take the above information into account.

3. Date of Next RAN2 Meetings:

TSG-RAN WG2 Meeting #114-e										19 - 27 May 2021, online

TSG-RAN WG2 Meeting #115-e 										16 – 27 August 2021, online