3GPP TSG RAN WG2 #113b-e	R2-2104364

Electronic Meeting, 12th Apr – 20th Apr 2021

Source: 	Huawei, HiSilicon

Title: 	Summary of offline 105 - [NTN] TAC update

Agenda Item:	8.10.3.1

Document for:	Discussion and decision

# Introduction

This document is to collect companies’ views for the following offline discussion:

- [AT113bis-e][105][NTN] TAC update (Huawei)

Initial scope: Continue the discussion on based on the proposals from R2-2103628, R2-2103749 and R2-2103076, including the need to send an LS to SA2 and/or CT1

Initial intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Wednesday 2021-04-14 22:00 UTC

Initial deadline (for rapporteur's summary in R2-2104364): Thursday 2021-04-15 02:00 UTC

Proposals marked "for agreement" in R2-2104364 not challenged until Thursday 2021-04-15 14:00 UTC will be declared as agreed via email by the session chair.

For the rest the discussion will continue in a second round of the offline discussion until Monday 2021-04-19. Further details on the scope/intended outcome/exact deadlines to be announced by the session chair after Thursday 2021-04-15 14:00 UTC.

# Discussion

Issue 1: how to make UE aware of TAC change in SI.

During online discussion, we have made the agreement as follows:

Agreements:

- When the network stops broadcasting a TAC, the UE needs to know it (FFS on further details)

And we can continue discussing how to make UE know this TAC change. Currently we have two options on table:

Option 1: reuse legacy paging mechanism [1]. When the network stops broadcasting a TAC, it could send a SI change indication.

Option 2: based on validity timers related to TAIs [2]. As the broadcasted TACs may be associated with validity times, especially in Earth-moving scenario, in this case the UE knows when it needs to recheck SI.

Question 1: how to make UE know this TAC change when the network stops broadcasting a TAC?

|  | Company |  | Option 1 or 2? |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Samsung |  | 1 |  | Option 1 has the drawback of increased signaling load for paging and Option 2 has the drawback of increased SIB overhead. |  |
|  | MediaTek |  | Option-1, but |  | The legacy SI change indication method can be reused here since the SI information is changing. However, it is not necessary to always send this indication to the UE when tracking area information changes. If the cell stopped covering a tracking area on the ground, all UEs in that tracking area will no longer be served by that cell. Therefore the indication will not bring any additional benefit. |  |
|  | Huawei, HiSilicon |  | Option1 |  | Legacy mechanism can be reused. Further enhancement can be considered when it’s clear how large the Tracking Area is and how many TAs one NTN cell can cover. |  |
|  | ZTE |  | Option 1 |  | - We understand it would be sufficient to reuse the existing procedure by sending SI change notification in short message in paging DCI when the broadcast TAC changes. - We would like to focus on how to make the whole procedure work in NTN and also to minimize the impact in the existing RAN2 and CT1/SA2 specs to support NTN. Thus, unnecessary enhancements like option 2 is not preferred from our perspective so that we can spare more time for issues that really need to be solved to support NTN in the very first release. |  |
|  | Lenovo |  | Option 1 |  | We can reuse legacy mechanism as a baseline and consider further enhancement to reduce signaling load. Option 2 has more spec impact and also additional overhead. |  |
|  | OPPO |  | Option 1 |  | Using legacy paging mechanism to indicate UE the TAC change is enough. |  |
|  | Qualcomm |  | Option 2 |  | Option 1 is not efficient in moving cell scenario. Such SI update notification would be frequent, and we have concern on signaling load, resource, and power inefficiency. For option 2, if increased SIB1 size is issue, such time information can be carried in other SIB or NTN specific SIB. |  |
| Apple |  | Option 1 is preferred for lack of better alternatives |  | As mentioned by Samsung and MediaTek, one has signaling node issues while the other has increased SIB broadcast message sizes. In the current situations it is still unclear why the network would stop broadcasting SI when cell coverage is huge where there is esp. a signaling overhead concern. |  |  |
|  | Xiaomi |  | Option 1 |  | The legacy SI change indication can be reused. For the earth fixed scenario, we don’t think the gNB should send SI change indication when the TAC is stop broadcasted. We think network can trigger the SI update when the broadcast TACs changed based on network implementation. |  |
|  | Spreadtrum |  | Option 1 |  | For option 2, it is difficult to guarantee all the UE to start the timer at the same time. |  |
|  | CMCC |  | Option 1 with comments |  | Only legacy solution may not be enough as mentioned by some companies (e.g the increasing signaling overhead for SI modification), some enhancements may be needed. |  |
|  | Rakuten Mobile |  | Option 2 is preferred but can agree with Option 1 for now. |  | We agree with Qualcomm, It would be better to control the SIB Size then to overload signaling. |  |
|  | NEC |  | Option 1 with comments |  | In quasi Earth-fixed cells, cell-wide CHO may be used to move UEs before the cell stops broadcasting the TAC and UEs will not get any more information from SI change indication. We agree to use Option 1 as a baseline and enhancements to lower the signalling load should be considered. |  |
|  | LG |  | Option 1 |  | It is much simple approach to reuse existing mechanism. Regarding the preceding comments that option 1 may increase signaling load for paging, we think generally UEs should reselect to other neighbor cell before its registered TAC is deleted in the broadcast SIB. So we think it is not really critical issue. |  |
|  | ETRI |  | Option 1 |  | We prefer Option 1 as a baseline. Enhancements such as option 2 can be discussed later if necessary. |  |

Issue 2: how many TACs for one PLMN does AS layer indicate to NAS layer.

When more than one TAC per PLMN is broadcasted in NTN cell, we need to figure out how to indicate TAC to NAS layer. In current TS 38.304, we only have the basic principle “In the UE, the AS shall report tracking area information to the NAS”. And normally the AS only indicates one TAC for the PLMN to NAS. But in order to provide the complete information to NAS, it seems necessary to indicate all received TACs to NAS layer. So we also have two options to consider:

Option 1: AS indicates only single TAC to NAS layer, even if more than one TAC for a PLMN is broadcasted in NTN cell;

Option 2: AS indicates all received TACs to NAS layer. It means when one TAC for a PLMN is broadcasted, AS layer indicates one TAC to NAS layer, and when more than one TAC for a PLMN is broadcasted, AS layer indicates all of them to NAS layer.

Question 2: how many TACs for one PLMN does AS layer indicate to NAS layer?

| Company | Option 1 or 2? | Comments |
| --- | --- | --- |
| Samsung | Any | Eventually, we would need to address the issue of relating the UE location to a specific TA when a beam covers multiple TAs, especially in the region with country borders. |
| MediaTek | Option 2 | AS layer should report all TACs to the NAS layer. |
| Huawei, HiSilicon | Option 2 | It would be good to provide all TAC information to NAS, and let NAS do its job. |
| ZTE | Option 1 | - We would like to follow the existing behavior, i.e. option 1 to provide only one TAC to NAS layer and it should be up to the AS layer to select the TAC to report. Otherwise, considerate impacts on other groups, e.g. CT1 and SA2, are foreseen and would bring much inter-WG coordination. - For option 2, we understand the intention is to provide all the broadcast TACs to NAS layer and TAU will be triggered only when none of them is in the registered TAI list. But it would be a little bit problematic as shown in the following example: Assume UE2 in the following figure moves from TAC1 to TAC2 with TAC2 not part of the current Registration Area but where the serving cell indicates support for both TAC1 and TAC2. No TAU will be triggered and when the UE sends a Service Request or other initial NAS message, the RAN may determine UE belongs to TAC2 and the AMF will receive TAC2 in the ULI. This may require new AMF treatment if TAC2 is not in the current Registration Area for the UE, as the AMF does not expect to receive such a TAC except as part of a Registration. So what we suggest is to let UE AS layer select one TAC from the broadcast ones based on its location information and the pre-configured mapping table between TAC and the location info. In this case, the TAC2 will be selected and reported from AS to NAS in the above example, and TAU will be triggered and AMF would be aware of theTAC2 that UE is actually in. |
| Lenovo | Option 2 | AS should provide full info to NAS and let NAS decide how to handle. |
| OPPO | Option 2 | When multiple TACs for a PLMN is broadcasted in a NTN cell, if only one TAC can be indicated to NAS layer, UE has difficulty to determine which TAC should be indicated to NAS layer without other information. NAS could choose one of the broadcasted multiple TACs for the UE. |
| Qualcomm | Option 2 | First of all, CT1 should be consulted on this issue. Option 1 requires RRC to define a new criterion of TAC selection. There is also a risk that AS indicates a TAC which is not in the UE Registration Area (RA) and does not indicate a TAC which is in the RA which might cause an unneeded Registration Update. For example, when a UE first accesses a cell, AS providing only one TAC would defeat the purpose of soft TAC update. |
| Apple | Option 2 | The concern with country borders can be taken up separately from the regular discussion. In normal situations the AS layer indicates all received TACs to NAS layer. We are open to sending an LS to CT1 on this as needed. |
| Xiaomi | Option 2 | For option 1, we don’t clear how UE to decide the TAC in AS layer, it may need some enhancements, for example, each TAC linked to different location, and UE decides the TAC by UE location. |
| Spreadtrum | Option 2 | AS shall indicate all the TACs to NAS, otherwise, it is complex for AS how to select the TAC to indicate to NAS. |
| CMCC | Option 2 | To avoid the complexity of UE behavior, all the received TACs should be known by NAS, and CT1 is responsible for the decision of this issue. |
| Rakuten Mobile | Option 2 | NAS layer should get all TAC information. |
| NEC | Option 2 |  |
| LG | Option 2 | All the broadcast TACs should be provided to NAS and how to trigger TAU based on the TAC list is up to NAS. As already NAS compares the broadcast PLMN+TAC combinations with TAIs of UE, we think it would not bring big specification impact. |
| ETRI | Option 1 | Single TAC is indicated as a legacy behavior without the NAS impact. Inconsistency of TACs should be handled in the AS layer (can be RAN3 or RAN2) |

Question 3: For Q2, if option 1 is adopted, do companies further agree that UE uses the timing information associated to the broadcasted TAC when selecting which TAC to update to NAS layer as well as when performing location update?

| Company | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | No | This may not work well because TAs may be irregular and multiple TAs could be valid in the beam where the UE is located. |
| MediaTek | No | Agree with Samsung that timing information is not needed. |
| Huawei, HiSilicon | No | For Q2, we suggest to go with option 2. |
| ZTE | No | As we commented under Q2, we suggest to let UE AS layer select one TAC from the broadcast ones based on its location information and the preconfigured mapping table between TAC and the location info. Furthermore, the idea of broadcasting the timing info of the TAC is not preferred from our side as commented under Q1. |
| Lenovo | No | We prefer Option 2 in Q2. |
| OPPO | No | We prefer Option 2 in Q2. |
| Qualcomm | Neither | Option 1 cannot reliably work as already commented. |
| Apple | No | Agree with Samsung and mediatek that the timing information is not needed in this case. |
| Xiaomi | No | We prefer Option 2 in Q2. |
| Spreadtrum | No | We prefer Option 2 in Q2 |
| CMCC | No | We prefer Opt.2 in Q2. |
| Rakuten Mobile | No | It would be hard to realize. |
| NEC | No | We agree with Samsung. |
| LG | No | As we commented in question 2, AS should provide all the TACs to NAS. |
| ETRI | No | It is unnecessary enhancement. |

Issue 3: RAN2 assume that if none of the TAC included in SI belongs to the TAI list, UE triggers TAU.

In both [1] and [2], the TAU principle has been mentioned and we suggest RAN2 to assume that if none of the TAC included in SI belongs to the TAI list, UE triggers TAU.

Question 4: Do companies agree that RAN2 assume UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area?

| Company | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| MediaTek | Yes |  |
| Huawei, HiSilicon | Yes |  |
| ZTE | No | - Firstly, as we commented in the first GTW session, it should be up to CT1 to decide the trigger for TAU thus this issue is out of RAN2 scope. - Secondly, it is related to the outcome of the Question 2: how many TACs for one PLMN does AS layer indicate to NAS layer and what kind of TAC would be reported to NAS layer. If AS layer selects one TAC and report to NAS layer, NAS will decide whether to trigger TAU based on the TAC provided from AS layer. |
| Lenovo | Yes |  |
| OPPO | Yes |  |
| Qualcomm | - | We assume this is current behavior and is handled by NAS (not by AS). While we agree with this statement, RAN2 needs to send an LS to CT1 and SA2 to confirm. |
| Apple | Yes |  |
| Xiaomi | Yes |  |
| Spreadtrum | Yes |  |
| CMCC | Yes |  |
| Rakuten Mobile | Yes |  |
| NEC | Yes |  |
| LG | Yes |  |
| ETRI | Yes |  |

Issue 4: the need of a LS to CT1 (CC RAN3 and SA2)

We see RAN2’s agreement on multiple TACs per PLMN may have some impact on CA1/SA2/RAN3, so a LS is recommended [2]. And according to our further discussion on Q2 and Q4, i.e. RAN2 assume UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area (if agreed), and AS indicates all received TACs to NAS layer (if agreed), we also need to inform CT1/SA2/RAN3 of our progress.

Question 5: Do companies agree that RAN2 should send a LS to CT1 and CC RAN3 and SA2? The content could include our agreement on multiple broadcasted TACs per PLMN, and our conclusion on Q2 and Q4 (if available)?

| Company | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | No | After RAN2 agrees on supporting one or more TAC update methods (hard, soft, virtual), such LS would be more helpful. If VTA is adopted, there is a single TAI on the radio interface and the N2 interface. |
| MediaTek | Yes | We think RAN2 needs to send LS to CT1 as there are significant overlaps between RAN2 and CT1 in this aspect. |
| Huawei, HiSilicon | Yes | A LS is needed, as multiple TACs related agreements have significant impact on CT1. |
| ZTE | No | The LS should only be sent when we agree on something requiring changes in CT1/SA2/RAN3 specs. For now, as the impact on CT1/SA2/RAN3 specs is not clear, we do not see the need for an LS out. |
| Lenovo | Yes | At least we can send an LS for information from RAN2. |
| OPPO | Yes | We are ok to send this LS to CT1 since there are some impacts on NAS layer that CT1 might need to discuss. |
| Qualcomm | Yes | We already have clear agreement on HARD and SOFT TAC update. CT1 should be informed on any NAS impact due to SOFT TAC update mechanism. |
| Apple | Yes | We think an LS is definitely needed here whether we include VTA into it or not. |
| Xiaomi | Yes |  |
| Spreadtrum | Yes | A LS is needed at least to CT1. |
| CMCC | Yes | If the opt.2 in Q2 is agreed, then an LS is essential. |
| Rakuten Mobile | No | We agree with ZTE and Samsung. Firstly, RAN2 should decide the approach for TAU. |
| NEC | Yes | We think RAN2 should send an LS to CT1 regarding this topic. |
| LG | Yes | We think Q1 and especially Q2 should be informed to CT1. |
| ETRI | No | Agree with ZTE. It is ok to send it after the influence of other groups is confirmed. |

Issue 5: Virtual Tracking Area

The following observation and proposal are quoted from [3]:

Observation 1. When multiple TAIs are broadcast by an NTN cell to implement Earth-fixed Tracking Areas, the TA management becomes quite complex at the gNB and the UE, SIB1 overhead increases, and reliability of SIB1 detection may also be affected adversely. Proposal 1. We suggest that RAN2 consider the concept of a Virtual Tracking Area as a candidate option to simplify the TA management and to continue to ensure reliable detection of SIB1.

And regarding how the mechanism works, it is described as below [3]:

In the VTA approach, the gNB transmits a single TAI in a cell like R16. The UE and the AMF are aware of the mapping between the VTA and TAIs in different time windows. Predictable platform movements (e.g., LEO satellites) can be used to easily determine such mapping. The AMF registers the UE in a Virtual Registration Area (VRA) that consists of VTAs. The VRA is equivalent to the R16 TAI List. The UE compares the TAI broadcast in SIB1 with the set of TAIs associated with the VRA at the current instant. In support of the mobility-based registration update, the UE does not send a Registration Request as long as the TAI in SIB1 is in the VRA.

Question 6: Do companies agree that RAN2 consider Virtual Tracking Area as the alternative to the soft TAC update approach?

| Company | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | Yes | We have serious concerns about the soft TAC update approach due to its practical implementation challenges. The VTA approach is usable for all types of beams: Earth-fixed, quasi-Earth-fixed, and Earth-moving. The VTA approach ensures Earth-fixed TAs called VTAs. In particular, in case of Earth-moving beams, the gNB and the UE processing are significantly simplified and the impact on NAS is minimal because there is a single TAI and existing TAU/registration signaling is essentially reused. In particular, the gNB does not to keep changing its broadcast TAs as its beams cover different Earth-fixed VTAs. The UE’s processing is simplified because the UE needs to compare a single broadcast TAI with its VRA List only once per cell selection/reselection instead of multiple times in a cell (e.g., when TAs are removed). Paging cost does not increase like the soft TAC update approach. SIB does not need to broadcast more than one TAI, reducing the SIB overhead. The VTA approach avoids the problems mentioned in various questions 1 to 3 above. The soft TAC update approach offers no benefits over the VTA approach in our view. Indeed, RAN2 will have to solve multiple problems such as identifying exactly what TAs to broadcast when TAs being covered by the beam change within SIB1 time period. Please see [3] for a comparison of hard, soft, and virtual TA approaches and details of the VTA approach. |
| MediaTek | No | We think VTA approach requires knowledge of when a TAI (broadcast by the satellite) serves a VTA, i.e. time information is needed by the UE. It is not clear how this time information provided to the UE and might create additional overhead. |
| Huawei, HiSilicon | No | We see this solution is even more complicated than soft TAU, and the concepts of VTA and VRA also impact CN operation, so it can’t be decided by RAN2. |
| Lenovo | No | We think VTA is more complicated and requires more knowledge. |
| OPPO | No | In the first release to support NTN, in our perspective, hard TAU and soft TAU are sufficient. The VTA approach has many impacts on both RAN and CN. |
| Qualcomm | No | The proposal seems quite complex. It has a lot impact to UE and AMF and it is too late to start discussing new solution other than hard and soft TAC update. |
| Apple | Yes | We support the VTA option. The time information can be obtained using SIB9 UTC data or if the network provides the entire ephemeris to the UE. We support the VTA option here. Alternately, we can have RAN do this mapping and let the cell broadcast only one TAI to the UE. |
| Xiaomi | No | It seems more complicated than soft TAU, we can consider it in the future release if the soft TAU is not sufficient in future. |
| Speadtrum | No | It is complex for UE to map between the VTA and TAIs in different time windows. |
| CMCC | No | The VTA solution may introduce some impacts on both RAN and CN, and bring more complexity than the soft TAU. |
| Rakuten Mobile | Yes | We support VTA option. Network can provide entire ephemeris to the UE and UE can decide TAU based on VTA approach. |
| NEC | No | We think that agreed Soft and Hard TAC update with broadcasted TACs controlled by the NW is sufficient. |
| LG | No | We think soft TAU is enough and VTA concept seems to bring too much overhead and complexity for the network. |
| ETRI | No | Soft TAC update is sufficient. It seems to have more impact on NAS standards. |

# Conclusion

# Reference

- R2-2103628	Discussion on remaining issues on soft TAU	Huawei, HiSilicon

- R2-2103749	Aspects concerning soft TAC switch	Ericsson

- R2-2103076	TAC Management and Neighbor Search in an NTN	Samsung Research America