3GPP TSG RAN WG2 #113b-e	R2-2104364

Electronic Meeting, 12th Apr – 20th Apr 2021

Source: 	Huawei, HiSilicon

Title: 	Summary of offline 105 - [NTN] TAC update

Agenda Item:	8.10.3.1

Document for:	Discussion and decision

# Introduction

This document is to collect companies’ views for the following offline discussion:

- [AT113bis-e][105][NTN] TAC update (Huawei)

Initial scope: Continue the discussion on based on the proposals from R2-2103628, R2-2103749 and R2-2103076, including the need to send an LS to SA2 and/or CT1

Initial intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Wednesday 2021-04-14 22:00 UTC

Initial deadline (for rapporteur's summary in R2-2104364): Thursday 2021-04-15 02:00 UTC

Proposals marked "for agreement" in R2-2104364 not challenged until Thursday 2021-04-15 14:00 UTC will be declared as agreed via email by the session chair.

For the rest the discussion will continue in a second round of the offline discussion until Monday 2021-04-19. Further details on the scope/intended outcome/exact deadlines to be announced by the session chair after Thursday 2021-04-15 14:00 UTC.

# Discussion

Issue 1: how to make UE aware of TAC change in SI.

During online discussion, we have made the agreement as follows:

Agreements:

- When the network stops broadcasting a TAC, the UE needs to know it (FFS on further details)

And we can continue discussing how to make UE know this TAC change. Currently we have two options on table:

Option 1: reuse legacy paging mechanism [1]. When the network stops broadcasting a TAC, it could send a SI change indication.

Option 2: based on validity timers related to TAIs [2]. As the broadcasted TACs may be associated with validity times, especially in Earth-moving scenario, in this case the UE knows when it needs to recheck SI.

Question 1: how to make UE know this TAC change when the network stops broadcasting a TAC?

|  | Company |  | Option 1 or 2? |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Samsung |  | 1 |  | Option 1 has the drawback of increased signaling load for paging and Option 2 has the drawback of increased SIB overhead. |  |
|  | MediaTek |  | Option-1, but |  | The legacy SI change indication method can be reused here since the SI information is changing. However, it is not necessary to always send this indication to the UE when tracking area information changes. If the cell stopped covering a tracking area on the ground, all UEs in that tracking area will no longer be served by that cell. Therefore the indication will not bring any additional benefit. |  |
|  | Huawei, HiSilicon |  | Option1 |  | Legacy mechanism can be reused. Further enhancement can be considered when it’s clear how large the Tracking Area is and how many TAs one NTN cell can cover. |  |
|  | ZTE |  | Option 1 |  | - We understand it would be sufficient to reuse the existing procedure by sending SI change notification in short message in paging DCI when the broadcast TAC changes. - We would like to focus on how to make the whole procedure work in NTN and also to minimize the impact in the existing RAN2 and CT1/SA2 specs to support NTN. Thus, unnecessary enhancements like option 2 is not preferred from our perspective so that we can spare more time for issues that really need to be solved to support NTN in the very first release. |  |
|  | Lenovo |  | Option 1 |  | We can reuse legacy mechanism as a baseline and consider further enhancement to reduce signaling load. Option 2 has more spec impact and also additional overhead. |  |
|  | OPPO |  | Option 1 |  | Using legacy paging mechanism to indicate UE the TAC change is enough. |  |
|  | Qualcomm |  | Option 2 |  | Option 1 is not efficient in moving cell scenario. Such SI update notification would be frequent, and we have concern on signaling load, resource, and power inefficiency. For option 2, if increased SIB1 size is issue, such time information can be carried in other SIB or NTN specific SIB. |  |
| Apple |  | Option 1 is preferred for lack of better alternatives |  | As mentioned by Samsung and MediaTek, one has signaling node issues while the other has increased SIB broadcast message sizes. In the current situations it is still unclear why the network would stop broadcasting SI when cell coverage is huge where there is esp. a signaling overhead concern. |  |  |
|  | Xiaomi |  | Option 1 |  | The legacy SI change indication can be reused. For the earth fixed scenario, we don’t think the gNB should send SI change indication when the TAC is stop broadcasted. We think network can trigger the SI update when the broadcast TACs changed based on network implementation. |  |
|  | Spreadtrum |  | Option 1 |  | For option 2, it is difficult to guarantee all the UE to start the timer at the same time. |  |
|  | CMCC |  | Option 1 with comments |  | Only legacy solution may not be enough as mentioned by some companies (e.g the increasing signaling overhead for SI modification), some enhancements may be needed. |  |
|  | Rakuten Mobile |  | Option 2 is preferred but can agree with Option 1 for now. |  | We agree with Qualcomm, It would be better to control the SIB Size then to overload signaling. |  |
|  | NEC |  | Option 1 with comments |  | In quasi Earth-fixed cells, cell-wide CHO may be used to move UEs before the cell stops broadcasting the TAC and UEs will not get any more information from SI change indication. We agree to use Option 1 as a baseline and enhancements to lower the signalling load should be considered. |  |
|  | LG |  | Option 1 |  | It is much simple approach to reuse existing mechanism. Regarding the preceding comments that option 1 may increase signaling load for paging, we think generally UEs should reselect to other neighbor cell before its registered TAC is deleted in the broadcast SIB. So we think it is not really critical issue. |  |
|  | ETRI |  | Option 1 |  | We prefer Option 1 as a baseline. Enhancements such as option 2 can be discussed later if necessary. |  |
|  | BT |  | Option 1 |  | We envision this scenario mainly for moving beams while in quasi Earth-fixed cells, it is not expected a TAC change. Based on that, even it is not the most optimal solution, we prefer to keep it as simple as possible in this first release. |  |
|  | Thales |  | Option 1 |  |  |  |
|  | ITRI |  | Option 2 |  | IDLE/INACTIVE UE should not be required to require system information whenever the broadcast TAC changed. Network have the knowledge that a UE may not re-acquire system information before the expiry of SI validity time. Considering NTN TA is fixed associated with geographical area, even when a cell stopped broadcasting a TAC (means the cell stopped serving the NTN TA), network can reach an IDLE/INACTIVE UE by sending paging message in a NTN cell which had served the NTN TA a period of time before. |  |
|  | Ericsson |  | 2 |  | When the information is in the SIB and UE reads it when it reads the TAC list it is readily available at the UE. There is no need to use network resources for paging the UEs for that. Paging capacity is limited anyway in NTN due to large cell size. Then, SI modification periods are not aligned with when network actually needs to stop broadcasting. Requiring such alignment causes unwelcome issues for network planning especially at country boarders. |  |
|  | Vodafone |  | Option 1 |  | Validity time, in practice, is complicated to implement and as stated above, there is no guarantee that all UE will start this timer simultaneously. More importantly , if TACs are beamed from the Satellites to the ground, the movement of the TACs can not be guaranteed as the orbits of the passing satellite and ‘beamed’ TAC fluctuate with the movement of the satellite and differing systems; this follows that the TAC based Validity timer cannot be uniform Option 1 is still a robust mechanism |  |
|  | Nokia |  | Option 1 |  | We think Option 2 has too large specification impact, while Option 1 reuses legacy mechanisms and can work even in EMC scenario. |  |
|  | Fraunhofer IIS/HHI |  | - |  | We agree with MediaTek, it might not be necessary to signal the TAC change in the SIB to the UE. Both options introduce overheads to the system, but we are not sure which solution imposes the least overhead to the system. |  |
|  | Intel |  | Option 1 |  |  |  |
|  | Eutelsat |  | Option 1 |  | Option 1 is the legacy way of proceeding, simple and proven and should be sufficient for both conversational and IoT use cases if/when a TAC change needs to be signaled. Noting that option 2 could be considered as an optimization at a later stage if felt more efficient in terms of energy / capacity aspects. |  |
|  | Sequans |  | Option 1 but |  | We think the SI change indication is costly both in terms of resources and power consumption. We can use legacy mechanism but in our view this doesn't need to be systematic. |  |
|  | CATT |  | Option 1 |  | We assume it’s sufficient to reuse the legacy procedure to notify UE the SI is changed. Not necessary to do the enhancement. |  |

Summary:

26 companies provided replies, and 22 companies prefer to reuse legacy paging mechanism while 4 companies thinks option 2 is better. As for reusing legacy paging mechanism, companies also share concern about increased paging signaling overhead, but are fine to consider it as a baseline. Whether and when to send a SI change indication still depends on network implementation. Further enhancement can be considered to lower the signaling overhead later if time allows.

Based on our preceding agreement “1.	When the network stops broadcasting a TAC, the UE needs to know it (FFS on further details)”, we make the following proposal:

(22/26)Proposal 1: When the network stops broadcasting a TAC, network can send a SI change indication like legacy to make UE aware of it. Whether and when to send a SI change indication still depends on network implementation. FFS on further enhancement to lower paging signaling overhead.

Issue 2: how many TACs for one PLMN does AS layer indicate to NAS layer.

When more than one TAC per PLMN is broadcasted in NTN cell, we need to figure out how to indicate TAC to NAS layer. In current TS 38.304, we only have the basic principle “In the UE, the AS shall report tracking area information to the NAS”. And normally the AS only indicates one TAC for the PLMN to NAS. But in order to provide the complete information to NAS, it seems necessary to indicate all received TACs to NAS layer. So we also have two options to consider:

Option 1: AS indicates only single TAC to NAS layer, even if more than one TAC for a PLMN is broadcasted in NTN cell;

Option 2: AS indicates all received TACs to NAS layer. It means when one TAC for a PLMN is broadcasted, AS layer indicates one TAC to NAS layer, and when more than one TAC for a PLMN is broadcasted, AS layer indicates all of them to NAS layer.

Question 2: how many TACs for one PLMN does AS layer indicate to NAS layer?

| Company | Option 1 or 2? | Comments |
| --- | --- | --- |
| Samsung | Any | Eventually, we would need to address the issue of relating the UE location to a specific TA when a beam covers multiple TAs, especially in the region with country borders. |
| MediaTek | Option 2 | AS layer should report all TACs to the NAS layer. |
| Huawei, HiSilicon | Option 2 | It would be good to provide all TAC information to NAS, and let NAS do its job. |
| ZTE | Option 1 | - We would like to follow the existing behavior, i.e. option 1 to provide only one TAC to NAS layer and it should be up to the AS layer to select the TAC to report. Otherwise, considerate impacts on other groups, e.g. CT1 and SA2, are foreseen and would bring much inter-WG coordination. - For option 2, we understand the intention is to provide all the broadcast TACs to NAS layer and TAU will be triggered only when none of them is in the registered TAI list. But it would be a little bit problematic as shown in the following example: Assume UE2 in the following figure moves from TAC1 to TAC2 with TAC2 not part of the current Registration Area but where the serving cell indicates support for both TAC1 and TAC2. No TAU will be triggered and when the UE sends a Service Request or other initial NAS message, the RAN may determine UE belongs to TAC2 and the AMF will receive TAC2 in the ULI. This may require new AMF treatment if TAC2 is not in the current Registration Area for the UE, as the AMF does not expect to receive such a TAC except as part of a Registration. So what we suggest is to let UE AS layer select one TAC from the broadcast ones based on its location information and the pre-configured mapping table between TAC and the location info. In this case, the TAC2 will be selected and reported from AS to NAS in the above example, and TAU will be triggered and AMF would be aware of theTAC2 that UE is actually in. |
| Lenovo | Option 2 | AS should provide full info to NAS and let NAS decide how to handle. |
| OPPO | Option 2 | When multiple TACs for a PLMN is broadcasted in a NTN cell, if only one TAC can be indicated to NAS layer, UE has difficulty to determine which TAC should be indicated to NAS layer without other information. NAS could choose one of the broadcasted multiple TACs for the UE. |
| Qualcomm | Option 2 | First of all, CT1 should be consulted on this issue. Option 1 requires RRC to define a new criterion of TAC selection. There is also a risk that AS indicates a TAC which is not in the UE Registration Area (RA) and does not indicate a TAC which is in the RA which might cause an unneeded Registration Update. For example, when a UE first accesses a cell, AS providing only one TAC would defeat the purpose of soft TAC update. |
| Apple | Option 2 | The concern with country borders can be taken up separately from the regular discussion. In normal situations the AS layer indicates all received TACs to NAS layer. We are open to sending an LS to CT1 on this as needed. |
| Xiaomi | Option 2 | For option 1, we don’t clear how UE to decide the TAC in AS layer, it may need some enhancements, for example, each TAC linked to different location, and UE decides the TAC by UE location. |
| Spreadtrum | Option 2 | AS shall indicate all the TACs to NAS, otherwise, it is complex for AS how to select the TAC to indicate to NAS. |
| CMCC | Option 2 | To avoid the complexity of UE behavior, all the received TACs should be known by NAS, and CT1 is responsible for the decision of this issue. |
| Rakuten Mobile | Option 2 | NAS layer should get all TAC information. |
| NEC | Option 2 |  |
| LG | Option 2 | All the broadcast TACs should be provided to NAS and how to trigger TAU based on the TAC list is up to NAS. As already NAS compares the broadcast PLMN+TAC combinations with TAIs of UE, we think it would not bring big specification impact. |
| ETRI | Option 1 | Single TAC is indicated as a legacy behavior without the NAS impact. Inconsistency of TACs should be handled in the AS layer (can be RAN3 or RAN2) |
| BT | Option 2 | As mention by several companies, CT1 needs to be involved. |
| Thales | Any |  |
| ITRI | Option 2 | The AS layer indicates all the received TACs to NAS. |
| Ericsson | Option 1 | We are fine to conclude RAN2 supports Option 1 and then send LS to CT1 asking for feedback and to take it into account. Also, it is fine to conclude RAN2 can see either Option 1 or Option 2 viable and then we ask CT1 opinion. RAN2 cannot really decide we should support only Option 2. |
| Vodafone | Any | UE can only be in one particular Tracking area, however the Access Stratum cam report as many Tracking areas as available to the NAS layer |
| Nokia | Option 2 | Option 1 would be better from simplicity point of view, but we agree it is a bit difficult to decide which TAC to choose (unless we agree any TAC chosen is fine to be signaled to NAS, even if that brings some inaccuracies) |
| Fraunhofer IIS/HHI | Option 2 | In any case, it should be possible for the UE to report multiple TACs per PLMN. Option 2 should be the baseline, while a reduction to a subset could be a potential future enhancement. |
| Intel | Option 2 | AS can provide all TACs to NAS. However, we think this more of NAS issue and CT1 should be consulted. |
| Eutelsat | Option 2 | TA selection and TAU triggering is typically a NAS feature. |
| Sequans | Option 2 | TAC is NAS information. |
| CATT | Option 2 | It’s straightforward for AS to forward all the received TACs to NAS. Or else, AS need to do know the real coverage of each TAC, and do corresponding mapping between UE location and the scope of TACs. |

Summary:

26 companies provided replies. 23 companies support to report all TACs to NAS, and 6 companies support to report only one TAC to NAS, while 3 companies are fine with both options.

The reason to support option 2 is it would be complex to make UE select one TAC to report in case multiple TACs are broadcasted in one cell, and it’s also because NAS is responsible for TAC management so AS should provide full information to NAS. And the proponents of option 1 prefer to stick to legacy mechanism, and let RAN2 handle the TAC selection.

(23/26)Proposal 2: AS indicates all received TACs to NAS layer when more than one TAC per PLMN is broadcasted in NTN cell.

Question 3: For Q2, if option 1 is adopted, do companies further agree that UE uses the timing information associated to the broadcasted TAC when selecting which TAC to update to NAS layer as well as when performing location update?

| Company | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | No | This may not work well because TAs may be irregular and multiple TAs could be valid in the beam where the UE is located. |
| MediaTek | No | Agree with Samsung that timing information is not needed. |
| Huawei, HiSilicon | No | For Q2, we suggest to go with option 2. |
| ZTE | No | As we commented under Q2, we suggest to let UE AS layer select one TAC from the broadcast ones based on its location information and the preconfigured mapping table between TAC and the location info. Furthermore, the idea of broadcasting the timing info of the TAC is not preferred from our side as commented under Q1. |
| Lenovo | No | We prefer Option 2 in Q2. |
| OPPO | No | We prefer Option 2 in Q2. |
| Qualcomm | Neither | Option 1 cannot reliably work as already commented. |
| Apple | No | Agree with Samsung and mediatek that the timing information is not needed in this case. |
| Xiaomi | No | We prefer Option 2 in Q2. |
| Spreadtrum | No | We prefer Option 2 in Q2 |
| CMCC | No | We prefer Opt.2 in Q2. |
| Rakuten Mobile | No | It would be hard to realize. |
| NEC | No | We agree with Samsung. |
| LG | No | As we commented in question 2, AS should provide all the TACs to NAS. |
| ETRI | No | It is unnecessary enhancement. |
| Thales | No | Agree with Samsung |
| ITRI | No | Agree with Samsung. |
| Ericsson | yes | When the single TAC is provided to NAS alyer, it should be selected such that it matches UE’s registration area and that is the TAC where UE most likely will be if paged. |
| Vodafone | No |  |
| Nokia | No | We do not think the timing information for TAC is needed. |
| Fraunhofer IIS/HHI | No | We prefer Option 2 in Q2 |
| Eutelsat | No | (Not applicable for option 2) |
| Sequans | No |  |
| CATT | No | We prefer Option 2 in Q2 |

Summary:

As option 2 is selected by majority companies for Q2, we don’t need to further discuss this enhancement.

Issue 3: RAN2 assume that if none of the TAC included in SI belongs to the TAI list, UE triggers TAU.

In both [1] and [2], the TAU principle has been mentioned and we suggest RAN2 to assume that if none of the TAC included in SI belongs to the TAI list, UE triggers TAU.

Question 4: Do companies agree that RAN2 assume UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area?

| Company | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | Yes |  |
| MediaTek | Yes |  |
| Huawei, HiSilicon | Yes |  |
| ZTE | No | - Firstly, as we commented in the first GTW session, it should be up to CT1 to decide the trigger for TAU thus this issue is out of RAN2 scope. - Secondly, it is related to the outcome of the Question 2: how many TACs for one PLMN does AS layer indicate to NAS layer and what kind of TAC would be reported to NAS layer. If AS layer selects one TAC and report to NAS layer, NAS will decide whether to trigger TAU based on the TAC provided from AS layer. |
| Lenovo | Yes |  |
| OPPO | Yes |  |
| Qualcomm | - | We assume this is current 9ehaviour and is handled by NAS (not by AS). While we agree with this statement, RAN2 needs to send an LS to CT1 and SA2 to confirm. |
| Apple | Yes |  |
| Xiaomi | Yes |  |
| Spreadtrum | Yes |  |
| CMCC | Yes |  |
| Rakuten Mobile | Yes |  |
| NEC | Yes |  |
| LG | Yes |  |
| ETRI | Yes |  |
| BT | Yes |  |
| Thales | Yes |  |
| ITRI | Yes |  |
| Ericsson | yes |  |
| Vodafone | Yes |  |
| Nokia | Yes | In our understanding this would be a correct 9ehaviour in this case. We are OK to share it as ‘RAN2 understanding’, if we decide to send the LS to CT1. |
| Fraunhofer IIS/HHI | Yes |  |
| Intel | May be | TAU is initiated by NAS. RAN2 should consult CT1 on this. |
| Eutelsat | Yes | (But this is in CT1 scope) |
| Sequans | Yes but | It's up to CT1 but ok for RAN2 to assume. |
| CATT | Yes | It's aligned with legacy TAU mechanism. |

Summary:

26 companies provided feedback. 25 companies agree to the understanding that RAN2 assume UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area. 1 company disagrees and points out it is still related to the RAN2’s answer to Q2.

(25/26)Proposal 3: RAN2 assume UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area.

Issue 4: the need of a LS to CT1 (CC RAN3 and SA2)

We see RAN2’s agreement on multiple TACs per PLMN may have some impact on CA1/SA2/RAN3, so a LS is recommended [2]. And according to our further discussion on Q2 and Q4, i.e. RAN2 assume UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area (if agreed), and AS indicates all received TACs to NAS layer (if agreed), we also need to inform CT1/SA2/RAN3 of our progress.

Question 5: Do companies agree that RAN2 should send a LS to CT1 and CC RAN3 and SA2? The content could include our agreement on multiple broadcasted TACs per PLMN, and our conclusion on Q2 and Q4 (if available)?

| Company | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | No | After RAN2 agrees on supporting one or more TAC update methods (hard, soft, virtual), such LS would be more helpful. If VTA is adopted, there is a single TAI on the radio interface and the N2 interface. |
| MediaTek | Yes | We think RAN2 needs to send LS to CT1 as there are significant overlaps between RAN2 and CT1 in this aspect. |
| Huawei, HiSilicon | Yes | A LS is needed, as multiple TACs related agreements have significant impact on CT1. |
| ZTE | No | The LS should only be sent when we agree on something requiring changes in CT1/SA2/RAN3 specs. For now, as the impact on CT1/SA2/RAN3 specs is not clear, we do not see the need for an LS out. |
| Lenovo | Yes | At least we can send an LS for information from RAN2. |
| OPPO | Yes | We are ok to send this LS to CT1 since there are some impacts on NAS layer that CT1 might need to discuss. |
| Qualcomm | Yes | We already have clear agreement on HARD and SOFT TAC update. CT1 should be informed on any NAS impact due to SOFT TAC update mechanism. |
| Apple | Yes | We think an LS is definitely needed here whether we include VTA into it or not. |
| Xiaomi | Yes |  |
| Spreadtrum | Yes | A LS is needed at least to CT1. |
| CMCC | Yes | If the opt.2 in Q2 is agreed, then an LS is essential. |
| Rakuten Mobile | No | We agree with ZTE and Samsung. Firstly, RAN2 should decide the approach for TAU. |
| NEC | Yes | We think RAN2 should send an LS to CT1 regarding this topic. |
| LG | Yes | We think Q1 and especially Q2 should be informed to CT1. |
| ETRI | No | Agree with ZTE. It is ok to send it after the influence of other groups is confirmed. |
| BT | Yes | At least if option 2 of issue 2 is agreed. |
| Thales | Yes |  |
| ITRI | Yes | We think some impact in NAS layer is foreseen. An LS to CT1 is supported. |
| Ericsson | yes |  |
| Vodafone | Yes | it would be a useful piece of information to both working groups |
| Nokia | Depends on the decisions taken | If we agree something that affects CT1 (like sending two TACs to NAS) then clearly we should send the LS. |
| Fraunhofer IIS/HHI | Yes |  |
| Intel | Yes | Share the view explained by other supporting companies. |
| Eutelsat | Yes | CT1 needs to be informed of what information AS is sending to NAS layer as this will impact NAS procedure related to TAC handling. |
| Sequans | Depends | Same view as Nokia |
| CATT | Yes, but | We are positive to send the LS when we finish the RAN2 discussions on the whole solutions. The LS could indicate the final solutions of RAN2, or ask for guidance from other groups. |

Summary:

22 of 26 companies support to send a LS to CT1 as we already support soft TAU according to our agreement made in last meeting, also considering for Q2 current majority view is to report multiple TACs (if available) to NAS. 4 companies have different views, one reason is we still don’t have conclusion on VTA, and another reason is the impact on other working groups is still not clear.

(22/26)Proposal 4: RAN2 to send a LS to CT1 and CC RAN3 and SA2 to inform them of RAN2’s agreements that affect NAS.

Issue 5: Virtual Tracking Area

The following observation and proposal are quoted from [3]:

Observation 1. When multiple TAIs are broadcast by an NTN cell to implement Earth-fixed Tracking Areas, the TA management becomes quite complex at the gNB and the UE, SIB1 overhead increases, and reliability of SIB1 detection may also be affected adversely. Proposal 1. We suggest that RAN2 consider the concept of a Virtual Tracking Area as a candidate option to simplify the TA management and to continue to ensure reliable detection of SIB1.

And regarding how the mechanism works, it is described as below [3]:

In the VTA approach, the gNB transmits a single TAI in a cell like R16. The UE and the AMF are aware of the mapping between the VTA and TAIs in different time windows. Predictable platform movements (e.g., LEO satellites) can be used to easily determine such mapping. The AMF registers the UE in a Virtual Registration Area (VRA) that consists of VTAs. The VRA is equivalent to the R16 TAI List. The UE compares the TAI broadcast in SIB1 with the set of TAIs associated with the VRA at the current instant. In support of the mobility-based registration update, the UE does not send a Registration Request as long as the TAI in SIB1 is in the VRA.

Question 6: Do companies agree that RAN2 consider Virtual Tracking Area as the alternative to the soft TAC update approach?

| Company | Yes or No? | Comments |
| --- | --- | --- |
| Samsung | Yes | We have serious concerns about the soft TAC update approach due to its practical implementation challenges. The VTA approach is usable for all types of beams: Earth-fixed, quasi-Earth-fixed, and Earth-moving. The VTA approach ensures Earth-fixed TAs called VTAs. In particular, in case of Earth-moving beams, the gNB and the UE processing are significantly simplified and the impact on NAS is minimal because there is a single TAI and existing TAU/registration signaling is essentially reused. In particular, the gNB does not to keep changing its broadcast Tas as its beams cover different Earth-fixed VTAs. The UE’s processing is simplified because the UE needs to compare a single broadcast TAI with its VRA List only once per cell selection/reselection instead of multiple times in a cell (e.g., when Tas are removed). Paging cost does not increase like the soft TAC update approach. SIB does not need to broadcast more than one TAI, reducing the SIB overhead. The VTA approach avoids the problems mentioned in various questions 1 to 3 above. The soft TAC update approach offers no benefits over the VTA approach in our view. Indeed, RAN2 will have to solve multiple problems such as identifying exactly what Tas to broadcast when Tas being covered by the beam change within SIB1 time period. Please see [3] for a comparison of hard, soft, and virtual TA approaches and details of the VTA approach. |
| MediaTek | No | We think VTA approach requires knowledge of when a TAI (broadcast by the satellite) serves a VTA, i.e. time information is needed by the UE. It is not clear how this time information provided to the UE and might create additional overhead. |
| Huawei, HiSilicon | No | We see this solution is even more complicated than soft TAU, and the concepts of VTA and VRA also impact CN operation, so it can’t be decided by RAN2. |
| Lenovo | No | We think VTA is more complicated and requires more knowledge. |
| OPPO | No | In the first release to support NTN, in our perspective, hard TAU and soft TAU are sufficient. The VTA approach has many impacts on both RAN and CN. |
| Qualcomm | No | The proposal seems quite complex. It has a lot impact to UE and AMF and it is too late to start discussing new solution other than hard and soft TAC update. |
| Apple | Yes | We support the VTA option. The time information can be obtained using SIB9 UTC data or if the network provides the entire ephemeris to the UE. We support the VTA option here. Alternately, we can have RAN do this mapping and let the cell broadcast only one TAI to the UE. |
| Xiaomi | No | It seems more complicated than soft TAU, we can consider it in the future release if the soft TAU is not sufficient in future. |
| Speadtrum | No | It is complex for UE to map between the VTA and TAIs in different time windows. |
| CMCC | No | The VTA solution may introduce some impacts on both RAN and CN, and bring more complexity than the soft TAU. |
| Rakuten Mobile | Yes | We support VTA option. Network can provide entire ephemeris to the UE and UE can decide TAU based on VTA approach. |
| NEC | No | We think that agreed Soft and Hard TAC update with broadcasted TACs controlled by the NW is sufficient. |
| LG | No | We think soft TAU is enough and VTA concept seems to bring too much overhead and complexity for the network. |
| ETRI | No | Soft TAC update is sufficient. It seems to have more impact on NAS standards. |
| BT | No | VTA may introduce benefits but at a highly implementation cost. At this point in time, it is not 100% sure the cell will require to broadcast more than one TAC per PLMN as this is up to network implementation based on RAN2#113-e agreement: In NTN, the network may broadcast more than one TACs per PLMN in a cell, which is to up to network implementation. If a cell ends up with one TAC per PLMN, legacy mechanism can be re-used without any additional complexity. |
| Thales | Yes | VTA may have a benefit only for Earth-moving beams. This requires FFS. In any case the TA over the NG interface shall be Earth fixed. |
| ITRI | No | We think VTA requires more knowledge to be established in UE, also the alignment of the knowledge between UE and network could be complex. |
| Ericsson | No | From UE perspective it does not look that simple as UE would need to keep track on time vs its local mapping info + read and reread SI. The assessment on UE complexity “Re-use R16. The UE simply compares its TAI List against a single TAI broadcast by the gNB.” Is not really so clear. Further it is not clear where these mapping tables would come. Benefit with the approach is that gNB does not need to change the broadcasted TAC, however it does not seem to be good enough tradeoff w complexities. |
| Vodafone | No | Agree with comments from BT that it is an unnecessary over-engineering and provides little benefit. |
| ZTE | / | We are fine to consider it as an option. |
| Nokia | No | In our understanding, it moves the problem elsewhere (instead of resolving it) and introduces the need to keep the mapping, etc. |
| Fraunhofer IIS/HHI | No | Same view as Nokia. |
| Intel | FFS |  |
| Eutelsat | No | May bring some complexity vs. genuinely evolving existing procedures. |
| Sequans | Yes | We are fine to consider it further (some more details would be required to assess feasibility). |
| CATT | No | VTA may bring more complicity for both UE, RAN and 5GC. |

Summary:

20 out of 26 companies don’t support VTA as the alternative to the soft TAU, because it seems quite complex and has significant impact to UE and AMF, in the meantime it is too late to start discussing new solution. 6 companies prefer to further consider this enhancement and more details would be required to assess feasibility.

Rapporteur’s view is we should focus on fundamental adaptations in the first release of NR NTN, as soft TAU has been agreed we can consider further enhancement in further release.

(20/26)Proposal 5: Virtual Tracking Area solution is not considered in R17 NR NTN WI.

# Conclusion

In this offline discussion, we focus on the remaining issues with soft TAU, and we have the following proposals. Since there is clear majority for each proposal, they are all marked "for agreement".

List of proposals for agreement:

(21/25)Proposal 1: When the network stops broadcasting a TAC, network can send a SI change indication like legacy to make UE aware of it. Whether and when to send a SI change indication still depends on network implementation. FFS on further enhancement to lower paging signaling overhead.

(22/25)Proposal 2: AS indicates all received TACs to NAS layer when more than one TAC per PLMN is broadcasted in NTN cell.

(24/25)Proposal 3: RAN2 assume UE does not do TAU if one of the currently broadcasted TAC belongs to UE’s registration area.

(21/25)Proposal 4: RAN2 to send a LS to CT1 and CC RAN3 and SA2 to inform them of RAN2’s agreements that affect NAS.

(19/25)Proposal 5: Virtual Tracking Area solution is not considered in R17 NR NTN WI.

# Reference

- R2-2103628	Discussion on remaining issues on soft TAU	Huawei, HiSilicon

- R2-2103749	Aspects concerning soft TAC switch	Ericsson

- R2-2103076	TAC Management and Neighbor Search in an NTN	Samsung Research America