3GPP TSG-RAN WG2 Meeting #113-b electronic				R2-21xxxxx

Online, 12 - 20 April 2021

Agenda Item:		4.5	Other LTE corrections Rel-15 and earlier

Source:			Huawei (Email rapporteur)

Title:				Report of [AT113bis-e][203][LTE] One-shot configurations

WI code(s):			TEI15

Document for:		Discussion and Decision

## 1	Introduction

This is the report of [AT113bis-e][203][LTE] One-shot configurations.

- [AT113bis-e][203][LTE] One-shot configurations (Huawei)

Scope:

    - Discuss whether something needs to be done for one-shot configurations in 36.331

Intended outcome:

    - Discussion summary in R2-2104323 (by email rapporteur)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur's summary):  1st week Fri, UTC 0900

Please companies provide the contact information in the table below so that it would be easy to find the relevant delegate for the comments.

| Company | Delegate contact (email) |
| --- | --- |
| Intel | Sudee.k.palat@intel.com |
| Qualcomm | Umesh Phuyal (uphuyal at qti.qualcomm.com) |
| Huawei | Jun Chen (jun.chen@huawei.com) |
| Samsung | Seungri Jin (seungri.jin@samsung.com) |
| Ericsson | Emre A. Yavuz (emre dot yavuz at ericsson dot com) |

## 2	Discussion

### 2.1	Relevant contributions

As indicated by the session chair, the following contributions are to be discussed by email [203].

Web Conf (Monday 1st week) (1)

Ambiguity in Need ON for one-shot configurations:

R2-2104013	Discussion on one-shot configuration	Huawei, HiSilicon	discussion	Rel-15	TEI15

Offline discussion [203] (Huawei)

### 2.2	Companies’ feedbacks

R2-2104013	Discussion on one-shot configuration	Huawei, HiSilicon	discussion	Rel-15	TEI15

Proposal 1: In TS 36.331, for the fields that are not defined in one-shot manner but they actually implement one-shot configuration, it is proposed to collect companies’ opinions. At least the reestablishRLC IE is observed as a problematic case.

Proposal 2: The following options can be discussed:

- Option 1: A general way to solve all possible issues, e.g. up to UE implementations. This option may or may not need any spec impacts

- Option 2: Discuss clarifications case by case. This option may need some spec impacts

Question 1: Do companies agree with proposal 1 in R2-2104013?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Intel | No | In our understanding, the statement that “In the ASN.1 definition, the reestablishRLC IE has Need ON, and it is not one-shot handling.” Is not correct. In LTE spec, ON is used both for one shot and to continue using the same value. Continue to use value is only where applicable “where applicable shall continue to use the existing value” and this is not the case for reestablishRLC. This issue has been discussed a few times before in LTE. If there is any ambiquity, it can be clarified in the field description. But for reestablishRLC field, we don’t think there is a possibility of confusion on whether UE continue to store it. |
| Qualcomm | No | Similar view as Intel - we don’t think there is any ambiguity in general. If there is ambiguity for specific field(s), field description clarification can be considered case by case. Regarding reestablishRLC, it seems to be clear that it is a one-shot parameter. |
| Huawei, HiSilicon | Yes | We are the proponent companies. Based on the comments from Intel and Qualcomm, it is understood that UE should implement the reestablishRLC following one-shot hanlding because it is obvious. Our concern is that there are 7 places in the filed descriptions in TS 36.331 to explicitly mention one-shot handling, and we think the following two definitions would lead to different UE behaviours: - (need ON) indicates that the UE shall XXX - (need ON) A one-shot field indicating that the UE shall XXX For now, we only observe the one-shot handling issue for the IE reestablishRLC, and we prefer to discuss it case by case. Our reference is shown in section 5 in R2-2104013, i.e. update the field description of the reestablishRLC to clearly indicate the one-shot filed. |
| Samsung |  | It seems we now have 7 cases in LTE RRC specification where we state a field is one-short as Huawei indicated but we don’t recall if RAN2 agreed a convention i.e. that from a certain release we would aim to clarify this. We tend to agree that this kind of clarification is needed consistently compared with what already indicated in the specification. It seems better to make general agreement how to handle the field(s) need to be clarified for one-shot configuration. |
| Ericsson | No | Clarifications may be acceptable only where it is necessary, but there is no need to start a campaign to go through the whole specification to discuss each case. In particular for the reestablishRLC IE; consider the procedure text for re-establishment: 4> if the rlc-Config is included: 5> if reestablishRLC is received: 6> re-establish the primary RLC entity of this DRB; The text for Need ON captures the following: “…where applicable shall continue to use the existing value” and the procedure text above indicates that “continue using the existing value” does not apply to this case, i.e. RLC should not be re-established every time a reconfiguration is received with a blank reestablishRLC parameter. The procedure text states “when received” so it directly connects the execution of the procedure with the presence of the field in given reconfiguration message, i.e., not stored in the UE. |

Question 2: For proposal 2 in R2-2104013, which option do companies prefer?

| Company | Preferred option | Comments |
| --- | --- | --- |
| Intel |  | It most cases, it should be fairly obvious on whether the field is a one-shot or not. If there is any confusion on the UE behaviour, it can be discussed on a case by case basis and if felt necessary, can be clarified specific for that field. |
| Qualcomm |  | Can consider case by case if needed. We do not see a need for a generalized approach. |
| Huawei, HiSilicon | Option 2 | Firstly, we suggest to discuss the one-shot handling for the IE reestablishRLC. Secondly, in order to avoid the ambiguities, we prefer the changes in section 5 in R2-2104013, i.e.: RLC-Config field descriptions: reestablishRLC A one-shot field that indicates that RLC shall be re-established. |
| Samsung |  | We assumed that there are many R15/R16 one-shot fields not having such indication, so it would be best to handle all fields together i.e. do not prefer to handle this issue on case by case basis, but we agree that it would be the easiest way to handle case by case. In short, we are fine for the suggestion of Huawei above at this moment, and further foundings will be added in Rap CR. |
| Ericsson |  | Agree with Intel that in most cases it should be clear whether the field is one-shot. If there is any confusion for a particular case, we can discuss and clarify if needed. |

## 3	Conclusion

[To be updated]