3GPP TSG-RAN WG2 Meeting #113-b electronic				R2-21xxxxx

Online, 12 - 20 April 2021

Agenda Item:		6.4.1	CHO/CPC Corrections

Source:			Huawei (Email rapporteur)

Title:				Report of [AT113bis-e][210][MOB] CHO/CPC corrections

WI code(s):			NR_Mob_enh-Core, LTE_feMob-Core

Document for:		Discussion and Decision

## 1	Introduction

This is the report of [AT113bis-e][210][MOB] CHO/CPC corrections.

- [AT113bis-e][210][MOB] CHO/CPC corrections (Huawei)

Scope:

    - Discuss which CHO/CPC corrections (for LTE and NR) marked for this discussion are seen agreeable. CRs that are editorial can be merged together

Intended outcome:

    - Discussion summary in R2-2104311 (by email rapporteur).

    - Agreeable CRs (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for company feedback):  1st week Thu, UTC 0900

    - Initial deadline (for rapporteur summary):  1st week Fri, UTC 0900

    - Deadline for CR finalization: 2nd week Tue, UTC 1000

Web Conf 2nd week or By Email (summary of [210])

R2-2104311	Summary of [AT113bis-e][210][MOB] CHO/CPC corrections (Huawei)	Huawei	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

Please companies provide the contact information in the table below so that it would be easy to find the relevant delegate for the comments.

| Company | Delegate contact (email) |
| --- | --- |
| MediaTek | Li-Chuan TSENG (li-chuan.tseng@mediatek.com) |
| Intel | Yi GUO (yi.guo@intel.com) |
| ZTE | Mengjie Zhang (zhang.mengjie@zte.com.cn) |
| Huawei, HiSilicon | Jun Chen (jun.chen@huawei.com) |
| Nokia | Jedrzej Stanczak - jedrzej.stanczak[at]nokia.com |

## 2	Discussion

### 2.1	Relevant contributions

As indicated by the session chair, the following contributions are to be discussed by email [210].

Web Conf (Monday 1st week, if time allows) or By Email [210] (1+1)

Ambiguity in WI agreements and captured text:

R2-2103114	Discussion on Applicable Cases for Failure Recovery via CHO	CATT	discussion	Rel-16	NR_Mob_enh-Core

Is CHO with SCG configuration allowed in (MR-)DC?

R2-2103332	Clarification on SCG configuration in CHO	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_Mob_enh-Core

By Email [210] (1+2+1+1+1)

CPC configuration via SRB1 after initial SRB3 configuration:

R2-2104000	Discussion on cross-SRB CPC reconfiguration	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

CHO evaluation after DAPS fallback:

R2-2103046	Conditional evaluation upon fallback to source cell after DAPS handover	Ericsson	CR	Rel-16	36.331	16.4.0	4613	-	F	LTE_feMob-Core

R2-2103047	Conditional evaluation upon fallback to source cell after DAPS handover	Ericsson	CR	Rel-16	38.331	16.4.1	2497	-	F	NR_Mob_enh-Core

Procedural text for section on" Inability to comply with RRCReconfiguration":

R2-2103331	38.331 CR: Revised inability to comply with conditional reconfiguration	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.4.1	2507	-	F	NR_Mob_enh-Core

Full config support for CHO:

R2-2104261	Full configuration for CHO	Google Inc.	CR	Rel-16	38.331	16.4.1	2565	-	F	NR_Mob_enh-Core

Miscellaneous CHO corrections to Stage-2:

R2-2104074	Miscellaneous corrections to 37.340 on mobility enhancement	ZTE Corporation (Rapporteur), Sanechips	CR	Rel-16	37.340	16.5.0	0262	-	F	NR_Mob_enh-Core

### 2.2	Companies’ feedbacks

R2-2103114	Discussion on Applicable Cases for Failure Recovery via CHO	CATT	discussion	Rel-16	NR_Mob_enh-Core

Proposal 1:RAN2 to confirm which of the following agreement is valid:

-	(A) Confirm the agreement made in #109e meeting is still valid, i.e. Failure recovery via CHO in Rel-16 is applicable only to RLF, Intra-RAT Handover Failure or Intra-RAT Conditional Handover Failure. Inter-RAT handover failure can’t trigger failure recovery via CHO. TP option 2 should be adopted.

-	(B) Confirm the agreement made in #113e is valid, Inter-RAT handover failure can trigger failure recovery via CHO. The agreement made in #109e meeting should be revised as following: Failure recovery via CHO in Rel-16 is applicable only to RLF, Intra-RAT Handover Failure or Intra-RAT Conditional Handover Failure or inter-RAT handover failure. TP option 1 should be adopted.

Question 1: Do companies agree with proposal 1 in R2-2103114?

[Rapp] The original question 1 is too general, so it is changed into the following:

Question 1: For proposal 1 in R2-2103114, which option (A or B) do companies prefer?

| Company | Yes/No Preferred option | Comments |
| --- | --- | --- |
| MediaTek | - | We prefer to have failure recovery via CHO only applicable for intra-RAT cases, i.e. the agreement made in #109-e |
| Intel |  | No strong opinion, considering there is no huge impact to specification. InterRAT failure handling is acceptable to us. |
| ZTE | - | We slightly prefer to allow CHO based recovery for inter-RAT HOF. It seems no harm for the UE to maintain the CHO configuration a little longer (i.e. not release the stored CHO configurations upon reception of inter-RAT HO command) and use them upon HOF, if available. |
| Huawei, HiSilicon | B | We see some benefits of applying failure recovery via CHO for inter-RAT handover failure, and the potential specification impacts are minor. |
| Nokia | A | We think CHO was meant to be not supported for inter-RAT cases. Thus, we would prefer to confirm the recovery/reestablishment in inter-RAT scenario cannot use of CHO configurations, if those are somehow available. |

R2-2103332	Clarification on SCG configuration in CHO	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_Mob_enh-Core

Proposal 1: RAN2 to clarify the scenarios for which CHO with SCG configuration shall be supported in Rel. 16.

Proposal 2: RAN2 to send an LS describing the RAN2 understanding on the support of CHO with SCG configuration and asking RAN3 to work on the specification changes, if needed.

Question 2: Do companies agree with proposal 1 and 2 in R2-2103332?

| Company | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes | It’s good the have such clarifications and get RAN3 confirmation. |
| Intel | No | The intention of agreements "We will not preclude SCG configuration in RRC Reconfiguration with conditional reconfiguration. Limit to cases without RAN3 impact." Was not increase the load to RAN3. But Nokia's analysis, seems all potential scenarios have additional RAN3 impact. Then none of them can be supported in Rel-16. |
| ZTE | Maybe no | We have captured that “In case MR-DC is configured, CHO is only supported in Master Node to eNB/gNB Change procedure in this release.” in TS 37.340. So it seems clear on scenarios for CHO with MR-DC. |
| Huawei, HiSilicon | No | Share the same views as Intel. |
| Nokia | Yes | Regarding the comment made by Intel and shared by Huawei: can you please point out what are the other cases, than those we have listed in R2-2103332, which still apply to CHO with SCG configured and will meet the agreement quoted in our paper? In our opinion, we have listed all possible scenarios, and indeed, each seems to have some potential RAN3 impacts. Thus, we should agree whether SCG configuration can be provided with CHO command in any of the cases and perhaps ask RAN3 to check what kind of impacts they see. That was a basic goal of our R2-2103332, to simply gather RAN2 understanding whether the agreement ‘’We will not preclude SCG configuration in RRC Reconfiguration with conditional reconfiguration. Limit to cases without RAN3 impact’’ has any practical meaning or is just a paper solution. |

R2-2104000	Discussion on cross-SRB CPC reconfiguration	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

In the contribution, two options are provided in order to solve the cross-SRB CPC reconfiguration issue:

•	Option1: For CPC configurations with the same condReconfigId, cross-SRB reconfiguration is not allowed to modify CPC configurations.

•	Option2: For a condReconfigId, the UE sends the CPC execution complete message following the last RRC Reconfiguration message. For example, condReconfigId#1 is initially configured via SRB3 but reconfigured via SRB1, the UE shall send an ULInformationTransferMRDC message to the MN as the CPC execution complete message.

Proposal 1: For CPC configurations with the same condReconfigId, cross-SRB reconfiguration is not allowed to modify CPC configurations.

Question 3: Do companies agree with proposal 1 in R2-2104000?

[Rapp] Proposal 1 is following option1. If companies have a preference (between option1 and 2), please indicate it in the table.

| Company | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes | We prefer a simpler method |
| Intel | Yes with comments | I assume, only modification is not allowed. But cross-SRB release of CPC configuration is still allowed. |
| ZTE | No | We see no strong need for such restriction. The simplest way seems to let the UE decide which SRB can be used for sending RRCReconfigurationComplete message according to whether SRB3 exists upon triggering CPC execution. I.e., if SRB3 exists upon CPC execution, the UE shall send the RRC complete message via SRB3, otherwise the UE shall send the RRC complete message via SRB1, regardless of whether the latest received RRCReconfiguration for CPC is transmitted via SRB1/SRB3 or not. |
| Huawei, HiSilicon | Yes | We are the proponent companies. In our paper, we provide option1 (network based solution) and option2 (UE based solution). For option2, we think it requires the UE to couple the condReconfigId with the relevant SRB, and thus it may introduce some complexities. So we prefer option1. |
| Nokia | No | We believe the NW can handle that and no restrictions (or NOTEs in the specs) are needed. |

R2-2103046	Conditional evaluation upon fallback to source cell after DAPS handover	Ericsson	CR	Rel-16	36.331	16.4.0	4613	-	F	LTE_feMob-Core

R2-2103047	Conditional evaluation upon fallback to source cell after DAPS handover	Ericsson	CR	Rel-16	38.331	16.4.1	2497	-	F	NR_Mob_enh-Core

Question 4: Do companies agree with the changes in R2-2103046/3047?

| Company | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes |  |
| Intel | Yes | Assume it covers both CHO and CPC. |
| ZTE | Yes with comments | It is reasonable to restart the conditional reconfiguration evaluation upon the UE falls back to the source cell. But we think this action is just applicable to CHO case. Since the source SCG shall be released before performing DAPS HO, there is no CPC configuration stored in the source configuration upon fall-back to the source cell. So the summary of change and impacted functionality in the cover sheet need to be updated to remove CPC. |
| Huawei, HiSilicon | No | RAN2 agreed that DAPS and CHO can not be configured together for the UE, and the following field description captures it: The RRCReconfiguration message contained in condRRCReconfig cannot contain the field conditionalReconfiguration or the field daps-Config. In the reason for change, it mentions: In case of fallback to the source cell at a DAPS HO (due to T304 expiry when RLF has not been detected for the source cell), the UE will however still have the conditional reconfigurations available. It means that the UE stores both CHO and DAPS configurations before DAPS fallback. We wonder whether it can happen or not. |
| Nokia | No | How come the UE was configured with CHO and DAPS simultaneously? We do not optimize for the cases which are forbidden in current release (CHO/CPC + DAPS). |

R2-2103331	38.331 CR: Revised inability to comply with conditional reconfiguration	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.4.1	2507	-	F	NR_Mob_enh-Core

Question 5: Do companies agree with the changes in R2-2103331?

| Company | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes |  |
| Intel |  | We agree the intention. But the changes seems no correct for SA NR and NR DC case. E.g. Following should not be applied for NR DC. 3> else: 4> initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7, upon which the connection reconfiguration procedure ends; |
| ZTE | - | Agree with the intention. But it seems an editorial change, so prefer to merge it to the rapporteur CR if agreed. |
| Huawei, HiSilicon | No | There are lots of changes (including removing some text), and there is no inter-operability issue for the changes (from the cover page). We are concerned about the actual impacts, i.e. whether it impacts the functionality or not. So we prefer to keep the original text. |
| Nokia | Yes | Proponent. We are OK to update the procedure in case somebody (e.g. Intel as commented above) finds a missing/incorrect case. We agree, the re-establishment via 36.331 procedure does not apply to NR DC. Regarding Huawei’s comment: there is no inter-operability issue, as functionally this section stays the same. |

R2-2104261	Full configuration for CHO	Google Inc.	CR	Rel-16	38.331	16.4.1	2565	-	F	NR_Mob_enh-Core

Question 6: Do companies agree with the changes in R2-2104261?

| Company | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | - | We do not see the confusion but if other companies do, we can have some clarifications. Prefer to use the term “RRCReconfiguration for SCG”, rather than “SN RRCReconfiguration”, to align with ASN.1 |
| Intel | - | Should not “during reconfiguration with sync” cover CHO case? |
| ZTE | - | The first change is OK. For the second change, the CHO is also covered by reconfiguration with sync. So this change seems not really needed. Besides, we think the description of absent condition in the field description can be removed to the condition explanation of FullConfig to align with the ASN.1 guide for condition tags. |
| Huawei, HiSilicon | - | According to the definition of FullConfig, it is optionally present, Need N, during reconfiguration with sync. We think here “reconfiguration with sync” covers CHO case, so we understand that the full configuration for CHO is allowed based on the current specification. |
| Nokia | No | We think the first change is OK, but actually not needed. Second change is incorrect/redundant, CHO case is already covered by reconfiguration with sync. |

R2-2104074	Miscellaneous corrections to 37.340 on mobility enhancement	ZTE Corporation (Rapporteur), Sanechips	CR	Rel-16	37.340	16.5.0	0262	-	F	NR_Mob_enh-Core

Question 7: Do companies agree with the changes in R2-2104074?

| Company | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | Yes |  |
| Intel |  | No essential, but ok to have this restructure from specification Editor. |
| ZTE | Yes |  |
| Huawei, HiSilicon |  | No strong opinion. We have some suggestions on the new changes: - The figure numbering should be uniform, it is using “-3a” and “-5” for the current changes - In figure 10.3.1-3a, the step 3 should have the same style as in other figures - Also in figure 10.3.1-3a, there are some “(containing CPC configuration)”, however they are missing in figure 10.3.1-5 |
| Nokia | No strong view | The changes are OK, aligned with what we have done in other section (10.3.2). However, the specification will be OK also without them. |

## 3	Conclusion

[To be updated]