3GPP TSG-RAN WG2 Meeting #113 bis electronic						R2-21xxxxx
Online, April 12 – April 20, 2021

Agenda Item:	8.2.2.2

Source: 	OPPO

Title:  	offline of RRM relax and DL fine Sync for SCG deactivation (OPPO)

Document for:	Discussion and decision

# Introduction

This document is to kick off the following offline discussion:

- Offline [240] on P11: Do we support RRM relaxations and why? Can discuss draft LS to RAN4 if there is sufficient support.

Proposal 11: Send LS to RAN4 and ask RAN4 to study the RRM relax for SCG deactivation and RRM requirement for maintaining DL fine Sync.

- Deadline: collect companies’ view, by 2021-04-19 18:00 UTC;

Contact Information

| Company | Email |
| --- | --- |
| OPPO | wangshukun@oppo.com |

# Discussion

In [3][6][8][10][15], RRM relax should be supported for UE power saving purpose, e.g. configure long measurement cycle for deactivated SCG as deactivated SCell, and [6][8][10] propose to ask RAN4 to study RRM relax for SCG deactivation.

In [3][4][5][10][16], companies agree that DL Sync can be achieved by RRM depending on how dense PSCell RRM measurements. However, it is up to RAN4 to study the requirement of PSCell RRM measurements for maintaining DL fine Sync.

Q1: Do companies agree that RRX relax is supported for SCG deactivation and why?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| OPPO | Yes | In NR CA, long measurement period is configured for deactivated scell measurement for UE power saving purpose. If the SCG is deactivated, RRM relax, e.g. long measurement period as deactivation scell, can be applied for the SN configured RRM measurement for UE power saving. |

Q2: Do companies agree it is up to RAN4 to study the requirement of PSCell RRM measurements for maintaining DL fine Sync for SCG deactivation?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| OPPO | Yes |  |

Q3: Do companies agree to send LS to RAN4 and ask RAN4 to study the RRM relax for SCG deactivation and RRM requirement for maintaining DL fine Sync?

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| OPPO | Yes | In order to make good progress on R17 eDCCA, It is better to trigger RAN4 to start their work on R17 eDCCA as soon as possible. |

Q4: Do companies agree the LS content as below?

==========================

- Overall Description:

RAN2 has started the work within the Rel-17 eDCCA. RAN2 agreed to support SCG deactivation for UE power saving purpose, i.e. no PDCCH monitoring, no PDSCH reception and no PUSCH transmission for UE during SCG deactivation.

RAN2 has discussed RRM measurements and support to perform RRM measurment based on SN confgured RRM configuration in order to support PSCell mobility. For UE power saving purpose, RAN2 agree to support RRM relax for SCG deactivaiton.

In order to reach the desired latency reduction after SCG activation from deactivation, the UE needs to maintains fine DL synchronization. RAN2 also understand that DL Sync can be achieved by RRM depending on how dense PSCell RRM measurements. However, it is up to RAN4 to study the requirements of PSCell RRM measurement for maintaining DL fine Sync.

RAN2 asks RAN4 the following questions:

Q1: Does RAN4 see potential benefits, or any issues, with relaxed RRM measurements during SCG deactivation? If yes, RAN2 respectfully asks RAN4 to study how to relax RRM for SCG deactivation?

Q2: Does RAN4 see the posiblity to achieve DL sync by RRM for PSCell during SCG deactivation? If yes, RAN2 respectfully asks RAN4 to study requirements of PSCell RRM measurements for maintaining DL Sync?

- Actions:

To RAN4:	RAN2 respectfully asks RAN4 to consider the information above including the above questions.

===========================

| Company | Agree? (Yes or No) | Comments |
| --- | --- | --- |
| OPPO | Yes |  |

# Conclusions

Based on the discussion above, we propose:

# Reference

[1] R2-2102749	Considerations on RLM during SCG deactivation	KDDI Corporation	discussion	Rel-17

[2] R2-2102872	UE behavior when SCG is deactivated	vivo	discussion	Rel-17	LTE_NR_DC_enh2-Core

[3] R2-2102897	UE measurements and reporting in deactivated SCG	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

[4] R2-2103036	Discussion on UE behaviour when SCG is deactivated	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

[5] R2-2103107	UE Behavior in Deactivated SCG	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

[6] R2-2103275	Measurements for deactivated SCG	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

[7] R2-2103398	UE behavior when SCG is deactivated	Lenovo, Motorola Mobility	discussion	Rel-17

[8] R2-2103569	UE Measurement Aspects in SCG Deactivation	LG Electronics	discussion	Rel-17	LTE_NR_DC_enh2-Core

[9] R2-2103682	Measurements and maintenance of UL synch with a deactivated SCG	InterDigital	discussion	Rel-17	LTE_NR_DC_enh2-Core

[10] R2-2103808	UE measurements and reporting in deactivated SCG	Ericsson	discussion	Rel-17	LTE_NR_DC_enh2-Core

[11] R2-2103885	TA Maintenance and other UE actions in SCG deactivated state	Apple	discussion	Rel-17	LTE_NR_DC_enh2-Core

[12] R2-2103893	UE measurements and reporting in deactivated SCG	Qualcomm Incorporated	discussion	Rel-17

[13] R2-2103913	UE assistance information use case for SCG deactivation	Convida Wireless	discussion	Rel-17	LTE_NR_DC_enh2-Core

[14] R2-2103978	UE behaviour in deactivated SCG	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core

[15] R2-2104124	Discussion for UE behaviour in deactivated SCG	SHARP Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core

[16] R2-2104160	UE behavior during SCG deactivation	MediaTek Inc.	discusstion