3GPP TSG-RAN WG2 Meeting #113bis-e                                                 draft-R2-2104450

E-Meeting: 12 – 20 April 2021

Agenda item:	9.1.3

Source:	Qualcomm Incorporated

Title:	Report of [AT113bis-e][301][NBIOT/eMTC R17] NB-IoT Carrier Selection (Qualcomm)

Document for:	Discussion and Decision

# Introduction

RAN2 initiated the following offline:

:

- [AT113bis-e] [301][NBIOT/eMTC R17] NB-IoT Carrier Selection (Qualcomm)

Scope: Use R2-2103015 as a starting point.

    - How options 1 and 2 work in the 2 cases – same cell, cell change.

    - Metrics needed from UE.

Intended outcome: Report in R2-2104450

Deadline: Monday 19 April 1200 UTC

This document is to capture the company views and provide summary on this email discussion.

# How options 1 and 2 work in the 2 cases – same cell, cell change

## Overview of the two options

The two options for coverage-based paging carrier selection are depicted in Figure 1.

Figure 1: The two options for coverage-based paging carrier selection

Note, with Option 1 a solution may require network to provide additional information during dedicated connection for UE to use when selecting coverage-based paging carrier.

For the purpose of discussion in this section, the following definitions are used:

- Optimal paging carrier: A paging carrier that has Rmax just sufficient for UE to reliably decode the page.

        - Just sufficient could be interpreted to mean Rmax is no more than 2x the number of repetitions required to reliably decode pages.

- Non-optimal paging carrier: A paging carrier that has Rmax higher than the Rmax for Optimal paging carrier.

## Paging carrier selection upon coverage change

Once a UE starts to use the Optimal paging carrier (selected following Option 1 or Option 2) according to its coverage level then the UE behaviour needs to be defined for the case where the coverage level changes. Coverage level change could mean coverage level is better or worse than the coverage level when UE started to use Optimal paging carrier.

Question 2.2.1: In the context of cover-based paging carrier selection, what is your understanding of coverage level change?

| Company | Option 1: Better/Worse/Both | Option 2: Better/Worse/Both |
| --- | --- | --- |

Question 2.2.2: If UE is using Non-optimal paging carrier and the radio conditions become suitable for Optimal paging carrier then should the UE switch to using the Optimal paging carrier?

| Company | Option 1: Yes/No | Option 2: Yes/No |
| --- | --- | --- |

The key point about defining mechanism for UE to switch to a Non-optimal paging carrier is because the UE cannot reliably decode paging messages using the Optimal paging carrier i.e. this paging carrier does not have sufficient repetitions and/or power level. Therefore, can UE determine from coverage level whether the UE should stop using the coverage-based paging carrier.

Question 2.2.3: How can UE determine from the coverage level whether the UE should switch to a different paging carrier?

| Company | Option 1 | Option 2 |
| --- | --- | --- |

Question 2.2.4:  Any other comments/suggestions to help decide between the two options?

| Company | Comments |
| --- | --- |

## Paging carrier selection upon cell change

For the case of cell change the following aspects must be considered:

  - Network cannot always know UE has changed cell i.e. UE has no need to trigger RRC connection, including EDT, after cell reselection.

  - Different cells could have different paging carrier configurations e.g. different cells have different number of paging carriers, different paging carrier parameters etc.

The mechanism defined for paging carrier selection upon cell reselection should aim to avoid the following:

- Excessing use of radio resources i.e. paging UE on multiple carriers in the same cell either simultaneously or sequentially should be avoided.

- Excessive delay to reach the UE reliably. If the solution could lead to long delay then core network may need to take this into consideration.

Question 2.3.1: How does both network and UE select paging carrier after cell reselection to a new cell?

| Company | Option 1: | Option 2: |
| --- | --- | --- |

A UE using Optimal paging carrier in cell A reselects cell B and before UE establishes a RRC connection (including EDT) in cell B it returns to cell A. In that case how should UE select paging carrier upon returning to cell A

Question 2.3.2: How does UE select paging carrier after UE reselects back to a cell where it was using Optimal paging carrier?

| Company | Option 1: | Option 2: |
| --- | --- | --- |

Question 2.3.3:  Any other comments/suggestions to help decide between the two options?

| Company | Comments |
| --- | --- |

# Metrics needed from UE

Contribution [1] highlighted issues with using dedicated channel metrics or serving cell measurements to determine whether the paging carrier, from UE perspective, is optimal or not. Furthermore, paging carrier selection based on short term measurements could lead to reduced paging reliability and/or increased network resource usage.

The overriding requirement is the UE should be able to decode paging message reliably, which is essentially what the current specifications support. To reduce UE power consumption, it is desirable for UE not to have to receive large number repetitions only to determine network did not send any paging message.

Question 3.1: Do you agree the paging channel metric is needed to determine suitability of the current paging carrier?

| Company | Yes/No | Comments |
| --- | --- | --- |

Question 3.2: If the answer to question 3.1 is No, then provide your view on how the UE determine whether the selected paging carrier provides sufficient reliability?

| Company | Answer |
| --- | --- |

Question 3.3: If the answer to question 3.1 is No, then provide your view on how the UE determine whether the selected paging carrier is optimal? For definition of optimal paging carrier see section 2.

| Company | Answer |
| --- | --- |

Question 3.4: If the answer to question 3.1 is Yes, then should this metric be the number of repetitions UE required to decode the page?

| Company | Yes/No | Comments |
| --- | --- | --- |

Question 3.5: If the answer to question 3.4 is No, then state details of other metrics?

| Company | Alternative metric details. |
| --- | --- |

Question 3.6: If the answer to question 3.4 is Yes, then should the UE gather paging channel metrics (e.g. the number of repetitions required to decode each page) over a period of time e.g. 24 hours?

| Company | Yes/No | Comments |
| --- | --- | --- |

The paging channel metrics UE can easily gather over a period are:

- Minimum number of repetitions required to decode a page (Min)

- Maximum number of repetitions required to decode a page (Max)

- Average number of repetitions required to decode a page (Ave)

- No of paging occasions UE failed decode (Fail)

Question 3.7: If the answer to question 3.6 is Yes, then should the UE compute the minimum, maximum, average number of repetitions required to decode pages as well as number of pages it failed to decode over the metric gathering period e.g. 24 hours? Your response could be one or more from the choice list.

| Company | Min, Max, Ave, Failed | Comments |
| --- | --- | --- |

Question 3.8:  Any other comments?

| Company | Comments |
| --- | --- |

# Conclusion

TBD

# References

- R2-2103015,	Determining paging carrier suitability, April 2021, RAN2#113bis-e

# Participants

| Company | Name | e-mail address |
| --- | --- | --- |