3GPP TSG-RAN WG2 Meeting #113bis-e                                                 draft-R2-2104450

E-Meeting: 12 – 20 April 2021

Agenda item:	9.1.3

Source:	Qualcomm Incorporated

Title:	Report of [AT113bis-e][301][NBIOT/eMTC R17] NB-IoT Carrier Selection (Qualcomm)

Document for:	Discussion and Decision

# Introduction

RAN2 initiated the following offline:

:

- [AT113bis-e] [301][NBIOT/eMTC R17] NB-IoT Carrier Selection (Qualcomm)

Scope: Use R2-2103015 as a starting point.

    - How options 1 and 2 work in the 2 cases – same cell, cell change.

    - Metrics needed from UE.

Intended outcome: Report in R2-2104450

Deadline: Monday 19 April 1200 UTC

This document is to capture the company views and provide summary on this email discussion.

# How options 1 and 2 work in the 2 cases – same cell, cell change

## Overview of the two options

The two options for coverage-based paging carrier selection are depicted in Figure 1.

Figure 1: The two options for coverage-based paging carrier selection

Note, with Option 1 a solution may require network to provide additional information during dedicated connection for UE to use when selecting coverage-based paging carrier.

Question 2.1.1: How is the paging carrier selection information signalled to the UE?

| Company | Option 1: Dedicated/broadcast/both | Option 2: Dedicated/broadcast/both |
| --- | --- | --- |
| Huawei, HiSilicon | unclear to us | Dedicated signalling to provide the information about the dedicated carrier, e.g. explicit configuration or a pointer to a carrier in system information FFS if carrier details are signalled in dedicated signalling or broadcast signalling |
| Spreadtrum | The paging carrier related configuration for coverage-based paging carrier selection is signalled in the broadcast signalling. The coverage level information is signalled in dedicated signalling. | A dedicated signalling is used for providing the dedicated paging carrier related information. |
| MediaTek | Agree with Spreadtrum. On top of it, the coverage level information has not to be defined as from network to UE. The network can estimate UE’s downlink channel quality by uplink, but this is not accurate enough, DL channel quality report on Msg3 was designed for the possible asymmetry DL-UL interference. It is possible for network to utilize the quality report result directly, but this feature is optional for UE, network may not rely on it. Therefore, a coverage level information report from UE to network could be considered. | Same as the comments in option 1, a coverage level information report from UE to network could also be considered. |
| Sequans | Paging carriers’ details are given in broadcast signalling. The carriers could also be grouped. UE-specific information for selection (e.g. Rmax) is given in dedicated signalling. If grouped, selection could be randomized (e.g. based on UE_ID) within a selected group. | Paging carriers’ details are possibly given in broadcast signalling. The carriers could also be grouped. Selected carrier / pointer to carrier in SI are given in dedicated signalling. If grouped, selection could be randomized (e.g. based on UE_ID) within a selected group. |
| Ericsson | If for Option 1, signaling during RRC connected mode is optional, we are not clear how eNB and UE align paging carrier selection information. | Dedicated signaling will be used for paging carrier selection. When fall back happens, it can use either dedicated or broadcast signaling. |

For the purpose of discussion in this section, the following definitions are used:

- Optimal paging carrier: A paging carrier that has Rmax just sufficient for UE to reliably decode the page.

        - Just sufficient could be interpreted to mean Rmax is no more than 2x the number of repetitions required to reliably decode pages.

- Non-optimal paging carrier: A paging carrier that has Rmax higher than the Rmax for Optimal paging carrier.

## Paging carrier selection upon coverage change

Once a UE starts to use the Optimal paging carrier (selected following Option 1 or Option 2) according to its coverage level then the UE behaviour needs to be defined for the case where the coverage level changes. Coverage level change could mean coverage level is better or worse than the coverage level when UE started to use Optimal paging carrier.

Question 2.2.1: In the context of cover-based paging carrier selection, what is your understanding of coverage level change?

| Company | Option 1: Better/Worse/Both | Option 2: Better/Worse/Both |
| --- | --- | --- |
| Nokia | If the coverage level becomes better than coverage level at the time of connection release, UE will miss the paging on PO because network selects carrier based on last known coverage condition which will be different from UE. And this cannot converge even after paging escalation. This can be mitigated if the UE selects the carrier based on last known coverage level in this case. Both options have issues for some coverage change scenario | If coverage becomes worser at the time PO after connection release, this option will lead to missing of paging unless fallback mechanism to carrier with higher repetition level. Both options have issues for some coverage change scenario |
| Huawei, Hisilicon | Worse (Assuming the UE is using the ‘selected’ carrier). | same as option 1 |
| Spreadtrum | When coverage becomes worse, since the UE have to select paging carrier based on the previously determined CEL, the UE might fail to decode paging message. | A same issue should be considered as that in option 1. |
| MediaTek | Worse | Worse |
| Sequans | Coverage level change must include the case when the coverage level becomes worse. An optimization could consider the case when the coverage level becomes better, but that also has downsides (e.g. possible ping-ponging) | Same as option 1 |
| Ericsson | We should ensure there is no discrepancy between where UE monitors and where NW thinks UE is monitoring. Paging success rate KPI should not deteriorate. Fallback options need to be provided | Same as Option 1 |

Question 2.2.2: If UE is using Non-optimal paging carrier and the radio conditions become suitable for Optimal paging carrier then should the UE switch to using the Optimal paging carrier?

| Company | Option 1: Yes/No | Option 2: Yes/No |
| --- | --- | --- |
| Nokia | No for same cell scenario. This will lead to mismatch in paging carrier selection of NW and UE for the same cell scenario. Yes for cell change scenario. As the NW does not refer to last known coverage level it is possible for UE and NW to choose optimal carrier for paging in new cell. | No. |
| Huawei, Hisilicon | FFS: How does the UE determine that the candidate carrier has become suitable. Which metrics is used ? The simplest option is that the UE remains on the fallback carrier. | Same as option 1 |
| Spreadtrum | No. In order to keep a consistent understanding between UE and eNB, the UE should remain on the carrier based on the previous coverage level. | No. Same as option 1. |
| MediaTek | Yes. We had a work assumption that when coverage level changes, UE will not report the coverage level update. If UE fall back to the non-optimal paging carrier due to radio condition deteriorate and then the radio condition gets better, the network would has no idea about it, network would still has to send paging message on the optimal and non-optimal paging carrier, simultaneously or sequentially. If the UE can return to the optimal paging carrier when the radio condition gets better, the UE and network can still benefit from it. | Yes, same as option 1. |
| Sequans | A-priori we think No, as this may result in ping-ponging which could lead to longer delays; The UE should go to the fallback carrier when required and stay on it. However, we are open to revisit this when the exact solution is clearer. | Same as option 1 |
| Ericsson | We should ensure there is no discrepancy between where UE monitors and where NW thinks UE is monitoring. Paging success rate KPI should not deteriorate. | Same as Option 1 |

The key point about defining mechanism for UE to switch to a Non-optimal paging carrier is because the UE cannot reliably decode paging messages using the Optimal paging carrier i.e. this paging carrier does not have sufficient repetitions and/or power level. Therefore, can UE determine from coverage level whether the UE should stop using the coverage-based paging carrier.

Question 2.2.3: How can UE determine from the coverage level whether the UE should switch to a different paging carrier?

| Company | Option 1 | Option 2 |
| --- | --- | --- |
| Nokia | UE select paging carrier whose Rmax is closest to the estimated required number of repetitions. Depending on the coverage level the selected paging carrier also needs to be switched. | Switching is not possible without definition of proper fallback mechanism. Legacy fallback mechanism will not work here. |
| Huawei, Hisilicon | If UE is on the selected carrier, it could be based on the estimated number of repetitions required to receive paging. If the number is larger than Rmax of the carrier, then the UE falls back. If the UE is using the fallback carrier, there is no metric that will be applicable directly to the ‘candidate’ carrier. One possibility could be to use the RSRP measurement and associate it with the candidate carrier – How to do the mapping is FFS. Then upon switch/selection to/of the candidate carrier, the UE will have to check that it is suitable to receive paging. We do not think that using the expected number of repetitions on the fallback carrier helps as this measurement is carrier specific. | Same as option1 |
| Spreadtrum | When the coverage becomes worse or the required number of repetition is larger than the Rmax of the determined carrier, the UE needs to switch to a fallback carrier. | Same as option 1. |
| MediaTek | Agree with Spreadtrum | Same as option 1. |
| Sequans | Based on some coverage level or estimation of required number of estimations. This could be based on a specified rule or UE implementation. | Same as option 1 |
| Ercisson | Agree with Huawei. It may be complicated on how UE judge which the alternate fallback carrier is | NW can provide either via broadcast or dedicated signalling which carrier can be fallback. |

Question 2.2.4:  Any other comments/suggestions to help decide between the two options?

| Company | Comments |
| --- | --- |
| Nokia | For same CEL scenarios both options work well. For coverage change each option have drawback for some scenarios. Additional changes required on the basic method of carrier selection of both options to address these issues. |
| Huawei, HiSilicon | There is no fundamental difference on the above aspects |
| Spreadtrum | Both the two options have the same issue on coverage change. Other changes required should be furtherly considered to help make the decision between the two options. |
| Sequans | Anything can be done with both options. Therefore, we should focus first on what we want to achieve rather than how. Then, based on the exact needs, one option may be found to be more efficient. In general, we believe option 2 is better from specification complexity (but this may be affected by what exactly we want to achieve) and forward compatibility aspects. |
| Ericsson | The fundamental difference is: should large impacts be in UE or should it be in NW. We think it is easier if NW implements. |

## Paging carrier selection upon cell change

For the case of cell change the following aspects must be considered:

  - Network cannot always know UE has changed cell i.e. UE has no need to trigger RRC connection, including EDT, after cell reselection.

  - Different cells could have different paging carrier configurations e.g. different cells have different number of paging carriers, different paging carrier parameters etc.

The mechanism defined for paging carrier selection upon cell reselection should aim to avoid the following:

- Excessing use of radio resources i.e. paging UE on multiple carriers in the same cell either simultaneously or sequentially should be avoided.

- Excessive delay to reach the UE reliably. If the solution could lead to long delay then core network may need to take this into consideration.

Question 2.3.1: How does both network and UE select paging carrier after cell reselection to a new cell?

| Company | Option 1: | Option 2: |
| --- | --- | --- |
| Nokia | UE: When UE reselect new cell, it checks the Rmax value associated with all the carriers. If all Rmax values are same or the difference is only to manage the power level difference, UE can use paging carrier selection of Rel-16 mechanism. If the Rmax value of some carriers are very low compared to the anchor carrier Rmax value which is baseline for maximum coverage, Based on its measurements UE estimate the required repetitions and select paging carrier(s) whose Rmax is closer to the estimated number of repetitions for the current coverage level. NW: When network sends the paging to cell other than last connected cell, it starts the paging on the carrier having lower number of Rmax. In case of no response NW can increase the repetition for paging then accordingly it can also choose another carrier suitable for the new value of repetition value. If the NW intends to select carrier based on Rel-16 mechanism in new cell and intends to change the repetition within the same carrier without changing the paging carrier, NW can disable the coverage based carrier selection mechanism after cell reselection. NW can decide to enable/disable based on its paging strategy for paging escalation. | Not applicable. Option 2 is usable for cell change scenario if all the cells supports same set of carriers. Here also it will work only if the CEL does not degrade after cell change. |
| Huawei, HiSilicon | Unclear for option 1 what is the information used by the UE and NW to decide a carrier. One option would be the fallback carrier (i.e. legacy mechanism). An alternative option would be to use a ‘generic’ information negotiated between UE and eNB (e.g. ‘authorisation of good coverage carriers’) plus a broadcast information (e.g. list of good coverage carriers, all carriers being equivalent so there is only one possible choice on both sides). A combination of the two above. We do not think that using a cell specific (e.g. RSRP or Rmax) negotiated in another cell is appropriate. | Fallback carrier (i.e. legacy mechanism) |
| Spreadtrum | When the UE moves to a new cell, it would evaluate its coverage level firstly. If the current coverage level is better than or equal to the previous negotiated result, it would select the paging carrier based on the previous coverage level plus the broadcast paging carrier configuration in the new cell. Otherwise, it would select a fallback carrier. The network would select the paging carrier based on the previous coverage level plus the broadcast paging carrier configuration. | Both the UE and the network would use a fallback paging carrier. |
| MediaTek | Agree with Spreadtrum. Additionally, the excessive use of radio resource and excessive delay to reach the UE reliably is inevitable due to the coverage level change scenario in the same cell. We should try to mitigate them in the multi-cell scenario. But the radio condition in the new cell tender to be as good as in the last cell for mobile UE in the most of time, the excessive use of radio resource and excessive delay to reach the UE would not be significant. | Same like option 1, except a new mechanism should be defined to allow UE select a paging carrier after cell change. |
| Sequans | In principle it is possible that UE evaluates its coverage level based on the same mechanism as in the intra-cell scenario (Q 2.2.3) and selects either a carrier with the same related Rmax (based on SI) as on the previous cell or the fallback legacy mechanism. However, here already the NW has to page to UE on twice as many carriers per cell as in the legacy mechanism. Add to that further reselections and that sounds really excessive. Limiting this mechanism, to a single reselection will complicate much for very little gain. Notice this also adds load to the good coverage carriers, making this feature less effective for e.g. static UEs in the cell. So, there is a trade-off between optimizing this feature for (temporarily) static and mobile UEs. We think the latter should be the focus of this WI | Similarly to option 1, if some SI information is available, the NW can indicate to the UE which (group of) carrier(s) to select (from) based on some self-estimation by the UE. Even without SI, the NW can give some clear parameters for selection. If nothing is applicable, UE goes to the fallback legacy mechanism. This has the same downsides as option 1. |
| Ericsson | If the required Rmax in the new cell is smaller or equal to the Rmax in the previous cell, UE will be paged with no delay. If the required Rmax in the new cell is larger to the Rmax in the previous cell, UE will switch to a new paging carrier with no notification to eNB, then UE cannot be paged at first paging attempt, eNB need to change to a new paging carrier with larger Rmax, finally the UE will be paged, but there will be some delays and paging resource waste. | Fall back carrier should be used as per-Rel17. |

A UE using Optimal paging carrier in cell A reselects cell B and before UE establishes a RRC connection (including EDT) in cell B it returns to cell A. In that case how should UE select paging carrier upon returning to cell A

Question 2.3.2: How does UE select paging carrier after UE reselects back to a cell where it was using Optimal paging carrier?

| Company | Option 1: | Option 2: |
| --- | --- | --- |
| Nokia | There is no difference between normal case and reselection back to same cell in this option. Because the paging carrier selection is based on latest coverage level. | It is possible if the NW does not change the set of paging carriers and configuration of Rmax before UE return back to same cell. |
| Huawei, HiSilicon | yes if still suitable | same as option 1 |
| Spreadtrum | If the coverage does not become worse or the broadcast paging configuration does not change, there will be no difference. | The assigned paging carrier can continue to be used. |
| MediaTek | Since the network has no idea about the cell change, UE should be allowed to select the optimal paging carrier if the radio condition is suitable. | Same as option 1. |
| Sequans | This is similar to the intra-cell scenario (Q 2.2.2) so a-priori we think No, as this may result in ping-ponging which could lead to longer delays; The UE should go to the fallback legacy carrier. However, we are open to revisit this when the exact solution is clearer. | Same as option 1 |
| Ericsson | Agree with MediaTek | Same a option1. |

Question 2.3.3:  Any other comments/suggestions to help decide between the two options?

| Company | Comments |
| --- | --- |
| Nokia | When cell is configured with carriers having different Rmax values, UE behaviour needs to be specified on how the UE will select subset of carriers for paging reception based on its current coverage condition in new cell. This is required irrespective of the option chosen for same cell scenario. Eventhough there are views that the solution need not work for cell change scenario.. the configuration of different Rmax values across carriers requires change in paging carrier selection irrespective of basic solution. This issue cannot be excluded from the scope of work. Fallback to UE-ID based paging carrier selection will not work when carriers have different Rmax values. |
| Huawei, HiSilicon | Option 1 could allow to use the new scheme in another cell. However, so that the UE and eNB has only one possible choice this will require that all the new carriers are equivalent in term of ‘coverage’. It is also unclear how the eNB can decide that the UE is allowed to use the ‘good coverage carriers’ in another cell (potential inputs could be ‘coverage restricted UE’, ‘UE specific DRX’, stationary subscription info, some eNB observed information (only possible for the UP solution?) …). Option1 requires a lot work to solve the details. Option 2 allows carriers with different coverage (i.e. different Rmax). Option 2 is a lot easier to specify. |
| Spreadtrum | Option 1 is more flexible, and more UEs would benefit from the coverage based paging. Option 2 seems slightly unreasonable, since the coverage level would remain unchanged with a certain probability when the UE moves a new cell. |
| MedaTek | Option 1 is better for the cell change scenario. |
| Ericsson | Option 2 is more stable for cell change scenario, we cannot predict the probability of cell level changes when UE moves to a new cell. We can also limit the feature to same cell case; in case of cell change then legacy means. |

# Metrics needed from UE

Contribution [1] highlighted issues with using dedicated channel metrics or serving cell measurements to determine whether the paging carrier, from UE perspective, is optimal or not. Furthermore, paging carrier selection based on short term measurements could lead to reduced paging reliability and/or increased network resource usage.

The overriding requirement is the UE should be able to decode paging message reliably, which is essentially what the current specifications support. To reduce UE power consumption, it is desirable for UE not to have to receive large number repetitions only to determine network did not send any paging message.

Question 3.1: Do you agree the paging channel metric is needed to determine suitability of the current paging carrier?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Nokia | No | For CEL selection for RACH and RAR search space UE already uses RSRP measurements as criteria. RSRP threshold is used to select specific pool. If the main intention is to support set of carriers for UE in normal coverage (equivalent to CEL-0) and other carriers for all the higher CEL, the mechanism used for CEL selection can be used. This may require RSRP threshold to be associated with carriers configured with lower number of Rmax. Assuming the main purpose is to separate UE in normal coverage against the UE in extended coverage , the network may need to support only two values of Rmax across all the carriers. In such case simple mechanism of RSRP based measurements will be sufficient. We don’t see need to have more granular Rmax value across different carriers which will require more accurate estimation of required repetition for paging reception. |
| Huawei, HiSilicon | yes |  |
| Spreadtrum | No | We think the paging carriers configured with the same CEL would be configured with the same Rmax value. The current RSRP measurement for the determination of CEL is sufficient for paging carrier selection and the paging reliability can be ensured. |
| MediaTek | No | Agree with Nokia. |
| Sequans | Yes | This is different from RACH and RAR as this procedure applies for a long period of time and conditions may change. |

Question 3.2: If the answer to question 3.1 is No, then provide your view on how the UE determine whether the selected paging carrier provides sufficient reliability?

| Company | Answer |
| --- | --- |
| Nokia | As per the UE distribution across different coverage levels referred in many simulations more than 90% of the UE in cell are expected to be in good coverage condition which is equivalent to CEL-0. Only 10% UE may be distributed across further coverage ranges or RSRP values. Primary objective of the work item is to differentiate paging messages for UE in normal coverage from all extended coverage cases, only 2 Rmax values will be sufficient or it could be 3. With these Rmax values sufficiently separated decision based on RSRP measurements will be sufficient. |
| Spreadtrum | The CEL is determined by the measured RSRP and the configured RSRP threshold of each CEL. And, if the paging carrier is configured based on the CEL associated with the corresponding Rmax value, the selected paging carrier would ensure its sufficient reliability. |

Question 3.3: If the answer to question 3.1 is No, then provide your view on how the UE determine whether the selected paging carrier is optimal? For definition of optimal paging carrier see section 2.

| Company | Answer |
| --- | --- |

Question 3.4: If the answer to question 3.1 is Yes, then should this metric be the number of repetitions UE required to decode the page?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Sequans | Probably at least partially | This sounds like a good candidate, but it may not be suitable for seldomly-paged UEs |

Question 3.5: If the answer to question 3.4 is No, then state details of other metrics?

| Company | Alternative metric details. |
| --- | --- |

Question 3.6: If the answer to question 3.4 is Yes, then should the UE gather paging channel metrics (e.g. the number of repetitions required to decode each page) over a period of time e.g. 24 hours?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | FFS | Need discussion. Probably need an average on a number of samples. One question is what the UE does if it has not acquired enough samples yet. |
| Sequans | probably |  |

The paging channel metrics UE can easily gather over a period are:

- Minimum number of repetitions required to decode a page (Min)

- Maximum number of repetitions required to decode a page (Max)

- Average number of repetitions required to decode a page (Ave)

- No of paging occasions UE failed decode (Fail)

Question 3.7: If the answer to question 3.6 is Yes, then should the UE compute the minimum, maximum, average number of repetitions required to decode pages as well as number of pages it failed to decode over the metric gathering period e.g. 24 hours? Your response could be one or more from the choice list.

| Company | Min, Max, Ave, Failed | Comments |
| --- | --- | --- |
| Sequans | FFS | Unclear how failed can be defined. |

Question 3.8:  Any other comments?

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | What metrics are used to return to the ‘candidate’ carrier when the UE is using the fallback carrier (if this is supported)? |

# Conclusion

TBD

# References

- R2-2103015,	Determining paging carrier suitability, April 2021, RAN2#113bis-e

# Participants

| Company | Name | e-mail address |
| --- | --- | --- |
| Nokia | Srinivasan Selvaganapathy | Srinivasan.selvaganapathy@nokia.com |
| Sequans | Noam Cayron | noam.cayron@sequans.com |
| Ritesh | Ritesh Shreevastav | Ritesh.shreevastav@ericsson.com |