3GPP TSG-RAN WG2 Meeting #113bis electronic	                                                                            R2-21xxxxx

12th April – 20th April 2021

Agenda Item:	7.2.2

Source: 	ZTE (email discussion rapporteur)

Title:  	                  Report of [AT113bis-e][401][eMTC R16] Paging DRX cycle (ZTE)

Document for:	Discussion and Decision

# Scope of the offline email discussion

This document contains the summary of the offline email discussion “[AT113bis-e][401][eMTC R16] Paging DRX cycle”, as indicated below:

- [AT113bis-e][401][eMTC R16] Paging DRX cycle (ZTE)

Status: Started

Scope: Check whether the intention is agreeable and there is sufficient support

in principle; collect initial comments.

Intended outcome: Report in R2-2104385

Deadline: Thursday 2021-04-15 10:00 UTC

# Offline email discussion

The following three contributions have been submitted to this meeting:

[1] R2-2103361	Discussion on correction for paging DRX cycle determination, ZTE Corporation, Sanechips, 	discussion

[2] R2-2104239	draft LS to RAN3 to clarify paging DRX cycle, ZTE Corporation, Sanechips, LS out, Rel-16

[3] R2-2104246	Correction on paging DRX cycle, ZTE Corporation, Sanechips, CR Rel-16	  36.304  16.3.0  0825  	-	F

Companies are invited to provide comments in the tables below.

## 2.1 The #1 issue and change

The #1 change is to clarify which cycle parameter does this condition “if allocated by upper layers” apply to, the UE specific paging cycle or the default paging cycle? The suggested change (#1 change) is copied here:

7.1 Discontinuous Reception for paging ……In RRC_INACTIVE state, if extended DRX is not configured by upper layers as defined in 7.3, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle- if allocated by upper layers and the default paging cycle. Otherwise, in RRC_INACTIVE state when extended DRX is configured by upper layers, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle, if allocated by upper layers and the default paging cycle during the PTW as defined in 7.3, and by the RAN paging cycle outside the PTW.

Q1: Companies are invited to indicate whether you agree with the intent of the #1 change?

| Company | Yes/No | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | We think this might be just an editorial error when introducing this part specification change for UE in RRC_INACTIVE. |
| Qualcomm | Yes | This specific issue happened from Release 15 hence we think it should be corrected from Release 15. At the very least the CR should also contain the WI which introduced this change in Release 15. |
| Huawei, HiSilicon | Yes | Agree with Qualcomm this is not only related to eMTC |

Conclusion:

Proposal:

## 2.2 The #2 issue and change

In RRC_IDLE, if a UE specific extended DRX value of 512 radio frames is configured, T always equals to 512. That means neither PTW (Paging Time Window) nor default DRX cycle/UE specific cycle would be applied to T determination. We can see such special process hasn’t been considered for the RRC_INACTIVE case. The proponent company think the similar special process should also be applied to the RRC_INACTIVE case with additional consideration on RAN paging cycle. Therefore, in [1], the following proposal is given:

Proposal 2: For UE in RRC_INACTIVE, if extended DRX value of 512 radio frames is configured by upper layers according to 7.3, T is determined by the shortest of the RAN paging cycle and 512 radio frames.

Q2a: Companies are invited to indicate whether you agree with the intent of Proposal 2 in [1]?

| Company | Yes/No | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | Moreover, per our understanding for SA2 and RAN3 specifications, if eDRX cycle of rf512 is configured, regular paging strategy is used and Paging Time Window would be absent in the PAGING message. |
| Qualcomm | Yes |  |
| Huawei, HiSilicon | yes |  |

The suggested change (#2 change) for Proposal 2 is copied here:

7.1 Discontinuous Reception for paging ……In RRC_INACTIVE state, if extended DRX is not configured by upper layers as defined in 7.3, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle, and the default paging cycle, if allocated by upper layers. Otherwise, in RRC_INACTIVE state, if extended DRX value of 512 radio frames is configured by upper layers according to 7.3, T is determined by the shortest of the RAN paging cycle and 512 radio frames. When extended DRX is configured by upper layers and it’s not 512 radio frames, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle, if allocated by upper layers and the default paging cycle during the PTW as defined in 7.3, and by the RAN paging cycle outside the PTW.

Q2b: If answer for Q2a is Yes, companies are invited to indicate whether you agree with the #2 change?

| Company | Yes/No | Detailed comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Qualcomm | Maybe | But the structure of the paragraph needs will need to be adjusted to make sure it precisely captures the intent (see example restructure shown for Issue #3 below). For example, ‘Otherwise’ is in relation to the first ‘if’ or the second ‘if’ in the previous sentence. |
| Huawei, HiSilicon | maybe | we agree with the intention but the wording should be improved as highlighted above. |

Conclusion:

Proposal:

## 2.3 The #3 issue and change

As mentioned in [1], as determination rule of DRX cycle(T) for RRC_IDLE is different from that for RRC_INACTIVE, and as T is involved in the calculation of PNB, it’s possible that PNB calculation for RRC_IDLE is different from PNB calculation for RRC_INACTIVE. Such difference would cause determined PNB for monitoring/sending CN paging are different in UE and network and further cause paging failure. Moreover, the similar issue also exists for i_s and wg.

In [1], the following proposal is given:

Proposal 3: To separately describe DRX cycle T determination for PF calculation and for PNB, i_s and wg calculation. The legacy description is still applicable to DRX cycle T determination for PF calculation. But for PNB, i_s and wg calculation, the DRX cycle T determination should follow RRC_IDLE mode rule. E.g.:

- If extended DRX is not configured, T is determined by the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value.

- If extended DRX is configured, T is determined by the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value during the PTW as defined in 7.3, and by the RAN paging cycle outside the PTW.

Q3a: Companies are invited to indicate whether you agree with the intent of Proposal 3 in [1]?

| Company | Yes/No | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | As mentioned in [1], an example has given to show the possible inconsistence: Common assumption: • RAN paging cycle = rf32 • UE specific DRX value = rf64 • Default paging cycle = rf128 • nB=4T • Paging Narrowband Number (Nn) =3 • GWUS group number (Nw) =5 • UE_ID = 1234 Calculation following RRC_IDLE rule; PF: 18, 82, 146, 210, 274, 338, 402, 466, 530, 594, 658, 722, 786, 850, 914, 978 i_s = floor(UE_ID/N) mod Ns = floor(1234/64) mod 4=19 mod 4 =3 PNB = floor(UE_ID/(N*Ns)) mod Nn =floor(1234/(64*4)) mod 3=4 mod 3 =1 =floor(floor(1234/(64*4)) /3) mod 5 =1. Calculation following RRC_INACTIVE rule; PF: 18, 50, 82, 114, 146, 178, 210, 242, 274, 306, 338, 370, 402, 434, 466, 498, 530, 562, 594, 626, 658, 690, 722, 754, 786, 818, 850, 882, 914, 946, 978, 1010 i_s = floor(UE_ID/N) mod Ns = floor(1234/32) mod 4=38 mod 4 =2 PNB = floor(UE_ID/(N*Ns)) mod Nn =floor(1234/(32*4)) mod 4=9 mod 3 =0 =floor(floor(1234/(32*4)) /3)mod5 =3. It can be seen the PF values calculated following RRC_INACTIVE rule contains the PF values calculated following RRC_IDLE rule. But the PNB, i_s and wg values calculated following RRC_INACTIVE rules are totally different from the values calculated following RRC_IDLE rule. |
| Qualcomm | Yes | - It is important for UE to use the same paging narrowband in RRC_IDLE and RRC_INACTIVE. This principle has been agreed in previous RAN2 meetings. - Paging frames to monitor during RRC_INACTIVE must also contain all of the paging frames required to be monitored in RRC_IDLE. - UE is required to monitor the same WUS group in RRC_IDLE and RRC_INACTIVE. |
| Huawei, HiSilicon | partially | We agree with the issue that the PNB and WUS group be the same when monitoring paging in RRC_INACTIVE and RRC_IDLE. For the paging frames in RRC_INACTIVE, they should cover all the paging frames for CN paging and RAN paging, so we think the current determination based on the shortest of the three (same as LTE) is fine It does not seem right to use RRC_IDLE paging parameters for distributing UEs in RRC_INACTIVE so it could be worth exploring other options, e.g. use the T of the cell. |

The suggested change (#3 change) for Proposal 3 is copied here:

7.1 Discontinuous Reception for paging ……In RRC_INACTIVE state, if extended DRX is not configured by upper layers as defined in 7.3, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle, and the default paging cycle, if allocated by upper layers for PF calculation. For i_s, PNB and wg calculation, T is determined by the shortest of the UE specific paging cycle, if allocated by upper layers and the default paging cycle. Otherwise, in RRC_INACTIVE state when extended DRX is configured by upper layers, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle, if allocated by upper layers and the default paging cycle for PF calculation during the PTW as defined in 7.3, and by the RAN paging cycle outside the PTW. For i_s, PNB and wg calculation, T is determined by the shortest of the UE specific paging cycle, if allocated by upper layers and the default paging cycle during the PTW as defined in 7.3, and by the RAN paging cycle outside the PTW.

Q3b: If answer for Q3a is Yes, companies are invited to indicate whether you agree with the #3 change?

| Company | Yes/No | Detailed comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Qualcomm | Maybe | Intent of the change is ok but it can be difficult to read. For this reason we propose to re-organize this long paragraph as follows: - In RRC_INACTIVE state, if extended DRX is not configured by upper layers as defined in 7.3: - for PF calculation T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle if allocated by upper layers, and the default paging cycle. - for i_s, PNB and wg calculation, T is determined by the shortest of the UE specific paging cycle, if allocated by upper layers, and the default paging cycle. In RRC_INACTIVE state when extended DRX is configured by upper layers: - for PF calculation during the PTW as defined in 7.3, T is determined by the shortest of the RAN paging cycle, the UE specific paging cycle, if allocated by upper layers and the default paging cycle. - for PF calculation outside the PTW, T is determined by the RAN paging cycle. Note, the structure above does not take into account the changes needed for issue #2. Therefore, exact structure and changes should be discussed with complete CR. |
| Huawei. HiSilicon | Maybe | We agree that a change is needed. we need to agree first on the change itself and discuss the wording. Then the proposed wording is very difficult to read and need improvements |

Conclusion:

Proposal:

## 2.4 The #4 issue and LS to RAN3

As mentioned in [1], according to the description of DRX cycle (T) determination in RAN2 spec TS 36.304, company understand for other ng-eNB(s) (except the anchor ng-eNB) in the RAN paging area, the only one DRX parameter (e.g. Paging DRX) in RAN PAGING message is not enough. There is no way for the other ng-eNB(s) to acquire whether extended DRX is configured and how the other parameters are configured for this UE. The possible issue is, the other ng-eNB(s) cannot follow same DRX cycle (T) determination as the UE.

Furthermore, based on the Proposal 3 in [1], the only one DRX parameter (e.g. Paging DRX) in RAN PAGING message is also not enough for other ng-eNB(s) to correctly calculate PNB, i_s and wg for sending CN/RAN paging.

In [1], the following proposal is given:

Proposal 4: Send LS to RAN3 to indicate that the meaning of Paging DRX in RAN PAGING message is not clear and it is also not enough for other ng-eNB(s) (except the anchor ng-eNB) in the RAN paging area to perform same DRX cycle T determination as UE.

Proposal 4a: In the LS to RAN3, RAN2 asks provision of the following information in RAN PAGING message:

- Paging eDRX information

- UE specific DRX

- RAN paging cycle.

Q4a: Companies are invited to indicate whether you agree with the intent of Proposal 4 and 4a in [1]?

| Company | Yes/No | Detailed comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Qualcomm | Maybe | We think RAN2 can provide the agreed CR to RAN3 and request they take this into consideration. Exactly what information needs to be passed from anchor RAN to non-anchor RAN can be left up to RAN3 to decide. |
| Huawei, HiSilicon | No | The need for a LS depends on resolution of issue #3. We need to agree on the change first and see if anything is needed, |

Q4b: If answer for Q4a is Yes, companies are invited to give brief comments to the draft LS in [2].

| Company | Detailed comments |
| --- | --- |
| ZTE | The LS to RAN3 is necessary. No more comments now. |
| Qualcomm | We don’t think the LS needs to contain detailed information, especially what should be transported over the XnAP interface. See answer to previous question. |

Conclusion:

Proposal:

Q5: Is there any other related issue for discussion?

| Company | Detailed comments |
| --- | --- |

Conclusion:

Proposal:

# Summary

Conclusion:

Updated CRs:

# Contact information

Please provide your contact information when responding:

| Company | Contact Name | Email |
| --- | --- | --- |
| ZTE | Ting Lu | lu.ting@zte.com.cn |
| Qualcomm | Mungal Dhanda | mdhanda@qti.qualcomm.com |

# References

[1] R2-2103361	Discussion on correction for paging DRX cycle determination	ZTE Corporation, Sanechips	discussion	LTE_eMTC5-Core	Late

[2] R2-2104239	draft LS to RAN3 to clarify paging DRX cycle	ZTE Corporation, Sanechips	LS out	Rel-16	LTE_eMTC5-Core	To:RAN3	Late

[3] R2-2104246	Correction on paging DRX cycle	ZTE Corporation, Sanechips	CR	Rel-16	36.304	16.3.0	0825	-	F	LTE_eMTC5-Core	Late