3GPP TSG-RAN WG2 Meeting #113bis electronic	R2-210xxxx

Online, April 12 – April 20, 2021

Title: 	[AT113bis-e][603][Relay] Proposals from summary of agenda item 8.7.4.1 (ZTE)

Source: 	ZTE

Agenda item:	8.7.4.1

Document for:	Discussion and Decision

# Introduction

This paper handles the following email discussion:

 [AT113bis-e][603][Relay] Proposals from summary of agenda item 8.7.4.1 (ZTE)

Scope: Continue discussion of the summary of AI 8.7.4.1 and try to reach agreeable proposals.

Intended outcome: Report in R2-2104405

Deadline:  Friday 2021-04-16 1000 UTC

This email discussion is divided in two phases:

- Phase I with the deadline on Thursday April 15 0600 UTC for companies to provide their views.

- Phase II with the deadline on Friday April 16 1000 UTC for companies to provide their views on the summary and suggested proposals.

The discussion is based on the contributions in AI 8.7.4.1 as well as the summary document of AI 8.7.4.1(R2-2104503).

Rapporteur encourages the participating delegates to provide your contact information in this table.

| Company | Contact: Name (E-mail) |
| --- | --- |
| Peng Cheng | chengp@qti.qualcomm.com |
| Xiaomi | Yangxing1@xiaomi.com |
| Qianxi Lu, Bingxue Leng | qianxi.lu@oppo,com, lengbingxue@oppo.com |
| Ericsson (Tony) | antonino.orsino@ericsson.com |
| Samsung (Hyunjeong) | hyunjeong.kang@samsung.com |
| Huawei, HiSilicon | Wangrui46@huawei.com |
| Lenovo, MotM (Prateek) | pmallick@lenovo.com |
| Boubacar | kimba@vivo.com |
| Apple (Zhibin) | zhibin_wu@apple.com |
| Sharp(Chongming Zhang) | chongming.zhang@cn.sharp-world.com |
| ZTE(Lin Chen) | chen.lin23@zte.com.cn |
| CMCC | huangxueyan@chinamobile.com |
| Nokia | Jakob.buthler@nokia.com |
| CATT | xuhao@catt.cn |

# Discussion

## Issues with potential cross-group dependency

According to the summary document of AI 8.7.4.1 [37], the following issues with potential cross-group dependency were identified. We will discuss these issues one by one.

- Establishment cause value of relay UE

- UAC of relay UE

- Security issue of paging monitoring

- Retrieval of context for remote UE and relay UE

- TAU/RNAU of remote UE

### Establishment cause value of relay UE

During RRC establishment or resume, relay UE need to indicate the establishment cause value for gNB to decide whether accept or reject the request. Although the request is sent by relay UE, but the intention is to relay remote UE’s traffic. So some companies suggests that relay UE should align cause value with remote UE or indicate the relaying purpose. [5] [27] suggests to define new establishmentCause value for relay UE’s RRC connection request since relay UE may initiate this procedure upon reception of a message (whether first RRC message or a trigger message) on PC5 from remote UE. [11] propose that relay UE align the cause value as remote UE in RRC establishment or resume. [11] [27] mentions that the Remote UE need to indicate its cause value to relay UE. [5] [27] mentions that Relay UE needs an establishment cause value not based on upper layer request to establish its own Uu RRC connection. [34] propose to further discuss how to set the establishment cause value or resume cause value when Relay enter RRC_CONNECTED triggered by PC5.

As we know, the RRC Connection Setup includes an IE “establishmentCause” that indicates a cause for the establishment. These establishment cause values are pre-defined in 3GPP TS 24.501 and the selected establishment cause value depends on the combination of AI and AC. For RRC connection re-establishment and resume procedure, the “reestablishmentCause” and “resumeCause” are indicated to gNB. Actually, the necessity of new establishment cause/re-establishment cause value had been discussed in IAB WI when IAB node initiate the RRC connection with parent node. Some people think that unless new AI/AC are defined for IAB, it does not make sense to define new cause value. Due to lack of support, it’s agreed in RAN2#108 meeting that no new Establishment Cause values in RRC Connection Setup are defined and no new Re-establishment Cause values are defined.

Question 1-1: Which establishment/resume cause value should be set when relay UE enter RRC_CONNECTED for relaying purpose? Please give your comments.

Option 1: Existing establishment/resume cause value;

Option 2: New establishment/resume cause value;

| Companies | Option | Comments |
| --- | --- | --- |
| Qualcomm | Slightly prefer Option 2 | We tend to think Option 2 (a new cause value) is a cleaner solution. However, its cos is that it will need CT1/SA2 work, and (if agreed) it also implies a followed discussion on its access category is required which also has cross-WG impacts. For Option 1, it needs signalling change in PC5 (including cause value since relay doesn’t decode remote UE’s SRB0). However, the relay UE can reuse the legacy Uu RRC establishment/resume procedure after that, and it seems no cross-WG is needed. Meanwhile, we believe (when remote UE initialize resume/establishment) the relay UE already in CONNECTED should be a typical case. Thus, we think RAN2 should not spend much time on this issue. In all, we don’t have strong opinion. Either option can work. We slightly prefer Option 2 |
| Xiaomi | Option 1 as baseline | Noted the WI supports both public safety and commercial use case. gNB shall not reject establishment/resume request triggered by PS remote UE whose case value is Emergency. It’s essential for gNB to know the cause value to decide whether to accept the request. Due to size limit, the request message from relay and remote could not be sent together in msg.3. Relay UE has to establish its own connection before relay remote UE’s request. If cause value of relay UE is independent with remote UE’s cause value, gNB has to accept request from relay UE unconditionally to avoid reject emergency call. It’s unfair to other in-coverage UE and disobey the design principle of cause value. Therefore, the cause value should be aligned between relay and remote. Existing cause value could be reused. However, in some scenarios, new cause value may be needed. For example, relay UE’s establishment/resume is triggered by remote UE’s re-establishment. Currently, there is no establishment/resume cause value corresponding to re-establishment. How to set this value could be discussed further. |
| OPPO | Option-2 | Option-1 does not work alone anyway since there might be multiple remote UEs trigger connection establishment simultaneously, then which remote UE to follow? And option-1 cannot save X-WG interaction since the relay UE, if originally in IDLE state, must trigger PDU session setup, but the data is from lower layer due to relayed message from remote UE, instead of from upper layer of relay UE, so it is a new event for SA2/CT1 to trigger PDU session setup. From that perspective, we see more disadvantages from Option-1 than from Option-2. |
| MediaTek | Option 2 | We think Option 2 (a new cause value) is a better solution. Meanwhile we agree SA2/CT1 should be informed for such agreement. |
| Ericsson | Option 2 | Probably a new Existing establishment/resume cause value would be needed to differentiate the handling based on the service. If we go this way, an LS to SA2 and CT1 is needed. |
| Samsung | Option 2 | This option is clearer. |
| Huawei, HiSilicon | Option 2 | We share the same view as OPPO that this is a new trigger for relay UE initiating RRC connection setup, the existing UE behaviour of cause value setting in relay UE side cannot cover this new case. So at least new UE handling is needed. One way is relay UE to copy remote UE’s cause, and the other way is to introduce new cause value. We slightly prefer new cause value. Moreover, considering the new trigger is only seen by relay UE’s AS layer but not NAS layer, we are wondering if it is possible that RAN2 to define an AS layer cause value and inform CT1 and SA2. So that they can work on NAS layer procedure alignment/adjustment for this AS triggered RRC connection setup. |
| Lenovo, MotM | Option 1 | The current establishment causes can appropriately reflect the remote UE’s establishment reason. There’s no reason at this point (of RRC Connection Est.) to let the network know that the establishment is for a remote UE (i.e. the relay UE role can be transparent until Msg3). We do not see any re-establishment cause issue here as there’s no re-establishment procedure on PC5 in our understanding. |
| vivo | Option 1 | RRC establishment procedure and corresponding cause values are extensively used in Uu. We do not think it is future proof to introduce new cause value just for the SL relay UE RRC establishment purpose. |
| Apple | Option 1 | For RRC establishment cause, we do not see this is a critical issue which need to be different from Layer 3 U2N. If RAN2 really intends to let gNB knows the exact reason, then we need duplicate a whole new set of cause value with a prefix “remote-“, such as “remote-emergency”, etc. I do not see why a single new cause value makes sense here. Honestly speaking, gNB does not need to know the RRC connection is shared with a remote UE at the time of RRC establishment. To save the RAN2 work time, we are OK with Option 1. |
| Sharp | Option 2 | It is possible for multiple remote UEs to trigger connection establishment, a new establishment/resume cause value refers to relay will make it simple and clear. |
| ZTE | Option 1 | We suggest to adopt the same approach with IAB. For IAB node, it is generally used to forward the BH traffic. The only potential traffic of itself is the OAM traffic. And no new establishment cause value is designed for IAB. Compared to IAB, it is more likely for relay UE transmit its own traffic and relayed traffic. We see no strong motivation to support new establishment cause value for relay UE. |
| CMCC | Option 2 | Option2 is a clear solution for this issues, especially for the case that multiple remote UEs trigger connection establishment at the same time. Of course, the cross-group work can not be avoided. |
| Nokia | Option 1 | Althouth option 2 may provide a cleaner solution, we think that the remote UEs establishment cause may be indicated as in legacy after relay UEs connection establishment. |
| CATT | Option 2 | Option2 is clearer. |

Question 1-2: Do you agree that RAN2 should send an LS to SA2 on how to set the establishment/resume cause value when relay UE enters RRC_CONNECTED for relaying purpose?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | See comments | If Option 2 is selected, it has to. But it should be sent to CT1 because it is CT1 to decide the cause value. If Option 1 is selected, we are fine to inform CT1 for check. |
| Xiaomi | Neutral | If existing cause value is reused, SA would not be impacted. But it’s OK to inform agreements to SA2 if necessary. |
| OPPO | Yes | To both SA2 and CT1. |
| MediaTek | Yes 2 |  |
| Ericsson | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | See comments | As we comment to Q1-1, for option 2 RAN2 can decide an AS cause value. But the point is some AS-NAS interaction and NAS procedure alignment maybe needed in SA2 and CT1. In legacy procedures, it is always the NAS layer to trigger AS to initiate a RRC connection, while now it is AS to initiate the RRC setup. That is why we think SA2/CT1 should be informed on the new RRC setup trigger. |
| Lenovo, MotM | No | If no new est. cause then there’s no new information expected from upper layers. |
| vivo | Yes,with comments | We can inform RAN2 preference based on the majority view of Question 1-1. But The final decision should be up to SA2 and CT1. |
| Apple | LS to CT1 | We think the CT1 needs to be informed if Option 1 is chosen. |
| Sharp | Yes |  |
| ZTE | Yes | No matter which option is selected, SA2/CT1 need to be informed of the decision. |
| CMCC | Yes | SA2 and CT1. |
| Nokia | Yes | Mainly we go for option 2 |
| CATT | Yes | To both SA2 and CT1 |

### UAC of relay UE

Considering that the access barring check for an access attempt associated with a given Access Category and one or more Access Identities are upon request from upper layers according to TS 24.501 or the RRC layer, [34] proposes to discuss how to set the AC in relay UE when it intends to access network only for the purpose of relaying but not for its own service. Similar issue was also discussed in IAB WI. It’s agreed in RAN2#109 meeting that IAB-MT is not under UAC control since IAB node is mainly used for relaying backhaul traffic. SA2 may be involved to jointly make decision on this.

Question 2-1: Should the relay UE be under UAC control when relay UE intends to access network only for the purpose of relaying but not for its own service?  If yes, please clarify how to set the AC in relay UE.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | Because relay is a UE different from IAB, we tend to think it should also perform UAC. On how to set AC, we think there are below 2 options: - Alt-1: New AC, which needs CT1/SA2 involvement - Alt-2: Reuse existing AC 8 (MO signalling on RRC level resulting from other than paging) We slightly prefer Alt-2 |
| Xiaomi | No | If remote UE has done UAC check, relay UE doesn’t need to. Double UAC may lead to different barring results, which makes remote UE more difficult to get allowance. |
| OPPO | Yes | we understand this Q is limited to “UE be under UAC control when relay UE intends to access network only for the purpose of” we are also fine to go for AS-layer defined AC value, e.g., value-8. |
| MediaTek | Yes | We think we should reuse existing AC (e.g. MO signalling on RRC level resulting from other than paging). |
| Ericsson | Yes | Baseline is that the remote UE should use the same UAC as of the relay UE. If time allows, we may think to enhance this. |
| Samsung | Yes | We can reuse existing AC 8. |
| Huawei, HiSilicon | Yes | We are fine with new AS defined AC or existing AC. |
| Lenovo, MotM | No | Assuming remote UE has received the System Information and has done UAC check. |
| Vivo | Yes | Agree with Qualcomm. |
| Apple | No | UAC can be reused for relay case. I do not see justification to define a new AC for relay purpose. Also, UAC is part of SIB1 which needs to be broadcasted to remote UE(s). So remote UE can do UAC by itself. |
| Sharp | Yes | We prefer reusing existing AC value for relaying purpose. |
| ZTE | Yes | Relay UE is a UE instead of network node like IAB. It is suggested that relay UE under UAC control. However, it is suggested to reuse legacy AC. |
| CMCC | Yes | Reuse existing AC 8. |
| Nokia | Ask SA2 | We think this should be investigated further as Relay and IAB are quite different use-cases. |
| CATT | Yes | Prefer to reuse existing AC value, e.g. AC 8. |

Question 2-2: Do you agree that RAN2 should send an LS to SA2 on the UAC control of relay UE when relay UE  intends to access network only for the purpose of relaying but not for its own service?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | It should be sent to CT1 because it is CT1 to decide the cause value. No matter Alt-1 or Alt-2 (in our comment to Q2-1) is adopted, it has to send CT1/SA2 for check. |
| Xiaomi | Neutral | We don’t see impact to SA. But it’s OK to inform agreements to SA2 if necessary. |
| OPPO | Yes | CT1/SA2 confirmation is needed anyway. |
| MediaTek | Yes |  |
| Ericsson | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes | Since the AC/AI is captured in CT1’s specification, CT1 should be informed with any RAN2 concluded solution. |
| Lenovo, MotM | Neutral | For information purposes only, one single LS can be sent with all relevant agreements. |
| vivo | Yes, with comments | We can inform RAN2 preference based on the majority view of Question 2-1. But The final decision should be up to SA2 and CT1. |
| Apple | LS to CT1 | We are fine to inform CT1 about the RAN2 agreement, although we do not see RAN2 dependency on CT1. |
| Sharp | Yes |  |
| ZTE | Yes | LS to both CT1 and SA2 |
| CMCC | Yes |  |
| Nokia | Yes |  |
| CATT | Yes | Inform the agreement to CT1/SA2. |

### Security issue of paging monitoring

In order to monitor the remote UE’s paging occasion on behalf of remote UE, relay UE need to first obtain the remote UE’s PO information. According to TS 38.304, the PO is calculated as follows:

The PF and PO for paging are determined by the following formula: SFN for the PF is determined by: (SFN + PF_offset) mod T = (T div N)*(UE_ID mod N) Index (i_s), indicating the index of the PO is determined by: i_s = floor (UE_ID/N) mod Ns T: DRX cycle of the UE (T is determined by the shortest of the UE specific DRX value(s), if configured by RRC and/or upper layers, and a default DRX value broadcast in system information. In RRC_IDLE state, if UE specific DRX is not configured by upper layers, the default value is applied). N: number of total paging frames in T Ns: number of paging occasions for a PF PF_offset: offset used for PF determination UE_ID: 5G-S-TMSI mod 1024

As we can see, the PF and PO are determined by the UE-specific DRX cycle T and UE_ID value as well as the cell-specific Ns, N and PF_offset value. In order to monitor the PO of remote UE, relay UE should at least obtain the remote UE’s DRX cycle T and UE_ID information. [5] [14] [34] propose that remote UE sends UE specific DRX cycle to  relay UE. With regard to UE_ID, the following alternatives are proposed for the acquisition of remote UE’s UE ID by relay UE:

- Alt-1: Remote UE sends its UE ID (5G-S-TMSI) to Relay UE. This is proposed in [9] [21] [27][34].

- Alt-2: Remote UE sends calculated PO(s) directly to Relay UE.  This is proposed in [2] [9] [11] [21].

- Alt-3: Remote UE sends a pseudo UE ID (e.g. 5G-S-TMSI mod 1024) to Relay UE, which derives the same PO(s) as 5G-S-TMSI. This is proposed in  [5] [9] [14] [21].

As far as we know, 5G-S-TMSI is also used for paging the remote UE in the paging message. With Alt 1, relay UE can precisely determine whether the remote UE is paged or not. Otherwise, the relay is unable to determine the specific remote UE(s) indicated in a received paging message. However, Alt 1 may face security issue since it expose the 5G-S-TMSI of remote UE to relay UE over PC5 interface. In addition, [27] [34] propose that RRC_INACTIVE remote UE sends the I-RNTI to relay UE so that relay UE can determine the monitor the RNA paging of remote UE. Similar to the UE ID, security concerns need to be considered since it expose the I-RNTI of remote UE to relay UE over PC5 interface.

Question 3-1: In order to monitor the remote UE’s paging (including RNA and CN paging) on behalf of remote UE, which of the following information should be provided to relay UE?

Option 1: 5G-S-TMSI of remote UE

Option 2: Pseudo UE ID (e.g. 5G-S-TMSI mod 1024) of remote UE

Option 3: Calculated PO(s) of remote UE

Option 4: I-RNTI of remote UE

Option 5: UE specific DRX cycle T

Option 6: Others. If this Option is selected, please give the detailed description and how to use it.

| Companies | Option | Comments |
| --- | --- | --- |
| Qualcomm | Option 3 | For Option 1/2/4, we think LS to SA3 is needed. Note that current RRCSetupRequest has included 5G-S-TMSI in plain text to send in air interface. However, according to TS 23.50, when the procedure is triggered by paging, the AMF shall allocate a new 5G GUTI so that it would not be reused. Option 3 has two benefits: - No security issue and LS to SA3 needed - Its signalling is aligned between CN paging (5G-S-TMSI) and RAN paging (I-RNTI) Option 5 is not a solution, right? It can only work with Option 1/2/4. If Option 3 is adopted, Option 5 is also not needed |
| Xiaomi | Option 3 | Option 1 and 4 have security risk. Option 2 and 5 can only work together. Regarding option 3 vs option 2+5, we prefer option3. Option 2+5 requires relay UE to do the calculation. The calculation load may be high in case relay UE serves multiple remote UEs. Furthermore, Redcap is discussing PO subgrouping, which is not available for relay. Option 2+5 can’t support PO subgrouping in furture. |
| OPPO | Option-2 + Option-5 | we understand option-1/2/3 are exclusive to each other: - option-1 is not preferred due to security concern, - option-3 is not preferred since it leads extra PC5 signaling exchange, i.e., firstly the relay UE send the input parameters for PO/PF calculation (N, Ns, PF_offset) to relay UE, and then relay UE calculate the PO/PF to relay UE, and this kind of signalling exchange has to be performed for every remote UE when relay UE moves to a new cell. Option-4 is not needed, since I-RNTI is not used as input for PO/PF calculation. |
| MediaTek | Option 3 | This issue was discussed at LTE also during the discussion of L2 relay. We may need to check with SA3. |
| Ericsson |  | Our preference is to not decide now which solution to adopt. In fact, we should send an LS to SA3 and ask them if there are security concerns for all the options. Once SA3 replies back then RAN2 can take a decision. |
| Samsung | Option 3 |  |
| Huawei, HiSilicon | Option 1 | We understand only option 1 is aligned with SI agreement on LTE solution of option 2 in which the relay UE needs to decode paging message and forward the paging to the exact UE. And considering S-TMSI and I-RNTI are both temp UE ID, we do not see security risk. Just for clarification, for option 2/3/5, it seems assistance information for relay to calculate PO for remote UE, but relay UE still needs to know which remote UE is paged, right? Otherwise, the relay UE needs to forward all the paging messages received to all the remote UEs connected, we feel this may be a more power/resource consumption approach. About the suggestion to list all solutions to SA3, we prefer at least RAN2 can give detailed/clear solution for SA3 evaluation (if the LS is to be sent). Otherwise, the progress will be slow in both SA3 and RAN2. |
| Lenovo, MotM | Option 1 and Option 3 | Option 1 for RRC Idle remote UE and Option 3 for RRC Inactive remote UE. As the Rapport. correctly pointed out, the relay UE must be able to understand/ recognize the identities when receiving a paging message as one of its linked Remote UE’s or not. Otherwise, how can it let the remote UE know that there’s a Paging for it – blindly forwarding all Paging messages (in every POs of the remote UE) is a waste of resources and therefore not acceptable. More importantly, we need to locate the security concern. We think the relay UE itself is not a source of security risk assuming it is an authenticated UE; the risk may be on PC5 link. But SA3 is a better judge. If the security risk is confirmed by SA3, at least one option would be that the remote UE monitors its own Paging (as long as the radio conditions allow) – and request the relay to monitor paging only when required. |
| vivo | Option 1 or 2, Option 5 | For Option-1/2/3, Option 1 is the best choice from PC5 signaling overhead and paging forward accuracy perspective. However, we need to consult SA3 if there is any security issue on exposing the 5G-S-TMSI of remote UE before making the final decision. If there is security concern, Option 2/3 can be taken as alternative solutions. Option 2 is preferred for less PC5 signaling overhead. For Option 4, it is not used for the remote UE’s PO calculation but used to identify the UE ID carried in the RAN paging message. Knowing I-RNTI can improve the paging forward accuracy for Inactive remote UE but it is not essential to have this. For Option 5, it is used together with UE ID for the remote UE’s PO calculation |
| Apple | Option 1 | Option 1 is straight-forward, Option 2 (TMSI mod 1024) and Option 3 are not really secure. They are just mathematical transformations of the TMSI value with no real significant value in regards of security. Unless we want SA3 to introduce a “fully secure” method, we can live with Option 1. |
| Sharp | Option 1/Option 2 + Option 5 | We prefer the PO calculation is done by relay UE, otherwise, the cell reselection of relay UE will result the updated Calculated PO(s) of remote UE if we go with Option 3. Both Option 1 and Option 2 are OK for calculation of PO(s). However, if UE full identity is also required for other purposes, e.g. identification of a PAGING message by relay UE, Option 1 is preferred. |
| ZTE | Option 2 and Option 5 | We think option 2 and Option 5 can be used by relay UE to determine the PO and start paging monitoring of remote UE. With regard to the paging message, since relay UE is not aware of the 5G-S-TMSI and I-RNTI, relay UE has to forward all the paging message received on the PO to remote UE. |
| CMCC | Option 3 |  |
| Nokia | Slight preference towards Option 3 | In the end, we think this would be relevant to include in an SA3 LS before making any decisions |
| CATT | See comments | We share the same view as Nokia. |

Question 3-2: Suppose 5G-S-TMSI/I-RNTI of remote UE are to be provided to relay UE, do you agree that RAN2 sends LS to SA3 to confirm if there is any security issue on exposing the 5G-S-TMSI/I-RNTI of remote UE to relay UE over PC5 interface?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | It is not required if Option 3 is agreed, to avoid sending LS to SA3 |
| Xiaomi | Yes |  |
| OPPO | Yes | Agree if option-1 is adopted. Disagree if option-2 is adopted since it is just partial UE ID I-RNTI is not useful for PO/PF calculation. |
| MediaTek | Yes |  |
| Ericsson | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | No | We do not see the need. |
| Lenovo, MotM | Yes |  |
| vivo | Yes,with comments | We should only consult SA3 about security concern on exposing the 5G-S-TMSI of remote UE. There is no security issue for exposing I-RNTI. Because in NR, I-RNTI is configured by the network and the bit length of the corresponding UE ID part and gNB ID part are flexible and totally up to NW implementation. This is unlike LTE where the I-RNTI is defined as fixed 20-bit UE ID part and 20-bit gNB ID part. As a result, it is hard to track the real UE-ID based on I-RNTI in NR. |
| Apple | Yes | SA3 needs to be informed about the potential risk. |
| Sharp | Yes |  |
| ZTE | Yes |  |
| CMCC | Yes |  |
| Nokia | Yes | SA3 needs to be asked for potential risks |
| CATT | Yes |  |

### Retrieval of context for remote UE and relay UE

In [24], it mentions that when the remote UE or relay UE perform an RRC resume or RRC reestablishment procedure and  the remote and relay UE are close enough to keep their PC5 link, it may be possible that PC5 link may still be kept while Uu link is changed for both of them. So [24] proposes that when the Retrieve UE Context procedure is performed, the new gNB should retrieve both the remote and relay UE context. This can be achieved either by the UE that trigger the procedure (either the remote or relay UE) to indicate to the new gNB that the UE context of both the remote UE and relay UE should be retrieved, or by the old gNB sending to the new gNB both the remote and relay UE context when getting the Retrieve UE Context Request. [24] propose that RAN2 to send an LS to RAN3 in order to address the case and include in the LS the following solutions and let RAN3 to decide which solution to adopt.

During the discussion of the summary document, some companies think that this scenario looks like an group mobility enhancement where relay and remote UE perform RRC resume/re-establishment simultaneously and keep the PC5 connection between them. Given that the group mobility is not included in the SL relay WI, some companies think that this should not be discussed at all.

Question 4-1: Do you agree to discuss the scenario where new gNB retrieves both the remote and relay UE context via single retrieve UE Context procedure in case of RRC re-establishment and RRC resume, and initiate LS to RAN3?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | We don’t fully understand the issue / solution. We are confused by the following 2 cases: - Case 1 (relay UE changed after resume/re-establishment): Because it is a different relay UE, how delta signalling on relay configuration can work? In addition, relay UE has to first enter RRC_CONNECTED before remote UE according to TS 38.836, then its UE context has been retrieved in target gNB. Then, why its UE context needs to be retrieved again? - Case 2 (relay UE unchanged after resume/re-establishment): It seems a group mobility scenario, which is out of scoping of this release |
| Xiaomi | Comments | This question is related to relay UE’s RLF handling. If remote UE performs relay re-selection upon relay UE’s RLF, this scenario doesn’t exist. Otherwise, this scenario is valid. We should first discuss how to handle relay UE’s RLF. |
| OPPO | No | Similar concern as QC, we assume one can always rely on independent procedure for relay / remote UE related context retrieves. Or if the intention is to go for group-mobility, that’s out of R17 scope. |
| MediaTek | No | We do not think this is a typical scenario that needs to handle. |
| Ericsson | Yes | We believe that the rapporteur did not understood our proposal and it also captured it in a quite wrong way. This is quite unfortunate as it also give a wrong impression on what we want to propose. Please note that there is no case on when the relay UE is changed or unchanged, but we would like to clarify the handling of the UE context retrieval when the remote UE or relay UE performs RRC reestablishment and/or RRC resume. For RRC reestablishment: - If the remote UE performs RRC reestablishment, we believe that there is no issue as the UE context retrieval, in case a new gNB is selected can be done using the legacy framework. - If the relay UE performs RRC reestablishment, it is unclear how this procedure will work. - If the relay UE release the relay connectivity/path upon initiating the reestablishment procedure, than there should be no issue - If the relay UE wish to keep the relay path with the remote UE and it selects a new gNB, then our point if that the new gNB should fetch both the relay UE and remote UE context, otherwise it would not know that also the context of the remote UE should be fetched. For RRC resume, the situation is similar: - If the remote UE performs RRC resume, we believe that there is no issue as the UE context retrieval, in case a new gNB is selected can be done using the legacy framework. - If the relay UE performs RRC resume, it is unclear how this procedure will work. - If the relay UE release the relay connectivity/path upon initiating the resume procedure, then there should be no issue - If the relay UE wish to keep the relay path with the remote UE and it selects a new gNB, then our point if that the new gNB should fetch both the relay UE and remote UE context, otherwise it would not know that also the context of the remote UE should be fetched. We believe that RAN2 should discuss and clarify how this aspects should work for the resume and reestablishment case. |
| Samsung | No | The issue seems to be discussed in RAN3 without any RAN2 discussion |
| Huawei, HiSilicon | No | After reading the clarification from Ericsson, we understand the target scenario is when a relay UE perform RRC reestablishment/resume, if the relay UE can take remote UE with it. Our feeling is for this case the remote UE and relay UE should manage itself RRC states, and perform corresponding RRC procedure separately. No specific optimization is necessary. For instance, if the relay UE experiences Uu RLF, it would release the PC5 link and inform remote UE (discussed in relay (re)selection summary), so that remote UE could perform RRC reestablishment procedure for itself. If relay UE decides to go from INACTIVE state to CONNECTED state, the relay UE can perform RRC resume procedure, but the remote UE can still stay in INACTIVE state. |
| Lenovo, MotM | No | We only understand the case that an “unchanged” relay has to perform re-est. or resume on Uu. This is then like rel. 16 – we do not see anything new. |
| Vivo | Comments | We prefer to follow NR Uu as the baseline, i.e, the new gNB retrieves the remote UE context and relay UE context via existing retrieve UE Context procedure separately. However, we are ok to send LS to inform RAN3 based on the outcome of this Question 4-1. |
| Apple | Postpone | We think group mobility is not in scope of current WID. We can discuss this when we have some time after finish baseline design. |
| Sharp | No | Sharing the same vies with OPPO. |
| ZTE | No | With regard to Ericsson’s clarification of the scenario, we have following comments: 1) RRC reestablishment: Suppose the relay UE detects Uu RLF, the relay UE may send indication message to its connected remote UE to trigger the relay reselection of remote UE. Actually some companies proposed to support this relay re-selection triggers in their papers. If that is the case, it is not necessary to maintain the PC5 connection between the relay UE and remote UE. Also it is not necessary to support the context retrieval of both relay and remote UE at new gNB. 2) RRC Resume: Suppose relay UE perform RRC resume at a new gNB, how to handle the connected remote UE is a good issue for discussion. Actually, a similar scenario is that relay UE perform HO at new gNB and some companies suggest that relay UE sends the indication/message to its connected remote UE(s) to trigger relay reselection since group mobility is not supported in this Release. With regard to context retrieval, we see no big difference between the relay UE HO and RRC resume at new gNB. Namely, relay UE RRC resume at new gNB may be a new trigger for relay re-selection of its connected remote UE and it is also not necessary to support the context retrieval of both relay and remote UE at new gNB. |
| CMCC | No | Group mobility is not included in the SL relay WI. |
| Nokia | Yes/Postpone | We agree to the fact that the group mobility is out of scope, but maybe we should be sure to clarify the scenario first and then decide whether retrieving both UE contexts should be supported or not |
| CATT | No | We all agree that group mobility is out of scope, so the proposal is not needed. |

### TAU/RNAU of remote UE

For RRC_IDLE/INACTIVE remote UE, [20] [22][24][29] discuss the support of TAU/RNAU for remote UE. [20] proposes that Remote UE sends its RNA/TA configuration to Relay UE when it is PC5 connected to a Relay UE. Then  relay UE monitor if the TAU/RNAU trigger and notify remote UE if the triggered condition is fulfilled. It should be noted that the TA/RNA configuration of remote UE is exposed to relay UE over PC5 interface. If this option is adopted, SA3 need to be consulted for the potential security issue.

On the other hand, [24] proposes that the remote UE performs RNAU and TAU procedures while in RRC_INACTIVE and RRC_IDLE, respectively. If the remote UE is out-of-coverage, the relay UE performs RNAU and TAU procedure for itself and on behalf of the remote UE. In this case, the new gNB/AMF should retrieve both the remote and relay UE context. [24] suggests that RAN2 to send an LS to SA2 and CT1 to inform that, from RAN2 perspective, the remote UE is allowed to perform RNAU and TAU procedure. In addition, [24] propose RAN2 to send an LS to RAN3 to inform that when the relay UE performs RNAU and TAU procedure for itself and on behalf of the remote UE, the new gNB/AMF should retrieve both the remote and relay UE context.

In [29], it is proposed that CN Registration/RNAU for the remote UE can be triggered by registration tracking change and periodic registration area update as legacy. A UE-to-Network relay UE could serve a lot of the remote UEs. The relay UE can perform the CN registration on behalf of the served remote UE(s) in order to save the signalling overhead. For example, the RRC_Connected relay UE can indicate the list of the served idle-mode remote UE(s) to network for the RNAU of the served remote UE(s) purpose. As the above proposal can affect SA2/ CT1 as well, RAN2 should liaise with them to progress on this.

Question 5-1: Do you agree that the remote UE should perform TAU/RNAU while in RRC_INACTIVE and RRC_IDLE?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We see no reason why remote UE can’t perform TAU/RNAU |
| Xiaomi | Yes | It’s essential for remote UE to perform TAU/RNAU. Otherwise, gNB is not able to page remote UE in the right TA or RAN area. |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Ericsson | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Lenovo, MotM | Yes |  |
| vivo | Yes |  |
| Apple | Yes |  |
| Sharp | Yes |  |
| ZTE | Yes |  |
| CMCC | Yes |  |
| Nokia | Yes |  |
| CATT | Yes |  |

Question 5-2: How could the remote UE perform the TAU/RNAU of remote UE? And should RAN2 liaise SA2/ CT1/RAN3 to further progress on this? Please give your comments and feedback.

| Companies | Comments |
| --- | --- |
| Qualcomm | We think legacy TAU/RNAU procedure can work for remote UE without spec change. The only delta part is how TAI list of relay’s serving cell is provided to remote UE: We think relay need to send the TAI and RAN area ID to the Remote UE. It can be signalled via SIB forwarding or dedicated PC5-RRC. That is all. In above solution, no LS to SA2/CT1/RAN3 is needed. We don’t see need for relay UE to perform TAU/RNAU on behalf of remote UEs. And we tend to think there may be security issue because relay UE can know remote UE’s location. |
| Xiaomi | Remote UE performs TAU/RNAU according to SIB1 provided by relay UE following legacy procedure. |
| OPPO | Exactly same view as QC. |
| MediaTek | We prefer to reuse legacy TAU/RNAU procedures for Remote UE |
| Ericsson | We agree that legacy procedure should be performed in performing TAU/RNAU procedure. However, it is unclear once again how the UE context retrieval should work in case the UE changes TA or RNA. Does it mean that, if there is a PC5 connection this is released, or does it mean that when the remote UE performs TAU/RNAU also the relay UE should do the same? Further, how it works when e.g., the remote UE changes TA or RNA and relay UE not (and vice versa)? How the new gNB/AMF is aware of the UE context of both the remote and relay UE? We believe that RAN2 should discussion this issues. |
| Samsung | Reuse the legacy TAU/RNAU procedures for remote UE. |
| Huawei, HiSilicon | Agree with above companies that legacy procedure is sufficient. Regarding Ericsson’s question, we think the TA list/RNA is configured to the relay UE and remote UE via their own NAS message or RRC message in the legacy way, so that the relay UE and remote UE can perform TAU/RNAU based on TA/RNA information in SIB. We do not see why relay UE behaviour would be affected by remote UE’s TAU/RNAU. |
| Lenovo, MotM | A UE-to-Network relay UE could serve a lot of the remote Ues. According to the legacy TAU/RNAU, all the remote UE with idle/inactive state will establish the connection and transmit the indication to network. The relay UE can perform the TAU/RNAU on behalf of the served remote UE(s) in order to save the signalling overhead since the relay UE serves a lot of remote Ues. For example, the RRC_Connected relay UE can report the list of the served remote UE(s) to network for registration purpose. |
| Vivo | We agree that remote UE performs legacy TAU/RNAU procedure, which is independent from the relay UE TAU/RNAU procedure. Therefore, the details can be FFS in RAN2 without LS for the moment, e.g, how the remote UE judges it moves out of its configured TA/RNA, based on relay UE’s serving cell information or some indication from the relay UE. |
| Apple | I think there are two questions here: - TAU procedure is NAS procedure which is transparent to relay UE. Regarding LS to other WGs, we do not see there is any dependency for this. - Regarding whether relay UE shall be involved in remote UE RNA update, this is a purely RAN2 discussion and no need to LS other WGs. For the design itself, we think all options shall be still on the table. If a RRC_IDLE or RRC_INACTIVE relay UE has multiple remote UE’s, it may not want to be bothered multiple times for the sake of RNA update, some optimizations to reduce the overhead can be considered. Hence, I think there is no need of LS. But RAN2 should still be open to RNAU procedure optimizations to enhance the legacy approach. |
| Sharp | Sharing the same view QC. |
| ZTE | We think the remote UE can independently perform its TAU/RNAU with legacy procedure. It is not necessary to LS SA2/CT1/RAN3. |
| CMCC | Reuse the legacy TAU/RNAU procedures. |
| Nokia | We also agree to follow legacy procedures |
| CATT | We share the same view as QC. |

## Connection management

### PC5 and Uu RLC channel configuration

During the connection establishment procedure of remote UE, PC5 connection needs to be setup between remote UE and relay UE. Meanwhile, the RRC connection needs to be setup between remote UE and gNB. In order to support the CP and UP packet relaying of remote UE, the PC5 and Uu RLC channel should be configured for remote UE and relay UE. Generally speaking, different configuration options may be used for different type of CP and UP packet relaying.

Option1: specified (fixed) configuration, can not be overridden

Option2: default configuration, can be overridden by dedicated signalling

Option3: network configuration via dedicated signalling

Option4: reuse the SLRB and/or Uu SRB to deliver remote UE’s CP signalling within RRC container

Question 6-1: For the delivery of remote UE’s SRB0 RRC message, which option(s) should be used for the configuration of PC5 RLC channel and Uu RLC channel respectively?

| Companies | Option(s) for PC5 RLC channel | Option(s) for Uu RLC channel | Comments |
| --- | --- | --- | --- |
| Qualcomm | Option 1 | Option 1 | First, note that existing 38.331 has included below 2 types of “default configuration” for Uu: - Specified configuration: fixed configuration specified in Section 9.1 of 38.331 which can’t be reconfigured by the Network. It is typically for SRB0 delivery - Default configuration: fixed configuration specified in Section 9.2 of 38.331 which can be reconfigured by the Network later via RRCReconfiguration message. It is typically for delivery of RRCResume/RRCReetablishment which are SRB1 although no NW configuration is available to UE Then For PC5 RLC, we think SRB0 RRC should follow Uu principle to use specified configuration, which is a reserved PC5 RLC/LCH resource for SRB0 so that configuration is fixed and relay can know it is for remote UE’s SRB0 transmission from the RLC/LCH ID. For Uu RLC, we agree the concept is a little confusing. But we think the similar Uu principle can be reused, and our understanding is: - It is a separate Uu RLC/LCH other than relay UE’s own SRB RLC/LCHs (i.e. for relay’s SRB0/1/2) - The relaying channel config is specified/fixed (for SRB0) and the Uu RLC channel for this a new LCH with some specified parameters is more suitable for relaying SRB0. |
| Xiaomi | Option 2 | Option 1 | Remote UE shall establish sidelink unicast connection with relay before transmitting its SRB0 message. It’s possible relay UE could reconfigure the associated PC5 RLC configuration. |
| OPPO | Option-1 | Option-3 | For PC5 RLC, exactly the same view as QC. For Uu RLC, - We agree with QC on “It is a separate Uu RLC/LCH other than relay UE’s own SRB RLC/LCHs (i.e. for relay’s SRB0/1/2)”; - We disagree with QC on “The relaying channel config is specified/fixed (for SRB0)”, since in that way, it would occupy a LCH in a fixed manner, which is not necessary, since the relay UE can always tell NW that the relayed-SRB0 is needed, since the relaying always happen after relay UE establish connection with network. And based on the report, NW can do the configuration for the RLC bearer of relayed-SRB0, we do not see obvious disadvantage to allow that flexibility. As long as the relayed-SRB0 is established, it would be used for all the subsequent SRB0 for all remote UEs, i.e., no need for separate relayed-SRB0 for different remote UEs. |
| MediaTek | Option 1 | Option 1 | We know there is difference between PC5 and Uu on this issue. We also acknowledge that there should be a single channel between relay UE and BS used to transmit all of the relayed-SRB0 for different remote UEs. However a fixed configuration may present the simplest solution. |
| Ericsson | Option 1 | Option 1 |  |
| Samsung | Option 1 | Option 1 |  |
| Huawei, HiSilicon | Option 1 | Option 2 | We slightly prefer option 2 which is used for Uu SRB1/2 configuration, as relay UE has a RRC connection with network already, so SRB0 is not allowed to be used. However, we also agree other solutions are feasible. |
| Lenovo, MotM | Option 1 | Option 1 |  |
| vivo | Option 1 | Option 1 | Using fixed PC5 and Uu RLC/LCH configuration is simple. We are not in favor of option 3 because NW configuration after Relay UE request for PC5 and Uu RLC/LCH would lead to unnecessary latency for RRC establishment procedure for remote UE. |
| Apple | Option 2 or Option 4 | Option 2 or Option 4 | It is unclear to me why it is important to rebuild the remote UE’s Uu-SRB0 concept in U2N relay. The relay UE is fully aware of all the contents of whatever this pseudo SRB0 carries, so there is no real secrecy or security here. Although Option 2 can work well, I think it is also perfectly fine to use Option 4 or even other options not listed here to convey the meaning of the signalling for RRC setup, without pretending that there is a “SRB0” for remote UE to use. Logically speaking, all functions of remote-SRB0 can be delegated to relay UE with PC5-RRC messages by remote UE. |
| Sharp | Option1 | Option 1 |  |
| ZTE | Option 1 | Option 3 |  |
| CMCC | Option 1 | Option 3 |  |
| Nokia | Option 1 | Option 3 | PC5 unicast may be established before the remote UE’s SRB0 signalling messages exchange, meaning that the configured PC5 RLC parameters can be used instead of fixed. |
| CATT | Option 1 | Option 1 | In TS 38.331, CCCH configuration is specified. |

Question 6-2: For the delivery of remote UE’s SRB1 RRC message other than RRCResume and RRCReestablishment message, which option(s) should be used for the configuration of PC5 RLC channel and Uu RLC channel respectively?

| Companies | Option(s) for PC5 RLC channel | Option(s) for Uu RLC channel | Comments |
| --- | --- | --- | --- |
| Qualcomm | Option 3 | Option 3 | For SRB1 message other than RRCResume/RRCRerestablishment, its configuration for delivery is configured in RRCSetup or RRCReconfiguration. So, Network configuration works. It is also aligned with current TS 38.331 |
| Xiaomi | Option 3 | Option 3 |  |
| OPPO | Option-1, Option-2 or Option-3 | Option-3 | for PC5 hop - Although option-3 works, one can ask whether the PC5 configuration can already be done before SMC procedure (which is not allowed in R16) - We see option-1/2 as feasible as well if one holds the concern on PC5 configuration restriction before SMC. Otherwise, we do not have strong view in-between. |
| MediaTek | Option 3 | Option 3 | In general, we think legacy approach can be reused for Remote UE SRB1. |
| Ericsson | Option 3 | Option 3 |  |
| Samsung | Option 3 | Option 3 |  |
| Huawei, HiSilicon | Option 1/2 | Option 2 | For PC5 link, prefer to align the approach for PC5 SRB1. For Uu link, prefer to align with Uu SRB1. Just for clarification, option 2 (default configuration) allows dedicated configuration later, that is why we say default configuration can be override by dedicated configuration, while option 3 requires dedicated configuration in the first place. |
| Lenovo, MotM | Option 3 | Option 3 |  |
| vivo | Option 3 | Option 3 |  |
| Apple | Option 1/2 | Option 3 | Agree with Huawei |
| Sharp | Option 3 | Option 3 |  |
| ZTE | Option 3 | Option 3 |  |
| CMCC | Option 3 | Option3 |  |
| Nokia | Option 3 | Option 3 |  |
| CATT | Option 3 | Option 3 |  |

Question 6-3: For the delivery of remote UE’s SRB1 RRC message such as RRCResume and RRCReestablishment, which option(s) should be used for the configuration of PC5 RLC channel and Uu RLC channel respectively?

| Companies | Option(s) for PC5 RLC channel | Option(s) for Uu RLC channel | Comments |
| --- | --- | --- | --- |
| Qualcomm | Option 2 | Option 2 | Please note that NR Rel-15 agreed to use SRB1 to deliver RRCResume/RRCRerestablishment, which is different from LTE (they are SRB0 in LTE). However, because they are msg4, UE can’t obtain configuration from Network. That is why TS 38.331 introduced Section 9.2 on default radio configuration. It is fixed configuration in spec, but it can be reconfigured by Network because it is SRB1. Following same mechanism in Uu RRC, default radio configuration of PC5 RLC/LCH and Uu PDCP should also be used to deliver SRB1 message of RRCResume/RRCRerestablishment. For Uu RLC in relay, we have the similar understanding in Q6-1: it is a separate Uu RLC/LCH other than relay UE’s own SRB RLC/LCHs, which however can be reconfigured by NW because it is for relaying SRB1. |
| Xiaomi | Optin 2 | Option 2 | It’s possible for gNB or relay to reconfigure the associated RLC channel. |
| OPPO | Option-2 | Option-3 | For PC5 RLC, exactly same view as QC. For Uu RLC, there is even less reason to adopt option-1/2 compared to SRB0, since that happens well after relay UE establish the connection, and network has got the SRB0 message, so - We agree with QC on “It is a separate Uu RLC/LCH other than relay UE’s own SRB RLC/LCHs (i.e. for relay’s SRB0/1/2)”; We disagree with QC on using option-2 (default configuration), since in that way, it would occupy a LCH in a fixed manner, which is not necessary, since the NW can base on the received SRB0 to know whether there is a need to setup a LCH for SRB1. We do not see obvious disadvantage to allow that flexibility. |
| MediaTek | Option 2 | Option 2 | Default configuration can be used. |
| Ericsson | Option 2 | Option 2 |  |
| Samsung | Option 2 | Option 2 |  |
| Huawei, HiSilicon | Option 1/2 | Option 2 |  |
| Lenovo, MotM | Option 2 | Option 2 |  |
| vivo | Option 2 | Option 2 | In NR Uu, the RRCResume and RRCReestablishment is received over SRB1 using default configuration. We see no motivation to revert this principle. See the quoted text using RRC resume procedure as example below: 5.3.13 RRC connection resume 5.3.13.2 Initiation [text omitted...] 1> apply the default SRB1 configuration as specified in 9.2.1; [text omitted…] |
| Apple | Option 2 | Option 3 | Agree with OPPO. |
| Sharp | Option-2 | Option-2 |  |
| ZTE | Option 2 | Option 3 |  |
| CMCC | Option 2 | Option 2 |  |
| Nokia | Option 2 | Option 3 | Same comment as in 6-1 |
| CATT | Option2 | Option2 |  |

Question 6-4: For the delivery of remote UE’s SRB2 RRC message, which option(s) should be used for the configuration of PC5 RLC channel and Uu RLC channel respectively?

| Companies | Option(s) for PC5 RLC channel | Option(s) for Uu RLC channel | Comments |
| --- | --- | --- | --- |
| Qualcomm | Option 3 | Option 3 | For SRB2 message, its configuration for delivery is configured in RRCReconfiguration. So, Network configuration works. It is also aligned with current TS 38.331. Uu RLC can also use NW configuration. |
| Xiaomi | Option 3 | Option 3 |  |
| OPPO | Option-3 | Option-3 |  |
| MediaTek | Option 3 | Option 3 |  |
| Ericsson | Option 3 | Option 3 |  |
| Samsung | Option 3 | Option 3 |  |
| Huawei, HiSilicon | Option 1/2 | Option 2 | Unified solution for SRB1 and SRB2. |
| Lenovo, MotM | Option 3 | Option 3 |  |
| vivo | Option-3 | Option-3 |  |
| Apple | Option 1/2 | Option 3 | Same as end-to-end SRB1 |
| Sharp | Option 3 | Option 3 |  |
| ZTE | Option 3 | Option 3 |  |
| CMCC | Option 3 | Option 3 |  |
| Nokia | Option 3 | Option 3 |  |
| CATT | Option 3 | Option 3 |  |

Question 6-5: For the delivery of remote UE’s Uu DRB packet, which option(s) should be used for the configuration of PC5 RLC channel and Uu RLC channel respectively?

| Companies | Option(s) for PC5 RLC channel | Option(s) for Uu RLC channel | Comments |
| --- | --- | --- | --- |
| Qualcomm | Option 3 | Option 3 | For DRB message, its configuration for delivery is configured in RRCReconfiguration. So, Network configuration works. It is also aligned with current TS 38.331. Uu RLC can also use NW configuration. |
| Xiaomi | Option 3 | Option 3 |  |
| OPPO | Option-3 | Option-3 |  |
| MediaTek | Option 3 | Option 3 |  |
| Ericsson | Option 3 | Option 3 |  |
| Samsung | Option 3 | Option 3 |  |
| Huawei, HiSilicon | Option 3 | Option 3 |  |
| Lenovo, MotM | Option 3 | Option 3 |  |
| vivo | Option-3 | Option-3 |  |
| Apple | Option 3 | Option 3 |  |
| Sharp | Option 3 | Option 3 |  |
| ZTE | Option 3 | Option 3 |  |
| CMCC | Option 3 | Option 3 |  |
| Nokia | Option 3 | Option 3 |  |
| CATT | Option 3 | Option 3 |  |

[1][10] [13] [15] propose that only the RLC/LCH configuration is necessary for the PC5 RLC channel of relay UE and remote UE. Actually, this principle also applies to the Uu RLC channel. For the remote UE’s SRB1/SRB2, the Uu PDCP configuration is necessary for remote UE.  For the remote UE’s DRB, the Uu PDCP/SDAP configuration is necessary for remote UE.

Question 6-6: For the PC5 RLC channel configuration, do you agree that only the RLC/LCH configuration should be provided to the relay UE and remote UE?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | For relay and remote UE, they need to know which PC5 RLC/LCH for relaying We need clarification that the “RLC/LCH configuration” can be fixed in spec (specified configuration or default configuration) or from Network |
| Xiaomi | Yes |  |
| OPPO |  | We fail to get the point of the Q: “For the PC5 RLC channel configuration, do you agree that only the RLC/LCH configuration should be provided to the relay UE and remote UE”, is it to ask whether there is something else to configure for PC5 RLC channel besides RLC/LCH configuration? If that is the Q, our answer is NO, but that is to us too obvious to ask. |
| MediaTek | Yes |  |
| Ericsson | Yes with comment | In principle we agree, but we also want to echo OPPO’s comment. Maybe is good to clarify the scope of this question. |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Lenovo, MotM | Yes |  |
| vivo | Yes |  |
| Apple | Yes |  |
| Sharp | Yes |  |
| ZTE | Yes | During the IAB discussion, it has been debated whether the IAB node’s DRB/SRB can be used for the relaying of backhaul traffic. For example, the RLC/LCH of IAB node’s DRB/SRB is reused to deliver the backhaul traffic. It is finally agreed that separate BH RLC channels are configured to deliver the backhaul traffic, which only contains the RLC/LCH configuration. For this question, we want to confirm similar design that separate PC5 RLC channel is configured to relay and remote UE. It only contains the RLC/LCH configuration and does not mix with other PC5 SRB/DRB. |
| CMCC | Yes |  |
| Nokia | Yes |  |
| CATT | Yes |  |

Question 6-7: For the Uu RLC channel configuration, do you agree that only the RLC/LCH configuration should be provided to the relay UE?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | For relay, it needs to know which Uu RLC/LCH for relaying. We also need clarification that the “RLC/LCH configuration” can be fixed in spec (specified configuration or default configuration) or from Network |
| Xiaomi | Yes |  |
| OPPO |  | See reply to Q6-6 |
| MediaTek | Yes |  |
| Ericsson | Yes with comment | See reply to Q6-6 |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Lenovo, MotM | Yes |  |
| vivo | Yes |  |
| Apple | Yes |  |
| Sharp | Yes |  |
| ZTE | Yes | See reply to Q6-6. |
| CMCC | Yes |  |
| Nokia | Yes |  |
| CATT | Yes |  |

Question 6-8: For the remote UE’s SRB1/SRB2 configuration, do you agree that only the Uu PDCP configuration should be provided to the remote UE?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | According to current protocol stack, Uu PDCP is used for remote UE for SRB. We also need clarification that the “Uu PDCP configuration” may be default radio configuration for RRCResume/RRCRerestablishment, according to Section 9.2 of TS 38.331. But it is legacy Uu procedure. |
| XIaomi | Yes |  |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Ericsson | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Lenovo, MotM | Yes |  |
| vivo | Yes | It’s end-to-end configuration only for remote UE. |
| Apple | Yes |  |
| Sharp | Yes |  |
| ZTE | Yes |  |
| CMCC | Yes |  |
| Nokia | Yes |  |
| CATT | Yes |  |

Question 6-9: For the remote UE’s DRB configuration, do you agree that only the Uu PDCP/SDAP configuration should be provided to the remote UE?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | According to current protocol stack, Uu PDCP/SDAP is used for remote UE for DRB. |
| Xiaomi | Yes |  |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Ericsson | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Lenovo, MotM | Yes |  |
| vivo | Yes | It’s end-to-end configuration only for remote UE. |
| Apple | Yes |  |
| Sharp | Yes |  |
| ZTE | Yes |  |
| CMCC | Yes |  |
| Nokia | Yes |  |
| CATT | Yes |  |

### The RRC state of remote UE and relay UE

[1][10] [24] proposes to support the RRC state combination of relay in IDLE and remote UE in INACTIVE, which could achieve power saving performance in case there is no active data transfer. [27] suggest to discuss whether it is desirable to allow RRC_IDLE state for the Relay UE which has already enabled relaying functionalities (e.g. by sending discovery message).

Question 7: Do you agree to support the RRC state combination of relay in IDLE and remote UE in INACTIVE?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We think allowing this state combination can achieve benefit of reducing RRC 31ignalling overhead. It is typical scenario that INACTIVE remote UE may move to another relay or another cell for resumption in below 3 cases: - If resumed in a new cell directly, the resumed gNB can reduce RRC signaling overhead via UE context retrieve procedure - If resumed in the same cell via a new relay (i.e. remote UE performs relay reselection), the new relay may be in RRC_INACTIVE or RRC_CONNECTED. - If resumed in a new cell via a new relay, the resumed gNB can also reduce RRC signaling overhead via UE context retrieve procedure Furthermore, if this state combination is prohibited by spec, we are not sure how the Network can preclude this state combination in deployment. Please note that UE autonomous state transition to IDLE or INACTIVE state is allowed in current spec. It means NW may not know whether this state combination has happened in some cases. |
| Xiaomi | No | As discussed in SI, this combination would delay remote UE’s connection resume. We don’t see much benefit to allow this combination. |
| OPPO | Yes | If we do not allow this, our concern is that we need to spend additional effort on triggering the IDLE relay UE enter into CONNECTED/INACTIVE state for newly camped INACTIVE remote UE. |
| MediaTek | No | As discussed in SI, this combination is not motivated. |
| Ericsson | Yes | We do not see the point to not allow this and we also agree with the comment from QC and OPPO. If we prevent this state combination, the standardization efforts may be much more to clarify how the state transition would work. |
| Samsung | Yes | We share the view that this limitation may require more standard efforts. |
| Huawei, HiSilicon | Yes, but | We share the similar concern as OPPO. In addition, we wonder if the states of IDLE and INACTIVE of remote UE can be the same from relay UE point of view (except the paging monitoring), in this case the spec impact to support this combination could be small. |
| Lenovo, MotM | No |  |
| vivo | Yes | This issue is already discussed and agreed in SI to support all RRC state combinations. |
| Apple | Yes | At the current stage, there is no need to restrict RRC states combinations. |
| Sharp | Yes | We share the view with Huawei. |
| ZTE | Yes |  |
| Nokia | Yes | Not allowing this combination will necessitate the standard to define how to ensure this, and we don’t think there is time for this |
| CATT | Yes |  |

## System information delivery

### Which system information should be forwarded to remote UE

- [1]proposes that before connected to Relay UE, remote UE can acquire a small set of essential parameters of system information broadcast/groupcast by relay UE, such as  cell access parameter (e.g. PLMN ID, cell ID, cell barring), configuration to initialize RRC establishment (e.g. T300/T319) and UAC configuration. [3] [16] proposes that MIB, SIB1 and SIB12 should be relayed to Remote UE as essential SIBs. [26] proposes that Minimum SI can be relayed to remote UEs via broadcast and group cast. [34] proposes that SIB1/2/3/4/5/6/7/8/12/13/14 can be relaying to the Remote UE by Relay UE. [14] proposes that only the most essential system information should be delivered to the remote UE. [24] proposes that whether a full SIB or selected SIB information are relayed by the relay UE to the remote UE is left to relay UE implementation. [5] proposes to define a set of SIBs which are mandatory for a relay UE to be forwarded.

Question 8: Which system information (e.g. MIB, SIB1/2/3/4/5/6/7/8/12/13/14) should be forwarded to remote UE?

| Companies | MIB/SIB type | Comments |
| --- | --- | --- |
| Qualcomm | See comments | Considering different remote UEs may have different interest/capability, we tend to think there is no need to restrict which SIBs should be forwarded by relay. And even for MIB/SIB1, some of their IEs are not needed for remote UE, e.g. common COREST/Search space config, initial BWP. Our preference is: - Before unicast PC5 connection with relay, remote UE can acquire some essential info on SIB (e.g. UAC config, TAI list) from relay UE in broadcast/groupcast way. This set of essential info may not be an existing Uu SIB because of overhead issue of broadcast/groupcast - After unicasting PC5 connection with relay, relay UE can forward the entire Uu SIB(s) to remote UE via unicast PC5-RRC. But when to forward and which SIB to forward can be left to relay UE implementation. |
| Xiaomi | Comment | We agree with QC that no need to define essential SIB for relay. MIB and SIB1 should always be provided by relay to remote UE. Other SIBs could be provided in an on demand way. It’s not forward compatible to only support part SIBs delivery. |
| OPPO | See comments | System information should be forwarded to the remote UE: - MIB and SIB1 which are related to access to the NW; - SIB6/7/8 are related to safety. - SIB12 related to sidelink configuration System information doesn’t need to be forwarded to the remote UE: - SIB5 is only relevant for inter-RAT cell re-selection; - SIB13/14 are for LTE V2X sidelink communication. Need to be further discussed: - SIB2/3/4, which depends on whether/how for remote UE to perform cell reselection. |
| MediaTek | comment | We have the similar understanding as QC and Xiaomi. We are thinking if the SIB forwarding can be UE implementation. |
| Ericsson | Too early to decide | We think that is too early to decide the actually context of the SIB or which SIB should be forwarded to the remote UE. Nevertheless, how understanding is that whatever is not related to sidelink and relay, should not be allowed to be forwarded. |
| Samsung | See comment | We understand the difficulty to decide specific SIB(s) but we think the forwarding of all the SIBs should be avoided and allow minimum SIB for NW access and relaying operation. |
| Huawei, HiSilicon | See comments | Similar view with OPPO that MIB/SIB1/SIB12/SIB6/7/8 should be forwarded to remote UE anyway. But for other SIBs, it could be forwarded in on-demand way, then which ones to be supported seems have no impact on spec, so maybe RAN2 does not need to discuss it more. |
| Lenovo, MotM | Comment | Assuming the relay UE is just a means to bring the network to otherwise not reachable remote UE, the SI delivery should imitate the rel. 16 behaviour as much as possible – BUT – as pointed out by other companies, there’s no reason to forward information that are guaranteed to be useless to remote UEs and that applies to even MIB, SIB1. RAN2 may spend some efforts and time to narrow down to a list of IEs from MIB/ SIB1/ SIB12. For the remaining SIBs, the remote UE can just request them from relay. A relay should be enabled to distribute SIBs to more than one linked remote UE at the same time (like groupcasting of SI/ SIB). |
| vivo | MIB, SIB1/2/3/4/5/6/7/8/12) | - Minimum SI (i.e., MIB and SIB1) ：request by remote UE, may depend on whether connected to relay UE - Other SIs (SIB2-SIB14): request by remote UE, may depend on whether connected to relay UE - Support: SIB2/3/4/5 related to cell re-selection, SIB6/7/8 related to public warning, SIB12 related to NR sidelink communication NOT support: SIB13/14 used to support LTE V2X SL. |
| Apple | Comment | We need focus on how to deliver SIB to remote UE(s) not connected to relay UE yet. For that, we think SIB1/MIB is needed. FFS how this is delivered. We also share the concern that we do not need to waste radio resources for IEs whch is meaningless to remote UEs, such as RACH configurations. |
| Sharp | MIB/SIB1/SIB12/SIB6/7/8 at least |  |
| ZTE | See comment | MIB contains subCarrierSpacingCommon and ssb-SubcarrierOffset which are useless to out of coverage remote UE while systemFrameNumber and cellBarred info which are necessary to be forwarded to the remote UE. Some information in SIB1, such as cell access control and cell configuration related information, should be acquired by the out of coverage Remote UE for relay selection and mobility purpose. SIB2~5 contain parameters used to control intra-frequency, inter-frequency and inter-RAT cell re-selection. For the out of coverage Remote UE connected with Relay UE, it is not necessary to perform the cell re-selection. Therefore, the SIB2~SIB5 do not need to be forwarded to Remote UE. For the SIB 12, we think it is necessary to be forwarded to remote UE for sidelink discovery and communication. |
| CMCC | Comments | MIB/SIB1/SIB12 should be forwarded to remote UE at least. |
| Nokia | To early | We need only to send the essential SIBs, meaning that first of all we need to define which of the SIBs are essential. Also, not all SIBs can be obtained on-demand if the Relay UE is in RRC_CONNECTED, and we should avoid allowing all to be obtained on-demand simply to allow the Relay to forward all. Althout the proposal from Qualcomm is the safest option, it will require much data to be forwarded which is otherwise not needed. |
| CATT | See comments | MIB/SIB1/SIB12 can be defined to essential SIB for relay. Other SIBs depends on remote UE’s request. |

### On-demand system information acquisition

For RRC_Connected remote UE, [7] [14] [21] [26] proposes that DedicatedSIBRequest procedure is re-used for the Remote UE to request the SI via relay UE and  network can respond by sending dedicated RRC message for concerned SIBs. Actually, it has been agreed in SL relay SI phase that DedicatedSIBRequest procedure is re-used for the Remote UE in RRC_CONNECTED to request SI via the Relay UE.

For RRC_Idle/INACTIVE remote UE, [7] [26] propose to support the Msg3 based on-demand SI request for remote UE. However [2] and [3] proposes that legacy on-demand SIB acquisition procedure can’t work for IDLE/INACTIVE remote UE. In order to support the on-demand system information acquisition of remote UE, [2] [3] [7] [14] [21] [29] [34] [36] proposes that  remote UE informs relay UE on requested SIB type(s) via PC5 RRC message. Then, relay UE triggers legacy on-demand SIB acquisition procedure and includes the acquired SIB in PC5 RRC message. [7] also mentions that the on demand SIB request is from remote UE to network, and relay UE is not aware of the requested SIB. Relay UE just forwards the on demand SIB request to gNB, and later forwards the SIBs to remote UE. Here the SIBs which are mandatory for a relay UE to be forwarded need to be specified, i.e. define some SIBs that the relay should keep monitoring. [16] propose to discuss the need of on-demand SI request for non-essential system information for Remote UE in RRC_IDLE or RRC_INACTIVE. [21] proposes to define the criteria for a remote UE to request a relay UE for system information delivery. It is also proposed in [21] that a relay UE may switch on/off system information delivery function by a remote UE announcement and be responsible for the first time and subsequent modified system information delivery.

Question 9-1: For RRC_Connected remote UE, could RAN2 confirm that DedicatedSIBRequest procedure is re-used for the Remote UE to request the SI via relay UE and  network can respond by sending dedicated RRC message for concerned SIBs?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We have agreed and captured in TR 38.836. The legacy DedicatedSIBRequest procedure can be reused |
| Xiaomi | Yes |  |
| OPPO | Yes | It was already captured in TR 38.836 |
| MediaTek | Yes |  |
| Ericsson | Yes with comments | We agree that current DedicatedSIBRequest procedure is re-used for the Remote UE to request the SI via relay UE but we strongly disagree that “network can respond by sending dedicated RRC message for concerned SIBs”. According to what has been agreed in Rel-16, it up to the network how to deliver the SIB and we have to stick to this principle. Therefore, we don’t need to say that “network can respond by sending dedicated RRC message for concerned SIBs” |
| Samsung | See comment | We share Ericsson’s comment. The legacy procedure should be reused except the on-demand request can be relayed. |
| Huawei, HiSilicon | Yes | We do not see the issue on network handling, because in the question it says network can respond by…. |
| Lenovo, MotM | Yes |  |
| vivo | Yes |  |
| Appple | Yes |  |
| Sharp | Yes |  |
| ZTE | Yes |  |
| CMCC | Yes |  |
| Nokia | Yes |  |
| CATT | Yes |  |

Question 9-2: For RRC_Idle/INACTIVE remote UE, which option(s) should be considered for the on demand SI acquisition?

Option1: Remote UE informs relay UE on requested SIB type(s) via PC5 RRC message. Then, relay UE triggers legacy on-demand SIB acquisition procedure and includes the acquired SIB in PC5 message.

Option2: Remote UE triggers legacy on-demand SIB acquisition procedure towards gNB, where relay UE just forwards the on demand SIB request from remote UE to gNB.

| Companies | Option(s) | Comments |
| --- | --- | --- |
| Qualcomm | Option 1 | We think legacy on-demand SIB acqusition procedure can’t work for IDLE/INACTIVE remote UE. The reason is that legacy procedure of IDLE/INACTIVE UE’s on-demand SIB acquisition (i.e. RRCSystemInfoRequest procedure as specified in Rel-15) needs to receive Msg4 addressed with TC-RNTI as response before monitoring SIB, according to section 5.1.5 of TS 38.321. However, if remote UE is connected to relay, there is no Msg1/2 and thereby TC-RNTI can’t be obtained in Msg2. Thus, some spec change is needed if supporting IDLE/INACTIVE UE. To resolve this issue, we think the simplest solution is to allow relay UE to be aware of the on-demand SIB request from remote UE (i.e. option 1). For option2, it implies that relay UE has to enter RRC_CONNECTED even if only a Uu SIB is on-demand requested from remote UE. We think it is not necessary |
| Xiaomi | Option 1 | For IDLE/INACTIVE remote UE, relay UE could be in IDLE/INACTIVE. Msg1 based SI request is not supported, since remote could not send preamble. It’s possible remote UE could send DedicatedSIBRequest to relay. However, this would trigger relay UE to enter connected to relay the message, which would result in signal overhead. Therefore, we prefer option 1. |
| OPPO | Option 2 | The relay UE can simply forward the on demand SIB request from remote UE to gNB and doesn’t need to be aware of the SIB(s) requested by the remote UE. The relay UE forwards the received SI directly to the remote UE. The SIBs which are mandatory for a relay UE to be forwarded need to be specified, i.e. define some SIBs that the relay should keep monitoring and forward if exist, as the output of Q8. Option1 will cause additional PC5 signalling, if multiple remote UEs are connected to the relay UE, it may be too complex for the relay UE to keep track of the request for each. Prefer a simpler way for the relay UE. |
| MediaTek | Option 1 | Meanwhile option 2 can not work, since Relay UE may not be aware of the on demand SIB request transmitted from remote UE to gNB |
| Ericsson | Option 1 | If we go with Option 2, this basically means that a relay UE in connected needs to send an on-demand request via SRB0 to the network. While this it may work somehow, it would require additional standardization effort to differentiate the UE behaviour with respect on how to use the legacy procedure for sidelink relay. |
| Samsung | Option 1 |  |
| Huawei, HiSilicon | Option 1 | Option1 seems more efficient, thus we slightly prefer option1. |
| Lenovo, MotM | Option 1 | Apart from the reasons mentioned above by other companies, the Option 1 allows the relay UE to combinedly serve the requests from linked multiple remote Ues i.e. combine the SI requests and distribute the Sis to them together. |
| Vivo | Option 1 | In Option 1, a relay UE may switch on/off system information delivery function by a remote UE announcement and be responsible for the first time and subsequent modified system information delivery. Option 2 cannot work in case of subsequent modified system information delivery. |
| Apple | Option 1 | We think this approach can be used for other remote-UE initiated procedures. |
| Sharp | Option 1 |  |
| ZTE | Option 1 |  |
| CMCC | Option 1 |  |
| Nokia | Option 1 |  |
| CATT | Option1 | Remote UE informs relay UE on requested SIB type(s) via PC5 RRC message. Relay UE acquires SIB using legacy procedure. |

### How to forward the system information to remote UE

- For the system information forwarding by relay UE, [2] proposes relay UE collect the information based on Uu SIB and then relay UE generate a E-SIB and transmit it via PC5 interface. FFS the groupcast/broadcast PC5 message is PC5 communication signal or a separate discovery protocol message. [7] proposes that the SI transmitted in broadcast manner by Relay UE is independent from the Relay discovery message sent by Relay UE. [3] proposes that relay UE forwards the original SIBs to Remote UE without changing the information and format of the SIB. [24] proposes to discuss whether a new SIB is needed for sidelink relay or whether the existing NR sidelink SIB can be re-used.

Question 10-1: When should the remote UE receive the system information via PC5?

Option 1: Before PC5 connection establishment with relay UE,

Option 2: After PC5 connection establishment with relay UE,

Option 3: Both of the above

| Companies | Option(s) | Comments |
| --- | --- | --- |
| Qualcomm | Option 3 | Before unicast PC5 connection (Option 1), we think remote UE needs to obtain at least below information from gNB when intended to establish RRC connection: - Cell access parameters (e.g. PLMN identity, cell identity, TAI list) - Configuration to initialize RRC connection establishment (e.g. values of timer T300/T319) - UAC configuration After unicast PC5 connection (Option 2), we also think it is needed because relay UE needs to forward updated Uu SIB to remote UE. In this case, relay UE can use unicast PC5-RRC to forward entire Uu SIB(s) |
| Xiaomi | Option 2 as baseline | Remote UE need to do UAC even in connected state. Therefore, option 2 is baseline. Before PC5 connection establishment, remote UE would perform relay selection. Cell information may be needed. If the cell information is provided by SI not by discovery message, remote UE has to receive SI. Otherwise, remote UE doesn’t need to receive SI. This may need further discussion. |
| OPPO | Option 3 | Before PC5-RRC connection, the SI is helpful for the remote UE to decide which relay UE to camp on - in this case the broadcast manner can be used. After PC5-RRC connection, the on-demand SI acquisition can be supported by relay forwarding the on demand request from remote UE, and the SIBs from gNB. |
| MediaTek | Option 3 |  |
| Ericsson | Option 3 |  |
| Samsung | Option 3 |  |
| Huawei, HiSilicon | Option 2 | Our thinking is that the information to be used for relay selection could be included in discovery message, then the system information could be forwarded after PC5 connection establish after the relay UE is selected by the remote UE. |
| Lenovo, MotM | Both | Needs for SIBs could be different at different point of time. |
| vivo | Option 2 | We slightly prefer option 2 by using PC5 RRC connection for more efficient system information delivery to remote UE. |
| Apple | Option 3 |  |
| Sharp | Option 3 |  |
| ZTE | Option 3 |  |
| CMCC | Option 3 |  |
| Nokia | Option 3 |  |
| CATT | Option3 |  |

Question 10-2: Which signaling shall be used to carry the system information forwarding via PC5?

Option 1: PC5-RRC message

Option 2: Discovery protocol message

Option 3:  Other new SL-SRB (e.g., for BCCH forwarding only)

| Companies | Option(s) | Comments |
| --- | --- | --- |
| Qualcomm | Option 1 and Option 2 | Before unicast PC5 connection, remote UE can acquire some essential info on SIB from relay UE in broadcast/groupcast way. So, it should use discovery protocol message (option 2). The details can be discussed further After unicast PC5 connection, at least PC5-RRC (Option 1) can be used. The entire Uu SIB can be encapsulated into existing PC5-RRC message or a new PC5-RRC message, which can be further discussed. Whether broadcast/groupcast can be used can also be further discussed. For option 3, is the intention to specify a new BCCH for SIB forwarding? Doesn’t introduce a new PC5 physical channel which is not in scoping of this release? |
| Xiaomi | Option 1 and option 3 | Option 1 can be used for on demand SI delivery, which is unicast. MIB/SIB1 delivery should be done in broadcast/groupcast way to save signaling. Option 3 is preferred. It’s easier to support RAN mechanism such as SI modification. Option 2 would require NAS support and it’s unclear how to support SI modification. |
| OPPO | Option-3 | If the SI is requested by a RRC_Connected remote UE, it may get forwarded SIB via broadcast or unicast (if network decide to respond using unicast). Otherwise, remote UE can get the forwarded SIB via broadcast. - For unicast-case, there is no difference compared to normal SRB1 messsage; - For broadcast-case, we believe option-3, a new SL-SRB message be the easiest way-out (option-2 cause extra interaction with SA2/CT1, and option-1 is not doable since that was designed for unicast) |
| MediaTek | Option 1 | We think option 2 make the discovery message complicated, as there is no need to multiplex two message together. They are sent at different occasions. |
| Ericsson | Option 1 but | If we talk on after the PC5 connection has been established, then we agree that PC5-RRC can/should be used. If we talk in general, we are not sure why PC5 broadcast has been left out from the options. |
| Samsung | Option 1 with comment | SIB can be delivered over PC5 in unicast manner or broadcast manner. For broadcast option, we prefer to use SCCH (like SL-SRB0) |
| Huawei, HiSilicon | Option 1 | For clarification, we agree some information (e.g. PLMN, cell ID, etc) for relay selection should be included in discovery message, but we do not consider it as SIB forwarding. |
| Lenovo, MotM | Option 2 for relay (re)selection and Option 1 afterwards | A remote UE needs to ensure that it is selecting a relay that is being served by a serving cell that supports the desirable features/ SIBs. So, for this purpose the Discovery (like) message should be used to indicate some basic SI to the remote UE e.g. a list of supported SIBs/ Feature by the relay’s serving cell. |
| vivo | Option 1 | For Option 2 using Discovery protocol message, which has potential SA2/CT1 impact and should be avoided. For Option 3, we have similar concern as Qualcomm on the BCCH-like forwarding. |
| Apple | Option 1 and Option 2 | For Option 1, We support RAN2 to define PC5-RRC message for broadcast in R17. |
| Sharp | Option 1 and Option 3 | Option 1 could be used after PC5 connection establishment with relay UE while Option 3 could be used before PC5 connection establishment with relay UE. Option 2 will require interaction with upper layer which we think it is not necessary. |
| ZTE | Option 1 and 3 | For the system information that is used for AS criteria of relay selection, such as PLMN, cell ID, load, it can be delivered via discovery message. For the other system information, we prefer Option 1 and 3 to support different scenarios. |
| CMCC | Option 1 |  |
| Nokia | Option 1 and option 2 |  |
| CATT | Option1 and 3 |  |

Question 10-3: If remote UE could receive the system information via PC5 before PC5 connection establishment with relay UE, which cast type (i.e. unicast/groupcast/broadcast) can be used for the system information forwarding via PC5?

| Companies | Cast type | Comments |
| --- | --- | --- |
| Qualcomm | Groupcast/broadcast | As we indicated in Q10-2, only groupcast/broadcast way is available before unicast PC5 connection. And discovery protocol message can be used for this purpose. |
| Xiaomi | Broadcast/groupcast |  |
| OPPO | broadcast | Broadcast as baseline for SI forwarding since there is no PC5-RRC connection and the relay UE doesn’t know how many remote UEs are there is in proximity. Group-cast is not feasible since the management of G-cast is up to upper layer as defined since R16, so there is no tools for AS-layer to establish/manage the group. |
| MediaTek | Groupcast | Broadcast transmission may be received by some Ues that camp on other gNB. |
| Ericsson | Groupcast, broadcast |  |
| Samsung | Broadcast | We think the transmission method of system information in Uu (broadcast or unicast) should be used via PC5. We do not think that existing system information includes group specific information. |
| Huawei, HiSilicon |  | We do not see the need of SIB forwarding before PC5 connection establishment. |
| Lenovo, MotM | Groupcast/broadcast | We may need to think about what a useful “DST L2 Id” could be for a GC/ BC for SI distribution purpose – we think, such DST L2 Id may not be coming from upper layer. |
| Vivo | None | Allowing system information via PC5 before PC5 connection establishment may bring more issues in RAN2 and potentially SA2, e.g., how the destination L2 ID is assigned just for the groupcast/broadcast-based SI forwarding. |
| Apple | Broadcast/groupcast | The only difference between GC and BC is the HARQ-FB mechamis, we are fine to support HARQ FB for SIB delivery. |
| Sharp | broadcast |  |
| ZTE | Broadcast |  |
| CMCC | Broadcast |  |
| Nokia | Broadcast (and groupcast) | We are not sure of the advantages of using groupcast for this procedure, and it may cause extra specification overhead to handle the group management |
| CATT | Broadcast |  |

Question 10-4: If remote UE could receive the system information via PC5 after PC5 connection establishment with relay UE, which cast type (i.e. unicast/groupcast/broadcast) can be used for the system information forwarding via PC5?

| Companies | Cast type | Comments |
| --- | --- | --- |
| Qualcomm | At least unicast | As we indicated in Q10-2, unicast PC5-RRC can be used to forward entire Uu SIB in this case. Whether groupcast/broadcast are still needed (e.g. for PWS) after unicast PC5 connection can be further discussed. It seems unicast is sufficient in this case. |
| Xiaomi | Unicast/broadcast/groupcast | On demand SI could be delivered in unicast. But broadcast/groupcast is also needed in following cases to avoid signalling overhead, - Upon SI modification, all remote UEs should be provided with the same update SI. - Multiple remote UEs require the same on demand SI. |
| OPPO | Unicast and broadcast | As our reply to Q10-2, - Unicast if the network respond using unicast Broadcast if the SIBs are broadcasted by NW. |
| MediaTek | Unicast/groupcast |  |
| Ericsson | At least unicast | We agree with QC |
| Samsung | Unicast and broadcast |  |
| Huawei, HiSilicon | Unicast should be the baseline |  |
| Lenovo, MotM | All Cast types | GC maybe useful if more than one linked remote UEs need the same SIB(s). |
| vivo | Unicast | As our reply to Q10-2, PC5 RRC is used for system information delivery. PC5 RRC is used in unicast way as in Rel-16 NR V2X. |
| Apple | All cast type from remote UE perspective, Unicast from relay UE perspective | From remote UE perspective, SIB information in any cast type shall be retrieved. For relay UE perspective, it may want to only send a dedicated PC5-RRC message as SL unicast for a particular relay UE. |
| Sharp | At least unicast |  |
| ZTE | At least unicast |  |
| CMCC | Unicast and broadcast |  |
| Nokia | At least unicast |  |
| CATT | Unicast and broadcast |  |

### SI acquisition of in coverage remote UE

[2][3] [5] [7] propose that in coverage remote UE can only acquire the system information from relay UE after connected to relay UE. It is more power efficient for remote UE. Also, it ensures that the SIB info acquired by remote UE is aligned with relay UE suppose the remote UE and relay UE camp on different cells. [14] [23] [24] [26] propose that in coverage Remote UE can be allowed to directly receive the system information via Uu from the network. Whether in coverage remote UE acquires the SI messages via the relay UE or via the broadcast shall be left to the remote UE implementation. [27] proposes that the specific details of system information delivery can be discussed once the L2-relaying design is more stable.

Question 11: Should the in coverage remote UE be allowed to directly receive the system information via Uu after connected to relay UE?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | After connecting to relay UE via unicast PC5 link, its connection is now managed via the relay UE. To make it efficient and simple, we think remote UE should receive SIB and paging only from relay UE, i.e. reception via Uu for in-coverage remote UE is unnecessary. |
| Xiaomi | No | The dual connectivity has been excluded. There is only one RRC for remote UE, either direct or indirect. |
| OPPO | See comments | From remote UE perspective, it will anyway perform cell reselection procedure in additional to relay reselection procedure, so SIB reading from neighboring cells (if any) is needed for remote UE. But we would like to highlight, the two are both needed - Relay forwarding SIB for remote UE camping on indirect connection; - Remote UE reading SIB from neighboring cells for cell reselection, i.e., for camping on direct connection. The two are independent from each other, and there is no need/possiblity to see one as the substitute of the other. |
| MediaTek | No | Agree with QC |
| Ericsson | Yes | What has been point out by OPPO is one example. Another example is that the remote UE should be able to acquire also the other SIBs that are not sent via relay UE (this is depended also to Question 8). How it should work in this case? |
| Samsung | Yes | Reading SIB via Uu should not be restricted for remote UE in coverage. Also, all the SIBs do not have to be forwarded via PC5. |
| Huawei, HiSilicon | No | Agree with Qualcomm and MediaTek. |
| Lenovo, MotM | Yes | We do not see any reason of artificially restricting this – at least for regularly broadcasted SIBs. We need to think about power efficient relays as well. The choice of which and when SIBs/ Paging is to be received via relay should be left to a remote UE. |
| vivo | Yes, with comments | If the in coverage remote UE is allowed to request SI directly from gNB, then it should also be allowed to directly receive the system information via gNB after connected to relay UE. So, the question should be firstly clarifying whether the in coverage remote UE is allowed to request SI directly from gNB after connected to relay UE. And our answer is still YES. Whether define some criterion to decide which path is used for SI request/reception or leave to UE implementation can be further studied. |
| Apple | No spec impact for R17 | I do not think we need to discuss this issue. For UE using a relay, it only has one connection to NW (via relay). So, we do not need to “add” any “new” procedures by assuming a UE can still read SIB via Uu. |
| Sharp | comments | We share the view with OPPO they are independent. Not clear the intention of this question. |
| ZTE | Yes |  |
| CMCC |  | Same understanding with OPPO. |
| Nokia | Yes | As mentioned in Q8, it may not be all SIBs which are easily forwardable, so it may simplify the process to allow for the UE to receive SIB from Uu. Allowing to receive on Uu may lower the burden of the PC5 interface as well as the Relay UE. |
| CATT | No | The serving cell of the Relay UE can be different to Remote UE’s source cell. |

## Paging

### Which remote UE’s paging should be monitored

In [9], it is proposed that a relay UE monitors paging occasions of all PC5-RRC connected remote UEs. However, monitoring paging for all connected remote UEs may consume significant power and may eliminate the savings achieved by connected mode DRX. The selective paging monitoring of remote UE is discussed for relay UE. In [9] [27], it is proposed a relay UE can skip monitoring of POs of one or more remote UEs based on network indication. [21] proposes that paging forwarding is switched on/off by remote UE announcement. [24] proposes that when both the remote UE and relay UE are in RRC_CONNECTED, the relay UE shall monitor only the SI change indication or the indication about PWS notification in each POs (its own and remote UE POs). [14] [28] proposes to discuss the necessity of some handshaking procedure to explicitly signal if and when a remote UE needs its linked U2N relay UE to monitor its paging and if and when the paging for the remote UE can really be monitored by the linked relay. [36] proposes that the Relay UE may not monitor the PO for paging Remote UE if it is informed that a paging message to Remote UE is not transmitted in the PO.

Question 12-1:  Should the relay UE monitor paging for all remote UEs connected to the relay UE? If not, please further clarify how relay UE ascertain which remote UE’s paging is to be monitored.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | See comments | We think this question should be discussed for different cases: - Case 1: both relay and remote UE in CONNECTED state. We don’t think relay UE needs to monitor paging for remote UE. For a connected UE, it needs to monitor paging for PWS and SIB update according to spec. Then, relay UE anyway needs to monitor paging for PWS and SIB update (and forward necessary ones to remote UE). - Case 2: Relay in CONNECTED state. Before this discussion, we think it needs to first discuss whether relay still needs to monitor paging for remote UE. In this case, because RRC connection is available in relay, maybe a better solution is that gNB just includes paging record for remote UE in dedicated RRC message towards relay UE. - Case 3: Relay in IDLE/INACTIVE. We think relay should monitor paging for all remote UEs. If the intention is to reduce power consumption of relay UE for monitoring, we think a better direction is to align POs of remote UE (e.g. group paging similar to Rel-17 power saving enhancement). |
| Xiaomi | Comment | We think this is related to Question 3-1, which is how to provide PO to relay UE. It’s up to remote UE whether to provide its PO to relay UE. If a remote UE provides its PO to relay, relay has to monitor corresponding PO. Otherwise, relay UE doesn't need to monitor. Relay UE shall not autonomously decide not to monitor remote UE’s PO, which is not future proof. |
| OPPO | See comments | The relay will - Monitor paging for a remote UE when it receives some paging configurations from remote UE, for example the DRX and pseudo UE_ID. This is to support RRC_IDLE/INACTIVE remote UEs; - And it will not monitor paging for the remote UE if the paging configuration is not setup or released. This is to support RRC_CONNECTED remote UEs; We do not think additional optimization for CONNECTED relay as mentioned by QC is needed. |
| MediaTek | comment | The answer can be yes for idle/inactive Remote UEs/Relay UE. For connected remote UEs, the network may have no need to page it. If Relay UE is connected but the Remotes are inactive or idle, the network can simply find the remote UE through Relay UE as long as PC5 between remote UE and relay UE is kept. |
| Ericsson | Comment | We fully agree with Qualcomm |
| Samsung | See comment | Same view as OPPO |
| Huawei, HiSlicon | Yes | The basic procedure should be to enable a relay UE to monitor paging message for the remote UEs connected to it. And if there is any optimizations for UE power saving or other cases could be discussed on top of the basic procedure. |
| Lenovo, MotM | Comment | We think the main question here is about cases when the remote UE is in RRC Idle/ Inactive. First, we do not think that a relay UE has infinite capacity in terms of battery life. Second, if there will be a BWP switch required to monitor paging for a linked remote UEs, the relay UE loses opportunity to be DL scheduled. Third, remote UE might be indeed able to receive the paging message – since paging messages can be transmitted more robustly. |
| vivo | No, with comments | Similar as system information delivery, we think paging message forwarding can also be switched on/off by remote UE announcement to relay UE. |
| Apple | Comment | We agree with QC and MediaTek that connected relay can receive dedicated signalling from gNB instead of listening to paging. |
| Sharp | Comments | The question could be discussed at least based on the state of relay UE. In a case the relay UE is in IDlE/INACTIVE state, the relay UE should monitor paging for all remote UEs connected to the relay UE if the paging monitoring information is provided to relay UE. In a case the relay UE is in Connected State, regarding to its connecting remote UEs - For the remote UEs also in Connected State, it is no necessary to monitor paging for them - For the remote UEs not in Connected State, it should be discuss if relay UE is able to monitor paging for them or if not, how to guarantee the delivery of paging to these UE. |
| ZTE | No | We think the relay UE does not need to monitor the POs of RRC_Connected remote UE. On the other hand, in coverage remote UE may choose to monitor paging on its own. In this case, relay UE does not need to monitor the paging of this remote UE. Relay UE may be notified of remote UE’s RRC state or remote UE may ask relay UE to start/stop monitor the paging. |
| CMCC | Comment | We agree the comments of MediaTek. |
| Nokia | Comment | Agree with Qualcomm |
| CATT | See comments | Relay UE monitors paging for RRC_IDLE/INACTIVE remote UE depending on its request. |

Question 12-2:  Suppose a relay UE needs to monitor paging for a remote UE, should the relay UE monitor all POs for the remote UE? If not, please further clarify how relay UE ascertain which POs are to be monitored.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | See comments | We think this solution can only work in case 2, i.e. relay in CONNECTED while remote UE in IDLE/INACTIVE because NW needs to indicate which PO to skip via RRC. Again, if the intention is to reduce power consumption of relay UE for monitoring, we think a better direction is to align POs of remote UE (e.g. group paging similar to Rel-17 power saving enhancement). It can also work when relay in IDLE/INACTIVE. |
| Xiaomi | Yes | We think this is related to Question 3-1, which is how to provide PO to relay UE. If a remote UE provides its PO to relay, relay has to monitor corresponding PO. It’s up to remote UE whether to provide its PO to relay UE. Relay shall not autonomously decide not to monitor remote UE’s PO, which is not future proof. |
| OPPO | Yes | As our reply to Q12-2, we do not think additional optimization for CONNECTED relay as mentioned by QC is needed. Either “relay UE monitor part of POs for the remote UE” or “gNB just includes paging record for remote UE in dedicated RRC message towards relay UE”, it is just optimization on the baseline scheme, there is no need to go to that in this release. |
| MediaTek | Yes |  |
| Ericsson | Maybe | We agree with Qualcomm |
| Samsung | Yes | Relay UE should monitor POs for corresponding remote UE(s) |
| Huawei, HiSilicon | Yes | Similar comments to Q12-1. RAN2 should first conclude on the basic procedure, the optimizations could be discussed later. |
| Lenovo, MotM | Yes, unless | Assuming relay UE is monitoring paging for a given remote UE, all POs need to be monitored unless RAN2 pursues some optimizations that involve CN/ RAN. |
| vivo | Yes | We consider the relay UE monitoring all POs for the remote UE as baseline. Relay UE skipped monitoring of POs of remote UEs is optimization on top of that. |
| Apple | Yes for Relay in IDLE/INACTIVE |  |
| Sharp | Yes |  |
| ZTE | Yes |  |
| CMCC | Yes |  |
| Nokia | Comments | Agree with Qualcomm |
| CATT | Yes |  |

### How to forward the paging message to remote UE

Suppose relay UE only knows the pseudo UE ID or calculated PO(s), the relay is unable to determine the specific remote UE(s) indicated in a received paging message.  As a result, [9] [11] proposes the relay UE forwards the all paging message received in a PO to all of the remote UEs configured with that PO. [11] also consider the short message forwarding. For example if short message set bit 1 or 2, relay UE send the short message to all remote UEs by broadcast or groupcast. If short message set bit 3 (stopPagingMonitoring), relay UE doesn’t send the short message to remote UE.

[24] proposes to discuss whether paging forwarding over PC5 is done using sidelink broadcast or unicast communication. [11] also proposes that relay UE may send the paging message by broadcast or groupcast if multiple remote UEs are paged. [21] [24] [27] [34] proposes that paging forwarding information is carried in PC5-RRC message.

Question 13: Which cast type (i.e. unicast/groupcast/broadcast) can be used for the paging forwarding via PC5?

| Companies | Cast type | Comments |
| --- | --- | --- |
| Qualcomm | Unicast only | It depends on whether PO or UE-ID is shared between remote and relay. But we think in both cases, unicast is enough: - If full UE-ID is shared, relay can maintain the mapping from L2 ID of remote UE to its full UE-ID (e.g. 5G-S-TMSI). Then it can just forward paging for MT data to the concerned remote UE via PC5-RRC - If pseudo UE-ID or PO is shared, relay can maintain the mapping from PO to a group of remote UEs (sharing same PO). Then it can just forward paging for MT data via PC5-RRC one by one. Because remote UE is in IDLE/INACTIVE state without active traffic, we think there is no signaling overhead issue to use unicast In addition, if paging is for PWS or SIB-update, relay don’t need to forward paging but just need to forward acquired PWS/SIB to remote UE. Whether PWS or SIB1 can be broadcast/groupcast after PC5 connection is another question, but we think at least groupcast/broadcast for paging is not necessary. |
| Xiaomi | Unicast/broadcast/groupcast | Unicast is used to carry paging message if only one or several remote UEs are paged in their PO. Broadcast/groupcast could save siganling overhead when multiple remote UEs are paged. |
| OPPO | Unicast | we do not think relay UE needs to filter the paging content based on UE ID, i.e., relay UE can base on the mapping between remote UE and its related PO/PF (calculated by relay UE), to forward the whole PCCH-message, without further changing the payload. And we do not think short-message need to be forwarded. |
| MediaTek | comment | We think the answer of this question depends on the discussion of Q11 and Q12. If the PC5 connection is kept between Remote UE and Relay UE, unicast forward should e adopted. |
| Ericsson | Unicast |  |
| Samsung | Unicast |  |
| Huawei, HiSilicon | Unicast | According to LTE option2 agreed to be adopt in SI, the relay UE should decode the paging message and forward the paging information to the exact remote UE. |
| Lenovo, MotM | All casts | UC: Depending on if there’s paging only for one linked UE GC/ BC: If there’s a paging for more than 1 linked UEs and there’s a L2 ID for this purpose. |
| vivo | unicast | We suggest to use a common solution for forwarding system information and paging message. |
| Apple | unicast |  |
| Sharp | Unicast |  |
| ZTE | Unicast |  |
| CMCC | Unicast |  |
| Nokia | Unicast | Seems like the obvious choice as group/broadcasts are clearly further enhancements |
| CATT | Unicast/broadcast | Forward the original PCCH-message. |

### Impact of BWP on paging monitoring

According to [28], a RRC Connected UE (that could be a U2N relay UE as well) is configured up to 4 BWPs (in UL as well as in DL) and not all may be configured with a common search space. If the current active BWP does not have a common search space, it’s network’s responsibility to ensure that System Information (changes) is dedicatedly sent to the UE. So far, a RRC Connected UE does not need to monitor paging to check for MT calls. However, a RRC connected relay UE may need to do so, if it needs to monitor paging for linked remote UE(s). If the U2N relay does not have a spare RF chain for monitoring paging on behalf of the linked remote UE(s), it would need to switch BWP to e.g. initialDownlinkBWP to be able to receive Paging for remote UE(s). To help the gNB to better schedule the U2N relay UE, it is proposed in [28] that RRC connected U2N relay UE informs its serving gNB about time periods (paging occasions of remote UE(s)) of its intended absence (away time) from the current DL BWP. As another possibility available to the network is configuring and activating a DL BWP that is overlapping with the initial DL BWP.

Question 14: Should RAN2 address the BWP switching issue of relay UE due to paging monitoring on behalf of linked remote UE? Please give comments to support your selection.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | Yes. We think it is a valid issue to resolve only when relay is in CONNECTED. (when relay in IDLE/INACTIVE, it stays in initial BWP). However, we think it needs to first discuss whether a RRC_CONNECTED relay still needs to monitor paging for remote UE. In this case, because RRC connection is available in relay, a better solution is that gNB just includes paging record for remote UE in dedicated RRC message towards relay UE. Then, this issue can be resolved via Network flexible dedicated RRC signalling in relay’s active BWP, without introducing monitoring gap. |
| Xiaomi | No | It’s up to gNB’s implementation to keep relay UE in the appropriate BWP to monitor remote UE’s PO. |
| OPPO | NO | The gNB of a connected relay UE can be aware of the remote UEs connected to the relay, so this issue can be solved by gNB implementation, and no need to pursue UE behaviour. |
| MediaTek | No | Network implementation issue |
| Ericsson | Yes | If the relay UE switch BWP and is not configured with commonSearchSpace or PagingSearchSpace, then we should clarify how SIB delivery and Paging would work. |
| Samsung | No | gNB should handle this issue. |
| Huawei, HiSilicon | No | Could be left to network implementation. |
| Lenovo, MotM | Yes | Agree with QC: RAN2 either needs to resolve this issue directly or go for optimized solutions where the network delivers the Paging dedicatedly for all linked remote UEs of a relay, to the relay. The latter can be quite an elaborate procedure. |
| vivo | No | Rely on Network implementation to solve the issue, E.g., activating a DL BWP that is overlapping with the initial DL BWP for relay UE. |
| Apple | No | Agree with OPPO, Huawei and MediaTek |
| Sharp | No | It could be resolved by gNB implementation. |
| ZTE | See comments | We think it may be hard to leave it to network implementation since the gNB does not know the relay UE shall monitor which remote UE’s paging and the exact PO since gNB does not keep the context of RRC_IDLE remote UE. However, we think this issue can be postponed until more critical issues have been solved. |
| CMCC | No | Left it to network implementation. |
| Nokia | Yes | We should go for dedicated signalling when Relay is connected |
| CATT | No | We don’t any spec impacts is needed. |

## Access control

- [2] [3] [5] propose to reuse legacy access control for Remote UE.‎ It is not needed to enhance the access control procedure of Remote UE. [2] [14] propose that UAC configuration is available to remote UE before it has established PC5 connection with a Relay UE and the access control check is performed at remote UE using the parameters of the cell it intends to access. [27] proposes that remote UE may obtain minimum SI over Uu (in-coverage) and/or the UAC parameters via relay UE (in-coverage or out-of-coverage) in a dedicated or broadcast message to decide UAC. [9] proposes that for access control, the remote UE should take into account the following aspects when performing access check: QoS characteristics (e.g. 5QI/PQI), SL channel conditions, loading over PC5 link (e.g. CBR, CR) and loading at relay UE (e.g. buffer level). [21] proposes that remote UE may report its Uu serving cell information to relay UE via PC5 RRC procedure and it is up to relay UE based on remote UE reporting to decide whether access control information needs to be delivered to remote UE in PC5 RRC procedure.

Question 15: Do you agree that Remote UE can reuse legacy access control and no need to enhance the access control procedure of Remote UE?

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | Yes | In our understanding, UAC configuration is available to remote UE before unicast PC5 connection. Thus, UAC configuration is available to remote UE either via essential SIB info before relay connection or via SIB forwarding after relay connection. Then, we don’t think spec change is required. |
| Xiaomi | Yes | No new AC or AI is introduced for remote UE. There is no need to enhance UAC. |
| OPPO | Yes |  |
| MediaTek | Yes |  |
| Ericsson | Yes |  |
| Samsung | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Lenovo, MotM | Yes |  |
| vivo | Yes |  |
| Apple | Yes |  |
| Sharp | Yes |  |
| ZTE | Yes |  |
| CMCC | Yes |  |
| Nokia | Yes | But we need to discuss further on how UAC will be performed |
| CATT | Yes |  |

## Others

Since both L2 relay architecture and L3 relay architecture would be specified in this WI, [38] analyzes the co-existence issue of L2 relay architecture and L3 relay architecture within a same area. From network deployment perspective, the co-existence scenarios are listed in [38]. In addition, the UE capability of L2 and L3 relay architecture support are also discussed. It is necessary to collect companies’ views on these co-existence scenarios.

Question 16-1: From network deployment perspective, which relay architecture option should be considered in RAN2?

- Option 1: Per area or PLMN, i.e., each cell support only one relay architecture

- Option 2: One relay architecture per cell, i.e., cell supports only one kind of relay architecture L2 or L3

- Option 3: Full relay architecture per cell, i.e., cell supports both L2 and L3 architecture

Option 4: Others. If this option is selected, please give detailed description.

| Companies | Option (s) | Comments |
| --- | --- | --- |
| Qualcomm | None | We think SA2 should work on it first. In our understanding, SA2 is discussing to introduce relay service code in discovery message for UE to implicitly derive the support of L2 or L3 relay. That is enough in this release, we don’t think any further mechanism is required to be specified in this release. |
| Xiaomi | Option 1 | We think option 1 makes more sense. There is no performance difference between these two architectures. One architecture per area or PLMN is enough. |
| OPPO | wait for SA2 | Same view as QC. |
| MediaTek | Option 1 |  |
| Ericsson | Wait for SA2 | This is currently discussed in SA2. We should wait for their conclusion. |
| Samsung | Wait for SA2 | Same view as Qualcomm |
| Huawei, HiSilicon | Wait for SA2 |  |
| Lenovo, MotM | Option 1 |  |
| vivo | Prefer Option 1 | We understand that the discussion happening in SA2 does not touch the network deployment but only the UE capability issue. It is worthwhile further thinking about this network deployment and potential issue from RAN2 perspective, and even inform SA2 about our consideration or preference if necessary. -For Option 1, it is simple from network and UE point of view. Based on PLMN and relay architecture support UE can select and establish relay connection with network. But, Inter-PLMN mobility may be problematic if the PLMNs do not support the same relay architecture. -For Option 2, within the cell the relay architecture support is simple. From SIB information UE can know whether it can select an establish relay connection with the concerned cell. But, for UE to fully support relay selection and establish relay connection with the network, UE has to have capability support for both L2 and L3 relay architecture. - For Option 3, it may be complicated deployment scenario. But it provides some flexibility for UE support and establishment of relay connection with network, as UE do not have to support both L2 and L3 architecture relay. As analyzed above, Option 1 is slightly preferred. But we are also ok to leave the final decision to SA2. |
| Apple | Option 1 | We can design based on Option 1, unless SA2 explicitly supports a co-existence of L2/L3 in the same cell. |
| Sharp | Wait for SA2 | Same view as QC |
| ZTE | Wait for SA2 | Same view as QC |
| CMCC | Option 1 | From our perspective, Option 1 makes sense for network deployment. We are also fine to wait decision of SA2 |
| Nokia | Wait for SA2 |  |
| CATT | Wait for SA2 |  |

Question 16-2: From UE capability perspective, which relay architecture option should be considered in RAN2?

Option 1: UE with only L3 relay architecture support capability

Option 2: UE with only L2 relay architecture support capability

Option 3: UE with both L2 relay architecture support capability

Option 4: Others. If this option is selected, please give detailed description

| Companies | Option(s) for relay UE | Option(s) for remote UE | Comments |
| --- | --- | --- | --- |
| Qualcomm | Wait SA2 | Waits | SA2 work is necessary first. We are even not sure whether both L2 and L3 relay will be deployed in the same geographic area. |
| Xiaomi | All | All | It’s up to UE’s capability. |
| OPPO |  |  | Same view as QC that we should wait for SA2. If discovery message design differentiates layer-2/3 relay, e.g., using RSC, we believe there is no coupling to RAN2 work, and RAN2 can freely decide on option-1/2/3 in UE capability discussion, i.e., business as usual. |
| MediaTek | L2 and/or L3 | L2 and/or L3 | Up to UE capability. |
| Ericsson | Wait for SA2 | Wait for SA2 | Agree with QC. It is too early anyway to talk about capabilities. |
| Samsung | Wait for SA2 | Wait for SA2 | Agree with Qualcomm |
| Huawei, HiSilicon |  |  | We are wondering if rapporteur’s intention is to discuss the impact on relay selection, in case remote UE needs to know if the selected relay supports L2/L3 relay? If so, it is similar as Q16-1. |
| Lenovo, MotM | All | All | It’s up to the capability. This also affects relay (re)selection. |
| Vivo | All | All | SA2 is considering the UE capability issue during discovery procedure and all UE capability options for remote and relay UE are on the table. Therefore, there may be no need to down select on any UE capability option now. |
| Apple | Too early to decide | Too early to decide |  |
| Sharp | all | all |  |
| ZTE | Wait for SA2 | Wait for SA2 |  |
| Nokia | Wait for SA2 | Wait for SA2 | But in principle we should allow the UE to support both options. |
| CATT | Option1,2,3 | Option1,2,3 | It is UE capability. |

# Conclusion

# Reference

[1] R2-2102693	RRC management procedures of L2 U2N relay	Qualcomm Incorporated.

[2] R2-2102695	System information, paging delivery and UAC in L2 U2N relay	Qualcomm Incorporated.

[3] R2-2102700	Control Plane Procedures of L2 Relay	CATT.

[4] R2-2102701	Service Continuity for L2 U2N Relay	CATT.

[5] R2-2102747	Discussion on Control Plane Aspects for L2 Relay	OPPO.

[6] R2-2102779	Connection establishment for L2 UE-to-Network Relay	MediaTek Inc..

[7] R2-2102780	Further details on System Information Delivery	MediaTek Inc..

[8] R2-2102809	Connection Management for L2 UE to NW Relays	InterDigital.

[9] R2-2102810	Control Plane Procedures for L2 UE to NW Relays	InterDigital.

[10] R2-2102891	Left issues on RRC procedure for L2 U2N Relay	OPPO.

[11] R2-2102968	Connection on L2 relay	Xiaomi communications.

[12] R2-2102969	Discussion on resouce allocation for remote UE	Xiaomi communications.

[13] R2-2102974	The connection management of SL relay	ZTE Corporation, Sanechips.

[14] R2-2102975	Discussion on system information paging and access control	ZTE Corporation, Sanechips.

[15] R2-2103087	Connection management in L2 U2N relay	Samsung.

[16] R2-2103088	System information delivery via relay UE	Samsung.

[17] R2-2103203	UE to Network Relay Connection Establishment	Futurewei.

[18] R2-2103231	RRC state transitions in L2 relaying 	Kyocera.

[19] R2-2103310	Support of idle mode mobility for remote-UE in SL UE-to-Nwk relay	Nokia, Nokia Shanghai Bell	.

[20] R2-2103325	RRC Connection Management for L2 relay	vivo.

[21] R2-2103326	Control Plane procedure for L2 SL Relay	vivo.

[22] R2-2103458	Discussion on RRC procedures for U2N Relay	ASUSTeK.

[23] R2-2103482	SIB Handling in Sidelink UE-to-Nwk Relay	Nokia Germany.

[24] R2-2103662	Discussion on control plane procedures for L2 sidelink relay	Ericsson.

[25] R2-2103663	Discussion on service continuity for L2 sidelink relay	Ericsson.

[26] R2-2103718	System information delivery for L2 U2N Relay	CMCC.

[27] R2-2103738	Control plane procedures for L2 U2N relaying	Intel Corporation.

[28] R2-2103742	Monitoring Paging by a U2N Relay	Lenovo, Motorola Mobility.

[29] R2-2103744	SI acquisition, CN Registration and RNAU	Lenovo, Motorola Mobility.

[30] R2-2103857	Discussion on QoS mechanism for Layer 2 UE-to-NW relay	Apple.

[31] R2-2103956	Control plane multi-connectivity for NR Sidelink Relay UE	AT&T.

[32] R2-2103996	L2 relay QoS handling procedure	LG Electronics Inc..

[33] R2-2104126	Service continuity of L2 U2N relay	Qualcomm communications-France.

[34] R2-2104131	Discussion on the CP procedures for L2 Relay	Huawei, HiSilicon.

[35] R2-2104132	Discussion on path switch for L2 UE to NW Relay	Huawei, HiSilicon.

[36] R2-2104245	discussion on Paging and SI delivery for L2 U2N relay	ETRI.

[37] R2-2104503	Summary of 8.7.4.1, ZTE.

[38] R2-2103328 Discussions on L2 and L3 relay co-existence	vivo.