- 3GPP TSG RAN WG2 Meeting #113bis-e      	               R2-210xxxx
E-Conference, 12th – 20th April 2021

Agenda item:	8.7.3

Source:	Qualcomm Incorporated

Title:	Remaining proposals on relay (re)selection

WID/SID:	FS_NR_SL_relay – Release 17

Document for:	Discussion and Decision

# Introduction

This is email discussion for below offline discussion:

- [AT113bis-e][611][Relay] Remaining proposals on relay (re)selection (Qualcomm)

Scope: Discuss the proposals for discussion from the (re)selection summary and converge where possible.

Intended outcome: Report in R2-2104415

Deadline:  Monday 2021-04-19 1000 UTC

Please note that in Tuesday online discussion, the following agreements on relay (re)selection were made:

Proposal 1: For relay (re)selection, RAN2 clarify that only the common parts of L2 and L3 relay is required to be completed by RAN#92. L2 specific design may be discussed in L2 relay agenda items in contribution driven manner.

Proposal 4: RAN2 confirm below NR relay (re)selection procedures which are same as LTE Prose relay:

1) PC5 Measurement: For relay(s) without unicast PC5 sconnection, remote UE uses RSRP measurements of sidelink discovery messages (i.e. SD-RSRP) to evaluate whether PC5 link quality of a Relay UE satisfies relay selection and reselection criterion

2) Trigger of relay selection: Triggered at remote UE when: a) direct Uu link quality is below a configured threshold for an in-coverage remote UE (in IDLE/INACTIVE and CONNECTED for L3 U2N relay; L2 case to be further discussed); or b) triggered by upper layer

3) Trigger of relay reselection: Triggered at remote UE when: a) PC5 measurement towards current relay UE is below a (pre)configured threshold; or b) Reception of an upper layer release message or similar indication from current relay UE; or c) Triggered by upper layer

4) How to choose relay UE in relay (re)selection: Remote UE searches for suitable relay UE candidates which meet all AS-layer & higher layer criteria. If multiple such candidate relay UEs available, it is up to Remote UE implementation to choose one Relay UE.

Proposal 5: Same as LTE, Uu link threshold (like threshHigh-r13), PC5 link threshold(like q-RxLevMin-r13), L3 filter coefficient for SD-RSRP/SL-RSRP (like filterCoefficient-r13) and hysteresis (like hystMax-r13 and minHyst-r13) can be provided via SIB/RRC by gNB or pre-configuration. Handling of Uu link threshold being absent can reuse LTE approach (i.e. when absence, remote UE considers condition to be met).

Proposal 6: In SD-RSRP measurement for relay (re)selection trigger and candidate relay evaluation, L3 filtering is applied across measurements on the DMRS of PSSCH transmission which carries discovery message from the concerned relay.

Proposal 8: RAN2 confirm that remote UE triggers relay reselection if PC5 RLF with current relay UE is detected by remote UE.  FFS if there is any impact to other RLF handling activities.

Proposal 14: Uu quality between relay UE and gNB is not included in discovery message as additional AS criteria for relay (re)selection

Proposal 16: Include the information required for agreed additional AS criteria in discovery message.

# Discussion

## gNB controlled relay (re)selection

The related proposals are:

Proposal 18: Same as LTE, CONNECTED remote UE in L2 U2N relay can also trigger relay selection when directly Uu link quality is below a configured threshold. It doesn’t exclude the option of gNB decision on relay selection.

Proposal 3: For L2/L3 relay common parts of relay (re)selection, RAN2 confirm that there is no support of service continuity from AS layer perspective. gNB controlled path switch for service continuity belongs to L2 relay service continuity agenda item.

Proposal 2: Because gNB decision on relay selection/reselection and QoS controlled relay (re)selection are L2 relay specific design, they are not treated in relay (re)selection discussion by RAN#92

First, as Proposal 18 implied, although majority companies agree to reuse same mechanism of LTE to CONNECTED remote UE L2 relay in NR. two companies showed concern during offline and online discussion. So, rapporteur would like to first get companies inputs. LTE related part in TS 36.331 is copied below:

#### 5.10.11.4	Selection and reselection of sidelink relay UE

A UE capable of sidelink remote UE operation that is configured by upper layers to search for a sidelink relay UE shall:

1>	if out of coverage on the frequency used for sidelink communication, as defined in TS 36.304 [4], clause 11.4; or

1>	if the serving frequency is used for sidelink communication and the RSRP measurement of the cell on which the UE camps (RRC_IDLE)/ the PCell (RRC_CONNECTED) is below threshHigh within remoteUE-Config :

2>	search for candidate sidelink relay UEs, in accordance with TS 36.133 [16]

2>	when evaluating the one or more detected sidelink relay UEs, apply layer 3 filtering as specified in 5.5.3.2 across measurements that concern the same ProSe Relay UE ID and using the filterCoefficient in SystemInformationBlockType19 (in coverage) or the preconfigured filterCoefficient as defined in 9.3(out of coverage), before using the SD-RSRP measurement results;

Q1: Do you agree the below LTE mechanism can be supported for RRC_CONNECTED remote UE in L2 U2N (Note that it doesn’t exclude gNB controlled relay (re)selection even if agreed)?

- [LTE mechanism] If RSRP measurement of PCell is below a configured threshold (i.e. Uu RSRP is below threshHigh by hystMax), the remote UE triggers relay selection

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | No | The legacy LTE mechanism is applicable for RRC_CONNECTED remote UE because the LTE relay mechanism is only work for L3 relay, so that gNB is not capable of being awareness of the existence of remote UE. However, in NR sidelink relay, for L2, gNB can be clearly aware of the existence of remote UE. Thus, the RRC_CONNECTED remote UE should be under gNB’s control. In other word, it means network may indicate the command to the remote UE to select one specific relay UE, means remote UE would switch from direct Uu link to indirect link, where it is more like handover procedure rather than relay UE selection/reselection procedure. Thereafter, it is proposed that for RRC_CONNECTED remote UE selecting relay UE, this issue shall be handled in service continuity topic in the following. |
| Fraunhofer | No | Agree with OPPO. Maybe the threshold based configuration should only be tied to the discovery procedure |
| Ericsson | comments | It is more accurate to say that remote UE is allowed to initiate discovery procedure. The procedure will be like as the following. Remote UE triggers relay selection, when its Uu RSRP is below the configured threshold. Remote UE collects measurements of different relay UE candidates, and sends a measurement report to the gNB, the measurement includes PC5 measurements, optionally also including Uu measurements. gNB can decide which relay UE or which cell the remote UE shall switch to. Meanwhile, the gNB may send a signaling (e.g., like handover command) to a remote UE at any time without receiving a measurement report from the remote UE first. Meanwhile, since the remote UE is in RRC CONNECTED, it is not allowed that the remote UE selects a relay UE or cell by itself without sending a measurement report to the gNB. The remote UE selects a relay UE by itself, is only allowed in case the remote UE has detected or occurred RLF. |
| Lenovo, MotM | No | We agree with Oppo to the extent that for a RRC Connected remote UE, the relay selection (direct to indirect link switch) needs to be under network control. Also, for reselections, the RRC Connected remote UE needs to report the target relay quality to the serving cell and the actual U2N relay reselection is better controlled with network signaling. |
| InterDigital | No | We agree with OPPO that for L2 relay, relay selection should be under network control as long as the remote UE is RRC_CONNECTED. Relay selection without NW involvement may be possible in certain cases (e.g. RLF) in order to support re-establishment with the network. In terms of the network controlled reselection, we see a procedure whereby: - NW sends an indication to relay UE to trigger reselection procedure, and a list of relays to measure - Remote UE provides the measurements of the relays to the NW - NW indicates the selected relay to the remote UE for PC5 connection establishment and/or path switch |
| Qualcomm | Yes | We appreciate companies’ interests on NW controlled relay (re)selection, but no company provides any technique reason why LTE baseline can’t be reused for L2 relay. From our perspective, NW controlled relay (re)selection is an optimization which can be considered on top of LTE baseline. We are not sure why people start from optimization instead of ensuring system workable from beginning of WI. As this proposal clearly indicated, reusing this LTE baseline doesn’t preclude the discussion on NW controlled relay (re)selection in L2 service continuity session. In addition, the approach mentioned by Ericsson and IntetDigatial can’t work in this release, because in NOTE4 of WID objective, it is clearly indicated indirect to indirect path switch is NOT supported: NOTE 4: Work specific to the mobility scenario of “between indirect (via a first Relay UE) and indirect (via a second Relay UE)”, and the group mobility is not supported in this release. Assuming such procedure mentioned by Ericsson and InterDigital can be considered, does it imply that if NW wants to switch from one relay path to another relay path, it has to do double HO like below? - Use HO cmd to switch remote UE to direct path - Then, use HO cmd to switch from gNB to another relay Then, my question is: how do you handle a remote UE in OOC (Note that 1st step can’t work because it is OOC)? Are you sure such double HO can be quicker than LTE baseline? Again, we hope before discussing optimization, companies can use more efforts to ensure the system can work |
| ZTE | No | Agree with Ericsson that before the gNB configure the UE to switch from direct link to indirect link, UE should be triggered to perform sidelink discovery and report the PC5 measurements of neighboring relay UE. During the online SL relay session on Tuesday, it is agreed that RRC_IDLE/RRC_INACTIVE remote UE is able to perform discovery message transmission if and only if Uu RSRP of serving cell is below a configured minimum threshold by hysteresis. We think this principle should also apply to RRC_Connected remote UE. We may further discuss this under the umbrella of relay discovery. With regard to QC’s comments, the triggering of discovery and subsequent PC5 measurement report from remote UE can be used by gNB to configure the path switch from direct link to indirect link, it is within the scope of the WI. I think QC raise a good point for the mobility between indirect and indirect link. According to the WID, work is specific to the mobility scenario of “between indirect (via a first Relay UE) and indirect (via a second Relay UE)”. Does it imply that the autonomous relay reselection based on configured threshold is allowed but the gNB controlled relay reselection based on dedicated signalling is not? My understanding with the note is that both autonomous and gNB controlled relay reselection could be considered. However, it is not necessary to consider the lossless support during the path switch from indirect link to another indirect link. I hope this can be further clarified. |
| MediaTek | No | Agree with InterDigital, except that in first steps, NW should send an indication to “remote UE”. |
| CATT | No | We share the same view as ZTE. |
| Huawei | No | We agree with OPPO/ Fraunhofer/ Lenovo and Interdigital that L2 CONNECTED remote UE should be totally under control of network. In details, For direct to indirect link, indeed it should be path switch following network command but not relay selection. While for direct to direct case, the point is that the remote UE should not just release the RRC connection/data transmission without network awareness. Even though Quacomm explain network can configure a threshold, the network may still not know when the remote UE will leave. In this case, gNB controlled mechanism is necessary, but not UE autonomous relay reselection. |
| vivo | See comments | We share the same view with Qualcomm that the NW control to switch one relay to another is not pursued in SI and we don’t need to discuss about that. In this sense, if we consider L2 architecture, NW controlled relay selection should be limited to path switch from direct link to indirect link, and this can be discussed in L2 specific session, and if that is supported, we can further decide if LTE mechanism is also used on top of that for L2 relay. |
| Samsung | See comment | We somewhat share the view of OPPO that this need to be discussed with service continuity scenarios for L2. |
| Intel | See comment | We understand that it does not exclude gNB-controlled reselection, but if by triggering relay selection, it means that Remote UE autonomously selects Relay UE as in LTE, then, we need to wait to converge with what we decide in Service continuity scenario for RRC_CONNECTED case. Therefore, we can keep FFS for this RRC state. |

Then, we discuss gNB controlled relay (re)selection / path switch. Note that we have online agreed relay (re)selection will not discuss L2 specific design by RAN#92:

Proposal 1: For relay (re)selection, RAN2 clarify that only the common parts of L2 and L3 relay is required to be completed by RAN#92. L2 specific design may be discussed in L2 relay agenda items in contribution driven manner.

In Rapporteur’s understanding, gNB controlled relay (re)selection is same as gNB controlled path switch. They mean the gNB to use dedicated RRC signalling to indicate remote UE to trigger path switch between indirect path and direct path. No matter what is difference between them, it is for service continuity enhancement and should belong to L2 relay specific issues. Rapporteur would like to confirm whether it is same understanding in RAN2

Q2: Do you agree that “For L2/L3 relay common parts of relay (re)selection, RAN2 confirm that there is no support of service continuity from AS layer perspective”?

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Yes | We agree that when discussing relay (re-)selection, it should be isolated with service continuity issue. To make it clearer, so far it is agreed that for remote UE in IDLE/INACTIVE state, both L2/L3 should commonly reuse LTE mechanism. The controversial part is for CONNECTED UE - For L3 CONNECTED remote UE, it can use legacy LTE mechanism as IDLE/INACTIVE. - For L2 CONNECTED remote UE, it should be discussed in service continuity topic rather than relay selection/reselection topic. |
| Fraunhofer | Ok to go with majority |  |
| Ericsson | Yes | Agree with OPPO. |
| Lenovo, MotM | Yes | Agree with Oppo analysis. |
| InterDigital | Yes, with comments | We agree with the statement in general. For the rapporteur understanding, we have the following comments: - NW controlled relay (re)selection may be needed to trigger PC5-RRC connection establishment without the need for immediate path switch - Relay (re)selection following RLF can be performed using LTE mechanism, even though the remote UE is still in RRC_CONNECTED. |
| Qualcomm | Yes | The proposal has clearly mentioned “common parts of L2 and L3”. So, we don’t need to discuss L2 connected UE issue. |
| ZTE | Yes |  |
| MediaTek | Ok to go with majority |  |
| CATT | Yes |  |
| Huawei, HiSilicon | Yes | Agree with OPPO and other companies. We also would like to echo InterDigital, the proposed solutions seem good as well. But maybe only for relay reselection for indirect link to indirect link. For direct to indirect link, path switch is sufficient. |
| vivo | Yes | Agree with OPPO. |
| Samsung | Yes |  |
| Intel | Yes | We agree with OPPO’s views. |

Q3: Do you agree that “gNB controlled relay (re)selection” or “gNB controlled path switch” belong to L2 relay service continuity agenda item, and they are not treated in relay (re)selection discussion by RAN#92?

| Company | Yes / No | Comments |
| --- | --- | --- |
| OPPO | Yes | See comment in Q1 |
| Fraunhofer | Yes |  |
| Ericsson | Yes |  |
| Lenovo, MotM | Yes |  |
| InterDigital | Yes |  |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| MediaTek | Yes |  |
| CATT | Yes |  |
| Huawei, HiSilicon | Yes |  |
| vivo | Yes |  |
| Samsung | Yes |  |
| Intel | Yes |  |

## QoS controlled relay (re)selection

The related proposals are:

Proposal 2: Because gNB decision on relay selection/reselection and QoS controlled relay (re)selection are L2 relay specific design, they are not treated in relay (re)selection discussion by RAN#92

The related contributions are:

| Tdoc# | Source | Related proposals |
| --- | --- | --- |
| R2-2103009 | Fraunhofer | Consider the QoS fulfilment based (re-)selection triggers: P5/P6. |
| R2-2103311 | Nokia | A relay-UE may perform the discovery procedure, only if the QoS requirements of the relay service can be fulfilled, based on the information obtained from gNB: P8 |
| R2-2103739 | Intel | Discuss whether to support the release of connected remote UE by the relay UE if its QoS requirements cannot be met: P4 |
| R2-2103995 | LG | Remote UE may have different criteria for selecting a Relay UE. These criteria are based on the QoS of the service to be transmitted/received: P3. |

Rapporteur’s analysis:

- For solution proposed in R2-2103311 and R2-2103739, it requires info from gNB on whether remote UE QoS can be satisfied, which seems to only work under L2 relay.

- For solution proposed in R2-2103009 and R2-2103995, it is not clear how relay UE can know the QoS status of remote UE. If it is based on info from core network, it seems to be out of scoping from RAN2.

Considering WID has an objective on QoS of L2 relay, Rapporteur also suggest RAN2 don’t discuss below proposals on QoS controlled (re)selection in relay (re)selection session by RAN#92. For R2-2103009 and R2-2103995, please provide a concrete solution for discussion if you disagree.

Q4: Do you agree that QoS controlled relay (re)selection is L2 relay specific design, and it is not treated in relay (re)selection discussion by RAN#92?

| Company | Yes / No | Comments (If “No”, please provide a concrete solution for discussion) |
| --- | --- | --- |
| OPPO | No | According to the objective listed in the WID: Specify mechanisms for U2N relay discovery and (re)selection for L3 and L2 relaying [RAN2, RAN4] Re-use LTE relay discovery and (re)selection as baseline Specify mechanisms for Relay and Remote UE authorization for L3 and L2 relaying [RAN3] Re-use LTE as baseline it is requested that for relay selection/reselection, the LTE mechanism shall be used as baseline. However, if to consider QoS in relay selection/reselection, it is more like that some companies want a brand-new design without any connection to legacy LTE principle. |
| Fraunhofer | Maybe, but okay to go with majority | Firstly, we agree with the rapp analysis on L2 relaying. However, we would like to clarify our intention on the applicability of this to L3. In the case of L3 U2N relays, our understanding is that when the remote UE initiates communication with the relay UE, it negotiates E2E QoS requirements i.e., across both PC5 and Uu links and maybe performs relay selection based on which relay can satisfy this requirement. In addition, the relay UE has the knowledge of the overall QoS requirements i.e., across both PC5 and Uu links. Subsequently during the U2N relaying, we believe there must be a mechanism for the relay UE to make sure it is able to hold up its end of the bargain i.e., relay UE able to support the remote UE E2E QoS requirement independent of the remote UE QoS status because in contrast to the remote UE, the relay UE QoS status is affected by additional factors like load (as being discussed). To this end, we also agree with the rapp that higher layer interaction is required. On the other hand, we believe a mechanism should be specified in RAN2 to deal with this problem and think triggering reselection (e.g., based on P4-3-b) would be a reasonable. |
| Ericsson | Yes | Agree with Rapporteur |
| Lenovo, MotM | Yes (= not required to be treated) | We think that QoS based triggers for reselection should also manifest in radio based reselections – so no reason to treat it separately. |
| InterDigital | Yes | Agree with the rapporteur |
| Qualcomm | Yes, but | After reading Fraunhofer’s comment, maybe one clarification could be: “If relay UE identifies the QoS of remote UE can’t be satisfied, it may send the upper layer indication/message to remote UE triggering relay (re)selection”? Note that “may” means it is up to relay UE implementation. Not sure whether it can resolve Fraunhofer’s concern |
| ZTE | Yes | We only need to consider the AS criteria, such as cell, PLMN, load and PC5 link quality. It is not necessary to consider QoS for relay (re)selection. |
| MediaTek | Yes | Agree with rapporteur |
| CATT | Yes | We share the same view as rapporteur. |
| Huawei, HiSilicon | See comments | We are not sure if this is only for L2 relay or common for both relay architectures. But anyway it could be treated as a relay (re)selection criteria. And we do not see the need to consider QoS other than the other criteria. |
| vivo | Yes | We are fine to leave it to L2 relay specific discussion. If during the discussion, companies are thinking the QoS factors need to be considered at the beginning of relay (re)selection then we can add that as criterion for relay (re)selection. |
| Samsung | Yes | Agree with Rapporteur |
| Intel | Yes | The intention from our perspective was to consider QoS support as an AS layer criterion, but since we have converged to primarily support Relay load, we are fine to exclude it from this discussion. |

## RLF handling during relay reselection

The related proposals are:

Proposal 9: When Uu RLF is detected by relay UE, relay UE sends the indication/message, e.g., in Proposal 4-3-b to its connected remote UE(s) to trigger relay reselection.

Proposal 10: When relay performs HO to another gNB, relay UE sends the indication/message, e.g., in Proposal 4-3-b to its connected remote UE(s) to trigger relay reselection.

Proposal 11: When PC5 RLF is detected by relay UE on a PC5 unicast link towards a remote UE, relay UE sends the PC5 RLF report including available PC5 measurements of the PC5 unicast link to gNB.

First, for Proposal 9 on Uu RLF detected by relay, it seems it is a majority view:

|  | Tdoc# | Source | Summary of their proposals |
| --- | --- | --- | --- |
| Option 1: reselection triggered upon Uu RLF detected | R2-2102692 | Qualcomm Inc. | Relay UE sends relay link release indication to all its connected remote UE(s) to trigger relay reselection. |
|  | R2-2102699 | CATT | Reselection triggered if Uu BH RLF is indicated to remote UE |
|  | R2-2102807 | InterDigital | Similar to QC/CATT |
|  | R2-2103994 | LG | Similar to QC/CATT |
| Option 2: reselection triggered upon Uu recovery failure | R2-2103390 | Lenovo | The remote UE should suspend the SL data transmission towards the relay UE once it receives the failure notification from relay UE. The remote UE should perform relay reselection once the it receives the recovery failure notification from relay UE. |

Rapporteur think it can be agreed. In addition, whether to trigger RRC re-establishment followed by it is L2 relay specific issues. This offline will not discuss it.

Q5: Do you agree that for both L2 and L3 relay, “When Uu RLF is detected by relay UE, relay UE sends the indication/message, e.g., in Proposal 4-3-b to its connected remote UE(s) to trigger relay reselection”?

| Company | Yes / No | Comments (if you think only L2 or L3 relay is applied, please indicate) |
| --- | --- | --- |
| OPPO | see comment | It is acceptable that relay UE sends the indication towards remote UE when it detects Uu RLF. However, we do not think - The indication has to be a PC5 link release message, or - Remote UE should perform relay selection/reselection immediately after receiving this indication, Especially considering the case that remote UE has both relay unicast link and non-relay unicast link with this specific relay UE. For the non-relay link, there should be no impact even if relay UE’s Uu RLF is detected and sidelink transmission should be able to continue. Besides, for L2 relay, this question is more related to CP procedure. Thus, it is proposed to agree PC5 link release can trigger relay reselection. The trigger of PC5 link release can be discussed in the following after #92 meeting. |
| Fraunhofer | Yes | Relay UE can also send an indication/message based on an early RLF detection mechanism |
| Ericsson | Yes with comments | As a high level proposal, it is fine. In other words, the details of indication/message need further study. It may be RRC signaling or control PDU. |
| Lenovo, MotM | Yes, but | Once the relay UE declares Uu RLF, the relay UE will send the indication/message, which just notify the remote UE. After the remote UE receives the notification, the remote UE may not release the source PC5 link immediately if no suitable relay/cell is discovered. In this case, once recovery succeeds, the relay UE needs to notify the connected remote UE. Therefore, both option1 and option2 can be supported. |
| InterDigital | Yes | We agree that the indication/message is sent upon Uu RLF, and reception of the indication/message should trigger reselection at the remote UE. As mentioned by the other companies, the reception of the message should trigger (re)selection, and whether there is a link release or not should be further discussed. |
| Qualcomm | Yes | Based on companies’ input so far, maybe we can change the proposal to: When Uu RLF is detected by relay UE, relay UE may send the indication/message, e.g., in Proposal 4-3-b to its connected remote UE(s) to trigger relay reselection. FFS other indication/message? |
| ZTE | Yes | We agree that relay UE need to send the indication/message to remote UE. However, the detailed format of the indication/message need to be further discussed. It can be an Uu RLF indication instead of upper layer release message. |
| MediaTek | Yes |  |
| CATT | Yes |  |
| Huawei, HiSilicon | Yes with comments | We are generally fine with the proposal. We understand the intention of the indication is let the remote UE know the current indirect link may be obstructed, and a straightforward UE behavior is to reestablish via another UE/cell. |
| vivo | Yes with comments | We are fine for the relay UE to send indication to remote UE upon Uu RLF, and the indication can be both in upper layer or in PC5-RRC. That can be discussed later. However, we echo OPPO’s comments that it doesn’t have to trigger relay (re)selection which can be left to remote UE implementation. We also prefer Qualcomm’s suggestion to add ‘may’ in the proposal. |
| Samsung | Yes with comment | We share the view that remote UE’s behavior should be further discussed when it receives the indication. |
| Intel | Yes | A generic indication which also supports other trigger options could be considered. |

Then, for Proposal 10 on HO performed by relay, Rapporteur’s understanding is that it is also applied to both L2 and L3 relay:

- For L2 relay, it has captured in WID objective that group mobility is not supported in this release. Thus, if relay performs HO, it seems service continuity can’t be ensured anymore.

- For L3 relay, remote UE traffic is carried via relay UE’s own PDU session. Thus, if relay performs HO, it is questioned why remote UE needs to keep the same relay.

Q6: Do you agree that for both L2 and L3 relay, “When relay performs HO to another gNB, relay UE sends the indication/message, e.g., in Proposal 4-3-b to its connected remote UE(s) to trigger relay reselection”?

| Company | Yes / No | Comments (if you think only L2 or L3 relay is applied, please indicate) |
| --- | --- | --- |
| OPPO | See comment | It is acceptable that relay UE sends the indication towards remote UE when it performs HO to another gNB. However, we do not think - The indication has to be a PC5 link release message, or - Remote UE should perform relay selection/reselection immediately after receiving this indication Especially considering the case that remote UE has both relay unicast link and non-relay unicast link with this specific relay UE. For the non-relay link, there should be no impact even if relay UE’s Uu RLF is detected and sidelink transmission should be able to continue. Besides, for L2 relay, this question is more related to the scope of service continuity (for which during SI, we conclude the need for release message is FFS). Thus, it is proposed to agree PC5 link release can trigger relay reselection. The trigger of PC5 link release can be discussed in the following after #92 meeting. |
| Fraunhofer | Yes | Group mobility is not supported |
| Ericsson | Yes | Same comments for Q5. |
| Lenovo, MotM | Yes | Assuming the proposal here is that the relay UE informs remote UE as soon as the HO CMD is received (T304 is started). |
| InterDigital | Yes | The remote UE can treat this as similar to the indication in case of Uu RLF. |
| Qualcomm | Yes | Similar to Q5, maybe we can change it to: When relay performs HO to another gNB, relay UE may send the indication/message, e.g., in Proposal 4-3-b to its connected remote UE(s) to trigger relay reselection. FFS other indication/message? |
| ZTE | Yes | Since group mobility is not supported in this release, the remote UE can not perform HO together with the connected relay UE. Instead, it can only perform cell/relay (re)selection to find a new cell/relay UE to connect. |
| MediaTek | Yes |  |
| CATT | Yes |  |
| Huawei, HiSilicon | Yes with comments | In general, we think this procedure is more about L3 relay. For L2 relay, the network could handle the remote UE appropriately before HO the relay UE. But in case the network does not move the remote UE to somewhere else before relay UE HO, the proposed procedure could apply. |
| vivo | Yes with comments (FFS for L3) | The case is similar to Q5 and please refer to our comments there. For L2 relay, this indication can be helpful to some extent to allow remote UE to trigger relay reselection. However, for L3 we are wondering why this indication is critical to remote UE because as the remote UE is not visible by the gNB and there is no E2E connection, why should the remote UE care about the relay UE’s change of gNB? And in LTE there is no such indication upon relay UE handover. Moreover, if this proposal is agreed, we think the indication for Q5 and Q6 can be a single indication to simplify the design. |
| Samsung | Yes | Similar to Q5 |
| Intel | Yes | Same comment as above. |

Finally, for Proposal 11 on PC5 RLF detected by relay, Rapporteur’s understanding is that it is legacy behavior for relay to report PC5 RLF indication to gNB upon detection of PC5 RLF according to TS 38.331:

#### 5.8.9.3	Sidelink radio link failure related actions

The UE shall:

1>	upon indication from sidelink RLC entity that the maximum number of retransmissions for a specific destination has been reached; or

1>	upon T400 expiry; or

1>	upon indication from sidelink MAC entity that the maximum number of consecutive HARQ DTX for a specific destination has been reached; or

1>	upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3:

2>	consider sidelink radio link failure to be detected for this destination;

2>	release the DRBs of this destination, in according to sub-clause 5.8.9.1a.1;

2>	release the SRBs of this destination, in according to sub-clause 5.8.9.1a.3;

2>	discard the NR sidelink communication related configuration of this destination;

2>	reset the sidelink specific MAC of this destination;

2>	consider the PC5-RRC connection is released for the destination;

2>	indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

2>	if UE is in RRC_CONNECTED:

3>	perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3 or sub-clause 5.10.X in TS 36.331 [10];

NOTE:	It is up to UE implementation on whether and how to indicate to upper layers to maintain the keep-alive procedure [55].

So, it seems the only delta part is whether to also include available PC5 measurements of the PC5 unicast link to gNB. Rapporteur suggests to first confirm PC5 RLF indication can be sent as Rel-16 legacy.

Q7: Do you agree that for both L2 and L3 relay, “When PC5 RLF is detected by relay UE on a PC5 unicast link towards a remote UE, relay UE in RRC_CONNECTED sends the PC5 RLF to gNB, as NR Rel-16 V2X”?

| Company | Yes / No | Comments (if you think only L2 or L3 relay is applied, please indicate) |
| --- | --- | --- |
| OPPO | Yes |  |
| Fraunhofer | Yes |  |
| Ericsson | Yes |  |
| Lenovo, MotM | Yes, but | For L2 relays, we assume for a variety of reasons (including DL scheduling of the relay itself), a RRC Connected U2N relay UE needs to keep its serving gNB informed about the remote UEs it is currently serving. In addition, the relay UE will report to gNB when receiving the sidelink RRC reconfiguration failure message from the remote UE in legacy. We also can confirm this same as PC5 RLF. |
| InterDigital | Yes |  |
| Qualcomm | Yes |  |
| ZTE | Yes |  |
| MediaTek | Yes |  |
| CATT | Yes |  |
| Huawei, HiSilicon | Yes |  |
| vivo | Yes |  |
| Samsung | Yes |  |
| Intel | Yes |  |

Then, on whether to also include available PC5 measurements of the PC5 unicast link to gNB, one company thought that it may not be useful for L3 relay. For L2 relay, Rapporteur understanding is that it is up to gNB implementation how to handle it (e.g. release remote UE context or suspend transmission with the remote UE).

Q8: If you agree Q7, do you also agree to include available PC5 measurements of the PC5 unicast link in PC5 RLF report to gNB, and it is up to gNB implementation how to handle it?

| Company | Yes / No for L2 relay? | Yes / No for L3 relay? | Comments |
| --- | --- | --- | --- |
| OPPO | No | No | PC5 RLF can be triggered by multiple case rather than only SL RSRP cannot fulfill the requirement. If the PC5 RLF happened due to other reason, e.g. RLC mismatch issue, security issue and etc, then even the RSRP measurement is reported and gNB may still consider the relay UE is available for reselection by referring to the reported RSRP measurement, but in fact the unicast link still cannot work.. Thus, to simplify this issue, only RLF shall be reported towards gNB. |
| Fraunhofer | Yes | No |  |
| Ericsson | Yes | No | PC5 measurement results may be useful, to help gNB understand the failure reason, and takes proper recovery action. However, whether or not to include PC5 measurements shall be optional. |
| Lenovo, MotM | No | No |  |
| InterDigital | No | No | Reselection following RLF needs to be performed by the UE on its own because the gNB cannot communicate with the remote UE at that point. Measurements of PC5 sent to the network may not be useful. |
| Qualcomm | Yes | No | Similar view as Ericsson, the PC5 measurement is useful for gNB to know the situation. Depending on PC5 measurements, gNB can directly release remote UE’s context or suspend the transmission (i.e. gNB may regard the PC5 link can be recovered) In addition, it is “available” PC5 measurement. So it is optional and no extra measurement efforts from relay UE side. |
| ZTE | No | No | We think the relay UE may send the PC5 RSRP to gNB before the PC5 RLF happens based on which the gNB may configure the remote UE to switch to Uu or other relay UE. However, if the PC5 RLF already happens, it is not necessary to send the PC5 RSRP together with the PC5 RLF report to gNB. |
| MediaTek | No | No | Agree with InterDigital |
| CATT | No | No | As the PC5 RLF failure reason is not agreed to report to gNB, we wonder why we need to discuss this “available” PC5 measurements of the PC5 unicast link. Compare to this “available” PC5 measurements, we think the PC5 RLF failure reason is more useful. |
| Huawei | Possible | Possible | But we feel this is more like an enhancement of RLF report, not sure why discuss/conclude here. |
| vivo | No | No | We don’t see much benefits to report the PC5 measurement to gNB as it may not be helpful because of variable PC5 failure cases, but just cause signaling overhead. And in R16 when PC5 RLF is detected for a CONNECTED UE, we just indicate it to gNB in a simple one bit indication with destination ID, without PC5 measurement report, which we think can be reused here. |
| Samsung | No | No | In case of PC5 RLF we don’t think that PC5 measurement report is useful for gNB. |
| Intel | No | No | It is not fully understood what the gNB could do with a report on localized link issues and may cause additional spec. impact. |

## Coupling with cell (re)selection

The related proposal is:

Proposal 17: When relay (re)selection is triggered, the remote UE may perform cell (re)selection and relay (re)selection procedure independently. When both a suitable cell and a suitable relay are available, the remote UE can select either one based on its implementation in this release, i.e. TS 38.304 will not specify this procedure.

And companies’ views in contributions are summarized below:

| Tdoc# | Source | Summary of their proposals |
| --- | --- | --- |
| R2-2103001 | Ericsson | In case of relay UE reselection, remote UE may select either a target Uu link or a target relay UE to re-establish the link according to radio signal strength. |
| R2-2103007 | OPPO | For L2 UE-to-Network Relay, cell selection procedure needs to allow the remote UE to camp on relay UE. For L2 UE-to-Network Relay, cell re-selection procedure needs to allow the UE to camp on relay UE. |
| R2-2104130 | Huawei | P4: Before connecting to a Relay UE, Remote UE can perform cell selection/reselection and relay selection independently. P5: After the Remote UE connects to a Relay UE, it performs Relay UE reselection. And if no suitable Relay UE to be selected, the Remote UE can perform cell selection. P6: The legacy cell (re)selection procedure and relay (re)selection procedure could go independently and no more combined procedure is needed. |

Because cell (re)selection is not in scoping of WID, Rapporteur don’t think RAN2 can make progress for a combined procedure of cell (re)selection and relay (re)selection. In LTE Prose relay, cell (re)selection and relay (re)selection seem to be two independent procedures. And when both a suitable cell and a suitable relay are available, it seems remote UE can select either based on its implementation, similar to LTE Prose relay. Otherwise, we may have cross-WG impact, e.g. RAN4/CT1/SA2.

Q9: Do you agree that when relay (re)selection is triggered, the remote UE may perform cell (re)selection and relay (re)selection procedure independently?

| Company | Yes / No | Comments (if you think it need specification, please provide your solution and indicate whether it has cross-WG impact, e.g. RAN4/CT1/SA2) |
| --- | --- | --- |
| OPPO | Partially Yes | The key point or the possible spec impact is - for cell selection: now the spec only allows direct camping via Uu, but with L2 relay, indirect camping via PC5 is also allowed; - For cell reselection, now the spec requires the UE to always camp on Uu directly if in-coverage, but with L2 relay, it would be an either-or selection for UE to do either direct camping or indirect camping, it is necessary at least to avoid the case that remote UE concurrently select one suitable cell and one suitable relay UE On the other hand, this issue is not purely related to L2/3 common relay (re-)selection, instead, it is more related to remote UE performing cell (re-)selection of L2 relay. Thus, it is proposed to postpone this discussion after RAN #92 meeting. |
| Fraunhofer | Yes | Could have dependency on SA2. One procedure (either cell (re)selection or relay (re)selection) maybe prioritized over the other based on a policy for priority or the need to have a fallback mechanism |
| Ericsson | Yes | Remote UE shall be able to select either Uu or relay UE. |
| Lenovo, MotM | Yes but | There are dependencies e.g. relay (re)Selection may depend on the serving cell of a candidate relay UE. |
| InterDigital | Yes | To keep this simple, we can operate these two procedures independently. |
| Qualcomm | Yes | @OPPO, your mentioned enhancement needs CT1/SA2 involvement (at least definition of “suitable” relay for camping) which is also mentioned by other companies’ comment. and RAN4 involvement is required if you want to avoid the case that remote UE concurrently select one suitable cell and one suitable relay UE. Do you remind sending LS to them for check impact? @Lenovo, the discussion on including serving cell ID in discovery is covered in offline#610 |
| ZTE | See comments | We agree that the remote UE may perform cell (re)selection and relay (re)selection independently. However, there is no precedence relationship between the cell (re)selection and relay (re)selection. It is suggested to remove the former part of the sentence and change the proposal as follows: “When relay (re)selection is triggered, tThe remote UE may perform cell (re)selection and relay (re)selection procedure independently” |
| MediaTek | Yes |  |
| CATT | Yes | The legacy cell (re)selection procedure and relay (re)selection procedure should go independently and no combined procedure is needed in Rel-17. |
| Huawei, HiSilicon | Yes | We understand OPPO’s point, but it seems UE implementation could address most cases. For how to capture in TS, we could discuss in a later stage. For the wording suggestion from ZTE, our intention of the original proposal is to say if the remote UE already connected to a relay, the cell selection may/may not be performed by the remote UE for power saving purpose. But we are also fine to make it a more general proposal. |
| vivo | Yes | We are ok to treat these two procedures independently for now, and when both a relay UE and a cell is suitable to camp, it can be left to UE implementation. Otherwise, it would need specification design to differentiate cases and related UE behavior which we think is too complicated. |
| Samsung | Yes |  |
| Intel | Yes | We can agree to handle them independently as it will reduce spec. impact. |

Q10: If you agree Q9, do you agree that if both a suitable cell (i.e. satisfying suitable criteria defined in 38.304) and a suitable relay (i.e. satisfying all AS and upper layer criteria of relay) are available, the remote UE can select either one based on its implementation in this release (i.e. TS 38.304 will not specify this procedure)?

| Company | Yes / No | Comments (if you think it need specification, please provide your solution and indicate whether it has cross-WG impact, e.g. RAN4/CT1/SA2) |
| --- | --- | --- |
| OPPO | Yes | See the comment in Q9 |
| Fraunhofer | Yes, that 38.304 need not specify this procedure. But… | Could have SA2 dependency. In addition to AS conditions, policy and charging could determine which link i.e., suitable cell or suitable relay is to be used |
| Ericsson | Yes | It is sufficient to leave to UE implementation. |
| Lenovo, MotM | No | If the candidate relay belonging to the same serving cell and one neighbor cell are suitable during relay reselection, it is better to select the candidate relay in priority, which could be helpful for service continuity. Specifically, data forwarding is not needed if the candidate relay belonging to the same cell can be selected. Maybe not 38.304 captures the relay (re)selection criteria in this regard but rather 38.331 needs a remote UE to ensure that the serving cell (of a potential relay) is indeed the one the UE should camp on/ be connected to – in regard to the RAN sharing, SIBs/ features supported etc. |
| InterDigital | No | The network should be able to control, for UEs in coverage, whether they connect to relay or directly via Uu (e.g. to balance the load of relays and/or avoid significant SL congestion). |
| Qualcomm | Yes at least for L3 relay | For L3 relay, it seems to be the only way For L2 relay, it seems companies still have concern. Let us see. Again, we may need to send LS to SA2/CT1/RAN4 for checking if combined procedure can work. |
| ZTE | Yes | For L3 remote UE and L2 RRC_IDLE/INACTIVE remote UE, it is suggested to follow the design in LTE and leave it to UE implementation. For L2 RRC_Connected remote UE, it can be up to network configuration. |
| MediaTek | Yes |  |
| CATT | Yes | We share the same view as ZTE. |
| Huawei, HiSilicon | Yes | From our point of view, no need to optimize in this release. |
| vivo | Yes | See Q9. |
| Samsung | Yes |  |
| Intel | Yes | Agree with ZTE’s view. |

# Conclusion

TBD

# References

[1] RP-210904, New WID on NR Sidelink Relay.

[2] RAN2#113-e, Chair Notes.

[3] TR 38.836, v-1.0.0, Study on NR sidelink relay.

[4] R2-2102692, Discussion on relay (re)selection, Qualcomm Incorporated

[5] R2-2102699	, Sidelink Relay (Re)Selection	CATT

[6] R2-2102807, Relay selection and reselection, InterDigital

[7] R2-2102960, Further considerations on relay (re)selection, ETRI

[8] R2-2102977, Discussion on Relay selection in Sidelink Relay, ZTE Corporation, Sanechips

[9] R2-2103001	Aspects for SL relay selection and reselection, Ericsson

[10] R2-2103007 Discussion on NR sidelink relay (re-)selection,	OPPO

[11] R2-2103009 NR Sidelink Relay (Re-)Selection, Fraunhofer IIS, Fraunhofer HHI

[12] R2-2103086 SL relay selection and reselection triggering criteria, Samsung

[13] R2-2103237 Discussion on relay selection and reselection, Spreadtrum Communications

[14] R2-2103311 UE-to-Nwk Relay Discovery and (Re)selection for Path Switching in SL Relay, Nokia, Nokia Shanghai Bell

[15] R2-2103324 Discussions on Relay (re-)selection procedure,	vivo

[16] R2-2103390 Relay (re)selection for L2 and L3 U2N case, Lenovo, Motorola Mobility

[17] R2-2103422 Sidelink Relay Reselection and Selection, proposal for outline procedure, Beijing Xiaomi Mobile Software

[18] R2-2103423 NR sidelink relay (re)selection	, MediaTek Inc.

[19] R2-2103584 Relay (re)selection, Sony Europe B.V.

[20] R2-2103667 Discussion on relay selection and reselection, Nokia, Nokia Shanghai Bell

[21] R2-2103717 Consideration on Relay selection and reselection, CMCC

[22] R2-2103739 Discussion on SL Relay (re)selection, Intel Corporation

[23] R2-2103884 Discussion on sidelink relay (re)selection, Apple

[24] R2-2103994 Relay (re-)selection and path switching, LG Electronics Inc.

[25] R2-2104130 Discussion on relay selection and reselection, Huawei, HiSilicon

[26] R2-2104262 Relay UE load as an additional AS criterion for relay (re-)selection, Philips International B.V.

[27] R2-2103993 Relay UE selection criterion using SL-unicast and discovery message, LG Electronics Inc.

[28] R2-2103995 Discovery message contents and relay selection criteria, LG Electronics Inc..