3GPP TSG-RAN WG2 Meeting #113bis electronic                                         R2-2104473

Online, April 12 – April 20, 2021

Title: 	[AT113bis-e][707][V2X/SL] Uu DRX impact to support SL

Source: 	CATT

Agenda item:	8.15.2

Document for:	Discussion and Decision

# Introduction

This paper handles the following email discussion:

[AT113bis-e][707][V2X/SL] Uu DRX impact to support SL (CATT)

Scope: Summarize and discuss Uu DRX impact to support SL based on the companies’ contributions (2.2 in R2-2102690, 2.4 in R2-2102771 and similar ones in others) including any need of separate timer to define additional Uu active time for SL mode 1 operation, etc.

Intended outcome: Discussion summary in R2-2104473.

Deadline: 4/19, 10:00am (UTC), R2-2104473 should be available before next Monday session on SL enhancement

This email discussion is divided in two phases:

- Phase I with the deadline on Thursday April 15 22:00 UTC for companies to provide their views on the questions.

- Phase II with the deadline on Monday April 19 10:00 UTC for rapporteur to finalize summary report based on input of Phase I.

# Discussion

In Rel-17, it is possible that SL DRX and SL resource allocation mode 1 are simultaneously configured for one UE. And in RAN2#113 meeting, it was agreed that:

Agreements on high-level principles for SL DRX

9: In mode 1, when in RRC_CONNECTED, if DRX is configured, the MAC entity monitors the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI in Uu DRX Active Time. MAC entity does not need to monitor the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI in Uu DRX in-active Time.

The above agreement means if UE is configured with SL resource allocation mode 1 and there is SL data transmission requirement, the Uu should be kept in active time. Hence, it is obvious that SL DRX will impact the Uu DRX. In this email discussion, we will discuss the detailed SL DRX impacts on Uu DRX.

## Impact on the drx-onDurationTimer and drx-InactivityTimer in Uu

In [2], it mentioned that “when both DRX and NR SL mode 1 are configured, NG-RAN will properly configure drx-onDurationTimer and drx-InactivityTimer e.g. considering SL traffic patterns to be reported and DL/UL traffic patterns”. According to the rapporteur’ understanding, we should first make clear whether it is necessary to introduce SL-specific drx-onDurationTimer and drx-InactivityTimer in Uu?

Question 1-1: Whether to introduce SL-specific drx-onDurationTimer in Uu? Please give your comments.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | No | In mode 1, UE would monitor PDCCH to obtain scheduled SL resource. The required UE behaviour is the same as in the legacy onduration timer. We prefer to reuse the existing onduration timer. |
| Ericsson | No | Agree with Xiaomi, it is not well motivated to have SL specific on-duration timer. if there is SL specific Uu DRX on-duration timer, which needs to be aligned with UL/DL DRX on-duration timer, in this case, there is no point to have specific timer in Uu DRX for SL Mode 1. gNB can configure a proper timer value for the common on-duration timer (i.e., common for both Uu UL/DL and SL mode 1), to accommodate PDCCH monitoring for both Uu UL/DL and SL mode 1. |
| LG | No |  |
| OPPO | No | The Uu drx-onDurationTimer can be directly used for SL traffic, nothing is different between SL and DL/UL grant reception regarding this timer. |
| Lenovo | No | There is no need to have some SL specific drx-On-DurationTimer in addition to to the drx-OndurationTimer. The configuration of the drx-onDurationTimer should also consider SL transmission activity. |
| CATT | No | Considering it had better align the Uu and SL DRX, there is no need to introduce SL-specific drx-onDurationTimer in Uu from the perspective of power saving. |
| InterDigital | No | The guaranteed wakeup time should be the same for SL scheduling and Uu scheduling. There is no need to separate these. |
| Spreadtrum | No |  |
| Huawei, HiSilicon | No | There is no need to introduce SL-specific onduration timer in Uu. |

Question 1-2: Whether to introduce SL-specific drx-InactivityTimer in Uu? Please give your comments.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | No | Currently, the inactivity timer would only be started by PDCCH indicating new UL or DL transmission. We could extend the triggering condition to include PDCCH indicating new SL transmission. |
| Ericsson | No | Agree with Xiaomi |
| LG | No |  |
| OPPO | No | The same reason as the reply to Q1-1. |
| Lenovo | No | There is in our understanding no need to have a SL-specific drx-InactivityTimer. However we need to introduce new SL related trigger conditions for drx-Inactivity (re)starting. |
| CATT | No | Drx-InactivityTimer is used for extending the Uu DRX active time, there is no need to identify Uu and SL. |
| InterDigital | Yes | Uu active time in DRX is controlled by an inactivity timer. For a SL UE, mode 1 scheduling is triggered by the UE sending SR/BSR when data arrives in the SL transmission buffers. Because the network schedules the UE in mode 1 based on the priority of the SL data, the amount of additional time the UE should remain active on PDCCH following each SL scheduling grant will depend on this priority. The timeline of such data arrival is therefore different than normal Uu traffic, and a different inactivity timer should be defined for the SL UE. |
| Spreadtrum | No |  |
| Huawei, HiSilicon | No | There is no need to introduce SL-specific inactivity timer in Uu. As in LTE DRX, the UE starts or restarts the drx-InactivityTimer if the UE receives a PDCCH indicating a new SL transmission. |

Regarding to the drx-onDurationTimer, it is no doubt that its maintainence should follow legacy Uu. Regarding to the drx-InactivityTimer, no matter which answer is chosen for Question 1-2, new drx-InactivityTimer maintenance mechanism should be considered if taking SL scheduling into account. In [1] and [5], it is proposed that the Tx UE should start or restart the Uu drx-InactivityTimer if the UE receives a PDCCH indicating a new SL transmission.

Question 1-3: For sidelink resource allocation mode 1, regarding to the drx-InactivityTimer maintenance, which options do you prefer? Please give your comments.

- Option 1: Tx UE should start or restart the Uu drx-InactivityTimer if the UE receives a PDCCH indicating a new SL transmission;

- Option 2: others (if companies have other options, it can be added here).

| Companies | Option | Comments |
| --- | --- | --- |
| Xiaomi | Option 1 |  |
| Ericsson | Option 1 |  |
| LG | Option 1 |  |
| OPPO | Option 1 | There is no difference between a PDCCH indicating a new SL transmission and DL/UL transmission, therefore the Tx UE should start or restart the Uu drx-InactivityTimer if the UE receives a PDCCH indicating a new SL transmission. |
| Lenovo | Option 1 With Comments | In addition to the option 1, RAN2 should also consider to ensure that Tx UE is in ActiveTime for the Uu interface in response to having sent an ACK on PUCCH for a SL data transmission in order to be ready to receive a further SL DCI for a new transmissions. This could be implemented by starting the drx-InactivityTimer. It should be noted that different to Uu, for SL gNB is not really aware of the current BS since UE selects TBS autonomoulsy (in Uu UL the TBS is indicated by DCI). Therefore gNB has for SL less information on UE’s status (buffer status). |
| CATT | Option 1 | Similar as Uu drx-inactivityTimer maintenance for UL/DL can be used. |
| InterDigital | Option 1 | The TX UE restarts the SL inactivity timer if the UE receives SL scheduling, and it restarts the Uu inactivity timer if it receives normal Uu scheduling. |
| Spreadtrum | Option 1 |  |
| Huawei, HiSilicon | Option 1 |  |

## Impact on the drx-HARQ-RTT-Timer and drx-RetransmissionTimer in Uu

According to TS38.321, two drx-HARQ-RTT-Timers are defined as below, one is for UL and another is for DL.

- drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity; - drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

In there is SL PUCCH configuration in Uu, according to the analysis in [1][2][3][4][5][6], SL-specific drx-HARQ-RTT-Timer is suggested to be introduced in Uu.

Question 2-1: Whether to introduce SL HARQ process related DRX timers, which including drx-HARQ-RTT-Timer and drx-RetransmissionTimer, in Uu? Please give your comments.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Yes | These timers could reduce UE’s monitoring time and bring power saving gain. |
| Ericsson | Yes | It is beneficial to introduce SL specific HARQ timers, they operate per SL HARQ process. |
| LG | Yes | drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL should be introduced for SL mode 1 UE supporting the DRX. |
| OPPO | Yes | The original RTT and Retx timers are used for UL and DL HARQ process respectively, therefore the corresponding drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL should be defined for SL HARQ process. |
| Lenovo | Yes | For SL Mode 1, if the UE is configured with sl-PUCCH-Config, in order to ensure that the SL Tx UE is ready to receive SL DCI(s) for retransmissions, SL TX UE should transit to ActiveTime on the Uu interface monitoring PDCCH in response to having transmitted a NACK to gNB on PUCCH triggered by the reception of a NACK on PSFCH from a Rx UE. Therefore a SL specific drx-HARQ-RTT-Timer as well as drx-RetransmissionTimer should be introduced. |
| CATT | Yes | SL-specific HARQ related timers can be introduced similar as the HARQ related timers introduced for UL and DL. |
| InterDigital | Yes | HARQ timeline on Uu and SL is different, so different values of timers are required to consider the additional latency over SL for feedback and/or reporting of HARQ feedback to the network. |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Yes with comments | We agree to define two HARQ related timer, i.e. drx-HARQ-RTT-TimerSL and drx-RetransmssionTimerSL for each SL HARQ process. But we think the corresponding UE behaviour is different under different cases. Case 1: UE is configured with PUCCH, how to start these two timers can follow the legacy Uu behaviour, i.e, - Start the drx-HARQ-RTT-TimerSL for the corresponding SL HARQ process in the first symbol after the end of the PUCCH resources carrying the SL HARQ feedback - Start the drx-RetransmissionTimerSL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerSL if the data of the corresponding SL HARQ process has not been successfully transmitted via the sidelink Case 2: UE is configured without PUCCH, as blind retransmission is supported, the UE still needs to extend the active time to monitor blind retransmission. Different from case 1, there is no HARQ feedback transmitted and when to schedule the blind retransmission is all up to the NW. Therefore, there is no need to start the drx-HARQ-RTT-TimerSL since there is no need to consider the processing time of HARQ feedback at the network side. The UE only needs to start the drx-RetransmissionTimerSL to monitor the blind retransmission. |

If the answer of Question 2-1 is Yes, the pre-condition for supporting such timers should be further discussed. According to the current NR Uu specification, drx-HARQ-RTT-Timer and drx-RetransmissionTimer for each downlink process are configured for Uu DRX since the UE needs to receive the retransmission resource allocation information from the gNB. In the rapporteur’s understanding, the similar principle is also applied for NR sidelink. For NR sidelink, if PUCCH for SL HARQ FB is not configured, SL retransmission will not happen, these timers does not need to be maintained; on the contrary, SL retransmission is possible and it should main these SL-specific HARQ retransmission related timers in Uu.

Question 2-2: Do companies agree that the SL HARQ process related timers (drx-HARQ-RTT-Timer and drx-RetransmissionTimer) only need to be supported when there is PUCCH configuration for SL HARQ FB in Uu? Please give your comments.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Yes | Agree with rapp that retransmission scheduled by gNB is only possible when PUCCH is configured for feedback. |
| Ericsson | No | Similar as discussions for SL DRX HARQ timers, we think at least retransmissionTimer needs to be supported in case of PUCCH config is absent. Regarding RTT timer, we don’t have strong views, we can follow the majority view. |
| LG | No | We prefer to use SL HARQ feedback such as Uu DRX design regardless of PUCCH resource. |
| OPPO | No | Even if without PUCCH configuration, there may be PSFCH feedback, and thus both Tx/Rx UE has to respect RTT interval, which means the RTT and Retx timers are essential for all the cases. As for the cases without PUCCH and PSFCH configuration, the start point of RTT timer may be different: - If PUCCH is configured, RTT timer is started after PUCCH transmission. - If PUCCH is not configured, but if PSFCH is configured, considering RAN1 has agreed on the PSFCH-to-PSSCH latency requirement, it seems to start RTT timer since PSFCH is reasonable. - If when neither PUCCH nor PSFCH is not configured, start RTT/Re-transmission timer after PSSCH could be feasible. The retransmission timer can be started upon the expiry of SL RTT timer. |
| Lenovo | No | Similar to the case were UE is configured with PUCCH configuration for SL HARQ FB, it should be also for Blind retransmissions ensured that UE extends its ActiveTime in order to receive SL DCI for SL retransmission resources. Otherwise the gNB needs to schedule the SL retransmission resource on the next OnDuration time. Therefore, UE should also a SL specific drx-RetransmissionTimer to extend the active time to monitor PDCCH for retransmission in case PUCCH is not configured. HARQRTTTimer wouldn’t be required though. |
| CATT | Yes | Regarding to the combinations of PSFCH and PUCCH configuration, there are four combinations: Case; PUCCH configuration; PSFCH configuration; Necessity for maintaining SL-specific RTT/Retx timer in Uu. 1; Yes; Yes; Yes. 2; No; Yes; ?. 3; No; No; ?. 4; Yes; No; Rel-16 V2X has excluded this case. In our understanding: - For case 1, it certainly needs to support SL-specific RTT and retx timer in Uu; - For case 2, this combination is agreed by RAN1 and it is mainly used for the case that SL blind retx is used. That is one scheduling inform in Uu can allocate more than one SL resources, e.g., one for initial tx and the others used for retx). Considering blind retx and HARQ-based retx was not supported till now, in this case, once the gNB sends the scheduling information to UE, it will not schedule the retransmission for the same sidelink HARQ process. Hence, there is no need to maintain the SL-specific RTT and retx timer in Uu. - For case 3, if there is neither PSFCH nor PUCCH, only blind retx is needed. In Uu, only one scheduling information for SL is enough and the scheduling information may include the blind retransmission resources for SL. Similar as case 2, it also does not need to maintain the SL-specific RTT and retx timer in Uu. |
| InterDigital | No | We can define similar HARQ RTT (for microsleep) and retransmission timer (for monitoring PDCCH) regardless of whether PUCCH resources are defined. |
| Spreadtrum | No | Both RTT timer and retransmission timer are needed even if PUCCH is absent. |
| Huawei, HiSilicon | Yes for HARQ RTT timer No for retransmission timer | See our reply above. Even the UE is configured without PUCCH, since blind retransmission is supported, the UE still needs to extend the active to monitor the blind retransmission, otherwise the gNB can only schedule the SL retransmission resource on the next OnDuration time, which is less-efficient. But as there is no HARQ feedback on PUCCH, when UE is configured without PUCCH, there is no need to start the HARQ RTT timer which is to consider the processing time of HARQ feedback at the network side. The UE only needs to start the retransmission timer at some point to monitor the blind retransmission. |

If the answer of Question 2-1 is Yes, it should also discuss how to define the SL-specific drx-HARQ-RTT-Timer and  drx-RetransmissionTimer. In Uu, the legacy corresponding descriptions are as below:

- drx-RetransmissionTimerDL (per DL HARQ process except for the broadcast process): the maximum duration until a DL retransmission is received; - drx-RetransmissionTimerUL (per UL HARQ process): the maximum duration until a grant for UL retransmission is received; - drx-HARQ-RTT-TimerDL (per DL HARQ process except for the broadcast process): the minimum duration before a DL assignment for HARQ retransmission is expected by the MAC entity; - drx-HARQ-RTT-TimerUL (per UL HARQ process): the minimum duration before a UL HARQ retransmission grant is expected by the MAC entity.

According to [2], [3] and [5], the similar definitions as Uu are suggested.

Proposals in [2]: - drx-RetransmissionTimerSL (per Sidelink process): the maximum duration until a grant for SL retransmission is received; - drx-HARQ-RTT-TimerSL (per Sidelink process): the minimum duration before a SL retransmission grant is expected by the MAC entity.

Proposals in [2]: - drx-RetransmissionTimerSLmode1 (per SL HARQ process): the maximum duration until a grant for SL retransmission is received; - drx-HARQ-RTT-TimerSLmode1 (per SL HARQ process): the minimum duration before a SL HARQ retransmission grant is expected by the MAC entity.

Proposals in [5]: Proposal 2: For the case when UE is configured with sl-PUCCH-Config, define two HARQ related timer, i.e. drx-HARQ-RTT-TimerSL and drx-RetransmssionTimerSL for each SL HARQ process. The values of these two timers are configured by gNB. - drx-RetransmissionTimerSL (per Sidelink process): the maximum duration until a grant for SL retransmission is received; - drx-HARQ-RTT-TimerSL (per Sidelink process): the minimum duration before a SL HARQ retransmission grant is expected by the MAC entity.

Question 2-3: Whether the following definitions for SL-specific drx-HARQ-RTT-Timer and drx-RetransmissionTimer can be agreed (note: please ignore the detailed name of the SL-specific drx-HARQ-RTT-Timer and drx-RetransmissionTimer, which will be discussed separately in section 2.3)? Please give your comments.

- drx-RetransmissionTimerSL (per Sidelink process): the maximum duration until a grant for SL retransmission is received;

- drx-HARQ-RTT-TimerSL (per Sidelink process): the minimum duration before a SL retransmission grant is expected by the MAC entity.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Yes | Follow Uu design |
| Ericsson | Yes | Same as Uu design |
| LG | Yes | Same as Uu design |
| OPPO | Yes |  |
| Lenovo | Yes | Uu DRX principles should be followed. |
| CATT | Yes |  |
| InterDigital | Yes | Conceptually these are the same for SL process |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Yes |  |

If the answer of Question 2-1 is Yes, how to maintain the SL-specific drx-HARQ-RTT-Timer and drx-RetransmissionTimer should also be further discussed.  Some similar proposals are suggested in the corresponding contributions [1] [4] [5], listed below:

Proposals in [1]: Proposal 5: For NR sidelink, if mode 1 resource allocation is used, UE should start the drx-HARQ-RTT-TimerUuSL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the SL HARQ feedback in Uu. Proposal 6: For NR sidelink, if mode 1 resource allocation is used, UE should start the drx-RetransmissionTimerULSL if drx-HARQ-RTT-TimerUuSL expiries and the data of the corresponding SL HARQ process is not successfully decoded.

Proposal in [4]: - if a SL HARQ feedback is received, start the drx-HARQ-RTT-TimerSL for the corresponding HARQ process in the first symbol after the end of the corresponding transmission carrying the SL HARQ feedback. - if the data of the corresponding SL HARQ process was not successfully decoded, start the drx-RetransmissionTimerSL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerSL.

Proposal in [5]: Proposal 3: If the UE needs to send the SL HARQ feedback via the PUCCH to the gNB, the UE shall: - Start the drx-HARQ-RTT-TimerSL for the corresponding SL HARQ process in the first symbol after the end of the PUCCH resources carrying the SL HARQ feedback; - Stop the drx-RetransmissionTimerSL for the corresponding HARQ process. Proposal 4: Upon the expiry of a drx-HARQ-RTT-TimerSL, and the data of the corresponding SL HARQ process has not been successfully transmitted via the sidelink, the UE shall: - Start the drx-RetransmissionTimerSL for the corresponding HARQ process in the first symbol after the expiry of drx-HARQ-RTT-TimerSL

Question 2-4: Whether companies can agree the following timer maintenance mechanism for SL-specific drx-HARQ-RTT-Timer? Please give your comments.

- UE should start the SL-specific drx-HARQ-RTT-Timer in Uu for the corresponding SL HARQ process in the first symbol after the end of the corresponding transmission carrying the SL HARQ feedback.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Yes |  |
| Ericsson | Yes |  |
| LG | No | We prefer to use SL HARQ feedback such as Uu DRX design regardless of PUCCH resource. We suggests following modification. UE starts the drx-HARQ-RTT-TimerSL for the corresponding HARQ process ID in the first symbol after the end of the corresponding PSFCH reception carrying the SL HARQ feedback, regardless of whether PUCCH is configured or not. If SL HARQ feedback is disabled, UE starts the drx-HARQ-RTT-TimerSL for the corresponding HARQ process ID in the first symbol after the end of the first repetition of the corresponding PSSCH transmission, regardless of whether PUCCH is configured or not. |
| OPPO | See comments | As our comments to Q2-2, the start point of the RTT timer may be different, can be either PUCCH, PSFCH or PSSCH. |
| Lenovo | Yes |  |
| CATT | Yes |  |
| InterDigital | No | This can work only if PUCCH is configured. We can use this definition for that case (same as Uu). - When PUCCH is not configured start the timer upon reception of PSFCH When PUCCH is not configured and PSFCH is not configured (or transmission is HARQ disabled), start timer upon SCI transmission |
| Spreadtrum | No | - RTT timer should start upon the corresponding PSFCH transmission if SL HARQ feedback is enabled. - RTT timer should start upon the corresponding PSSCH transmission if SL HARQ feedback is disabled and PUCCH presents. - RTT timer is not need if SL HARQ feedback is disable and PUCCH is absent. |
| Huawei, HiSilicon | See comments | As we replied above, we think the maintenance of SL-specific HARQ RTT timer only applies to the case when PUCCH is configured to the UE. Therefore, to make it more clear, we prefer to reword as below: If the UE is configured with sl-PUCCH-Config, UE should start the SL-specific drx-HARQ-RTT-Timer in Uu for the corresponding SL HARQ process in the first symbol after the end of the corresponding transmission carrying the SL HARQ feedback |

Question 2-5: Whether companies can agree the following time maintenance mechanism for SL-specific drx-RetransmissionTimer? Please give your comments.

- If the data of the corresponding HARQ process was not successfully decoded in sidelink, UE should start the SL-specific drx-RetransmissionTimer in Uu for the corresponding HARQ process in the first symbol after the expiry of the SL-specific drx-HARQ-RTT-Timer.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Yes with comment | Unlike Uu, the data decoding and feedback transmission in PUCCH is done at RX UE and Tx UE respectively. Therefore, we suggest rewording such as, If the feedback of the corresponding HARQ process was NACK, UE should start the SL-specific drx-RetransmissionTimer in Uu for the corresponding HARQ process in the first symbol after the expiry of the SL-specific drx-HARQ-RTT-Timer. |
| Ericsson | Yes and No with comments | For PUCCH config present case, agree with Xiaomi’s suggested update. For PUCCH config absent case, if RTT timer is not supported, UE starts the retransmission timer in the first symbol after the last PSSCH transmission using the scheduled grant for the corresponding SL HARQ process |
| LG | Yes with comment | Agree with Xiaomi’s modification. |
| OPPO | No | In sidelink, whether the corresponding HARQ process was successfully decoded is judged by the Rx UE, and there may be no HARQ FB to the Tx UE. However, whether a SL grant for retransmission is needed is judged by the Tx UE based on the FB from the Rx UE or other reasons. Therefore, suggest to change the expression as follows: If retransmission of the data of the corresponding HARQ process is needed in sidelink, UE should start the SL-specific drx-RetransmissionTimer in Uu for the corresponding HARQ process in the first symbol after the expiry of the SL-specific drx-HARQ-RTT-Timer. |
| Lenovo | Yes with comments | For BL case also the SL specific drx-RetransmissionTimer should be started. |
| CATT | Yes with comment | We agree with the modification of Xiaomi, If there is PUCCH configuration in Uu and there must be PSFCH configuration in SL, hence whether the data can be decoded successfully can be judged based on the PSFCH feedback. |
| InterDigital | No | Agree with the wording from OPPO – the need for retransmission does not depend on decoding, and may not depend on HARQ feedback if no HARQ feedback is expected. |
| Spreadtrum | No | If RTT timer is used, agree the wording from OPPO. If RTT timer not used, the retransmission timer should start upon the transmission of the corresponding PSSCH. |
| Huawei, HiSilicon | See comments | As we replied above, we think the described maintenance of SL-specific retransmission timer above only applies to the case when PUCCH is configured to the UE. When PUCCH is not configured, there is no need to start the SL-specific HARQ RTT timer and when to start the retransmission timer for blind retransmission monitoring needs FFS. Also from the TX UE’s perspective, it can only judge whether the data is successfully transmitted or not while from the RX UE’s side, the RX UE determines whether the data is successfully decoded or not. Therefore, to make it more clear, we prefer to reword as below: If the UE is configured with sl-PUCCH-Config, if the data of the corresponding HARQ process was not successfully decoded transmitted in sidelink, UE should start the SL-specific drx-RetransmissionTimer in Uu for the corresponding HARQ process in the first symbol after the expiry of the SL-specific drx-HARQ-RTT-Timer. |

According to the proposal in [5], it also mentioned that if the UE needs to send the SL HARQ feedback via the PUCCH to the gNB, the UE shall start the drx-HARQ-RTT-TimerSL for the corresponding SL HARQ process in the first symbol after the end of the PUCCH resources carrying the SL HARQ feedback. And in the meanwhile, it should stop the drx-RetransmissionTimerSL for the corresponding HARQ process.

Question 2-6: Whether companies can agree to stop the SL-specific drx-RetransmissionTimerSL in the meanwhile of the start of the drx-HARQ-RTT-TimerSL? Please give your comments.

| Companies | Yes/No | Comments |
| --- | --- | --- |
| Xiaomi | Comment | We think the proposal should be modified as following, since there are RTT and RTX timer in both Uu and SL. It’s not clear which timer is in current wording. Stop the SL-specific drx-RetransmissionTimer in Uu in the meanwhile of the start of the drx-HARQ-RTT-TimerSL in SL |
| Ericsson | Yes | Same as Uu DRX |
| LG | Yes | Same as UU DRX design as following; “stop the drx-RetransmissionTimerSL for the corresponding sidelink process at the first transmission of the corresponding PSSCH transmission.” |
| OPPO | Yes |  |
| Lenovo | Yes |  |
| CATT | Yes | Similar as Uu DRX |
| InterDigital | Yes | Same as Uu |
| Spreadtrum | Yes |  |
| Huawei, HiSilicon | Yes | Similar as in Uu. |

## Name of the SL-specific DRX timers in Uu

How to define the name of the SL-specific DRX timers in Uu has been discussed in [1][2][3][5]5]:

- drx-HARQ-RTT-TimerUuSL and drx-RetransmissionTimerUuSL in  [1];

- drx-HARQ-RTT-TimerSLmode1 and drx-RetransmissionTimerSLmode1 in  [3][1];

- drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL in  [2][1].

Based on the rapporteur’s understanding, using suffix –SL may cause confusion with the SL DRX timers used in PC5 interface. In order to facilitate the consecutive discussion, it had better make clear how to define the SL-specific DRX related timers in Uu.

Question 3-1: Regarding to the name of the SL-specific DRX related timers in Uu, which suffix should be used:

- Option 1: UuSL;

- Option 2: SLmode1;

- Option 3: SL;

- Option 4: others (if company has different views, please add it here).

| Companies | Option | Comments |
| --- | --- | --- |
| Xiaomi | Too early to decide | We think this is stage 3 discussion. |
| Ericsson | Option 2 |  |
| LG | Option 3 |  |
| OPPO | Option 3 |  |
| Lenovo |  | We think it might be too early for this. However currently for RRC signalling, similar things happen that SL related signalling can be transmitted on both Uu and SL, maybe we could use it as a reference 1. for SL associated signalling transmitted on Uu: use “SL-“ prefix, e.g. SL-ConfigDedicatedNR, SL-ConfiguredGrantConfig etc. 2. For SL signalling transmitted on PC5 (PC5-RRC signaling): use “-Sidelink” suffix, e.g. RRCReconfigurationSidelink, UECapabilityEnquirySidelink etc. |
| CATT | Option 1 or Option 2 | It had better distinguish the SL-specific DRX timer used in Uu from the SL-specific DRX timers used in PC5 since different value may be needed. |
| InterDigital | Option 3 | No strong view, but we think option 3 is sufficient |
| Spreadtrum | No strong views |  |
| Huawei, HiSilicon | Option 3 | We prefer to use drx-HARQ-RTT-TimerSL and drx-RetransmissionTimerSL |

## Others

Regarding the issue that Uu DRX timer impact to support SL, besides the issues listed in the above sections, if companies still have other issues, please listed in the table below:

| Companies | Issue description |
| --- | --- |
| Huawei, HiSilicon | If PUCCH is not configured, when to start the SL-specific retransmission for blind retransmission monitoring considering a single DCI can schedule up to 3 retransmission opportunities? |

# Conclusion

# Reference

- R2-2102690	DRX Active Time Alignment between Uu and SL  CATT

- R2-2102771	Further discussion on Sidelink DRX   LG

- R2-2103003	General aspects of SL DRX	  Ericsson,Qualcomm Incorporated

- R2-2102979	Discussion on  Coordination between Uu DRX and SL DRX	ZTE

- R2-2104113	Discussion on SL communication impact on Uu DRX	Huawei, HiSilicon

- R2-2103470	Coordination between Uu DRX and SL DRX 	 Lenovo, Motorola Mobility