3GPP TSG-RAN WG2 Meeting #113-bis-e	                                                                                  R2-210xxxx

Electronic meeting, 12th April – 20th April 2021

Agenda item:	8.13.3.2

Source:	CMCC

Title:	Report of [AT113b-e][804][NR/R17 SON/MDT] Logged MDT (CMCC)

Document for:	Discussion, Decision

# Introduction

This is to report the result of the following email discussion [1]:

 [AT113b-e][804][NR/R17 SON/MDT]  ]  Logged MDT (CMCC)

-     Step 1: Collect the companies’ view on the proposals in R2-2103708. Deadline is 16:00 UTC 15/04/2021.

-     Step 2: Based on majority view provide the agreeable proposals

-     Step 3: Collect the companies’ view on the new proposals and goto Step 2 until 04:00 UTC Friday 16/04/2021.

-     Step 4: Providing final proposals for CB session. Note that in CB session there will not be technical discussion again. We try to directly agree the final proposals.

Intended outcome: Easily agreeable proposals and LS draft

Deadline: Friday 16/04/2021

# Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |

# Discussion

## Logging early measurements

As discussed in [2], the proposal that “let the network to understand the reason why early measurement results are not included in the logged measurement report anymore is due to coverage hole or not” is not pursued based on the majority view.

[Step1-Q1]: Do you agree that “let the network to understand the reason why early measurement results are not included in the logged measurement report anymore is due to coverage hole or not” is not pursued?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |

Summary:

## MDT for On-demand SI

Following proposals are provided in [2]:

Proposal 1: UE reports the SIBs that UE actually intends to request.

Proposal 2: UE does not report the time elapsed since the SI request initiation or the UE modem realizes the need for on demand SI until the successful SI acquisition or the acquisition failure.

Proposal 3: Both Msg1-based and Msg3-based SI request related information are supported.

Proposal 4: It is FFS whether to extend current RA-report to include the on demand SI related information.

As these proposals seem to be agreeable to majority during email discussions, we can check if these are agreeable to all.

[Step1-Q2] Are proposals 1-4 agreeable to you?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |

Summary:

## Logged MDT in EN-DC

Following proposal is provided in [2] since none of the option obtains enough support:

Proposal 5: It is FFS which option is used for logged MDT in EN-DC.

(1) LTE and NR logged MDT configurations are independent, and UE performs logging based on the logged MDT configuration of the same RAT it camps. (CATT, Qualcomm, Ericsson, ZTE)

(2) In EN-DC where UE cannot camp on NR cells, UE logs the NR measurements based on network configurations, i.e. for the NR logged MDT measurements in NSA, LTE SIB24 broadcasts the NR frequencies and indicates the UE does not need to reselect to NR cells. (Huawei)

(3) Do not introduce SN configuration for logged MDT (neither for camping nor for non-camping/ EMR specific frequencies). (Samsung)

(4) Enhanced areaConfiguration wherein a list of areaConfig could be provided to the UE wherein one list is associated to LTE and the other is associated is NR. (Vivo, Ericsson)

[Step1-Q3] Is proposal 5 agreeable to you?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |

Summary:

## Logged MDT in other MR-DC

It was proposed to firstly discuss EN-DC case and then other MR-DC cases for logged MDT. So we will not discuss other MR-DC cases at this email discussion.

## Override discussion (based on RAN3 LS R3-207176)

Following proposals are provided in [2]:

Proposal 6: It is FFS whether there is a need to avoid logged MDT configuration in the following cases:

- Logged MDT is configured, but no results are available e.g. so far nothing stored, or all previously stored results retrieved

- Logged MDT configuration is released, but UE still has un-retrieved results that would be discarded upon accepting a new configuration

Proposal 7: UE provides assistance by which network can avoid overwriting of an MDT configuration.

Proposal 8: Introduce the logged MDT type (i.e. the management based MDT or the signalling based MDT) in the logged MDT configuration.

[Step1-Q4] Are proposal 6-8 agreeable to you?

| Company | Agree or not | Reasons/Comments/Suggestions |
| --- | --- | --- |

Summary:

# Conclusions

Following proposals are made based on above discussion:

# References

- Draft session report SON/MDT, see https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_113bis-e/Inbox/Chairmans_Notes

- R2-2103708 Report of [Post113-e][854][NR17 SON MDT] Logged MDT (CMCC)