3GPP TSG RAN WG2 Meeting #114-e	                                        R2-210xxxx

- Electronic, 19th – 27th May 2021

Agenda item:		8.14.2.2

Source:			Lenovo

Title:		Report from email discussion [AT114-e][027][QoE] Start and Stop (Lenovo) – Phase 2 (LS out)

Document for:	 	Discussion and decision

# Introduction

As result of the online discussion of the report [AT114-e][027][QoE] Start and Stop (Lenovo) in R2-2106661 the following agreement was made on QoE report handling during QoE pause:

- On whether to store reports in the AS or the application layer at Pause, Send LS to SA4/SA5/SA3 to inform them about the options and their pros/cons (if possible) and ask them for feedback. RAN2 will continue work on this topic based on the feedback received.

Scope of this email discussion is to discuss and agree on the content of the LS to SA4/SA5/SA3:

- On the description of the pros/cons for each option.

- On the specific actions to ask to SA4/SA5/SA3.

# Discussion

## Description of Option 1

As result of the email discussion the description of the pros/cons for Option 1 was updated as shown below.

|  | Advantages | Disadvantages |
| --- | --- | --- |
| Option 1, RAN transparent approach (i.e. QoE reports during “pause” are stored at application layer) | - There is no need to store the QoE reports in AS layer, which utilizes BP memory which is a scarce resource. It makes no sense to buffer a very limited amount in AS layer. - Considering UE storage is large (e.g. 256G), QoE data amount could be stored as much as possible. - Buffering in application layer gives the chance to transfer stored QoE data when the application layer is terminated. - Future-proof to consider QoE measurement in IDLE and Inactive state. - Very limited impact to RAN2 specifications, e.g. there is no need to discuss details of QoE reports storing in AS layer (e.g. maximum storing time, maximum size of stored reports, priorities etc.) or to define reporting of stored QoE reports after resume is indicated (i.e. QoE reports are handled in the same way as during normal operation) | - Application layer behavior upon reception of pause/resume indications needs to be specified by SA4. - Indicating pause to the applications basically means telling the applications that RAN is overloaded. This could be a security risk, if there are malicious applications that would like to take down the network. This needs to be checked with SA3. - Handling of reports from different applications when the overload situation has passed may need to be specified by SA4, This needs to be checked with them. |

Question 1: Companies are requested to provide their comments on the description of the pros/cons for Option 1, i.e. which of the bullet points are agreeable / not agreeable.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |

## Description of Option 2

As result of the email discussion the description of the pros/cons for Option 2 was updated as shown below.

|  | Advantages | Disadvantages |
| --- | --- | --- |
| Option 2, Application transparent approach (i.e. QoE reports during “pause” are stored at AS layer) | - Application layer is unaffected | - The QoE reports need to be stored at AS layer, which has more limited storage capacity - May impact non-QoE data processing performance due to reduced AS buffer size. - High workload and specifications impact in RAN2, e.g.to discuss the details of QoE reports storage and reporting after UE receives pause/resume indications - Collide with SA5 specification “Temporary stop and restart of QoE information reporting during RAN overload.” In TS 28.405. - Requires App Layer – AS layer coordination, besides impacts to RRC the interaction goes beyond RAN2, thus impact functionality definition and maintenance in cross-WGs specifications |

Question 2: Companies are requested to provide their comments on the description of the pros/cons for Option 2, i.e. which of the bullet points are agreeable / not agreeable.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |

## Description of Option 3

As result of the email discussion the description of the pros/cons for Option 3 was updated as shown below.

|  | Advantages | Disadvantages |
| --- | --- | --- |
| Option 3, RAN transparent approach (i.e. AS discard the QoE data during “pause”) | - There is no need to store the QoE reports in AS layer, which utilizes RAM memory which is a scarce resource - Not touch application behavior. - Very limited impact to RAN2 specification. - Fits purpose of using RAN as interface to pass available QoE measurements, without introducing new requirements going beyond RAN | - Application layer may or may not buffer the QoE data during pause, which can be decided by SA4. - Restrict the supporting for partially Pause - No benefit of pause mechanism if the reports are anyhow discarded. Then the network might as well deconfigure QoE. |

Question 3: Companies are requested to provide their comments on the description of the pros/cons for Option 3, i.e. which of the bullet points are agreeable / not agreeable.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |

## Specific actions to ask to SA4/SA5/SA3

Referring to the 3GPP calendar SA4, SA5 and SA3 have their next meetings in Q3 as follows:

- SA3#103-e: 16-27 August 2021

- SA4#115-e: 18-27 August 2021

- SA5#138-e: 23-31 August 2021

That means, responses from them we can expect in our meeting(s) in Q4/2021 at the earliest. Therefore, we should be very specific about the actions we want to ask them. Referring to the discussions we had, rapporteur drafted a first description of the LS and actions to the concerned groups as shown below.

Overall description RAN2 discussed the handling of QoE reports during RAN overload and agreed that the RAN node can send a QoE pause indication to instruct the UE to temporarily stop sending QoE reports of the affected QoE measurements configuration until receiving a QoE resume indication from the RAN node. With regards to the handling of QoE reports in the UE which are generated during QoE pause, RAN2 identified three options: - Option 1: Application layer is responsible for storing QoE reports when the UE receives QoE pause indication. - Option 2: AS layer is responsible for storing QoE reports when the UE receives QoE pause indication. - Option 3: The QoE container received from application layer is discarded during pause. In the table below the description of the pros/cons for each option has been summarized. RAN2 did not reach consensus on the option to pursue and seek guidance from SA4, SA5 and SA3. <Table to add> Action to SA4: RAN2 kindly asks SA4 to take the information about the options for handling QoE reports during QoE pause into account and provide their feedback whether QoE reports during QoE pause can be stored at the application level. Action to SA5: RAN2 kindly asks SA5 to take the information about the options for handling QoE reports during QoE pause into account and provide their feedback on the feasibility of each option. Action to SA3: RAN2 kindly asks SA3 to provide their feedback on the potential security issue raised for Option 1.

Question 4: Companies are requested to provide their comments on the action to SA4.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |

Question 5: Companies are requested to provide their comments on the action to SA5.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |

Question 6: Companies are requested to provide their comments on the action to SA3.

| Company | Agree/disagree | Additional comments |
| --- | --- | --- |

# Conclusion