3GPP TSG-RAN WG2 Meeting #115-e	R2-210xxxx

E-meeting, 9th – 27th August 2021

Title:	DRAFT LS Reply to SA3 on security protection on RRCResumeRequest message

Response to:	S3-212349

Release:	Release-17

Work Item:	FS_5GFBS

Source:	Apple [To be RAN2]

To:	SA3

Cc:	RAN3, RAN

Contact Person:

Name:	Fangli XU

E-mail Address:	fangli_xu@apple.com

# 1. Overall Description

RAN2 would like to thank SA3 for the LS on security protection on RRCResumeRequest message.

Regarding SA3’s questions, following are the RAN2’s feedback.

Q1: For the capability negotiation method between UE and gNB/ng-eNB as mentioned above, if there are other preferable alternatives from RAN2 perspective?

Q2: Is there any mechanism for the source gNB/ng-eNB to know the target gNB/ng-eNB capabilities?

[RAN2 response]

RAN2 discussed the capability negotiation methods and made the initial conclusion as follows:

- The feature requires the support of the UE, the source gNB and the target gNB;

- From UE perspective

  - The UE indicates its capability in the AS capability and reports to network via RRC signaling;

  - The UE enables the feature only when it knows both source node and target node support it;

- From the source node perspective

  - The source node indicates its capability via the RRC dedicated configuration (i.e. RRCRelease with SuspendConfig) or the SIB (depending on the method);

  - The source node only performs the new ResumeMAC-I verification when the UE is configured with the new feature and the target node supports the new ResumeMAC-I;

- From the target node perspective

  - The node as the target node role indicates its capability via SIB or binds its capability together with the source node’s capability (depending on method);

  - How to indicate the target node’s capability to the source node should be discussed in RAN3.

Q3: The possibility of specifying the solution in RAN2 specification in Rel-17 timeframe, if the solution is concluded by SA3.

[RAN2 response]

The solution is technically feasible from RAN2 perspective. However, RAN2 observed that the solution spans multiple WGs (i.e. RAN2 and RAN3), and thus it should be first discussed in RAN plenary if SA3 decides to specify it in R17.

2. Actions:

RAN2 respectfully asks SA3 to take the above information into account for future work.

3. Date of Next RAN2 Meetings:

TSG RAN2 Meeting #116-e					Nov. 1 – 12, 2021

TSG-RAN2 Meeting #117                            Feb. 21 – 25, 2022