3GPP TSG-RAN WG2 Meeting #115 electronic	                             R2-xxxxxx

9th August – 27th August 2021

Agenda item:	8.4.2

Source:	Interdigital (rapporteur)

Title:	Report of [AT115-e][042][eIAB] fairness, latency and congestion (Interdigital)

Document for:	Discussion and Decision

# Introduction

This document is the summary of the offline email discussion [AT115-e][042][eIAB] fairness, latency and congestion (Interdigital), as indicated below:

- [AT115-e][042][eIAB] fairness, latency and congestion (Interdigital)

Scope: Continuing from on-line discussion, treat further P7 P8 P11 and variants thereof. Based on complexity and benefits, identify at least one agreeable or tolerable variant (if possible).

Intended outcome: Report, possible way forward.

Deadline: Tuesday 23 August (for CB)

Listed below are the concerned proposals to be discussed in this offline discussion [21]:

Proposal 7.a ([1][3][6][9][12][18]):  IAB-node and donor DU can be configured with downstream number of hops per routing ID.

Proposal 7.b ([1][3][6][9][12][18]):  IAB-node can be configured with upstream number of hops per routing ID.

Proposal 8 ([9][13][20][3][17]):  RAN2 to introduce a timing related information in the BAP header to help in enforcing the packet delay budget in a multi-hop IAB network. Details of the timing information are FFS (e.g., a timestamp corresponding to the time of the packet creation, a timestamp corresponding to the time of the packet expiry, the remaining PDB, etc)

Proposal 11 ([6][14][18]): RAN2 to discuss whether UL hop-by-hop flow control is supported in rel-17 IAB.

Proposals 7 and 8 are discussed under the latency enhancement section, and proposal 11 is discussed under the flow control section.

# Contact information

Please provide your contact information when responding:

| Company | Contact Name | Email |
| --- | --- | --- |
| Huawei, HiSilicon | Yulong Shi | shiyulong5@huawei.com |
| Interdigital | Oumer Teyeb | Oumer.teyeb@interdigital.com |

# Latency enhancement

In rel-16 IAB, IAB nodes are configured with the packet delay budget of the BH RLC channels (i.e., per-hop PDB), but they are not aware of the E2E packet delay budget of the bearers that are mapped to the BH RLC channel.

In the case of 1:1 mapped bearer, a network implementation could try to split the packet delay budget among the different hops. However, the actual latency experienced by a packet over a given hop may be shorter or longer than the configured PDB. In case it was shorter, the configured PDB over the next hops is more restrictive than necessary for that packet. If it was longer, the configured PDB over the next hops will not be strict enough to meet the E2E PDB. And just discarding the packet because its PDB was not met over one hop is an overkill, as the packet may be transported faster on subsequent links and the extra delay it has experienced so far can be compensated.

For the case of N:1 mapping, the BH RLC channels may be multiplexing bearers with different E2E PDBs, and as such, the splitting of the E2E PDB into per-hop PDB is not directly applicable. Even if it was (e.g., all the bearers multiplexed have the same E2E PDB), the same issues mentioned above for the 1:1 mapped bearer case still apply.

Q1: Do companies agree that, for both 1:1 and N:1 mapping, just splitting the E2E PDB to per-hop PDB is not optimal/sufficient for enforcing the E2E PDB of packets?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | Good CU implementation on the splitting can enforce the E2E PDB requirement. We don’t agree on the statement “the splitting of the E2E PDB into per-hop PDB is not directly applicable” for N:1 mapping. |
| Interdigital | Yes | For the case of 1:1 mapped BH RLC channels: The CU can split the E2E PDB among the different hops could help, but the actual latency experienced by the packets on each hop will not be the same as the per-hop PDB, and as such it wont guarantte the E2E PDB will be met. If there are only a small number of bearers concerned, then the network may be able to ensure each per-hop PDB is met, but the more bearers in the network, the more difficult it will become to ensure that on each hop. Also, the scheduler has to consider other QoS parameters like the throughput, which makes it difficult to guarantee the per-hop PDB on every hop. In summary, on some hops (e.g. due to temporary congestion), it is very likely that the per-hop PDB will not be met, while on other hops (e.g. due to less congestion), the packet may be delivered well before the per-hop PDB for that hop has expired. For the case ôf N:1 mapped BH RLC channels: On top of the problems mentioned for 1:1 mapping, here we will have a problem of having multiple bearers with different E2E per-hop PDBs. It is not clear on how the network will decide the E2E per hop PDB in this case. For example, if there are 2 bearers multiplexed on a certain BH RLC channel, with E2E PDBs of L1 and L2 (assume L1 < L2), then will the CU average the E2E PDB as the average, min, max of the two and split it on the number of hops? The averaging or considering the max may hurt the performance of the packets of the first bearer, while considering the min will be an over provisioning as packets of the second bearer do not need such a strict latency. On top of that, things become complicated if the two bearers belong to UEs that are different hops away. |

In order to properly schedule/prioritize packets (and if possible, also re-route packets via another path) so that E2E PDBs are met, IAB nodes/donor DU ideally need to know the following:

a)	the remaining packet delay budget (or the E2E PDB and the time the packet has been in flight)

b)	the expected delay the packet is going to experience on subsequent hops until the destination

If an IAB node becomes aware of the above two, it can make optimal scheduling decisions (e.g., prioritize those packets with very short remaining PDB).

Regarding a), the proposals from companies fall into one of the following:

- Timestamp included in the BAP header (inserted at the donor DU in the DL or at the access IAB node in the UL)

  - Time stamp of packet creation ([9][13][20])

  - Time stamp of packet expiry ([3][8])

- Remaining PDB included in the BAP header, which is included in the donor DU in the DL or at the access IAB node in the UL, and updated on each subsequent IAB node by considering the delay the packet has experienced on the previous hop ([3][8][17])

Q2:  Do companies support the inclusion of the timing related information on the BAP header? If so, which of the above is the preferred solution?

| Company | Yes/no | Detailed comments (if the answer is yes, please include the preferred solution) |
| --- | --- | --- |
| Huawei, HiSilicon | No | It cause more complexity that BAP marks the timing info per packet. Not sure the timing info is accurate, since we will not specify when exactly to add this timing info. If the timing info is not accurate, maybe it will make it worse. |
| Interdigital | Yes | From the options listed, we tend to prefer option 1.b or option 2. option 1.a requires the knowledge of the E2E PDB of the packet on each IAB node and the DU, which could be difficult to do in the case of N:1 mapping (unless we have a bearer ID on every packet, which we have more or less agreed not to have during the online discussion). Option 1.b and option 2 requires only the donor DU and the access IAB node to be aware of the E2E PDB, and since these two are the nodes where the bearer is mapped to a BAP for the first time, providing this information to them will work whether we have 1:1 or N:1 mapping. Option 1.b is the simplest solution because only the access IAB node or the donor DU have to do the calculation, but it will have a higher overhead than option 2. On the other hand, option 2 will lead to less overhead, but requires all the intermediate IAB nodes to update the BAP header. As some companies have proposed ([9]), it is not necessary to include the full timestamp for option 1.b and some relative time unit based on a shared reference time in the IAB network could be used. |

The rapporteur’s understanding is that for any of the timing information related enhancements described above, the IAB nodes and/or donor DU require information related to the E2E PDB. For example,

For solution 1a: IAB nodes have to know the E2E PDB of the packet to make use of the timestamp of the packet creation

For solution 1b: donor DU has to know the E2E PDB to calculate the expiry time for DL packets, access IAB node has to know the E2E PDB to calculate the expiry time for UL packets

For solution 2a: donor DU has to know the E2E PDB to put the remaining PDB for DL packets, access IAB node has to know the E2E PDB to calculate the remaining PDB for UL packets

Q3:  If the answer to Q2 is Yes, do companies agree that at least the access IAB node and the donor DU must be configured with the E2E PDB of the bearer?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| Huawei, HiSilicon |  | In R16, the configuration on adding BAP header at donor-DU is on the level of IP address + flow label/DSCP, rather than per bearer. No sure this is feasible for donor-DU. |
| Interdigital | Yes | See comment to Q2 |

Regarding b, the proposals from companies is to configure the donor DU and IAB nodes with the number of hops per routing ID, and this is used to estimate the extra latency the packet has yet to experience before reaching the destination and/or to prioritize one packet over another (([1][3][6][9][12][18])).

During the email discussion in [21] and online meeting, it was discussed that the number of hops is mostly relevant for the DL, as the number of hops in the UL is the same from a given IAB node. However, the rapporteur’s understanding is that this is not necessarily true (e.g., if the IAB node is operating in DC with two different donor CUs).

Q4a:  Do companies support the configuration of IAB nodes and donor DU with the number of hops per downstream destination routing ID?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| Huawei, HiSilicon | No |  |
| Interdigital | Yes | In general, in a multi-hop setting, it is almost impossible to know the exact remaining latency, and this has to be estiamted somehow. The number of hops can be considered as a crude estimate of the remaining latency, and as such, maynot be directly used to enforce the latecny of a certain packet. However, it can be a very useful information that the scheduler can employ when it has to prioritize among packets of different bearers. |

Q4b:  Do companies support the configuration of IAB nodes with the number of hops per upstream destination routing ID (e.g., in case of DC with different donor CUs)?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| Huawei, HiSilicon | No |  |
| Interdigital | Maybe | It can be useful in the case there are IAB nodes that are operating in DC with different donor CUs, where the number of hops in the UL that packets could take from a given IAB node could be different, depending whether the packet is destined for the MN or the SN. |

There were also some proposals that addresses the problem of latency enforcement in a way different from the above. In ([8][15]), it was proposed to configure the BH RLC channels with a multitude of per-hop PDB values corresponding to different destination routing IDs. In [15], it is further proposed that each IAB node can include the per-hop PDB deficit/surplus in the previous hop in the BAP header, which can be used to relax/restrict the scheduling of the packet over the next hop.

Q5:  Do companies support the configuration of BH RLC channels with multitude of per-hop PDBs corresponding to different destination routing IDs?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| Huawei, HiSilicon | No |  |
| Interdigital | No | It seems to us that the intention is to faciliate the per-hop PDB splitting for the case of N:1 mapping? For example, if there are 2 bearers, one two hops away, and the other 3 hops away, and both have an E2E PDB of x, is the inention to have them mapped on the same BH RLC channel on the fist and second hops where this backhaul RLC channel has PDB values of x/3 for destination address of bearer 2, and x/2 for destination address of bearer 1? If that is the correct understanding, then: - This is not relevant for 1:1 mapped bearers, where the latency enforcement is probably more important (as our understanding is that packet with very strict latency requirements are to be mapped 1:1) - Even if we have two packets that are destined at the same IAB node, it doesn’t mean they will have a similar E2E PDB. So splitting the per-hop PDB per destination will not help in this case. - Couldn’t the same behavior be achievable even in rel-16 via implemenation (e.g. creating multiple BH RLC channels, each with different per-hop PDB, and mapping the packets with certain routing IDs to the corresponding BH RLC channel) |

# UL flow control

In [6], [14] and [18], UL hop-by-hop flow control is proposed for better congestion mitigation via improved UL scheduling and resource utilization (including the case of DC where only the MCG or SCG is congested), while [3][13][15] and [19] propose not to introduce UL flow control as current mechanisms (parent controlling child’s UL traffic flow based on BSR received and grants provided in response to that).

Q6:  Do companies support the introduction of UL hop-by-hop flow control mechanisms?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | As in the chair minutes, the discussion point is for supporting UL flow control triggered local re-routing. One Further round of offline discussion: - P7 P8 - Consider also P11, consiering that the purpose to trigger local rerouting, at situations when there would be no local build up of buffers. - Consider complexity and gain.. One of the typical use case is to avoid data congestion, for the IAB node with dual parents but one of them is UL congested. Triggering local re-routing can be efficient solution to reduce congestion and latency. Also, the spec impact/standard effort could be quiet small (almost 0), since RAN2 will anyway define the DL local rerouting triggered by flow control and can fully reuse the R16 flow control format. Note the UL scheduling can only throttle the data transmission in one MAC (or on backpressure the data in one cell group). BAP layer is the only way to switch data from one MAC to another MAC by local re-routing. |
| Interdigital | Yes | We agree with the explanation from Huawei. |

# Summary

# References

| [1] | R2-2107063 | Fairness Latency and Congestion | CATT |
| --- | --- | --- | --- |
| [2] | R2-2107113 | Discussion on flow control for congestion mitigation | CANON Research Centre France |
| [3] | R2-2107177 | Fairness, latency and congestion – solutions to identified issues | Samsung Electronics GmbH |
| [4] | R2-2107178 | Enhancements to LCG space and BSR triggering including pre-emptive BSR | Samsung Electronics GmbH |
| [5] | R2-2107250 | Enhancements for topology-wide fairness, multi-hop latency and congestion mitigation | Huawei, HiSilicon |
| [6] | R2-2107289 | IAB topology-wide fairness, latency, and congestion enhancement | Intel Corporation |
| [7] | R2-2107635 | Way forward for IAB enhancements to improve topology-wide fairness multi-hop latency and congestion mitigation | Apple |
| [8] | R2-2107851 | An elaboration of required PDB for multi-hop latency | ITRI |
| [9] | R2-2107859 | Discussion on multi-hop latency and LCG extension issues | vivo |
| [10] | R2-2107892 | Discussion on LCG extension for IAB | Lenovo, Motorola Mobility |
| [11] | R2-2107998 | Possible solutions for topology-wide fairness, multi-hop latency and congestion mitigation in eIAB | Kyocera |
| [12] | R2-2108053 | Number of hops information to improve topology-wide fairness and latency | Sony |
| [13] | R2-2108139 | Discussion on fairness, multi-hop latency and congestion mitigation | ZTE, Sanechips |
| [14] | R2-2108241 | Fairness, latency, congestion | Nokia, Nokia Shanghai Bell |
| [15] | R2-2108421 | On Topology-wide Fairness, Multi-hop Latency, and Congestion in IAB Network | Ericsson |
| [16] | R2-2108437 | Multi-hop scheduling enhancements for IAB | AT&T |
| [17] | R2-2108492 | Timing information for latency enhancement in multi-hop IAB | InterDigital |
| [18] | R2-2108493 | Latency enforcement, fairness and congestion mitigation in multi-hop IAB | InterDigital |
| [19] | R2-2108743 | Discussion on topology-wide fairness, multi-hop latency and congestion mitigation | LG Electronics Inc. |
| [20] | R2-2108753 | Rel. 17 IAB enhancements for fairness, multi-hop latency reduction, and congestion mitigation | Futurewei Technologies |
| [21] | R2-2109032 | Feature summary of 8.4.2 (Rel-17 IAB contributions on fairness, latency and congestion) | Interdigital |