3GPP TSG-RAN2 Meeting #115 Electronic                               R2- 210xxxx

- 9th August – 27th August 2021

Agenda item:	8.17.3

Source:	Samsung

Title:	Report of [AT115-e] [053] [feMIMO] Beam Failure Handling

Document for:	Discussion & Decision

# Introduction

This document is a report from the following email discussion:

- [AT115-e] [053] [feMIMO] Beam Failure Handling (Samsung)

Scope: Progress P4 P5 from R2-2107007. Can discuss also alternative options.

Intended outcome: Agreements, Report.

Deadline: EOM (can CB if needed)

Deadline for companies' comments: Thursday, 26th August, 0800 UTC

# Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Samsung (rapporteur) | Anil Agiwal | anilag@samsung.com |
| OPPO | Xin You | youxin@oppo.com |
| ASUSTeK | Xinra Kung | Xinra_Kung@asus.com |
| Lenovo | Lianhai | Wulh5@lenovo.com |
| Xiaomi | Yumin Wu | wuyumin@xiaomi.com |
| LG | Hanul Lee | hanul.lee@lge.com |
| Intel | Youn Heo | Youn.hyoung.heo@intel.com |
| Apple | Fangli XU | Fangli_xu@apple.com |
| ZTE | Fei Dong | Dong.fei@zte.com.cn |
| Qualcomm | Ruiming Zheng | rzheng@qti.qualcomm.com |
| CATT | Erlin Zeng | erlin.zeng@catt.cn |
| Fujitsu | Meiyi Jia | jiameiyi@fujitsu.com |
| Vivo | Chenli | Chenli5g@vivo.com |

# Discussion

## Indicating failed TRP in BFRQ MAC CE

According to RAN1 agreement, BFRQ MAC-CE is transmitted upon beam failure detection for a TRP. Following information needs to be included in MAC CE:

1) Identity of Serving cell of failed TRP

2) Identity of failed TRP

3) candidate beam is available for failed TRP or not

4) candidate beam of TRP if available

1), 3) and 4) can be conveyed using existing fields (Ci/SP, AC and Candidate RS ID) in BFR MAC CE. The issue is how to convey 2).

Option 1: According to RAN1 agreements, number of TRPs (i.e. BFD-RS sets) supported is 2. So BFD-RS set ID (0 or 1) can be included in BFR MAC CE to identify the failed TRP [1][3]

Option 2: Candidate beams are indexed across candidate beam list of both the TRPs. Based on included candidate RS ID in BFR MAC CE, gNB can identify the failed TRP. In case the candidate beam is not available, candidate RS ID is not included in BFR MAC CE. In this case failed TRP info i.e. BFD-RS set ID is included in BFR MAC CE. [1]

| Company | Preferred Option | Comments (rationale) |
| --- | --- | --- |
| Nokia | Option 1 | Since we need this indication in any case when there is no candidate for the failed TRP, we don’t see a need to indicate differently for the case when there is a candidate beam for the failed TRP. It should be noted that number of candidates that can be configured for a single TRP is still likely 64 which means using the candidate RS ID brings no value in practice. |
| Samsung | Option 1 |  |
| OPPO | Option 1 | Agree with Nokia, option 1 can handle the case that no candidate beam is available. |
| ASUSTeK | Option 1 | According to current RAN1 discussion, it is not decided yet whether the number of BFD-RS sets could be more than 2 (in RAN1 section 8.1.2.3). Therefore, the number of bits used to indicate the identity of failed TRP could be 1 or 2, depending on the final decision. |
| Lenovo | Option 1 | Option1 looks straightforward. |
| Xiaomi | Wait for RAN1 | It seems that the RAN1 discussion is still on-going, according to the following RAN1 FFS(s): FFS: whether/what information of failed TRP(s) is conveyed in the MAC-CE FFS: whether/how to support indication of more than one TRP failure, corresponding BFR procedure |
| LG | Option 1 | We don’t prefer to interpret MAC CE differently depending on the presence of the candidate beam. |
| Intel | Option 1 | It seems RAN1 agreed to indicate failed BFD-RS set. Agreement For multi-TRP BFR, a single MAC-CE is used at least for BFRQ for all TRPs in all CCs in a cell group, which includes - Indices of failed BFD-RS set (as an indication of failed TRP link) - Indices of CC containing the failed TRP link - An indicator whether a new candidate beam is identified in the NBI-RS set associated with the failed BFD-RS set, and an resource indicator representing the new candidate beam (if identified) based on the number of NBI-RS resources in the corresponding NBI-RS set. - FFS: Content of MAC-CE related to SpCell when transmitted on msg3, msgA - Note: MAC-CE signaling design details are up to RAN2 - The term “failed TRP link” is used here for discussion purposes only |
| Apple | Option 1 | It’s the RAN1 agreement. |
| ZTE | Option 1 | Wait for RAN1 agreement |
| Qualcomm | Option 1 | We are basically fine with the principle of option 1. But the detailed design of BFR MAC CE can be FFS. If RAN1 are still under discussion on this issue, we also agree to wait for RAN1 more agreements. |
| CATT | Option 1 |  |
| Fujitsu | Option 1 |  |
| Ericsson | Wait for RAN1 |  |
| vivo | Option 1 | We think option 1 is quite straightforward, which is RAN1 intention. We are fine to wait for more details from RAN1 before working on MAC CE design. |

## Handling beam failure detection for both TRPs of a serving cell

It is possible that beam failure is detected on both TRPs (i.e. BFD-RS sets) of a serving cell. This scenario is similar to R16 scenario wherein BFD criteria is met for all BFD RSs of serving cell. In this case i.e. when the condition ‘BFI_COUNTER >= beamFailureInstanceMaxCount’ is met for both BFD-RS sets of a serving cell, one of the following approaches can be considered:

Approach 1: If the serving cell is SCell, BFR (as in legacy) is triggered for the serving cell [1] [8]

If the serving cell is SpCell, random access procedure is initiated on SpCell (as in legacy) [1][3][8][7][6].

Approach 2: Trigger BFR for both the BFD-RS sets of the Serving Cell. In this case beam failure recovery information for both TRPs are included in BFR MAC CE [1].

| Company | Preferred Approach | Comments (rationale) |
| --- | --- | --- |
| Nokia | Approach 1 | Approach 2 is not possible for SpCell since there is not necessarily any SCells configured in which case the RA needs to be triggered in any case. Hence, Alt.1 is basically the only option. |
| Samsung | Approach 1 |  |
| OPPO | Approach 1 |  |
| ASUSTeK | Approach 1 |  |
| Lenovo | See comments | If BFR (as in legacy) is triggered for the Scell, how to intercept the candidate RS ID included in legacy BFR MAC CE without TRP information? The candidate RS per TRP will be configured to UE according to the RAN1 agreement. After receiving the legacy BFR MAC CE including candidate RS ID, which TRP is associated with the received candidate RS ID? Therefore, If the serving cell is SCell, UE can trigger BFR for both the BFD-RS sets of the Serving Cell. In addition, for the SpCell, there are two different cases. The first case is that the TRP based BFR MAC CE has been transmitted. The second TRP fails before the response has been received. In this case, the UE can continue to monitor PDCCH within a period, which can avoid RACH. The other case is that no TRP based BFR MAC CE has been transmitted when two TRP fail. RACH for BFR can be triggered. Based on the above analysis, we suggest the following proposal. Approach#: - If the serving cell is SCell, trigger BFR for both the BFD-RS sets of the Serving Cell. In this case beam failure recovery information for both TRPs are included in BFR MAC CE [1]. - If the serving cell is SpCell and TRP based BFR MAC CE for the first failed TRP has been transmitted, the second TRP fails before response has been received, the UE can continue to monitor PDCCH within a period. If the UE fails to receive response within a period, random access procedure is initiated on SpCell (as in legacy). - If the serving cell is SpCell and no TRP based BFR MAC CE for two failed TRPs has been transmitted, random access procedure is initiated on SpCell (as in legacy). |
| Xiaomi | Approach 1 |  |
| LG | Approach 1 | We think that Approach 1 aligned with legacy is sufficient. The benefit from Approach 2 is not clear. |
| Intel | Approach 1 for SCell. FFS for SpCell | Approach 2 is unclear. Does it mean that BFR is triggered for both TRPs even if BFR condition is met in only one of TRP? Or Does it mean that MAC should wait until both of TRPs meets BFR condition? We don’t know yet how UL is supported in multi-TRP operation especially in inter-cell operation. If serving cell UL is stable, there is no need to trigger PRACH when TRP with different PCI on SpCell trigger BFR. |
| Apple | Approach 1 |  |
| ZTE | Approach 1 can be baseline | Instead of discussing beam failure recovery (i.e BFR or RACH) itself, we would like to discuss the event to trigger the beam failure recovery, it seems at least two possibilities: P1：beam failure is detected for both TRP transmission P2: beam failure is detected for the current active TRP transmission Obviously, we just assume the P1 is the event in this questionnaire. It is cannot sure at this stage, we think this mainly is relying on the RRC structure modeling. |
| Qualcomm | Approach1 with comments | It is unclear for us whether the condition can happen often, i.e. ‘BFI_COUNTER >= beamFailureInstanceMaxCount’ is met for both BFD-RS sets of a serving cell. It seems that it is not a usual case that the BFI counter of each TRP reaching the Max. at the same time. The beam failure instance indication from lower layer to MAC should be one by one. Then how can the criteria is met at the same time for both TRPs? ‘BFR (as in legacy) is triggered’ in approach 1 may have different understandings. In legacy BFR triggering procedure, there is no TRP information associated with the candidate beam in the BFR MAC CE. If we go to the approach 1, does UE indicate both failed TRP IDs (BFD-RS set) as well as the beam failure recovery information for both TRP? (this is not the legacy) |
| CATT | Approach 1 |  |
| Fujitsu | Approach 1 |  |
| Ericsson | Apprach 1 |  |
| vivo | Approach 1 |  |

# Conclusion

TBD

# References

[1] R2-2107007	Multi TRP Beam Failure Detection and Recovery	Samsung Electronics Co., Ltd

[2] R2-2107655	RAN2 impacts of beam failure detection and recovery	Fujitsu

[3] R2-2108246	Beam failure with mTRP	Nokia, Nokia Shanghai Bell

[4] R2-2108655	BFD and BFR for feMIMO	LG Electronics

[5] R2-2107832	Discussion on RAN2 impacts of TRP-specific BFR	OPPO

[6] R2-2107907	Beam failure recovery in multi-TRP	Lenovo, Motorola Mobility

[7] R2-2107995	Discussion on multi-TRP BFR and new MIMO MAC CE	Qualcomm Incorporated

[8] R2-2108443	Support of multi-TRP	Huawei, HiSilicon

[9] R2-2108806	Overview of RAN2 impacts for BFR and BFD for mTRP operation