3GPP TSG RAN WG2 #115-e	                                            (draft) 	R2-2108905

e-Meeting,  16th – 27th August, 2021

Title:	(draft) LS on Msg3 repetition in coverage enhancement

Response to:

Release:	Release 17

Work Item:	NR_cov_enh-Core

Source:	ZTE [to be RAN2]

To:	RAN1

Cc:

Contact Person:

Name:	Jing Liu

E-mail Address:	liu.jing30@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

During RAN2#115_e meeting, RAN2 discussed the potential impact of supporting Msg3 repetition, and would like to check the following with RAN1:

From RAN2 perspective, RAN2 observed that it is feasible to support Msg3 repetition on both NUL and SUL, and would like to ask if RAN1 has any concern on this. If RAN1 confirms the feasibility, then RAN2 would like to know whether different RSRP thresholds for requesting Msg3 repetition are needed for NUL and SUL?

RAN2 also observed that it is feasible to configure random access preamble Group B together with  Msg3 repetition, in which case a separate set of Group B related parameters such as ra-Msg3SizeGroupA, messagePowerOffsetGroupB, numberOfRA-PreamblesGroupA may be configured. RAN2 would like to ask if RAN1 has any concern on this.

In addition, RAN2 has been discussing parameters for the Msg1 transmission used to request Msg3 repetition. The question is whether RAN1 see any issue and benefit to optionally configuring a separate set of RACH parameters for the UE (The RACH parameters may include e.g. preambleReceivedTargetPower, powerRampingStep, powerRampingStepHighPriority, preambleTransMax, groupBConfigured)?

2. Actions:

To RAN4

ACTION: 	RAN2 respectfully asks RAN1 to answer the following questions.

- Question 1:	Does RAN1 think it is feasible to support Msg3 repetition on both NUL and SUL?

- Question 2:	If it is feasible for Q1, whether different RSRP thresholds for requesting Msg3 repetition are needed for NUL and SUL?

- Question 3:	Does RAN1 think it is feasible to configure random access preamble Group B together with Msg3 repetition?

- Question 4:	For Msg1 transmission used to request Msg3 repetition, does RAN1 see any issue and benefit of optionally configuring a separate set of RACH parameters?

3. Date of Next TSG-RAN2 Meetings:

3GPP  RAN2#116-e 	                1st  – 11th November 2021			Electronic Meeting