3GPP TSG-RAN WG2 Meeting #115 electronic	                                 draft R2-2108907

9th August – 27th August 2021

Agenda item:	7.2.2

Source:	ZTE (email discussion rapporteur)

Title:	Report of [AT115-e][402][eMTC R16] Paging resource determination (ZTE)

Document for:	Discussion and Decision

# Introduction

This document is the summary of the offline email discussion “[AT115-e][402][eMTC R16] Paging resource determination”, as indicated below:

- [AT115-e][402][eMTC R16] Paging resource determination(ZTE)

Scope: Check whether the intention is agreeable and there is sufficient support in principle; collect initial comments.

Intended outcome: Report in R2-2108907

Deadline: Wednesday 2021-08-18 12:00 UTC

# Contact information

Please provide your contact information when responding:

| Company | Contact Name | Email |
| --- | --- | --- |
| ZTE | Ting Lu | lu.ting@zte.com.cn |
| Qualcomm | Mungal Dhanda | mdhanda@qti.qualcomm.com |
| Sequans | Noam Cayron | noam.cayron@sequans.com |
| Ericsson | Tuomas Tirronen | tuomas.tirronen@ericsson.com |

# Discussion

This offline is to check whether the intention of the following CRs is agreeable.

[1] R2-2107769	36304_Correction on paging resource determination	ZTE Corporation, Sanechips, Qualcomm Incorporated	CR	Rel-16	36.304	16.4.0	0832	-	F	LTE_eMTC5-Core

The intention of this CR is to resolve the issue of different paging resources determination between eMTC UE in RRC_INACTIVE and the network. In order to resolve this issue, in RAN2#114 e- meeting, RAN2 have already achieved the following working assumption:

=> Working assumption: For an eMTC UE in RRC_INACTIVE same rules as for RRC_IDLE to are used to determine the PNB and i_s.

Q1: Companies are invited to indicate whether the above working assumption can be confirmed?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Qualcomm | Yes | For eLTE RAN2 unlikely to agree to similar change from Release 15 or 16 hence the change can be restricted specifically to BL UE and UE in CE from Release 16 as this will avoid the need for any UE capability. |
| Sequans | Yes | For eLTE R15 NW implementation was agreed. Based on current contributions for R16 any eLTE agreement (if any) would require UE capability and NW indication due to UEs already being in the field, an unnecessary complication for BL UEs. |
| Ericsson | Yes |  |

Conclusion:

All companies agree to confirm the working assumption made in RAN2#114e meeting. Some companies also mentioned that it’s not clear whether eLTE change can be agreed from R15 or R16. And the eLTE change may require UE capability and NW indication. So the change in this CR can be restricted specifically to BL UE and UE in CE from Release 16.

Proposal:

Proposal 1: RAN2 confirms the working assumption: For an eMTC UE in RRC_INACTIVE same rules as for RRC_IDLE  are used to determine the PNB and i_s.

Q2: If the answer to Q1 is Yes, companies are invited to indicate whether the intent of the change in [1] is agreeable and whether there are comments on the actual proposed change?

| Company | Do you agree with the intent of the change? Yes/no | Detailed comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Qualcomm | Yes |  |
| Sequans | Yes |  |
| Ericsson | Yes | Cover page should be updated to refer to an agreement, if we make such. |

Conclusion:

All companies can agree with the CR. One company mentions the cover page should be updated if the working assumption can be confirmed.

Proposal:

Proposal 2: The CR in R2-2107769 is agreeable with only change in cover page of updating the working assumption to agreement.

# Conclusion

Proposal 1: RAN2 confirms the working assumption: For an eMTC UE in RRC_INACTIVE same rules as for RRC_IDLE are used to determine the PNB and i_s.

Proposal 2: The CR in R2-2107769 is agreeable with only change in cover page of updating the working assumption to agreement.