3GPP TSG-RAN WG2 Meeting #115-electronic	 R2-2107054

Online, August 16th – August 27th, 2021                        Revision of R2-2104761

Source: 	vivo

Title:  	Discussion on RRC-controlled Small Data Transmission

Agenda Item:	8.6.3

Document for:	Discussion and Decision

Introduction

Currently, the following open issues highlighted have not been resolved, regarding the general NR SDT procedure (from the RRC point of view that are not relying on LS reply from RAN3/SA3/CT1). Without a doubt, they need to be further investigated:

RAN2#112e Agreement [1]: - For both RACH and CG based solutions, upon initiating RESUME procedure for SDT initiation (i.e. for first SDT transmission), the UE shall re-establish at least the SDT PDCP entities and resume the SDT DRBs that are configured for small data transmission (along with the SRB1). FFS for non-SDT DRBs. FFS on implicit vs. explicit. FFS on whether we a new Resume cause. FFS on whether we need to deal with suppressing PDCP status report. RAN2#113bis-e Agreements [2]: - FFS Switching from CG-SDT to RA-SDT is not allowed. - UE switches from SDT to non-SDT in following cases: Case 1 (27/0): UE receive indication from network to switch to non-SDT procedure. Network can send RRCResume. FFS whether network can send indication in RAR/fallbackRAR/DCI to switch to non-SDT procedure. FFS Case 2 (18/9): Initial UL transmission (in msgA/Msg3/CG resources) fails configured number of times. - RAN2 to decide whether SDT failure detection timer: 1) has an extended duration to accommodate subsequent SDT (13/25); or 2) is restarted upon (re)transmission or reception of small data (12/25) [CB next Tuesday or postpone it to next meeting].

Therefore, in this contribution, we would like to provide our understanding on the open issues of the RRC-controlled small data transmission procedure in terms of:

- Data volume threshold

- SDT initialization procedure

- Handling of new data arrival

- Switching

Discussion

Data volume threshold

According to the WID [3], it can be concluded that small data transmission shall be performed in RRC_INACTIVE state. Then the first question that comes to us is how small the data volume should be and what’s the minimum/maximum boundary of date volume that can still be considered as small data. Considering the data volume threshold configuration might have large impacts on transmission reliability and efficiency, it seems necessary to send an LS to RAN asking the value range of the data volume threshold for NR SDT.

Proposal 1: Send an LS to RAN1 asking the value range of data volume threshold for NR SDT.

SDT initialization procedure

Currently, RAN2 had agreed that small data transmission is configured by the network on a per DRB/SRB1/SRB2 basis. With this, during the SDT, the UE will only resume the allowed SDT-DRB and/or SRB indicated by the gNB.

For Rel-16 NR, an RRC INACTIVE UE can store MR-DC and/or MCG SCell configurations. To reduce the UE complexity and normative workload, in RAN2#113bis-e meeting, it was agreed that gNB can only configure MN terminated MCG bearer type for SDT. To further reduce UE complexity, we think not only the the mrdc-SecondaryCellGroup (if any) but also the radio bearer configuration for DRB(s) belonging to MCG SCell should not be restored from the stored UE INACTIVE AS context upon the transmission of RRC resume request message for SDT procedure. On the network side, it shall not indicate any DRB associated with MCG SCell for SDT. Therefore, we have the following proposals,

Proposal 2: If NR SDT is initiated, UE does not restore the mrdc-SecondaryCellGroup (if any) upon the transmission of RRC resume request message.

Proposal 3: For NR SDT, the UE is not expected to resume any radio bearer associated with MCG SCell.

Moreover, in the existing RRC resumption procedure, the UE does not restore the pdcp-Config from the stored UE Inactive AS context upon the transmission of RRC Resume Request message. As no default PDCP configuration is specified for DRB, there is no way to re-establish the SDT-DRB during the NR SDT procedure as per the current RRC specification. To get rid of this issue, the UE should restore the pdcp-Config stored in the UE Inactive AS context upon initiating the RRC resumption procedure for NR SDT initiation.

Proposal 4: For NR SDT, UE restores pdcp-Config upon initiating the RRC resumption procedure.

Further, some might think that the resume case can be used as a kind of UE assistance information in NR-SDT. However, in our understanding, as a separate preamble resource or dedicated CG resource is generally used for NR SDT, and the service types supported in NR SDT procedure are basically the same in legacy NR, we don’t see the need to identify an NR SDT UE via new resume case or introduce SDT-specific UAC.         Thus, we propose that,

Proposal 5: No NR-SDT specific resume cause is introduced.

Next, regarding the operation of the SDT failure detection timer, we think its operation should be the same as the legacy T319 since the functionality of them is similar. This understanding might also facilitate the UE implementation. If the SDT failure detection timer is (re)start upon each attempt of data transmission or reception, it seems the timer may never enter into an expiry state in some cases. For example, given that both the CG resources and UL traffic data can be periodic within a very very long period, if the SDT failure detection timer is restarted at each UL transmission after initiating the CG-SDT, how can the UE determine that the UL transmission has been failed since the timer might never expire without other new-designed mechanisms? An example is shown in the following figure.

Figure 1: SDT failure detection timer operation

Based on the above, we propose the following,

Proposal 6: Restarting of SDT failure detection timer is not supported.

Handling of new data arrival

After triggering SDT, there may be more arriving data mapping to SDT-RB from upper layers. Regarding the data arrival from SDT-DRB, we think the existing BSR mechanism can be reused to report the potential pending data volume to the NW. At the NW side, it can fully control the subsequent transmission with/without state transition. In our understanding, even though the potential data volume of pending data has exceeded the configured SDT data volume threshold during the SDT procedure, the UE should continue the ongoing SDT procedure since NW can freely reach and carefully control the UE with both UL grant scheduling and RRC state transition.

Proposal 7: If the data volume of data to be transmitted has exceeded the data volume threshold during the SDT procedure, UE will continue the ongoing SDT procedure.

Switching

For NR SDT, considering that far more resource blocks need to be allocated to the Msg3/MsgA/CG PUSCH for UL UP data transmission, the transmission becomes less reliable, compared to the legacy Msg3/MsgA/CG PUSCH transmission. As a result, the gNB may not successfully decode the PUSCH with UL UP data due to the time-varying and fading characteristics of the wireless channel even during the PUSCH retransmission. Also, considering that the radio resources of the initial BWP are a bit strained, it is very likely that gNB cannot reserve sufficient radio resources for the NR SDT UE.

To improve transmission efficiency, a fallback mechanism should be considered. For example, the network can control the number of times 'N'. If the RACH-based SDT procedure is not successfully completed even after 'N' times of RACH attempt, the UE will fallback to use legacy RA resources (e.g. 4-step RA resources or 2-step RA resources) and re-initiate the RRC connection resume procedure. For another example, the network can explicitly indicate a fallback indication via Msg2/MsgB. Upon receiving the fallback indication, the UE will abort the NR SDT procedure and may re-initiate the RRC connection resume procedure (i.e. non-SDT).

Proposal 8: Autonomous switching from NR SDT to non-SDT is supported.

Further, regarding the switching from CG-SDT to RA-SDT, we think the use case of this switching not so valid. Specifically, when CG-SDT is not suitable for small data transmission (due to poor radio condition or there is no available suitable SSB based on the configured beam list), it generally implies that either coverage performance or network load becomes a big issue at this time. In this sense, even switching to RA-SDT, the size of RAR UL grant size provided by the NW might be not large. And then the NW may immediately transit the UE into RRC CONNECTED. After that, a refined scheduling scheme can be provided based on the measurement report from the UE (e.g. CQI, PMI, and CSI). In our understanding, it might be more safe and efficient to directly switch from CG-SDT to legacy RESUME procedure. In other words, we propose that switching from CG-SDT to RA-SDT is not supported.

Proposal 9: Switching from CG-SDT to RA-SDT is not supported.

Conclusions

In this contribution, we have provided our understanding of the RRC-controlled small data transmission procedure. The following proposals are made:

Data volume threshold:

Proposal 1: Send an LS to RAN1 asking the value range of data volume threshold for NR SDT.

SDT initialization procedure:

Proposal 2: If NR SDT is initiated, UE does not restore the mrdc-SecondaryCellGroup (if any) upon the transmission of RRC resume request message.

Proposal 3: For NR SDT, the UE is not expected to resume any radio bearer associated with MCG SCell.

Proposal 4: For NR SDT, UE restores pdcp-Config upon initiating the RRC resumption procedure.

Proposal 5: No NR-SDT specific resume cause is introduced.

Proposal 6: Restarting of SDT failure detection timer is not supported.

Handling of new data arrival:

Proposal 7: If the data volume of data to be transmitted has exceeded the data volume threshold during the SDT procedure, UE will continue the ongoing SDT procedure.

Switching:

Proposal 8: Autonomous switching from NR SDT to non-SDT is supported.

Proposal 9: Switching from CG-SDT to RA-SDT is not supported.

- 4. Reference

[1] RAN2#112-e Session Chair Notes (Small data transmission), E-meeting, 2nd Nov. - 13th Nov. 2021.

[2] RAN2#113bis-e Session Chair Notes (Small data transmission), E-meeting, 12th Apr. - 20th Apr. 2021.

[3] 3GPP RP-200954, “New study item: NR smalldata transmissions in INACTIVE state”, ZTE Corporation, RAN#88, E-meeting, June 2020.