3GPP TSG-RAN WG2 Meeting #116 Electronic	R2-2109804

Elbonia, 01 November – 12 November 2021

Agenda item:	6.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Further issues on multiplexing sidelink logical channels with HARQ feedback enabled vs. disabled

WID/SID:	5G_V2X_NRSL-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#108, the following agreement on MAC [R2-1916123] wrt to the multiplexing of logical channels with HARQ feedback enabled vs. disabled has been made:

7: The logical channel with disabling the HARQ feedback cannot be multiplexed with a logical channel which enabling the HARQ feedback.

Hence a LCH with HARQ feedback enabled and a LCH with HARQ feedback disabled cannot be multiplexed into the same MAC PDU. Whether HARQ feedback is enabled or disabled for a sidelink logical channel is configured by the IE sl-HARQ-FeedbackEnabled.

In this TDoc we will discuss a discrepancy between RRC [TS 38.331] and MAC [TS38.321] specification on the presence of the IE sl-HARQ-FeedbackEnabled.

# 2	Discussion

In TS38.331 the IE sl-HARQ-FeedbackEnabled in the SL-LogicalChannelConfig is marked as OPTIONAL with need code R:

-- ASN1START

-- TAG-SL-LOGICALCHANNELCONFIG-START

SL-LogicalChannelConfig-r16 ::=            SEQUENCE {

sl-Priority-r16                            INTEGER (1..8),

sl-PrioritisedBitRate-r16                  ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

sl-BucketSizeDuration-r16                  ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

sl-ConfiguredGrantType1Allowed-r16         ENUMERATED {true}                                                OPTIONAL,   -- Need R

sl-HARQ-FeedbackEnabled-r16                ENUMERATED {enabled, disabled }                                  OPTIONAL,   -- Need R

sl-AllowedCG-List-r16                      SEQUENCE (SIZE (0.. maxNrofCG-SL-1-r16)) OF SL-ConfigIndexCG-r16

OPTIONAL,   -- Need R

sl-AllowedSCS-List-r16                     SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing                OPTIONAL,   -- Need R

sl-MaxPUSCH-Duration-r16                   ENUMERATED {ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, spare2, spare1}

OPTIONAL,   -- Need R

sl-LogicalChannelGroup-r16                 INTEGER (0..maxLCG-ID)                                           OPTIONAL,   -- Need R

sl-SchedulingRequestId-r16                 SchedulingRequestId                                              OPTIONAL,   -- Need R

sl-LogicalChannelSR-DelayTimerApplied-r16  BOOLEAN                                                          OPTIONAL,   -- Need R

...

}

-- TAG-SL-LOGICALCHANNELCONFIG-STOP

-- ASN1STOP

The corresponding field description for the IE in TS38.331 states

sl-HARQ-FeedbackEnabled

If present, indicate the HARQ feedback enabled/disabled restriction in LCP for this sidelink logical channel. If set to enabled, the sidelink logical channel will be multiplexed only with a logical channel which enabling the HARQ feedback. If set to disabled, the sidelink logical channel cannot be multiplexed with a logical channel which enabling the HARQ feedback. Corresponds to 'sl-HARQ-FeedbackEnabled' in TS 38.321 [3]. If this field of at least one sidelink logical channel for the UE is set to enabled, sl-PSFCH-Config should be mandatory present in at least one of the SL-ResourcePool.

From the specification TS38.331 it seems valid to configure a logical channel with this IE absent. If the IE is absent, then that would mean according to the field description that the logical channel (with absent IE) is not subject to multiplexing restriction based on the value of sl-HARQ-FeedbackEnabled in the other logical channels.

Observation 1: According to RRC specification, it is valid to configure a sidelink logical channel without the IE sl-HARQ-FeedbackEnabled  i.e. the IE is absent.

If one compares the issue in RRC [TS 38.331] and MAC [TS38.321], a sidelink logical channel without the IE sl-HARQ-FeedbackEnabled seems to be not in-line with the behaviour specified in MAC [TS38.321]as the following discussion shows:

According to clause 5.22.1.1 in TS38.321, for such a logical channel (without the IE) the UE can “select any pool of resources among the pools of resources”, so PSFCH does not need to be configured in the pool. However, according to 5.22.1.4.1.2 in TS38.321, such a logical channel (without the IE) cannot be selected unless the pool is configured with PSFCH resources. So, strictly following the text in TS38.321, a UE may end up selecting a resource pool without PSFCH resources for this logical channel but then being unable to select this logical channel for transmission in that pool.

Observation 2: A sidelink logical channel without the IE sl-HARQ-FeedbackEnabled may behave strange in the selection of a resource pool for such a logical channel according to TS38.321.

Further, from the conditions in clause 5.22.1.4.1.2 in TS38.321, such a logical channel cannot be multiplexed with any other logical channel.

Observation 3: The MAC specification was written under the assumption that the IE sl-HARQ-FeedbackEnabled must be present or has a default value.

Proposal 1: RAN2 to confirm that there is a discrepancy between RRC and MAC specification for the presence of the IE  sl-HARQ-FeedbackEnabled for the logical sidelink channel configuration.

In order to correct the mandatory presence of the IE sl-HARQ-FeedbackEnabled in the sidelink logical channel configuration in TS38.331 an ASN.1 change is required. The following changes wrt the current represrntation need to be made:

-- ASN1START

-- TAG-SL-LOGICALCHANNELCONFIG-START

SL-LogicalChannelConfig-r16 ::=            SEQUENCE {

sl-Priority-r16                            INTEGER (1..8),

sl-PrioritisedBitRate-r16                  ENUMERATED {kBps0, kBps8, kBps16, kBps32, kBps64, kBps128, kBps256, kBps512,

kBps1024, kBps2048, kBps4096, kBps8192, kBps16384, kBps32768, kBps65536, infinity},

sl-BucketSizeDuration-r16                  ENUMERATED {ms5, ms10, ms20, ms50, ms100, ms150, ms300, ms500, ms1000,

spare7, spare6, spare5, spare4, spare3,spare2, spare1},

sl-ConfiguredGrantType1Allowed-r16         ENUMERATED {true}                                                OPTIONAL,   -- Need R

sl-HARQ-FeedbackEnabled-r16                ENUMERATED {enabled, disabled}                                   OPTIONAL,   -- Cond Mandatory

sl-AllowedCG-List-r16                      SEQUENCE (SIZE (0.. maxNrofCG-SL-1-r16)) OF SL-ConfigIndexCG-r16

OPTIONAL,   -- Need R

sl-AllowedSCS-List-r16                     SEQUENCE (SIZE (1..maxSCSs)) OF SubcarrierSpacing                OPTIONAL,   -- Need R

sl-MaxPUSCH-Duration-r16                   ENUMERATED {ms0p02, ms0p04, ms0p0625, ms0p125, ms0p25, ms0p5, spare2, spare1}

OPTIONAL,   -- Need R

sl-LogicalChannelGroup-r16                 INTEGER (0..maxLCG-ID)                                           OPTIONAL,   -- Need R

sl-SchedulingRequestId-r16                 SchedulingRequestId                                              OPTIONAL,   -- Need R

sl-LogicalChannelSR-DelayTimerApplied-r16  BOOLEAN                                                          OPTIONAL,   -- Need R

...

}

-- TAG-SL-LOGICALCHANNELCONFIG-STOP

-- ASN1STOP

| Conditional Presence | Explanation |
| --- | --- |
| Mandatory | The field is mandatory present. |

However as the ASN.1 for Rel-16 is frozen RAN2 may consider other options to address the issue. Our proposal is to modify the field description for the IE in RRC specification and leave the MAC specification as well as the ASN.1 unchanged.

Proposal 2: RAN2 to discuss how to resolve the discrepancy between RRC and MAC wrt the IE sl-HARQ-FeedbackEnabled.

Proposal 3: RAN2 to adopt the RRC CR in R2-2109806.

# 3	Conclusion

This paper aims to clarify the presence of the IEs sl-HARQ-FeedbackEnabled in RRC specification and proposes to change the field description.

Observation 1: According to RRC specification, it is valid to configure a sidelink logical channel without the IE sl-HARQ-FeedbackEnabled  i.e. the IE is absent.

Observation 2: A sidelink logical channel without the IE sl-HARQ-FeedbackEnabled may behave strange in the selection of a resource pool for such a logical channel according to TS38.321.

Observation 3: The MAC specification was written under the assumption that the IE sl-HARQ-FeedbackEnabled must be present or has a default value.

Proposal 1: RAN2 to confirm that there is a discrepancy between RRC and MAC specification for the presence of the IE  sl-HARQ-FeedbackEnabled for the logical sidelink channel configuration.

Proposal 2: RAN2 to discuss how to resolve the discrepancy between RRC and MAC wrt the IE sl-HARQ-FeedbackEnabled.

Proposal 3: RAN2 to adopt the RRC CR in R2-2109806.