3GPP TSG RAN WG2 Meeting#116-e       	                                                       R2-2109827
Online, 01 – 11 November 2021

Agenda item:	8.11.7

Source:	Lenovo, Motorola Mobility

Title:	Support of Positioning Reference Units

Document for:	Discussion and Decision

Introduction

During the RAN2#115-e meeting [1], the following agreements were made with respect to Positioning reference units:

Agreements: Proposal 1 (modified): For purposes of RAN2 discussion, the PRU functionality as described in the RAN1 LS can be considered as UE with known location (to some degree of accuracy) at least (16/17). PRU modelled as a gNB can be discussed in RAN3 (no RAN2 action). Agreements: RAN2 confirm that the PRU considered as a UE supports the normal LPP procedures for assistance data transfer and location information transfer.

This contribution extends the discussion of supporting positioning reference units (PRUs)/ positioning reference devices (PRDs) in Rel-17 based on the above agreements.

Positioning Reference Units - Motivations

The positioning reference device (PRD) or positioning reference unit (PRU) with a known fixed location has been discussed and agreed during the in RAN1#105-e meeting to assist in improving the accuracy of the location estimate, especially for timing-based positioning methods. This achieved via compensating the pseudo-range errors arising from the Rx/Tx timing delays from both the UE and gNB-TRP side for DL-only, UL-only and DL+UL positioning methods. The LMF can exploit the PRU’s known precise location to compensate for timing-based errors such as delays embedded within the received measurements by applying (double) differential computations. This is similar in concept to the reference stations employed by GNSS systems to compensate for pseudo-range errors by the GNSS receivers by exploiting the fact that receivers in the same general area, e.g. within a few hundred km between the receivers, experience the same atmospheric impairments.  The actual known precise location of the reference stations is an important pre-requisite.

Observation 1: PRU can improve the accuracy of NR timing-based positioning methods to compensate for timing/range errors in a similar manner to differential-GNSS based methods.

# Management of PRUs

During the [AT115-e][610] discussion, the management of PRUs was discussed under the following broadly categorised options:

- Option 1: Impacts related to PRU Access and registration.

  - Registration at LMF or AMF

  - Using supplementary service messages

- Option 2: Using existing LPP procedures such as capability signalling

The configuration of the PRU as a UE should be more dynamic and fluid, which would align well with using existing LPP procedures as mentioned in Option 2.  This would be in some sense a similar approach to current MDT operations, where the gNB would take of the management and configuration of UEs performing MDT. The PRU UE can therefore use similar procedures as normal UE, with the only difference is that the LMF consumes location information rather than an External LCS client. In this case, depending on the UE capability the LMF need not require for an external trigger to manage the PRU UE.

Proposal 1: LMF is responsible for the management of the PRU (e.g., configuration) via existing LPP procedures.

# Specification Impacts

## Capability and Assistance Data Transfer

A key open issue is to understand how the PRU/reference device may be configured in terms of capability transfer and scheduling of assistance data. The PRU UE capabilities in terms of supported positioning methods can share similar characteristics to current UEs, e.g., PRU can support DL-TDOA, UL-AOA, DL-AOD, and Multi-RTT. For example, the existing capability information transfer messages including Request and Provide Capabilities may be used with an additional configuration or indication that the UE should be configured or capable of acting as a PRU UE. Similarly, the current assistance data mechanism for a normal UE can be extended to configure and signal assistance data requiring the PRU UE to perform the necessary reference measurements.

In addition, it is also clear that separate capabilities will be need to be defined for PRU UEs. This would be separate for normal UEs, although certain functionalities may overlap.

Proposal 2: Separate UE capabilities for PRU are to be defined. RAN1 input is required.

## Acquiring the known PRU Location information

As mentioned earlier, the precise and known position/location of the PRU/PRD is key pre-requisite to enabling the differential correction of pseudo-range errors. Furthermore, the location estimate accuracy of the reference device is therefore a critical piece of information required by the LMF in order to ascertain that the device meets the criteria to accurately and reliably compensate for pseudo-range errors based on the received reference measurements.

In the case of reference TRPs, the know location is fixed (static) and can be easily requested and acquired by the LMF, however in the case of PRU UEs, this aspect becomes more challenging depending on the movement dynamics of the UE. In the case of CPEs deployed in a IIoT environment and having a fixed location, this may not be a serious issue.

Observation 2: PRU UEs can have a fixed location (in the case of CPEs) or dynamic location.

The LMF can request the PRU UE to provide its available known location using a variety of existing specified methods depending on if the location information is already readily available to be shared by the UE:

- Via existing LPP reporting message (ProvideLocationInformation), e.g., using RAT-independent techniques such as A-GNSS, etc.

- Alternatively, the reference UE location can be obtained via the gNB, since the gNB can request a UE’s location via NRPPa and RRC signalling as seen in Figure 1.

Figure 1: LMF request of known location via NRPPa and RRC

- Other location sources can include an manual/offline/preconfigured calibration of the location information, which can be stored in the UE in the event that the reference UE is known to be stationary/fixed, e.g., in an IIoT environment. This stored location can be reported to the LMF.

Furthermore, the LMF can be further aware about the location information source used to obtain the known location, which could be based on GNSS, manual/offline/preconfigured calibration, etc.

Proposal 3: The PRU UE can report its known available location information to the LMF via:

- LPP signalling;

- RRC signalling (e.g. using CommonLocationInfo message) via gNB.

Proposal 4: Support reporting of the known location information source by a PRU UE (e.g., RAT-independent methods, manual/offline/preconfigured location, etc.) to the LMF.

Since the UE location may be dynamic, the PRU UE may also be configured to autonomously update its location to the LMF in an unsolicited manner based on a set of triggered events, which indicate a change to the previous reported location and may include but not limited to:

- Delta of 2D/3D latitude and longitude coordinates when compared to the previous 2D/3D location, which may exceed a certain distance/range/boundary.

- Delta orientation compared to the previous orientation of the PRU UE.

- Delta velocity change compared to previous reported value of PRU UE.

- Delta height compared to previous reported height of reference PRU UE.

- Time elapsed from previous known location report exceeds a PRU UE.

Any change in the UE location can drastically affect the type of reference measurements to be supported. Existing LPP signalling may be used to configure the PRU UE to update its location in the event that the latest location information changes depending on the previous known location.

Observation 2: Change in PRU UE location can impact the reference measurements and the associated differential corrections at the LMF.

Proposal 5: PRU UE to support change/update of the location information to the LMF. FFS the signalling (e.g., solicited/unsolicited request) and any relevant event-triggered criteria.

A further issue relates to location information uncertainty of PRU UEs, which will also affect the accuracy of the differential corrections. There are a existing mechanisms in the current specification to derive the quality of the location estimate. Furthermore, the PRU UE can report the positioning QoS of its location estimate to ensure that the LMF ascertains the accuracy of the location estimate from the reference UE. Such QoS information may encompass horizontal or vertical accuracy, or confidence intervals of the reference position estimate.

Proposal 6: PRU UEs can include positioning QoS information as part of its location estimate report to determine the quality/uncertainty of the location estimate. FFS whether an existing IE may be reused, or any new information is needed (e.g., confidence levels).

## Support for UE-based Positioning

The discussion has thus far focused on the LMF compensation of the Tx/Rx timing errors, which are especially relevant for the UE-assisted positioning methods. The LMF may also share the differential corrections with UEs performing UE-based positioning in the proximity of the reference UEs since the positioning calculation is performed at the UE. In this case, the LMF may have already compensated for the differential corrections related to e.g., DL-RSTD measurements and can therefore directly share the differential corrections to UEs performing UE-based positioning, e.g., via a new dedicated posSIB that carries the differential error corrections, which can be used by the UE for range error compensation.

Observation 3: Similar to UE-assisted methods, UE-based positioning methods can benefit from receiving differential correction information derived from the LMF and PRU UE.

Proposal 7: LMF may provide DL-PRS differential correction information via a new posSIB to assist UEs in compensating differential errors for UE-based positioning. Send LS to RAN1.

Conclusions

This contribution has noted the following observations with regard to on-demand PRS:

Observation 1: PRU can improve the accuracy of NR timing-based positioning methods to compensate for timing/range errors in a similar manner to differential-GNSS based methods.

Observation 2: PRU UEs can have a fixed location (e.g., in the case of CPEs) or dynamic location.

Observation 3: Similar to UE-assisted methods, UE-based positioning methods can benefit from receiving differential correction information derived from the LMF and PRU UE.

Based on the aforementioned discussions, the following on-demand DL-PRS proposals are presented:

Proposal 1: LMF is responsible for the management of the PRU (e.g., configuration) via existing LPP procedures.

Proposal 2: Separate UE capabilities for PRU are to be defined. RAN1 input is required.

Proposal 3: The PRU UE can report its known available location information to the LMF via:

- LPP signalling;

- RRC signalling (e.g. using CommonLocationInfo message) via gNB.

Proposal 3: The PRU UE can report its location information to the LMF via 1) LPP signalling; 2) combination NRPPa and RRC signalling via gNB; 3) offline/pre-configured location calibration.

Proposal 4: Support reporting of the known location information source by a PRU UE (e.g., RAT-independent methods, manual/offline/preconfigured location, etc.) to the LMF.

Proposal 5: PRU UE to support change/update of the location information to the LMF. FFS the signalling (e.g., solicited/unsolicited request) and any relevant event-triggered criteria.

Proposal 6: PRU UEs can include positioning QoS information as part of its location estimate report to determine the quality/uncertainty of the location estimate. FFS whether an existing IE may be reused, or any new information is needed (e.g., confidence levels).

Proposal 7: LMF may provide DL-PRS differential correction information via a new posSIB to assist UEs in compensating differential errors for UE-based positioning. Send LS to RAN1.

References

- R2-2108835, “Report from session on positioning and sidelink relay”, Session Chair (MediaTek), 3GPP RAN2#115-e, August 2021.